3GPP TSG RAN WG1 AH-1901	 R1-1901281

Taipei, Taiwan, 21st – 25th January 2019

Agenda item:	7.2.8.6

Source: 	Samsung

Title: 	Evaluation on SINR metrics for beam selection

Document for:	Discussion and Decision

# Introduction

The following agreement was reached in RAN1#95 [1].

Agreement - Support L1-SINR measured from - For signal part, SSB and/or NZP CSI-RS - FFS: For interference part - Companies are encouraged to provide simulation results on how to measure/define L1-SINR, e.g. whether interference is measured from dedicated IMR - For example, take Rel-15 L1-RSRP and/or SINR specified in 38.215 as a comparative reference for evaluation purposes

This contribution provides simulation results comparing the Rel.15 L1-RSRP, CSI-SINR definition according to TS 38.215 and the proposed L1-SINR metric [2].

# Simulation results for L1-SINR metric

For performance evaluation, the non-full-buffer system-level evaluation setup is used with the Dense Urban Macro scenario assuming medium (50% target RU) traffic loading scenario. SU-MIMO transmission is considered in the simulation. The relevant simulation assumptions and parameters are listed in Table 1 in the Appendix. The following beam selection metrics are simulated.

- Alt 0: Rel.15 L1-RSRP

- Alt 1: CSI-SINR metric as in TS 38.215

- Alt 2: the weighted L1-SINR metric as proposed in [2]. Let t=0, 1, …,NT-1 and r=0, 1, …,NR-1 be indices of transmit (TX) and receive (RX) beams. The weighted SINR metric for a given TX-RX beam pair t,r is then given by

Mt,r=γt,r×SINRt,r,

where

  - γt,r=Pt,rr=0NR-1t=0NT-1Pt,r is a weight assigned to TX-RX beam pair (t,r)

  - Pt,r= RSRP for TX-RX beam pair t,r

  - SINRt,r= 38.215 SINR TX-RX beam pair t,r.

Evaluation 1

The performance comparison between Alt 0 and Alt 1 is provided first. The distribution of gNB and UE selected beam pair (TX-RX beam across 16 possible TX beams and 8 possible RX beams) indices for the two alternatives is shown in Figure 1. In addition, the CDFs of the L1-RSRP metric and the TS 38.215 CSI-SINR metric are shown in Figure 2 and Figure 3, respectively. We can observe the following.

Observation 1: There is no observable difference in distributions of gNB and UE beam pair selection according to Rel.15 L1-RSRP and TS 38.215 CSI-SINR metrics.

Figure 1: Distribution of gNB and UE beam pairs

Figure 2: CDF of RSRP metric

Figure 3: CDF of 38.215 SINR metric

Evaluation 2

The user perceived throughput (UPT) performance comparison between the three metrics is shown in Figure 4. We can observe the following.

Observation 2:

- The TS 38.215 CSI-SINR metric results in lower average and 5% UPT performance than Rel.15 L1-RSRP metric. Therefore, there is no perceivable benefit from 38.215 SINR in UPT.

- The proposed RSRP-weighted SINR metric shows large performance gain over both Rel. 15 L1-RSRP and TS 38.215 CSI-SINR metrics.

Figure 4: User throughput performance comparison between Rel.15 L1-RSRP, TS 38.215 SINR, and weighted SINR metrics

# Conclusions

In this contribution, beam selection based on Rel.15 L1-RSRP, 38.215 CSI-SINR, and weighted SINR [2] metrics is studied using the agreed evaluation methodology. Based on the results, the following observation is made.

- There is no observable difference in distributions of gNB and UE beam pair selection according to Rel.15 L1-RSRP and TS 38.215 CSI-SINR metrics.

- The TS 38.215 CSI-SINR metric results in lower average and 5% UPT performance than Rel.15 L1-RSRP metric. Therefore, there is no perceivable benefit from 38.215 SINR in UPT.

- The proposed RSRP-weighted SINR metric shows large performance gain over both Rel. 15 L1-RSRP and TS 38.215 CSI-SINR metrics.

# References

- Chairman’s notes, RAN1#95

- R1-1901077, Enhancements on multi-beam operations, Samsung

# Appendix

Table 1: Simulation assumptions

| Parameters | Values |
| --- | --- |
| Scenarios | Dense Urban Macro layer Option 1: 2 tier (7 sites with 21 cells) |
| Mode | DL SU-MIMO |
| Simulation bandwidth | 80MHz (DL+UL), TDD |
| Subcarrier Spacing for data | 120kHz |
| Channel Model | Following related assumption in TR 38.802/38.901 |
| TXRU mapping to antenna elements | 2D DFT based beam per polarization |
| TXRU mapping weights | gNB: 16 2D DFT beams (4 in azimuth and 4 in zenith) UE: 8 DFT DFT beams (4 in azimuth and 2 in zenith) |
| Criteria for selection for serving TRP | (Similar to sub-6GHz), based on 1 TXRU at gNB sweeping 16 beams and all TXRUs at UE sweeping 8 beams; metric = max sum received power |
| Criteria for beam selection for serving TRP | Based on RSRP or 38.215 CSI-SINR metrics |
| Constraints for the range of selective beams per TRP sector | Uniform in azimuth and zenith: azimuth within 65 degree, and zenith within [0,180] |
| Scheduling algorithm | PF based |
| Link adaptation | Based on CSI-RS |
| Traffic Model | FTP model 1 with packet size 0.5Mbytes |
| BS antenna configurations | (M, N, P, Mg, Ng; Mp, Np) = (4, 8, 2, 1, 1; 1, 1). (dV, dH) = (0.5, 0.5) λ |
| BS antenna element radiation pattern | According to TR38.802 |
| UE antenna configurations | (M, N, P, Mg, Ng; Mp, Np) = (2, 4, 2, 1, 2; 1, 1); (dV, dH) = (0.5, 0.5) λ. (dg,V, dg,H) = (0, 0) λ. *Θmg,ng=90°; Ω0,1=Ω0,0+180° |
| UE antenna element radiation pattern | See Table A.2.1-8 in TR 38.802 |
| Inter-panel calibration for UE | Ideal |
| Beam correspondence | Ideal |
| Control and RS overhead | DMRS, CSI-RS, PDCCH |
| Control channel decoding | Ideal |
| UE receiver type | MMSE-IRC |
| BF scheme | 1 TXRU per polarization per panel |
| Transmission scheme | Rank 1 only |
| UE mobility feature | Not modelled |