3GPP TSG RAN WG1 Ad-Hoc Meeting 1901	R1-19xxxxx

Taipei, January 21 – 25, 2019

Agenda Item:	7.2.6.1.1

Source:	Huawei

Title:	Summary of 7.2.6.1.1 Potential enhancements to PDCCH

Document for:	Discussion and decision

Introduction

This document summarizes the key issues discussed under agenda item 7.2.6.1.1 based on the views expressed in the contributions submitted to this agenda. The agreements related to PDCCH enhancements achieved in the previous meetings are listed in Appendix A.

PDCCH enhancements

Many companies have provided analysis and/or evaluation to study PDCCH enhancements [1][2][3][4][5][6][7][8][9][10][11][12][13][14][15][16][17][18][19][20][21][22], mainly focusing on compact DCI, PDCCH repetition and increased PDCCH monitoring capability.

Compact DCI

In the RAN1#95 meeting, several aspects were agreed to further study a DCI for URLLC with a size potentially smaller than that of Rel-15 fallback DCI, including targeting a reduction of at least 10 to 16 bits for the DCI format size compared to the size of DCI formats 0_0 and 1_0, the link level performance gain from PDCCH reliability perspective, PDCCH resource utilization considering all UEs in the cell, PDCCH blocking probability, performance impact to PDSCH/PUSCH capacity, impact on PDCCH blind decoding and DCI size budget, and impact on PDSCH/PUSCH scheduling flexibility.

PDCCH reliability

As discussed in contributions submitted to section 7.2.6.1.1, for some use cases, e.g. transport industry and differential protection, the requirement of reliability is 99.999%, while for some other use cases, e.g. factory automation, the requirement of reliability is 99.9999%. The evaluation of PDCCH reliability should consider the requirement of reliability for the identified use cases. What operating target BLER for PDCCH to use depends on many factors, like the reliability requirement of data and/or whether to consider retransmissions. If HARQ retransmissions are allowed, the operating target BLER for PDCCH can be relaxed. However, it can be expected that when the latency budget is not sufficient, one-shot transmission has to be supported. In this case, the operating target BLER for PDCCH should be significantly smaller than the overall reliability requirement, e.g. target BLER of 1e-6 for PDCCH is required to meet the requirement of 1e-5 for data. In the RAN1#95 meeting, it was agreed that the target BLER of DCI(s) scheduling HARQ-less PDSCH/PUSCH should be smaller than 1e-x in Rel-16 NR URLLC, at the 5%-tile SINR geometry, where x is the reliability requirement given in the table of representative use case for evaluation and the 5%-tile SINR geometry is obtained by system-level simulation assuming full buffer.

Some companies have provided some initial evaluations on the PDCCH reliability. The summary of the evaluation results is shown in the Table 1 below:

Table 1 The required SINR (dB) to achieve different target BLER

| Urban Macro, carrier frequency 4 GHz, 4 Tx/4 Rx, TDL-C 300 ns, 30 KHz, AL=16, 3 km/h |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source | 40 bits | 30 bits | 24 bits | Target BLER | 5%-tile SINR1 | 5%-tile SINR2 | 5%-tile SINR3 | 5%-tile SINR4 |
| 1 (Huawei, R1-1900043) | -7.5 |  | -8.1 | 1e-6 | -2.2 | -4 | - | - |
| 2 (ZTE, R1-1900069) | -8.3 | -8.8 |  | 1e-6 | -0.06 | -1.04 | - | - |
| 3 (Intel, R1-1900493) | -7.9 |  | -8.6 | 1e-6 | -2.282 | -2.542 | - | - |
| 4 (MediaTek, R1-1900208) | -7.5 |  | -8.5 | 1e-6 | -3.1 |  | - | - |
| 5 (Vivo, R1-1900126) | -5.829 |  | -6.748 | 1e-6 | -2.696 |  |  |  |
| 6 (CATT, R1-1900331) | -8.3 |  |  | 1e-5 | -0.3 |  | - | - |
| 7 (OPPO, R1-1900281) | -8.2 |  |  | 1e-5 | -2.7 | -3.35 | - | - |
| 8 (Ericsson, R1-1900158) | -6.6 |  | -7.2 | 1e-5 |  |  | - | - |
| 9 (LG, R1-1900591) | -8.6 |  | -9.4 | 1e-5 |  |  | - | - |
| 10 (Pansonic, R1-1900399) | -9 |  | -10 | 1e-5 | -3.3 |  | - | - |
| 11 (Sequans, R1-1900680) | -5.5 |  | -6.2 | 1e-5 |  |  | - | - |
| Urban Macro, carrier frequency 4 GHz, 4 Tx/4 Rx, TDL-C 300 ns, 30 KHz, AL=16, 60 km/h |  |  |  |  |  |  |  |  |
| Source | 40 bits | 30 bits | 24 bits | Target BLER | 5%-tile SINR1 | 5%-tile SINR2 | 5%-tile SINR3 | 5%-tile SINR4 |
| 1 (Huawei, R1-1900043) | -7.8 |  | -8.5 | 1e-6 | - | - | -2 | - |
| 2 (Intel, R1-1900493) | -8.2 |  | -9.2 | 1e-5 | - | - | -2.337 | - |
| Urban Macro, carrier frequency 700 MHz, 2 Tx/2 Rx, TDL-C 300 ns, 30 KHz, AL=16, 60 km/h |  |  |  |  |  |  |  |  |
| Source | 40 bits | 30 bits | 24 bits | Target BLER | 5%-tile SINR1 | 5%-tile SINR2 | 5%-tile SINR3 | 5%-tile SINR4 |
| 1 (Huawei, R1-1900043) | -3.8 |  | -4.5 | 1e-6 | - | - | -2.6 | - |
| 2 (Intel, R1-1900493) | -4.3 |  |  | 1e-6 | - | - | -2.536 | - |
| Urban Macro, carrier frequency 700 MHz, 2 Tx/2 Rx, TDL-C 300 ns, 30 KHz, AL=16, 3 km/h |  |  |  |  |  |  |  |  |
| Source | 40 bits | 30 bits | 24 bits | Target BLER | 5%-tile SINR1 | 5%-tile SINR2 | 5%-tile SINR3 | 5%-tile SINR4 |
| 1 (Huawei, R1-1900043) | -3.8 |  | -4.5 | 1e-6 | -3.2 | -3.2 | - | - |
| 2 (MediaTek, R1-190028) | -4.8 |  | -5.7 | 1e-6 | -3 |  | - | - |
| 3 (ZTE, R1-1900069) | -5 | -5.3 |  | 1e-6 |  |  | - | - |
| 4 (Vivo, R1-1900126) | -1.693 |  | -2.752 | 1e-6 | -1.729 |  | - | - |
| 5 (Intel, R1-1900493) | -3.7 |  |  | 1e-6 | -2.595 |  | - | - |
| 6 (OPPO, R1-1900281) | -5 |  |  | 1e-6 | -2.6 | -2.55 | - | - |
| 7 (InterDigital, R1-1900803) |  |  | -1.6 | 1e-5 | -3.4 |  | - | - |
| Indoor hotspot, carrier frequency 4 GHz, 4 Tx/4 Rx, TDL-C 100 ns, 30 KHz, AL=16, 30 km/h |  |  |  |  |  |  |  |  |
| Source | 40 bits | 30 bits | 24 bits | Target BLER | 5%-tile SINR1 | 5%-tile SINR2 | 5%-tile SINR3 | 5%-tile SINR4 |
| 1 (Intel, R1-1900493) | -7.5 |  |  | 1e-6 | - |  | - | -5 |
| Notes: 5%-tile SINR1: The 5%-tile SINR for power distribution 5%-tile SINR2: The 5%-tile SINR for Rel-15 enabled use case with urban Macro 5%-tile SINR3: The 5%-tile SINR for transport industry 5%-tile SINR4: The 5%-tile SINR for factory automation |  |  |  |  |  |  |  |  |

Based on the above inputs, we can get the following observations on the performance of Rel-15 NR PDCCH:

Observation 2.1-1: For carrier frequency 700MHz with antenna configuration of 2 Tx/2 Rx and channel model of TDL-C 300 ns, four sources show that Rel-15 NR PDCCH (e.g. DCI payload size 40 bits and AL=16) can meet the reliability of 99.9999% at the 5%-tile SINR geometry, and two sources show that Rel-15 NR PDCCH (e.g. DCI payload size 40 bits and AL=16) cannot meet the reliability of 99.9999% at the 5%-tile SINR geometry.

Observation 2.1-2: For carrier frequency 4 GHz with antenna configuration of 4 Tx/4 Rx and channel model of TDL-C 300 ns, Rel-15 NR PDCCH (e.g. DCI payload size 40 bits and AL=16) can meet the reliability of 99.9999% at the 5%-tile SINR geometry.

The evaluation on PDCCH reliability is meaningful and the observations can give some guidance for the performance of Rel-15 NR PDCCH and also show the outcome of our study. The above observations should be captured in the TR 38.824.

Based on the above inputs, the following observations on compact DCI can be achieved. The gain are generally aligned with that observed in Rel-15 URLLC discussion.

Observation 2.1-3: Compact DCI targeting a reduction of 16 bits compared to (e.g. 40 bits) fallback DCI can provide 0.6dB ~ 1 dB gain for AL=16 assuming 4 GH, 1e-5 or 1e-6 target BLER, 4 Rx at UE side, TDL-C and a CORESET with 1 or 2 symbols in time domain and 40 MHz in frequency domain.

Observation 2.1-4: Compact DCI targeting a reduction of 16 bits compared to (e.g. 40 bits) fallback DCI can provide 0.7dB ~ 1 dB gain for AL=16 assuming 700 MHz, 1e-6 target BLER, 2 Rx at UE side, TDL-C and a CORESET with 1 or 2 symbols in time domain and 20 MHz in frequency domain.

As defined in the SID, Rel-16 URLLC should consider both FR1 and FR2 as well as TDD and FDD. Based on the above observations, PDCCH reliability can be met for 4 GHz, and it can be observed that there is sufficient room between the required SINR and the 5%-tile SINR to achieve even higher PDCCH reliability. However, for 700 MHz case with TDL-C 300 ns, it is possible that PDCCH reliability lower than 1e-6 cannot be met due to very small gap between the required SINR and the 5%-tile SINR. In addition, according to the summary of the evaluations in R1-1805630 in Rel-15, the current Rel-15 NR PDCCH cannot meet the PDCCH reliability in case of 700 MHz with TDL-A 30 ns. Compact DCI can further ensure to meet the requirement of reliability for all cases.

| Company | View |
| --- | --- |

PDCCH resource utilization

As DOCOMO mentioned, it is important to keep improving BLER performance of PDCCH by using as lower AL as possible for better system efficiency. Some companies provide some preliminary simulations on the PDCCH resource utilization:

Contribution [MediaTek, R1-1900208] BLER; PL; AL16; AL8; AL4; AL2; AL1; Average NR-PDCCH resources (CCE); NR-PDCCH resources saving of PL24. 1e-5; PL40; 6%; 38%; 24%; 16%; 16%; 5.44; 20.22%. PL24; 1%; 29%; 33%; 17%; 20%; 4.34. 1e-6; PL40; 14%; 41%; 20%; 16%; 9%; 6.73; 18.12%. PL24; 7%; 36%; 27%; 13%; 17%; 5.51. Table 3 : 700MHz, 2Rx, 30 kHz, 2 OS CORESET, TDL-C 300ns BLER; PL; AL16; AL8; AL4; AL2; AL1; Average NR-PDCCH resources (CCE); NR-PDCCH resources saving of PL24. 1e-5; PL40; 0%; 5%; 32%; 29%; 34%; 2.6; 13.07%. PL24; 0%; 2%; 28%; 28%; 42%; 2.26. 1e-6; PL40; 0%; 11%; 34%; 26%; 29%; 3.05; 14.42%. PL24; 0%; 5%; 33%; 27%; 35%; 2.61. Table 4: 4GHz, 4Rx, 30 kHz, 2 OS CORESET, TDL-C 300ns We observe that using DCI with PL24 allows for 18% CCE resource saving compared to using PL40 for target BLER 10-6 at 700MHz carrier frequency. It also allows for 14% CCE resource saving for target BLER 10-6 at 4GHHz carrier frequency. Observation 6: The Use of compact DCI allows to improve the reliability and also enhances the spectral efficiency.

Contribution [Huawei, R1-1900043] Table 4 – AL distributions for 24 bits DCI payload compared to 40 bits DCI payload Carrier frequency; Use cases; Payload; AL=16; AL=8; AL=4; AL=2; AL=1. 700MHz; Remote Driving; 40bits; 13.57%; 30.42%; 29.18%; 16.11%; 10.72%. 24bits; 8.56%; 22.76%; 32.65%; 21.12%; 14.82%. R15 enabled cases; 40bits; 14.49%; 25.11%; 26.10%; 21.78%; 12.52%. 24bits; 10.08%; 22.75%; 25.57%; 21.67%; 19.94%. Power Distribution; 40bits; 14.11%; 25.29%; 26.06%; 21.83%; 12.71%. 24bits; 9.79%; 22.60%; 25.87%; 21.67%; 20.06%. 4GHz; Remote Driving; 40bits; 0%; 2.58%; 21.93%; 35.99%; 39.51%. 24bits; 0%; 0.83%; 13.07%; 39.32%; 46.77%. R15 enabled cases; 40bits; 3.55%; 4.52%; 21.06%; 29.31%; 41.56%. 24bits; 2.75%; 2.64%; 14.63%; 32.32%; 47.66%. Power Distribution; 40bits; 0.45%; 3.70%; 20.63%; 31.18%; 44.04%. 24bits; 0.07%; 1.52%; 14.42%; 33.49%; 50.50%. The relaxed AL distribution results into fewer resources that have to be used in average. We calculated the number of used CCEs that have been occupied during the blocking evaluations for the two different DCI payload sizes. The result is shown in Table 5, where it can be seen that the number of needed resources can be reduced with about 15% when 24bit DCI payload is. Table 5 – PDCCH resource utilization of 24 bits vs 40bits DCI payload Carrier frequency; 700MHz; 4GHz. Use case; Remote Driving; R15 enabled cases; Power Distribution; Remote Driving; R15 enabled cases; Power Distribution. Number of used CCE; 40bit; 44648; 37373; 37087; 8441; 12816; 9956. 24bit; 36493; 32054; 31778; 7080; 10900; 8309. Resource Reduction; 18.27%; 14.23%; 14.31%; 16.12%; 14.95%; 16.54%. Observation 3: When using compact DCI, around 15% PDCCH resource will be saved.

Contribution [LG, R1-1900591] BLER; PL; AL16; AL8; AL4; AL2; AL1; Average NR-PDCCH resources (CCE); NR-PDCCH resources saving of PL24. 1e-5; PL40; 26.35%; 26.45%; 36.04%; 10.84%; 0.32%; 3.1524; 14.5%. PL24; 34%; 28.3%; 30.9%; 6.6%; 0.1%; 2.694. Observation 1: Compact DCI can provide BLER performance gain of about 1 dB.

Based on the above inputs, the following observation can be achieved:

Observation 2.1-5: Compact DCI targeting a reduction of 16 bits compared to fallback DCI (e.g. 40 bits) can save 14% ~ 20% PDCCH resource assuming 700 MHz, 1e-5 or 1e-6 target BLER, 2 Rx at UE side, TDL-C and a CORESET with 1 or 2 symbols in time domain and 20 MHz in frequency domain.

Observation 2.1-6: Compact DCI targeting a reduction of 16 bits compared to fallback DCI (e.g. 40 bits) can save 14% ~ 16% PDCCH resource assuming 4 GH, 1e-5 or 1e-6 target BLER, 4 Rx at UE side, TDL-C and a CORESET with 1 or 2 symbols in time domain and 40 MHz in frequency domain.

PDCCH blocking

Some companies have provided some initial evaluations and/or analysis on the PDCCH blocking:

Contribution [Huawei, R1-1900043] Table 6 - Percentage of users satisfying reliability and latency requirements, 40 bits and 24 bits DCI payload ### Carrier frequency; ### Use cases; ### Number of UEs; ### 40bits - baseline; ### Compact DCI. ### 700MHz; ### Remote Driving; ### 2; ### 50%; ### 50%. ### 6; ### 33.33%; ### 50%. ### 10; ### 10%; ### 30%. ### R15 enabled cases; ### 5; ### 20%; ### 40%. ### 10; ### 10%; ### 30%. ### 15; ### 0%; ### 20%. ### 20; ### 0%; ### 0%. ### Power Distribution; ### 5; ### 0%; ### 20%. ### 10; ### 0%; ### 10%. ### 4GHz; ### Remote Driving; ### 2; ### 100%; ### 100%. ### 6; ### 66.67%; ### 66.67%. ### 10; ### 30%; ### 50%. ### R15 enabled cases; ### 5; ### 40%; ### 60%. ### 10; ### 30%; ### 40%. ### 15; ### 20%; ### 26.67%. ### 20; ### 5%; ### 10%. ### Power Distribution; ### 5; ### 20%; ### 40%. ### 10; ### 10%; ### 30%. Observation 4: When using compact DCI, the PDCCH blocking rate is decreased significantly，and the number of users that can be scheduled within the given latency bound is increased by approximately 20%.

Contribution [Ericsson, R1-1900158] Blocking probability is computed based on the AL distribution and search space design, assuming: - each UE is scheduled with one DCI and - all UEs are scheduled simultaneously and - the number of PDCCH candidates for each AL 1, 2, 4, 8, 16 are 8, 8, 4, 2, 1, respectively. Figure 4: Blocking probability as a function of DCI size, number of UEs, and CORESET sizes - Blocking probability depends on several parameters such as CORESET size, number of UEs, and traffic load. Reducing DCI size by 40% (40bits to 24 bits) provides only small improvement for blocking probability. Using more control resources such as larger CORESET size can provide much significant improvement to the blocking probability.

Contribution [NTT DOCOMO, R1-1900969] As can be seen in Figure 4, with 3 or 4 DCIs per CORESET in the same PDCCH monitoring occasion, the PDCCH blocking probability cannot be lower than 10-2 for 4 GHz. Fig. 4 PDCCH blocking probability. Proposal 5: - Take into account traffic models and UE density in the cell to evaluate the PDCCH blocking probability, and make the conclusion based on the analysis.

Contribution [ZTE, R1-1900069] Figure 3. PDCCH blocking probability of compact DCI Observation 3: PDCCH blocking probability could be reduced by compact DCI.

Contribution [LG, R1-1900591] Figure 2. PDCCH blocking probability

Based on the above evaluation results and analysis, we can get the following observations:

Observation 2.1-7: PDCCH blocking probability depends on several parameters such as CORESET size, number of UEs, and traffic model.

Observation 2.1-8: When the number of users per cell is 2 to 10, PDCCH blocking probability could be higher than 1e-5. PDCCH blocking probability becomes worse with the increase of number of users.

Based on the discussion in the contributions and the above observations, it can be observed that some tools/mechanisms are needed to address the PDCCH blocking (e.g. compact DCI, PDCCH repetition, spending more resources on control). In addition, InterDigital (R1-190083) proposed adaptive PDCCH blind detection to address the PDCCH blocking issue. CATT (R1-1900331) and OPPO (R1-1900281) proposed configured scheduling assignments in conjunction with some DCI indication to solve the problem brought by PDCCH blocking. Intel (R1-1900493) proposed two-stage DCI towards achieving better trade-off between the blocking and scheduling flexibility.

| Company | View |
| --- | --- |

Performance impact on PDSCH/PUSCH

Some companies provides some simulation results and/or analysis on the performance or flexibility impact on PDSCH/PUSCH from compact DCI as below:

Contribution [Huawei, R1-1900863] Table 1 The ratio of UEs satisfying the required 1 ms latency and the reliability of 1e-5 in case of 10 UEs per cell in the DL transmission for Urban Macro deployment. DCI payload size; Arrival rate = 120 packet/s; Arrival rate = 500 packet/s. Ratio; RU; Ratio; RU. Compact DCI (6 bits RA); 93.98%; 5.85%; 46.19%; 16.15%. Normal DCI (12 bits RA); 89.10%; 3.18%; 33.33%; 11.84%. Gain; 5.48%; -; 38.6%; -. Observation1: Compact DCI with coarser scheduling granularity performs better than normal DCI in terms of 1ms latency satisfaction ratio under 1e-5 reliability requirement for Rel-15 enabled use case. Observation2: At least for the use case with small packet size (e.g. 32 bytes), the bandwidth for data channel is not the bottleneck so that congestion of the data channel would rarely happen regardless of the scheduling granularity.

Contribution [CATT, R1-1900331] For 200 bytes a coarse resource allocation (using e.g. RBG scaling) may be beneficial, whereas for the smaller packet size the usefulness would depend on the selected MCS. Table 1 Minimum PRB allocation for one-shot transmission for in a BWP of 106 RBs (40 MHz, 30 KHz SCS) and low-SE MCS table IMCS; Packet size = 32 bytes; Packet size = 200 bytes. 0; 101; N/A. 1; 76; N/A. 2; 61; N/A. 3; 48; N/A. 4; 39; N/A. 5; 31; N/A. 6; 26; N/A. 7; 20; N/A. 8; 16; 99. 9; 13; 76. 10; 10; 62. 11; 8; 50. 12; 7; 43. 13; 6; 36. 14; 6; 32. Observation: the benefits of a compact DCI by reducing or eliminating DCI fields is highly dependent on the deployment scenario and should be UE-specific.

Contribution [Ericsson, R1-1900158] Figure 5: PDSCH blocking probability as a function of UEs per cell with Poisson arrival traffic pattern, for different resource block group (RBG) sizes. PDSCH resources are assumed to be 40 MHz BW. Latency bound is 1 ms.

Based on the above inputs, it can be observed that compact DCI is beneficial at least for some cases, e.g. for the cases that large bandwidth would be needed for PDSCH/PUSCH transmission while congestion is not a problem for PDSCH/PUSCH transmission.

Observation 2.1-9: Compact DCI with coarse scheduling granularity is beneficial at least for the cases where large bandwidth would be needed for data transmission.

Observation 2.1-10: Compact DCI with coarse scheduling granularity may result in larger PDSCH/PUSCH blocking for the cases where data congestion would happen.

Impact on PDCCH blind decoding/DCI size budget

One concern about compact DCI is that it may increase the number of blind decodes. [R1-1812221, Huawei] provides the view that it is not an issue for pure URLLC UEs because compact DCI can be monitored instead of DCI format 0_0/1_0 and/or DCI format 0_1/1_1 thus the number of DCI size to be monitored is not increased. For UEs with eMBB and URLLC service, the number of blind decodes can be controlled by appropriate gNB configuration. [R1-1900069, ZTE] provides the view that compact DCI and full-fledge DCI can be configured depending on the service and scheduling. [R1-1900208, MediaTek] provides the view that compact DCI can be used at least for the case with small SCS where the limit of BD is larger.

Observation 2.1-11: Compact DCI may increase the number of blind decoding and/or increase the number of DCI sizes to be monitored if a new DCI format size different with Rel-15 fallback DCI is introduced. However, appropriate configuration can eliminate or reduce the impact (if any) on PDCCH blind decoding and/or DCI size budget from compact DCI.

URLLC DCI design

In the RAN1#95 meeting, it was agreed to consider using Rel-15 fallback DCI as a starting point for Rel-16 URLLC DCI. Many companies provide the views as summarized below:

Table 1 Potential DL DCI design for Rel-16 URLLC

| Fields | DCI format 1_0 | DL DCI for R16 URLLC |
| --- | --- | --- |
| Identifier for DCI formats | 1 bit | 1 bit |
| Frequency domain resource assignment | bits | Reduced # of bits, e.g. by using RBG instead of RB, or in a configurable manner Support: Vivo, MediaTek, ZTE, Panasonic, Mitsubishi, Huawei, NTT DOCOMO, Nokia, Samsung, CATT, Spreadtrum, China Telecom, CAICT, Samsung, Intel, OPPO, Ericsson, InterDigital, Sequans |
| Time domain resource assignment | 4 bits | Reduced # of bits, e.g. by changing the reference from slot boundary to some PDCCH symbol, or in a configurable manner Support: Pansonic, ETRI, NTT DOCOMO, Huawei, Vivo, ZTE, Mitsubishi, CATT, Spreadtrum, China Telecom, Nokia, CAICT, Samsung, Intel, OPPO, Ericsson, ETRI |
| VRB-to-PRB mapping | 1 bit | 0 bit Support: MediaTek, Mitsubishi, Nokia, Samsung, Vivo, Intel, Ericsson 0 bit or 1 bit in a configurable manner Support: NTT DOCOMO, Spreadtrum, CATT 1 bit Support: ZTE |
| Modulation and coding scheme | 5 bits | Reduced # of bits, e.g. limiting the number of rows to be indicated, or in a configurable manner Support: NTT DOCOMO, Huawei, Vivo, ZTE, Mitsubishi, CATT, China Telecom, Nokia, CAICT, Samsung, Intel, InterDigital, Sequans |
| New data indicator | 1 bit | Reduced # of bits, 0 bit always, or under some condition 0 bit: NTT DOCOMO, Vivo 1 bit Support: ZTE, Mitsubishi, CATT, Spreadtrum, Nokia, Samsung, Intel, Joint encoding of NDI and RV Support: AT&T |
| Redundancy version | 2 bits | Reduced # of bits, e.g. limiting the number of used RV (1 bit) , or in a configurable manner Support: NTT DOCOMO, Vivo, MediaTek, Mitsubishi, China Telecom, Nokia, Samsung, Intel, Ericsson, InterDigital, Sequans 2 bits Support: CATT, Joint encoding of NDI and RV Support: AT&T |
| HARQ process number | 4 bits | Reduced # of bits, e.g. reduce the supported number of HARQ process, or joint indication with other fields, or in a configurable manner Support: Pansonic, ETRI, NTT DOCOMO, Huawei, Vivo, MediaTek, ZTE, Mitsubishi, CATT, Spreadtrum, China Telecom, Nokia, Samsung, CAICT, Intel, OPPO, Ericsson, ETRI, InterDigital |
| Downlink assignment index | 2 bits | Reduced # of bits, e.g. setting to 0 or in a configurable manner Support: NTT DOCOMO, ZTE, Mitsubishi, , Nokia, Samsung, Intel, Ericsson 2 to 4 bits: CATT |
| TPC command for scheduled PUCCH | 2 bits | 0 bit: NTT DOCOMO 2 bits: Huawei, ZTE, Mitsubishi, Spreadtrum, Nokia, Intel, Ericsson 0 bit (open loop) or 2 bits (otherwise): Vivo, CATT, 0 bit to 3 bit in configurable manner: Samsung |
| PUCCH resource indicator | 3 bits | Reduced # of bits, e.g. by reducing to a fix number, or in a configurable manner Support: ETRI, NTT DOCOMO, Huawei, Vivo, ZTE, Mitsubishi, CATT, Spreadtrum, Nokia, Samsung, OPPO, Ericsson, ETRI |
| PDSCH-to-HARQ_feedback timing indicator | 3 bits | Reduced # of bits, e.g. by reducing to a fix number, or remove the field, or in a configurable manner Support: NTT DOCOMO, Vivo, MediaTek, ZTE, Mitsubishi, Spreadtrum, China Telecom, Nokia, Samsung, Intel, OPPO, Ericsson 3 bits: CATT |
| New field(s) proposed to be added compared to DCI format 1_0 |  |  |
| Carrier indicator | N/A | 1 or 2 bits: Qualcomm 0 bits: Pansonic, Huawei, Nokia, Intel Configurable (0 to 3):Samsung, CATT, OPPO, Ericsson |
| PRB bundling size indicator | N/A | Configurable # of bits (0 or 1 bit) Support: NTT DOCOMO, Nokia, CATT 0 bit Support: Intel |
| Rate matching indicator | N/A | Fixed # of bits or configurable # of bits (0 or 1 bit or 2 bits) Support: NTT DOCOMO, Qualcomm, CATT, Nokia, Samsung, Ericsson 0 bit Support: Huawei, Intel, |
| ZP CSI-RS trigger | N/A | Configurable # of bits (0 or 1 bit or 2 bits) Support: NTT DOCOMO, CATT, Ericsson 0 bit Support: Nokia, Intel |
| Antenna port(s) | N/A | Configurable # of bits (0 or 1 or 2 bits) Support: NTT DOCOMO, CATT, Intel, Ericsson 0 bit Support: Nokia |
| Transmission configuration indication | N/A | Configurable # of bits (0 or 3 bits) Support: NTT DOCOMO, CATT, Ericsson 0 bit Support: Nokia, Intel, |
| SRS request | N/A | Configurable # of bits (0 or 1 or 2 or 3 bits) Support: NTT DOCOMO, CATT, Samsung, Intel, Ericsson 0 bit Support: Nokia |
| DMRS sequence initialization | N/A | 0 bit Support: CATT, Nokia 1bit: Intel |
| Repetition factor | N/A | Configurable # of bits (0 or 1 or 2 bits) Support: NTT DOCOMO, Nokia, Sequans 2 bits Support: ZTE 0 bit Support: Intel |
| A-CSI triggering | N/A | 1 bit Support: Qualcomm, Huawei, OPPO Configurable: Ericsson 0 bit: Intel |
| Waveform indicator | N/A | Support: Qualcomm |
| BWP indicator | N/A | Support: OPPO |
| New format indicator | N/A | Support: Ericsson |

Table 2 Potential UL DCI design for Rel-16 URLLC

| Fields | DCI format 1_0 | DL DCI for R16 URLLC |
| --- | --- | --- |
| Identifier for DCI formats | 1 bit | 1 bit |
| Frequency domain resource assignment | bits | Reduced # of bits, e.g. by using RBG instead of RB, or in a configurable manner Support: Vivo, MediaTek, ZTE, Panasonic, Mitsubishi, Huawei, NTT DOCOMO, Nokia, Samsung, CATT, Spreadtrum, China Telecom, CAICT, Samsung, Ericsson |
| Time domain resource assignment | 4 bits | Reduced # of bits, e.g. by changing the reference from slot boundary to some PDCCH symbol, or in a configurable manner Support: Pansonic, ETRI, NTT DOCOMO, Huawei, Vivo, ZTE, Mitsubishi, CATT, Spreadtrum, China Telecom, Nokia, CAICT, Samsung, Ericsson |
| Frequency hopping flag | 1 bit | Reduced # of bits, e.g. setting to 0, or in a configurable manner Support: NTT DOCOMO, Vivo, Nokia, Samsung, Ericsson 1 bit Support: ZTE, Huawei, Mitsubishi, Spreadtrum, Intel, |
| Modulation and coding scheme | 5 bits | Reduced # of bits, e.g. limiting the number of rows, or in a configurable manner Support: NTT DOCOMO, Huawei, Vivo, ZTE, Mitsubishi, China Telecom, Nokia, CAICT, Samsung, Intel |
| New data indicator | 1 bit | Reduced # of bits, 0 bit always or 0 bit under some condition 0 bit: NTT DOCOMO, Vivo, 1 bit Support: ZTE, Mitsubishi, Spreadtrum, Nokia, Samsung, Intel Joint encoding of NDI and RV Support: AT&T |
| Redundancy version | 2 bits | Reduced # of bits, e.g. limiting the number of used RV (1 bit), or in a configurable manner Support: NTT DOCOMO, Vivo, MediaTek, Mitsubishi, China Telecom, Nokia, Samsung, Intel, Ericsson Joint encoding of NDI and RV Support: AT&T |
| HARQ process number | 4 bits | Reduced # of bits, e.g. reduce the supported number of HARQ process, or joint indication with other fields, or in a configurable manner Support: Pansonic, ETRI, NTT DOCOMO, Huawei, Vivo, MediaTek, ZTE, Mitsubishi, China Telecom, Nokia, CAICT, Samsung, Intel, |
| TPC command for scheduled PUSCH | 2 bits | 0 bit: NTT DOCOMO 2 bits: Huawei, ZTE, Mitsubishi, Nokia, Intel, Ericsson 0 bit (open loop) or 2 bits (otherwise): Vivo 0 bit to 3 bit in configurable manner: Samsung |
| UL/SUL indicator | 1 bit | 0 bit: NTT DOCOMO, ZTE, Mitsubishi, Nokia, Ericsson 1 bit: Intel |
| New field(s) proposed to be added compared to DCI format 1_0 |  |  |
| Carrier indicator | N/A | 2 bits: Qualcomm 0 bits: Pansonic, Nokia, Intel Configurable (0 to 3):Samsung |
| SRS resource indicator | N/A | Configurable # of bits (0 or 1 or 2 bit) Support: NTT DOCOMO 0 bit Support: Nokia |
| Precoding information and number of layers | N/A | Configurable # of bits (0 or 1 or 2 bits) Support: NTT DOCOMO, Ericsson 0 bit Support: Nokia, Intel, |
| Antenna port(s) | N/A | Configurable # of bits (0 or 1 or 2 bits) Support: NTT DOCOMO, 0 bit Support: Nokia, Intel, Ericsson |
| SRS request | N/A | Configurable # of bits (0 or 1, 2 or 3 bits) Support: NTT DOCOMO, Samsung, Intel, Ericsson 0 bit Support: Nokia |
| CSI request | N/A | Configurable # of bits (0 or 1 or 2 bits) Support: NTT DOCOMO, Nokia, Intel, OPPO, Ericsson |
| beta offset indicator | N/A | Configurable # of bits (0 or 1 or 2 bits) Support: NTT DOCOMO, Huawei, Samsung 0 bit Support: Nokia, Intel |
| Repetition factor | N/A | Configurable # of bits (0 or 1 or 2 bits) Support: NTT DOCOMO, Nokia 0 bit: Intel |
| BWP indicator | N/A | Support: OPPO |
| New format indicator | N/A | Configurable: Ericsson |

Based on the above two tables, it can be observed that there is common interest to reduce the number of bits for some fields in the DCI scheduling URLLC compared to Rel-15 DCI format 0_0 and DCI format 1_0, though the motivation may be different among companies, e.g. some want to achieve a smaller DCI size while some others want to give the chance to add some new fields.

Observation 2.1-12: For the DCI format scheduling Rel-16 NR URLLC, there is common interest to reduce the number of bits for the following fields compared to Rel-15 fallback DCI:

- Frequency domain resource assignment

- Time domain resource assignment

- Modulation and coding scheme

- HARQ process number

- PUCCH resource indicator

- PDSCH-to-HARQ_feedback timing indicator

- Downlink assignment index

In addition, as shown in the above two tables, many companies [Qualcomm, R1-1900896][NTT DOCOMO, R1-1900969][Nokia, R1-1900927][CATT, R1-1900331][Intel, R1-1900493][Samsung, R1-1901066][Ericsson, R1-1900158][Sequans, R1-1900680][ZTE, R1-1900069][OPPO, R1-1900281] also proposed to add some fields either from the non-fall back DCI or from some new potential supported enhancements for URLLC compared to Rel-15 DCI format 0_0 and DCI format 1_0. The motivation is either to provide more flexibility or achieve better URLLC performance. For example, several companies mentioned that some MIMO related fields like antenna port(s) and or TCI fields would be needed for multi-TRP transmission. And also several companies proposed to add rate matching indicator which is beneficial to let the PDSCH utilize all available resources in the mini-slot that are not occupied by PDCCH or other channels. Some companies mentioned that SRS request would be needed especially for TDD.

Observation 2.1-13: Compared to Rel-15 fallback DCI, some new field(s) need to be added to the DCI format scheduling URLLC, e.g. Rate matching indicator for DL DCI.

In addition, some companies also proposed to make some fields configurable to provide more flexibility either from DCI size perspective or from scheduling flexibility perspective.

As to the total size of the DCI for Rel-16 URLLC, there are different views. Among the companies supporting compact DCI, it is common understating that the DCI size should target a reduction of 10~16 bits compared to Rel-15 fallback DCI. The benefits include achieving better spectral efficiency, improving PDCCH reliability and reducing PDCCH blocking. Some other views are also observed. For example, some companies proposed to reuse the same size as fallback DCI [Panasonic, R1-1900399][NTT DOCOMO, R1-1900969][Ericsson, R1-1900158][ETRI, R1-1900686] to reduce the impact on blind decoding and DCI size budget. Some companies proposed configurable DCI format for URLLC [CATT, R1-1900331][Intel, R1-1900493][Samsung, R1-1901066], the potential benefits including providing considerable flexibility to the network when considering tradeoffs between scheduling flexibility and possible blocking, providing possibility to solve the DCI size budget issue. [Nokia, R1-1900927] proposed to configure to align the size of the new DCI format with the size of DCI format 0_0 and 1_0. [MediaTek, R1-1900208] thinks that the total number of size can depend on the subcarrier spacing.

Observation 2.1-14: It is beneficial to support configurable DCI sizes for more flexibility.

Summary

According to the discussion and observations in section 2.1.1, PDCCH reliability can be met for 4 GHz. However, for 700 MHz case with TDL-C 300 ns, it is possible that PDCCH reliability lower than 1e-6 cannot be met due to very small gap between the required SINR and the 5%-tile SINR. In addition, according to the summary of the evaluations in R1-1805630 in Rel-15, the current Rel-15 NR PDCCH cannot meet the PDCCH reliability in case of 700 MHz with TDL-A 30 ns. Therefore, from PDCCH reliability perspective, it seems the motivation to introduce compact DCI is not that strong. However, if compact DCI is supported, it can help further ensure the PDCCH reliability for all use cases, e.g. for both carrier frequency of 4 GHz and 700 MHz, and for both TDL-C channel model and TDL-A channel model.

From the discussion and observations in section 2.1.2, it can be observed that compact DCI targeting a reduction of 16 bits compared to fallback DCI (e.g. 40 bits) can save much PDCCH resource and thus improve spectral efficiency. Spectral efficiency is also very important for URLLC use case as mentioned in NTT DOCOMO contribution.

From the discussion and observations in section 2.1.3, it can be observed that compact DCI can help reduce PDCCH blocking. There is no converged views on whether the gain is significant or not though.

From the discussion and observations in section 2.1.4, it can be observed that compact DCI with coarse scheduling granularity is beneficial at least for the cases where large bandwidth would be needed for data transmission and/or the cases where data congestion is not the bottleneck from performance perspective.

From the discussion and observations in section 2.1.5, it can be observed that appropriate configuration can eliminate or reduce the impact on PDCCH blind decoding if any.

From the discussion and observations in section 2.1.6, it can be observed that for the DCI format scheduling Rel-16 NR URLLC, it is beneficial to reduce the number of bits for some fields, while allow to add some new field(s) and allow some fields to be configurable. As to the total DCI size, it seems configurable DCI sizes similar as Rel-15 non-fallback DCI can provide more flexibility. However, to make sure that the requirement of reliability it would be better to limit that the maximum DCI size is the same as Rel-15 fallback DCI, while the minimum DCI size can go down to target a reduction of at least 16 bits to achieve enough benefits.

Based on the above discussion, we can have the following proposal:

Offline proposal from Monday morning offline session:

Proposal 2.1-1: For the DCI format scheduling Rel-16 NR URLLC,

- Support reduction of the number of bits for some fields compared to Rel-15 DCI

- FFS support adding some new field(s) compared to Rel-15 DCI

- Support configurable number of bits for some fields

- FFS whether any limitation on the final DCI sizes

  - Option 1: limit the maximum size equal to the size of Rel-15 fallback DCI

  - Option 2: No limitation depending on the configurations

  - Option 3: limit the size equal to the size of Rel-15 fallback DCI

Proposal 2.1-1: For the DCI format scheduling Rel-16 NR URLLC,

- Support reduction of the number of bits for at least the following fields compared to Rel-15 fallback DCI:

  - Frequency domain resource assignment

  - Time domain resource assignment

  - Modulation and coding scheme

  - HARQ process number

  - PUCCH resource indicator

  - PDSCH-to-HARQ_feedback timing indicator

  - Downlink assignment index

- Support adding some new field(s) compared to Rel-15 fallback DCI

- Support configurable number of bits for some fields

- The DCI format size depends on the higher layer configuration

  - Minimum DCI size should target 10~16 bits reduction compared to the DCI format size of Rel-15 fallback DCI

  - Maximum size should be equal to the DCI format size of Rel-15 fallback DCI

PDCCH repetition

Evaluation of PDCCH repetition from PDCCH blocking perspective

Some companies have provided evaluation of PDCCH repetition from PDCCH blocking perspective:

Contribution [Huawei, R1-1900043] Table 8 - Percentage of users satisfying reliability and latency requirements when compact DCI is applied together with the PDCCH repetition scheme ### Carrier frequency; ### Use cases; ### Number of UEs; ### 40bits - baseline; ## PDCCH Repetition; ## PDCCH Repetition ### &feedback; ## PDCCH Repetition ## &feedback ### & Compact DCI. ### 700MHz; ### Remote Driving; ### 2; ### 50%; ### 100%; ### 100%; ### 100%. ### 6; ### 33.33%; ### 83.33%; ### 100%; ### 100%. ### 10; ### 10%; ### 80%; ### 90%; ### 100%. ### R15 enabled cases; ### 5; ### 20%; ### 60%; ### 100%; ### 100%. ### 10; ### 10%; ### 50%; ### 100%; ### 100%. ### 15; ### 0%; ### 46.67%; ### 93.33%; ### 100%. ### 20; ### 0%; ### 40%; ### 90%; ### 95%. ### Power Distribution; ### 5; ### 0%; ### 40%; ### 100%; ### 100%. ### 10; ### 0%; ### 30%; ### 80%; ### 100%. ### 4GHz; ### Remote Driving; ### 2; ### 100%; ### 100%; ### 100%; ### 100%. ### 6; ### 66.67%; ### 83.33%; ### 100%; ### 100%. ### 10; ### 30%; ### 90%; ### 100%; ### 100%. ### R15 enabled cases; ### 5; ### 40%; ### 80%; ### 100%; ### 100%. ### 10; ### 30%; ### 60%; ### 100%; ### 100%. ### 15; ### 20%; ### 53.33%; ### 100%; ### 100%. ### 20; ### 5%; ### 50%; ### 95%; ### 100%. ### Power Distribution; ### 5; ### 20%; ### 60%; ### 100%; ### 100%. ### 10; ### 10%; ### 50%; ### 90%; ### 100%. Observation 7: When using PDCCH repetition with fast feedback in combination with compact DCI, the PDCCH blocking rate is eliminated for most of the investigated cases: - For up to 10 users in the Remote Driving use case - For up to 10 users in the Power Distribution. - For up to 15 users in the R15 enabled cases. For the extreme scenario of up to 20 users and 700MHz, still 95% of the UEs can be scheduled within the latency bounds

Contribution [MediaTek, R1-1900208] Figure 5: Blocking rate of PDCCH repetitions. Observation 4: Blockage rate doesn’t improve by using repetition within the same CORESET and is degraded by using repetitions across CORESETs in time.

Contribution [Qualcomm, R1-1900896] Figure 6: PDCCH transmission for two UEs with (a) AL = 16, and (b) AL = 8 with 2 repetitions. Figure 7: PDCCH transmission for 3 UEs at different PDCCH occasions. UE 1 requires 16 CCEs overall to decode PDCCH, while UE 2 and 3 each require 4 CCEs for PDCCH decoding. Observation 4: The PDCCH performance with n candidates of AL = X/n is an upper bounded by that of the single candidate with AL = X. Observation 5: The blind PDCCH repetition does not reduce blocking issue for PDCCH scheduling. Proposal 3: To meet the eURLLC requirements, PDCCH repetition over a single TCI state is not needed.

Contribution [ZTE, R1-1900069] Note that repetition is only applied for AL16 in the simulation, which means one transmission of AL16 candidate in current occasion is divided into two transmission of AL8 in current occasion and next occasion. Figure 4. PDCCH blocking probability of PDCCH Repetition Observation 6: PDCCH repetition is beneficial to reduce PDCCH blocking probability. In case of using both compact DCI and repetition, one additional UE could be served under the same blocking probability.

Based on the above simulation results and analysis, some show that PDCCH repetition can reduce the PDCCH blocking while some others show that there is no gain. It was observed that different repetition schemes were assumed in the above evaluations.

Observation 2.2-1: PDCCH repetition may show benefits improving reliability and/or reducing PDCCH blocking depending on the PDCCH repetition scheme.

Summary

Many companies have provided analysis to study the potential benefits of PDCCH repetition [R1-1900043, R1-1900864, Huawei][R1-1900591, LG][R1-1900927, Nokia][R1-1900969, NTT DOCOMO][R1-1900399, Panasonic][R1-1900680, Sequans][R1-1900370, Sony][R1-1900069, ZTE], the observed benefits including potential improving reliability and reducing PDCCH blocking. Some companies also mentioned that PDCCH repetition is able to achieve intermediate ALs that are not the powers of 2. However, there are concerns on the need of PDCCH repetition and the increased UE complexity and may increase the latency [R1-1900208, MediaTek][R1-1900686, ETRI].

Observation 2.2-2: PDCCH may increase latency and UE complexity depending on the PDCCH repetition scheme.

Several companies expressed the view that PDCCH repetition should consider different transmission assumptions and configurations (e.g. time/frequency resource, TCI/QCL/TRP assumptions, etc.). And from the above discussion, it can be observed that the concern on PDCCH repetition may be based on different assumptions of PDCCH repetition. Therefore, for progress maybe we can try to discuss the potential PDCCH repetition mechanisms first and then further check the pros and cons based on converged or reduced set of PDCCH repetition options.

As to the possible PDCCH repetition, Pansonic R1-1900399 and MediaTek R1-1900208 provide nice description and provide detailed analysis on the candidate solutions. According to other contributions, it seems three options as shown in the following proposal achieved most interest. It was observed that different options have different pros and cons. PDCCH repetition study can focus on the following three options.

Proposal 2.2-1: Further study the following options for PDCCH repetition considering aspects like UE complexity, decoding performance, scheduling flexibility and latency.

- Option 1: PDCCH repetitions prior to PDSCH transmission(s)

- Option 2:Independent PDCCH schedules each PDSCH repetition

- Option 3:Multi PDCCHs schedule PDSCH repetition with indication of the number of repetitions in each DCI (LTE HRLLC similar scheme)

Among the above three solutions, one common point is whether to do PDCCH combination for PDCCH repetition. Several companies think that it would increase UE complexity to do combination between different PDCCH repetitions, thus it is not preferred [Vivo, R1-1900126][Intel, R1-1900493][CAICT, R1-1901129]. [Nokia, R1-1900927] also provides some simulation and analysis on whether to do combination as below. However, [Sony, R1-1900370] proposed to do combining due to more performance gain and they don’t see that PDCCH repetition increase the complexity.

Contribution [Nokia, R1-1900927] Figure 4-2: PDCCH repetition performance with varying SNR levels (due to intra-cell interference) with SNR offset for the repeated DCI / second CORESET symbol only Figure 4-3: PDCCH repetition performance with varying SNR levels (due to intra-cell interference) with SNR offset applied for both DCI transmissions / CORESET symbols Observation 4-1: DL control decoding gains of about 1 to 1.5 dB can be provided with blind PDCCH repetition (without UE combining) with very little specification and UE implementation effort. To show the performance difference on PDSCH/PUSCH decoding for Alt. 2, we compare the case of a single DCI transmission compared to the case of m DCI transmission based on a statistical analysis (in contrast to the LL evaluations above) which can be noted as: - P=(1-εDCI)(1-εData) for the case of a single transmitted DCI - P=(1-(εDCI)m)(1-εData) for m DCI transmissions on different PDCCH candidates, where εDCI is the probability of missed DCI/DL assignment/UL grant of a single PDCCH candidate, εData is the PDSCH BLER and P is the overall reliability of the PDSCH/PUSCH reception assuming statistical independent DCI missed detection probabilities on the different PDCCH candidates. In Figure 4-3, we present the error probability given by Pfailure=(1-P) of Alt. 2 with varying DCI missed detection probabilities and a PDSCH/PUSCH BLER of εData=10-5. Figure 4-3: Error probability of scheduled DL-SCH/UL-SCH as a function of the PDCCH missed detection probabilities with varying number of PDCCH repetitions for εData=10-5

Proposal 2.2-2: Further discuss whether to do PDCCH combination for PDCCH repetition.

| Company | View |
| --- | --- |

Increased PDCCH monitoring capability

Several companies have provided analysis to study the potential benefits of increased PDCCH monitoring capability [R1-1900158, Ericsson][R1-1900281, OPPO][R1-1900803, InterDigital] [R1-1900591, LG] [R1-1900927, Nokia][R1-1900969, NTT DOCOMO][R1-1900126, Vivo][R1-1900686, ETRI] [R1-1900896, Qualcomm][CATT, R1-1900331], the observed benefits includes potential reducing latency and improving the PDCCH blocking.

Contribution [Qualcomm, R1-1900896] - Case 2 with SCS = 30KHz, N1 = N3 = 4.5 symbols, a half-symbol propagation delay and four monitoring occasions per slot: Figure 2: eURLLC latency assuming four PDCCH monitoring occasion per slot (Case 2). - Case 2 with SCS = 30KHz, N1 = N3 = 4.5 symbols, a half-symbol propagation delay and seven monitoring occasions per slot: Figure 3: eURLLC latency assuming seven PDCCH monitoring occasion per slot (Case 2). Observation 1: For eURLLC with stringent latency requirements, a frequent PDCCH monitoring, e.g., in units of every 2, is necessary. Proposal 1: To enable fast scheduling for eURLLC, RAN1 considers the feasibility of increasing the number of BD/CCE limit. The required conditions and relaxations should be studied.

Contribution [Ericsson, R1-1900158] Table 9 PDCCH blocking probability within a slot with 1, 2, or 3 PDCCH occasions for different numbers of UEs per cell. (DCI size = 40 bits, CORESET duration = 1 symbol) Blocking prob.; #UE = 10; #UE = 20; #UE = 30; #UE = 40. After 1 PDCCH occasion; 7.91%; 39.03%; 58.01%; 68.46%. After 2 PDCCH occasions; 0; 1.42%; 19.50%; 37.75%. After 3 PDCCH occasions; 0; 0; 0.17%; 4.15%. Increase the limits of number of blind decodes and CCEs for channel estimation to allow flexible, multiple PDCCH monitoring occasions in a slot and reduce PDCCH blocking.

However, there are concerns on the increased UE complexity and power consumption [R1-1901066, Samsung][R1-1901129, CAICT].

Contribution [Samsung, R1-1901066] there is no need to increase the Rel-15 maximum number of PDCCH candidates that a Rel-16 URLLC UE is required to monitor per slot, considering that - The corresponding resource availability for PDSCH receptions or PUSCH transmissions and not the number of PDCCH candidates is likely to be the limiting factor - For UEs that support only URLLC services, it can be considered to reduce the maximum number of PDCCH candidates per slot that the UE is required to monitor. - For UEs that support both URLLC and eMBB services, whether or not to increase the maximum number of PDCCH candidates per slot and, if so, by how much can depend on the SCS and the DCI format used for URLLC (whether or not DCI format(s) for URLLC has same size as fallback DCI formats for eMBB) - With wideband RS (i.e. value of higher layer parameter precoderGranularity = allContiguousRBs), the UE can combine after descrambling the DMRS in the two symbols of the CORESET before filtering in the frequency domain (single channel estimate) as the time variation over 2 adjacent symbols is negligible. Observation 1: There is no need to increase the maximum number of monitored PDCCH candidates per slot for a UE supporting Rel-16 URLLC services relative to a Rel-15 UE. Reductions should be considered to reduce the complexity for machine type communications Rel-16 URLLC UEs Proposal 1: Consider reducing the maximum number of monitored PDCCH candidates per slot for a UE supporting only Rel-16 URLLC services relative to a Rel-15 UE. Observation 3: Rel-15 UE capabilities for the maximum number of non-overlapping CCEs per slot can support Rel-16 URLLC even for 15 kHz SCS with reasonable network configurations. Observation 4: If network flexibility is desirable in the configuration of CORESETs for URLLC, an increase in the number of non-overlapped CCEs can be supported particularly for the smaller SCS. Proposal 2: Consider increasing the maximum number of non-overlapping CCEs per slot for UEs supporting both eMBB and URLLC services and for the smaller SCS. FFS for UEs supporting only URLLC services.

Observation 2.3-1: Increasing the limit of the number of CCEs/BDs may reduce latency. However, there is concern on increasing UE complexity and power consumption.

In theory, the increase of PDCCH monitoring capability can provide more flexibility for URLLC scheduling and provide more chances to reduce the latency. However, according to the discussion in Rel-15, it was observed that the limit of the number of CCEs/BDs do have much impact on UE complexity. Qualcomm proposed that some conditions and relaxations would be introduced to support increased PDCCH monitoring to reduce the UE complexity, which is a good way to study. For example, URLLC could use a smaller number of CCs compared to eMBB operation to keep the overall value is small, although the number of BDs/CCEs per serving cell might be increased. Another important constraint could be to limit the number of BDs/non-overlapping CCEs per monitoring occasion so that the UE’s processing burden for decoding PDCCH can be made manageable, even though the total number of BDs/CCEs per slot is increased.

Another aspect related to whether to enhance the PDCCH capability is the potential number of PDCCH monitoring occasions needed for URLLC per slot. Several companies provides the views as below:

- 3: CATT, MediaTek

- 7: Vivo, Qualcomm, Nokia, NTT DOCOMO

According to the above inputs, it seems there is much difference on the views from different companies. Further evaluation is needed on this aspect.

As to details of the enhancements, [CATT, R1-1900331][Intel, R1-1900493] mentioned that the enhancements should focus on characterization of minimum requirements on CCEs for channel estimation. [MediaTek, R1-1900208] proposed to define different UE capabilities to support different number of BDs/CCEs. Several companies proposed to define both the maximum number of BD/CCE per slot and the maximum number of BD/CCE per monitoring occasion [Vivo, R1-1900126]. [OPPO, R1-1900281] proposed to report the UE capability in a finer granularity, e.g. the maximum number of blind decoding per Y symbols.

Proposal 2.3-1: Further study and evaluate increasing the limit of the number of CCEs/BDs for Rel-16 NR URLLC at least from the following aspects:

- What is the maximum number of PDCCH monitoring occasions needed per slot?

- Whether only the limit of the number of CCEs needs to be enhanced, or both the limit of the number of CCEs and BDs need to be enhanced?

- Whether and what condition needed for supporting increasing the limit of the number of CCEs/BDs?

- Whether and how to define the UE capability for the enhanced PDCCH monitoring capability?

| Company | View |
| --- | --- |

Other PDCCH enhancements

Some companies provide some other thinking on PDCCH enhancements.

- [CATT, R1-1900331][OPPO, R1-1900281] proposed configured scheduling assignments in conjunction with a group-common DCI to indicate one out of the respective assignments. The motivation us to solve the PDCCH monitoring capacity from a system perspective.

- Intel [R1-1900493] proposed two-stage DCI towards achieving better trade-off between the blocking performance and the scheduling flexibility, where the first stage DCI conveying equivalent functionalities of the RRC indications can be transmitted less frequently, and the second stage DCI conveying the activation equivalent functionalities can be transmitted more frequently.

- MediaTek [R1-1900208] proposed that a UE can be configured with two PDCCH monitoring configurations where the secondary configuration has more PDCCH monitoring occasions and it is triggered when the SR is transmitted, the reason is that the UL latency is longer than DL in case of grant based PUSCH transmission.

- [Qualcomm, R1-1900896][OPPO, R1-1900281] proposed to reduce the SPS periodicities and send ACK for SPS activations/de-activations, to further reduce the latency and improve the reliability.

- [Ericsson, R1-1900158] proposed to introduce an offset parameter to the configuration of regular CORESETs to align the 6-PRB grid of regular CORESETs with the grid of CORESET#0.

- [InterDigital, R1-190083] proposed to introduce adaptive blind detection by configuring multiple configured PDCCH monitoring patterns, and/or multiple configured durations and monitoring periodicities, then determine the active configured PDCCH monitoring patterns based on the total numbers of monitored PDCCH candidates and non-overlapped CCEs per slot.

Proponents of the above enhancements are encouraged to talk to other companies to achieve more views from other companies.

- Reference

- R1-1900043	PDCCH enhancements for URLLC	Huawei, HiSilicon

- R1-1900069	On PDCCH enhancements for URLLC	ZTE

- R1-1900126	PDCCH enhancements for URLLC	vivo

- R1-1900158	PDCCH Enhancements for NR URLLC	Ericsson

- R1-1900208	Evaluations and enhancements of NR PDCCH for URLLC	MediaTek Inc.

- R1-1900281	PDCCH enhancement for URLLC	OPPO

- R1-1900331	PDCCH enhancements for URLLC	CATT

- R1-1900370	L1 enhancements on PDCCH for URLLC	sony

- R1-1900399	PDCCH enhancements for NR URLLC	Panasonic Corporation

- R1-1900437	On Potential enhancements to PDDCH for URLLC	AT&T

- R1-1900493	On PDCCH enhancements for eURLLC	Intel Corporation

- R1-1900591	PDCCH enhancements for NR URLLC	LG Electronics

- R1-1900680	PDCCH enhancements for URLLC	Sequans Communications

- R1-1900686	Potential PDCCH enhancements for NR URLLC	ETRI

- R1-1900803	On Potential PDCCH enhancements for URLLC 	InterDigital, Inc.

- R1-1900896	PDCCH Enhancements for eURLLC	Qualcomm Incorporated

- R1-1900927	On the PDCCH enhancements for NR URLLC	Nokia, Nokia Shanghai Bell

- R1-1900938	Potential PDCCH enhancements for URLLC operation	Motorola Mobility, Lenovo

- R1-1900969	PDCCH enhancements for URLLC	NTT DOCOMO, INC.

- R1-1901066	DL control enhanements for URLLC	Samsung

- R1-1901129	Potential enhancements to PDCCH for URLLC	CAICT

- R1-1901148	On PDCCH enhancements for URLLC	Mitsubishi Electric RCE

- Appendix A: Agreements in the past meetings

RAN1#94 meeting

Agreements: Further evaluate the potential PDCCH enhancements for NR Rel-16 URLLC. - Further evaluate PDCCH reliability - Further evaluate PDCCH blocking - Companies describe the resource utilization - Complexity should be considered - Latency of the enhancement(s) should be considered

RAN1#95 meeting

Agreements: For link-level PDCCH evaluation, the target operating BLER of DCI(s) scheduling HARQ-less PDSCH/PUSCH should be smaller than 1e-x in Rel-16 NR URLLC, at the 5%-tile SINR geometry. - x is the reliability requirement given in the table of representative use case for evaluation agreed in the RAN1#94bis meeting. - The 5%-tile SINR geometry is obtained by system-level simulation assuming full buffer for a given evaluation scenario. - This target assumes no HARQ re-transmssion Agreement: - No change of DCI format 0_0/1_0 in CSS from Rel-16 URLLC study item perspective Agreements: - To further study DCI for URLLC with a size potentially smaller than that of Rel-15 fallback DCI - Consider using Rel-15 fallback DCI as a starting point for Rel-16 URLLC DCI - Target a reduction of at least 10-16 bits compared to Rel-15 fallback DCI - Companies report how to achieve the DCI size reduction - The link level performance gain from PDCCH reliability perspective - Check at least AL=16 - PDCCH resource utilization considering all UEs in the cell - Check AL=1/2/4/8/16 - If retransmission is feasible with the latency bound, different BLER target can be used - The PDCCH blocking probability when applicable - The performance impact from compact DCI including impact to PDSCH/PUSCH capacity when applicable - The impact on PDCCH blind decoding/DCI size budget - The impact on PDSCH/PUSCH scheduling flexibility - At least Rel-15 enabled use cases should be evaluated for the above study