3GPP TSG RAN WG1 Ad-Hoc Meeting 1901                 R1-1901337

Taipei, Taiwan, 21st – 25th January, 2019

Agenda Item:	7.2.5

Source: 	LG Electronics

Title: 	Work plan for Rel-16 WI on Cross Link Interference (CLI) handling and Remote Interference Management (RIM) for NR

Document for:	Discussion and Decision

- Introduction

In RAN #81, a new Rel-16 work item [1] on Cross Link Interference (CLI) handling and Remote Interference Management (RIM) for NR was approved. The objectives of this WI includes for both CLI and RIM as below:

The work item should specify cross-link interference mitigation techniques to support flexible resource adaptation. Furthermore, it also specifies remote-interference management techniques.

The detailed objectives for cross-link interference mitigation to support flexible resource adaptation for unpaired NR cells are: - Specify cross-link interference measurements and reporting at a UE (e.g., CLI-RSSI and/or CLI-RSRP) [RAN1, RAN2, RAN4] - Specify network coordination mechanism(s) including at least exchange of intended DL/UL configuration [RAN1, RAN3] - Perform coexistence study to identify conditions of coexistence among different operators in adjacent channels [RAN4] - Target no or very minimal impact on RF requirement Note: Measurement and coordination mechanisms should be applicable to IAB nodes.

The detailed objectives for remote-interference management are: - Specify RIM RS resource and configurations, including [RAN1] - A basic RIM-RS resource - Configuration of RIM-RS and distinguishable RIM RS-1/2 resources, including sequence type, time and frequency transmission pattern - Determine gNB set identification information through detection of RIM-RS(s) by implicit or explicit indication. Determine further information that can be carried by the RIM-RS, such as “Ducting phenomenon exists”, “Enough mitigation” & “Not enough mitigation”, [RAN1, SA5] - Specify the inter-set RIM backhaul signalling via the core network to convey the messages of “RIM-RS detected” and “RIM-RS disappeared [RAN3] Identify corresponding OAM functions to support RIM operation [RAN1, RAN3].

In this contribution, we provide the work plan for this Rel-16 work item.

- Work Plan for Rel-16 WI (RAN1)

NR_CLI_RIM WI is going to be finished by RAN#84 (June, 2019). As shown the TU assignment in Table 1, this WI should be finalized by RAN1#96 (February, 2019).

Table 1. TU assignment for CLI_RIM WI in Rel-16.

| Leading WG | RAT | WI or SI | Title | 96 | 96bis | 97 |
| --- | --- | --- | --- | --- | --- | --- |
| R1 | NR | WI | New WID on Cross Link Interference (CLI) handling and Remote Interference Management (RIM) for NR | 2 |  |  |

RAN1 #NRAH1901 meeting (January)

CLI

- Check whether previously made agreements can be applied

- Decide whether both measurement metric (e.g., CLI-RSSI, CLI-RSRP) are supported or downselected

- Identify the details of measurement resource configuration and measurement reporting

- Identify the details of intended DL/UL configurations for network coordination mechanism, and discuss whether any other network coordination mechanism can be agreed

RIM

- Generation of basic RS

- Determine the Functionality/Cause to be supported for RS configuration

- Determine the bandwidth of basic RS and frequency multiplexing of basic RS

- Identify the parameters/procedures to be configured by OAM for RS transmission and detection

RAN1 #96 meeting (February, 2TU)

CLI

- Finalize measurement resource configuration and measurement reporting

- Finalize configurations for network coordination mechanism

RIM

- Determine the maximal number of basic RS configuration to be supported from gNB point of view

- Determine the parameters to be configured by OAM for RS transmission and detection

- Identify the procedures to be done by SA5

- Reference

[1] RP-181652, “Revised WID on Cross Link Interference (CLI) handling and Remote Interference Management (RIM) for NR”, LG Electronics

[2] RP-182864, “Revised WID on Cross Link Interference (CLI) handling and Remote Interference Management (RIM) for NR”, LG Electronics