3GPP TSG RAN WG1 Ad-Hoc Meeting 1901		      R1-1901385

Taipei, Taiwan, 21st – 25th January, 2019

Agenda Item:	7.2.10.1.4

Source: 	LG Electronics

Title: 	Discussion on Performance Evaluation for RAT-Dependent Positioning Techniques

Document for:	Discussion and decision

Introduction

In 3GPP #94bis and #95 meeting, the following agreements were made for the Requirements for NR Positioning [1-2].

Agreement: - Regulatory requirements are considered as a minimum performance targets for NR Positioning studies - Additional requirements based on commercial use cases can be used as input performance targets that are subject to further analysis in terms of performance / complexity tradeoffs in different evaluation scenarios Agreement: - For regulatory use cases, the following requirements are considered as a minimum performance targets for NR positioning - Horizontal positioning error <= 50m for 80% of UEs - Vertical positioning error [<5 m] for [80%] of UEs - Note: The regulatory requirements refer to floor level vertical accuracy - End to end latency and TTFF < 30 seconds - As a starting point for commercial use cases, the following requirements are considered as performance targets for RAT dependent solutions, which are subject to further analysis in terms of performance / complexity tradeoffs of NR positioning radio-layer solutions - Horizontal positioning error < [3]m for [80]% of UEs in indoor deployment scenarios - Horizontal positioning error < [10]m for [80]% of UEs in outdoor deployments scenarios - Vertical positioning error < [3]m for [80]% of UEs in indoor deployment scenarios - Vertical positioning error < [3]m for [80]% of UEs in outdoor deployment scenarios - End to end latency < [1]s - Note: This does not eliminate more or less demanding commercial use cases. Agreement: - At least CDFs of horizontal and vertical (vertical error not necessarily applicable to all solutions and/or scenarios) positioning errors are used as a performance metrics in NR positioning evaluations - At least the following percentiles of positioning error are analyzed 50%, 67%, 80%, 90% - Physical layer latency, UE power consumption, scalability/capacity, network deployment complexity, availability, UE and gNB complexity can be considered as important design factors for NR positioning solutions and can be evaluated analytically for proposed solutions Agreement: - For commercial use cases, remove brackets from target performance requirements on horizontal positioning error that were agreed at the RAN1#94bis meeting - Add Note: - For commercial use cases, indoor deployment means indoor deployed UEs and gNBs - For commercial use cases, outdoor deployment means outdoor deployed UEs and gNBs Agreement: The following note is added under the regulatory and commercial NR positioning target requirements - Note: It is understood that no single positioning technology has to meet all the requirements for every scenario

In this contribution, we reveal our simulation results for NR positioning especially for DL only based RAT-dependent techniques.

Evaluation Result of OTDOA Techniques

For start point, we investigate the potential performance of NR OTDOA technique depending on the size of transmission bandwidth. In this evaluation, UMa scenario at FR1 (4GHz) is assumed. In Figure 1, system level simulation result is depicted.

Figure 1. Horizontal Positioning Detection Performance of OTDOA

From the Figure 1, it is observed that the horizontal positioning detection performance of OTDOA using 5MHz, 10MHz, and 20MHz BW for PRS transmission satisfies the regulatory requirement (Horizontal positioning error <= 50m for 80% of UEs). On the other hand, the above performance results are not sufficient to satisfy the commercial requirement (Horizontal positioning error < 10m for 80% of UEs in outdoor deployments scenarios).

Also, it is observed that when wider transmission bandwidth is applied, the detection accuracy is enhanced. From the tendency, we may expect that when wider bandwidth (e.g. 100MHz BW) for NR PRS transmission is applied, commercial requirement could be satisfied.

Observation 1:

- It is observed that the horizontal positioning detection performance of OTDOA using 5MHz, 10MHz, and 20MHz BW for PRS transmission satisfies the regulatory requirement (Horizontal positioning error <= 50m for 80% of UEs).

Evaluation Result of PDoA in frequency selective fading channels

In this section, we firstly summarize our analysis on PDoA and discuss accurate propagation delay estimation methods in frequency selective channel and its evaluation results.

Summary of technical analysis on PDoA:

The advantage of PDoA is that it can measure the delay of a signal in a continuous domain regardless of the bandwidth of the channel. For example, at 5 MHz bandwidth, a measurement error of about 39 meters may occur due to sampling error.

PDoA can also measure delay without using all resources in the frequency domain. The phase ambiguity according to distance can be solved by using multi tone. For example, a rough delay can be measured using phase rotation between adjacent tones, and a fine delay can be measured by two far distant tones.

PDoA can be used in all situations that measure time delay rather than just only in DL. In other words, PDOA can be used for UTDoA and RTT measurements as well. However, PDoA may not accurately measure the delay due to the frequency selective fading channel.

We consider two methods to mitigate the effect of frequency selective fading;

Method 1) Spatial filtering: multipath fading can be mitigated spatial filtering. Multipath delay can be suppressed by spatial domain filtering. This may results in mitigating the distortion of delay measurement via PDoA. However, residual multipath components may remain after spatial filtering, so further analysis seems necessary.

Method 2) Utilization of multipath delay profile: The UE can estimate a multi-path delay profile through a channel estimation process while the UE monitors a signal of a specific base station e.g. PDCCH/PDSCH decoding or CSI measurement. Some QCL relation between PRS and other RS may be indicated. Through the delay profile estimation, it is possible to estimate an accurate delay in selective fading by removing phase components contributed by selective fading. Detailed procedure is summarized in Appendix.

Evaluation result

We measured delay estimation performance at 5MHz system bandwidth. 15 kHz SCS is assumed. TDL-B (NLOS) channel, normal delay (100ns scaling), and delay profile estimation using regularized LS. Two tone scheme is assumed to be 3RBs apart, and delay is generated uniformly in the region without ambiguity. In proposed method, it is assumed that 1 tone is allocated in every RB, so total 25 tones are used in the proposed scheme. Figure 2 shows the delay measurement error performance according to SNR. For reference, the time correlation based scheme via wideband signaling at 5MHz system bandwidth will converge to 2Ts error even at high SNR because sampling rate is 4Ts (where Ts is LTE basis (i.e. Ts = 1 / 15000/2048)). The evaluation result shows that the proposed method can measure error less than 0.5Ts even in selective fading. In addition, note that in LOS delay profile, better performance can be observed and we have not considered phase differences for far distant tones, so there is room for further performance improvements.

Figure 2. Delay estimation performance comparison in frequency selective channel (TDL-B)

Conclusion

In this contribution, we reveal our system level simulation result for OTDOA technique, and summarize our Observation as follows:

Observation 1:

- It is observed that the horizontal positioning detection performance of OTDOA using 5MHz, 10MHz, and 20MHz BW for PRS transmission satisfies the regulatory requirement (Horizontal positioning error <= 50m for 80% of UEs).

Reference

[1] Chairman’s Notes 3GPP RAN1 #94bis meeting

[2] Chairman’s Notes 3GPP RAN1 #95 meeting

[3] 3GPP TR 38.901 “Study on channel model for frequencies from 0.5 to 100 GHz” V15.0.0 (2018-06)

Appendix: PDoA based delay estimation method in frequency selective fading

First, it is assumed that the UE transmits a reference signal at two or more frequency tones. At this time, it is assumed that the size and phase information of the reference signal are known by the transceiver in advance. In the frequency domain m-th tone (subcarrier), the received signal after removing reference signal contribution can be written as follows.

, 	(1)

where ,  are the amplitude of the multipath channel at the kth frequency tone and the phase response of the multipath channel, respectively.  is the subcarrier spacing, and  is the time offset between transceivers due to propagation delay. When receiving signals at two tones, the phase difference at each tone (assuming that the phase of the multipath channel in the two tones is the same) can be expressed by the following equation.

(2)

On the other hand, if the phase of the channel between two tones is different, Eq. (2) can be rewritten as

(3)

Then, the estimated distance can be written as

(4)

As a results, when the phase difference due to the multipath channel occurs more than the original distance, the error in the distance estimation or delay measurement will increase.

To solve this problem, we propose the following method. First, let's review the received signal of the kth tone.

, (5)

where W(k) represents the noise at the kth frequency tone. The conjugate product of the kth tone and the (k + m)th tone is as follows.

(6)

Here, the conjugate product of the frequency response of the k-th tone and the frequency response of the k + m-th tone can be rewritten as,

(7)

where .

If the average of the conjugate products between the k-th tone and the k + m-th tone is calculated,

(8)

Assuming there is no correlation between different channel taps,. Therefore, the phase distortion due to the multipath channel can be compensated by using the phase value at the mth tone after the IFFT operation of the channel delay profile as shown in Equation (8). For reference, we can obtain  via averaging the conjugate product between two tones separated by m tones. Based on the analysis, the timing difference between the transmitter and the receiver and the distance between the two devices can be obtained by the following equation,

Timing difference:  (9)

distance:  (10)