3GPP TSG RAN WG1 Ad-Hoc Meeting 1901		R1-1901403

Taipei, Taiwan, 21st – 25th January, 2019

Agenda item:	7.2.8.1

Source: 	Samsung

Title: 	Summary of Email Discussion on MU-MIMO CSI Enhancement

Document for:	Discussion and Decision

Introduction

To facilitate agreement in RAN1#96, the following issues are discussed over email: Evaluation criteria for down selection of N3 when NSB×R>13. This issue is to address the following agreement:

Agreement

Values of N3: For  and NSB is # CQI subbands, when ,

Values of N3: For  and NSB is # CQI subbands, when , down select among the following alternatives in RAN1#96

- Alt1:  is smallest multiple of 2, 3, or 5 which is

- Alt2:  is a multiple of 2, 3, or 5. Segment into 2 parts with overlapping between 2 parts. Note: no padding is needed to align the DFT size with the multiple of 2, 3, or 5

Summary

Interested companies commented on each of the aspects. While the comments diverge in many aspects (for instance, a few companies propose different solutions for each of the two alternatives), only the common ground is summarized below and proposed.

Proposal: Use the following criteria and guideline to compare Alt1 and Alt2 for down selection of N3 when NSB×R>13.

Table 1 Criteria for evaluation

| Criteria | Comment |
| --- | --- |
| Performance-overhead tradeoff (UPT vs. overhead) | - On Alt1, whenever padding is necessary, companies should describe their (assumed) padding scheme/algorithm. - On Alt2, companies should describe their (assumed) segmentation scheme. - For both alternatives, gNB receiver operation (e.g. how to handle padding or segmentation and the associated precoders/PMIs) can also be stated. - Bandwidth = 20 MHz and/or 50 MHz, 30kHz SCS, R=1 and/or 2, different N3 values (ranging from small to large) |
| Specification and/or implementation complexity | - Additional specification components, e.g. for Alt2, consider the benefit(s) over using 2 reporting settings - Additional effort to implement PMI search algorithm and UCI processing |