3GPP TSG RAN WG1 Meeting AH1901	R1-1901437

Taipei, 21st – 25th January, 2019

Agenda item:    7.2.2.2.3

Source:             Huawei

Title:                  Feature lead summary of HARQ enhancements for NR-U

Document for:  Discussion and Decision

# Introduction

This paper summarizes proposals on HARQ enhancement for NR-U submitted at the January 2019 ad-hoc meeting.

- The aspects on HARQ feedback mechanism are discussed in Section 2

- The aspects on scheduling are discussed in Section 3

The corresponding WI objectives are:

- 	HARQ operation: NR HARQ feedback mechanisms are the baseline for NR-U operation with extensions in line with agreements during the study phase (NR-U TR section 7.2.1.3.3), including immediate transmission of HARQ A/N for the corresponding data in the same shared COT as well as transmission of HARQ A/N in a subsequent COT. Potentially support mechanisms to provide multiple and/or supplemental time and/or frequency domain transmission opportunities. (RAN1)

-	Scheduling multiple TTIs for PUSCH in-line with agreements from the study phase (TR 38.889, Section 7.2.1.3.3). (RAN1)

The SI agreements on potential HARQ enhancements are documented in TR38.889. The proposals generated by the feature lead are appended at the end of each subsection. The summary focuses on decisions needed before diving into more detailed design aspects, hence not all details of proposals submitted by companies are summarized here.

Some aspects related to HARQ are primarily discussed in other agenda items, so no proposals are made in this summary on those aspects. Companies are encouraged to submit the corresponding proposals to the relevant agendas:

- LBT category for UE transmission of HARQ A/N feedback and the related signalling (e.g. in DCI)

- Adaptation of contention window size in case of configuration of CBG HARQ

# Discussion

## HARQ A/N feedback

### PDSCH-to-HARQ-timing-indicator

The past agreements and proposals impacting the PDSCH-to-HARQ-timing-indicator are addressed in this section.

TR38.889: to support the HARQ feedback corresponding to some or all the PDSCHs of a channel occupancy to be reported in the same channel occupancy, it is found beneficial to extend the PDSCH-to-HARQ-timing to support indicating timings up to the end of the longest COT allowed by regulations. One or more of the following would be needed: Allow values larger than 15 by RRC signalling; Allow more bits for the PDSCH-to-HARQ-timing-indicator.

TR38.889: Introduce signaling a value of the PDSCH-to-HARQ-timing-indicator in the DCI scheduling the PDSCH that tells the UE that the timing and resource for HARQ-ACK feedback for the corresponding PDSCH will be determined later.

A decision is needed on:

- The largest value to target for scheduling HARQ feedback within the longest COT (e.g. 10 ms assuming a paused COT), considering the largest subcarrier spacing supported for control and data (60 kHz).

- Details of configuring and signalling that the timing and resource for HARQ-ACK feedback for a PDSCH is not determined at the time of receiving the DCI scheduling the corresponding PDSCH.

TR38.889: one candidate scheme is Alt3 (gNB requests feedback outside the COT by PDSCH-to-HARQ-timing-indicator in the DCI scheduling the PDSCH), where the UE will attempt reporting at the indicated time and resource even if the PDSCH-to-HARQ-timing-indicator indicates a slot that falls outside the gNB-initiated COT.

Alt3, which is a candidate scheme for transmitting the UE HARQ A/N feedback in a later COT than the corresponding PDSCH, may have some impact on the K1 values supported to be signalled by the PDSCH-to-HARQ-timing-indicator.

No company indicated a position against Alt3 in contributions submitted to the January 2019 ad-hoc meeting. On the other hand, no company provided specific values of K1 that would need to be introduced specifically for supporting Alt3. It may be concluded that Alt3 works sufficiently well with the largest value of K1 targeted for scheduling HARQ feedback within the longest COT.

TR38.889: one candidate scheme is Alt5a (multiple candidate opportunities by providing multiple timings in PDSCH-to-HARQ-timing-indicator and/or other DCI fields), where the UE will attempt reporting at the preconfigured/pre-indicated times and resources (e.g. in a UE-initiated channel occupancy).

Alt5a is a candidate scheme that has a direct impact on the PDSCH-to-HARQ-timing-indicator.

Accordingly, proposals that impact the PDSCH-to-HARQ-timing-indicator are summarized below:

- Extend the PDSCH-to-HARQ-timing-indicator (RRC parameter: dl-DataToUL-ACK)

  - extend the largest value of PDSCH-to-HARQ-timing-indicator beyond 15

    - what is the largest value that needs to be supported?

    - companies expressed different views on whether to increase the number of bits

  - support a value of PDSCH-to-HARQ-timing-indicator indicating that A/N feedback is deferred

    - discussion will be needed on how the UE knows which LBT category to use

  - support that a state of PDSCH-to-HARQ-timing-indicator can indicate multiple timings

    - there were also proposals to support this functionality with a new field in the DCI instead, e.g. to instead signal a window providing multiple time-domain opportunities for UE transmission of HARQ A/N feedback. Discuss in section 2.1.2.

Proposed agreement:

- The largest value supported by RRC parameter dl-DataToUL-ACK is larger than 15 for NR-U

  - FFS between 31 and 63

  - The number of bits of PDSCH-to-HARQ-timing-indicator is not increased for this purpose

- RRC parameter dl-DataToUL-ACK supports a value that can be signaled by PDSCH-to-HARQ-timing-indicator, which indicates that the UE needs to store the HARQ A/N feedback result for the corresponding PDSCH, and which does not provide any timing for the transmission of this HARQ A/N feedback result

- Support that gNB requests feedback outside the COT by PDSCH-to-HARQ-timing-indicator in the DCI scheduling the PDSCH (i.e. Alt3 in TR38.889 section 7.2.1.3.3)

  - Further discuss the details for determining the LBT type for the UE transmission of HARQ A/N feedback for this case

  - No additional value of K1 signaled by PDSCH-to-HARQ-timing-indicator is introduced specifically for Alt3

Some feedback from companies was received during the review of the summary:

[DCM] For HARQ feedback within the same COT containing corresponding PDSCH, extension of PDSCH-to-HARQ-timing-indicator should be unnecessary or low priority since it is just an optimization for specific DL heavy COT situation while multiple DL-UL switching within gNB-initiated COT will be allowed (i.e., no need to put HARQ feedback at the end of COT). For HARQ feedback outside the same COT containing corresponding PDSCH, either another gNB-initiated COT or UE-initiated COT can be used. Since alt.1 (support a value of PDSCH-to-HARQ-timing-indicator indicating that A/N feedback is deferred, and support gNB request of feedback) is most flexible and efficient, we think the support of alt 5 (PDSCH-to-HARQ-timing-indicator in DCI scheduling PDSCH indicates multiple timings) seems unnecessary. So, we can agree only on second bullet in above proposals and we can further discuss on detailed mechanism to trigger/request HARQ feedback outside the COT such as in another gNB-initiated COT and in UE-initiated COT as in section 2.1.2.

[MTK] Support the 1st, 2nd, and 3rd bullets. However, for the 3rd bullet, channel access aspect for a HARQ-ACK feedback transmitted outside a gNB-initiated COT should be discussed in AI of channel access.

Results of the first online session are copied below:

Agreement:

- RRC parameter dl-DataToUL-ACK supports a value that can be signaled by PDSCH-to-HARQ-timing-indicator, which indicates that the UE needs to store the HARQ A/N feedback result for the corresponding PDSCH, and which does not provide any timing for the transmission of this HARQ A/N feedback result

Conclusion:

- No additional value of K1 signaled by PDSCH-to-HARQ-timing-indicator needs to be introduced for the purpose of requesting feedback outside the COT (i.e. Alt3 in TR38.889 section 7.2.1.3.3)

  - Further discuss the details for determining the LBT type for the UE transmission of HARQ A/N feedback for this case

  - Further discuss whether additional values need to be introduced for the purpose of requesting feedback inside the COT

    - If additional values are introduced these can also be used for requesting feedback outside the COT

  - Note: This does not necessitate that the UE needs to know whether the feedback is inside or outside the COT at the time when the feedback is requested

Proposal:

- The largest value supported by RRC parameter dl-DataToUL-ACK is larger than 15 for NR-U

  - FFS between 31 and 63

  - The number of bits of PDSCH-to-HARQ-timing-indicator is not increased for this purpose

### Multiple opportunities for HARQ A/N transmission

The candidate alternatives documented in the NR-U TR are listed in the table below.

|  | Cross-COT HARQ-ACK feedback | Multiple opportunities for HARQ-ACK feedback |
| --- | --- | --- |
| Alt1: gNB requests/triggers feedback for PDSCH from earlier COT(s) or additional reporting of earlier HARQ feedback, where the exact HARQ feedback timing and resource is provided to the UE in another DCI (in the same or in another COT) | Alt1a: request/trigger reporting of HARQ feedback for earlier COT(s) or additional reporting of earlier HARQ feedback without explicit signaling of HARQ process ID, possibly along with other HARQ feedback reports (e.g. for the current COT) Alt1b: request/trigger reporting for a set of HARQ processes, either for all configured HARQ processes (e.g. group feedback), or for a set of HARQ process IDs or HARQ process ID groups |  |
| Alt2: UE is configured/allowed to report HARQ feedback for PDSCH from earlier COT(s) without an explicit request/trigger | UE autonomously reports UCI with additional information about HARQ processes e.g. corresponding to PDSCH from earlier COT(s) – that are reported in PUSCH [or PUCCH] along with the HARQ-ACK feedback. |  |
| Alt3: gNB requests feedback outside the COT by PDSCH-to-HARQ-timing-indicator in the DCI scheduling the PDSCH | The UE will attempt reporting at the indicated time and resource (e.g. in a UE-initiated channel occupancy), even if the PDSCH-to-HARQ-timing-indicator indicates a slot that falls outside the gNB-initiated COT. | Not a solution if PDSCH-to-HARQ-timing-indicator can only indicate a single value |
| Alt4: preconfigured/pre-indicated multiple opportunities in frequency domain in different LBT subbands | Possible if this is combined with Alt1 or Alt2 or Alt3 | Possible for indicating multiple candidate PUCCH or PUSCH carrying HARQ-ACK feedback |
| Alt5: preconfigured/pre-indicated multiple opportunities in time domain | The UE will attempt reporting at the preconfigured/pre-indicated times and resources (e.g. in a UE-initiated channel occupancy) | Alt5a: Multiple candidate opportunities by providing multiple timings in PDSCH-to-HARQ-timing-indicator and/or other DCI fields Alt5b: Multiple candidate slots in a window with size configured by RRC. There could be some activation/deactivation by DCI |

Offline consensus

- At least Alt1 above is supported

Summary of proposals submitted to the January 2019 ad-hoc meeting:

- Alt1: gNB requests/triggers feedback for PDSCH from earlier COT(s) or additional reporting of earlier HARQ feedback, where the exact HARQ feedback timing and resource is provided to the UE in another DCI (in the same or in another COT)

  - Broadly supported by: Intel, Mediatek, Qualcomm, Ericsson, VIVO, Nokia, Huawei, Panasonic, Docomo, Fujitsu, OPPO, Sony, Lenovo, AT&T, interdigital, Sharp, CAICT, Spreadtrum, AT&T, LG, ZTE

- Alt2: UE is configured/allowed to report HARQ feedback for PDSCH from earlier COT(s) without an explicit request/trigger

  - Supported by: VIVO

  - Not supported by: Mediatek, Nokia, Samsung, Huawei, Intel, DOCOMO

- Alt3: gNB requests feedback outside the COT by PDSCH-to-HARQ-timing-indicator in the DCI scheduling the PDSCH

  - Broadly supported

  - See section 2.1.1 for impact to PDSCH-to-HARQ-timing-indicator

- Alt4: preconfigured/pre-indicated multiple opportunities in frequency domain in different LBT subbands

  - Broadly supported by: Mediatek, Qualcomm, Huawei, ZTE, Nokia, Sony, Samsung

  - Ericsson mentioned that details of wideband operation need to be agreed first

- Alt5: preconfigured/pre-indicated multiple opportunities in time domain

  - Broadly supported by: Mediatek, Huawei, Nokia, OPPO, Sony, Lenovo, Motorola Mobility, Samsung, Xiaomi, LG, ZTE

  - Needs further investigations: Ericsson, InterDigital, DOCOMO

Alt1 and Alt2 are the only two candidate solutions being proposed for transmitting HARQ A/N feedback in resources not indicated at the time of scheduling the PDSCH. At RAN1#95, some observations were discussed but no conclusion was made on a comparison of the use cases that can be handled by Alt1 or Alt2. The analysis from RAN1#95 is summarized in the Table below. Companies also raised concerns on Alt2 due to increased blind detection and detection robustness at the gNB, and specification impact for UCI plus additional information on PUCCH and PUSCH.

| HARQ feedback failure case | Alt 1 | Alt 2 |
| --- | --- | --- |
| case of LBT failure for PUCCH/PUSCH transmission | Ok | ok |
| case of PUCCH/PUSCH detection failure at gNB | Ok | Not possible |
| case of PDCCH detection failure at UE | Ok | Not possible |
| case of HARQ-ACK feedback pending from earlier COT(s) | Ok | ok |

Proposed agreement

- An explicit trigger from the gNB is required for UE transmission of HARQ A/N feedback on resources that were not indicated at the time of scheduling the PDSCH.

Among the proposals for Alt1, a majority of proposals suggested that triggering should apply to groups of PDSCHs that are determined dynamically by fields in DCI, rather than semi-statically configured groups of HARQ process IDs. A group could be indicated as a group of PDSCHs or a group of HARQ processes or a group of slots, or derived as a dynamic window. Further discussions will be needed to determine the details of the signalling of the dynamic grouping.

Proposed agreement

- For Alt1 (explicit triggering of HARQ feedback of PDSCHs scheduled earlier), the triggering applies to groups of PDSCHs where a group is not semi-statically configured

  - In a DCI carrying PDSCH-to-HARQ-timing-indicator and PUCCH resource indicator, one or more fields allow determining the group(s) of PDSCHs for which feedback is triggered

    - Supported at least in a DCI carrying a DL scheduling assignment

    - FFS: also supported in a UL grant or in a DCI not carrying a scheduling grant

There were also proposals to additionally support one-shot group HARQ ACK feedback:

- The group can be tied to a window, or bitmap of HARQ IDs, or all configured HARQ processes

- Trigger can be in PDSCH grant, in PUSCH grant, or in DCI not carrying a scheduling grant

An observation from the feature lead is that this could be seen as complementary to a solution based on Alt1 if the trigger for one-shot group HARQ ACK feedback would be carried in a PUSCH grant or in a DCI not carrying a grant.

Proposed agreement

- Further discussion on one-shot group HARQ ACK feedback should focus on the following alternatives:

  - signalling in a DCI carrying a PUSCH grant

  - signalling in a DCI not carrying a scheduling grant/assignment

  - if supported, feedback is based on semi-static HARQ codebook

Regarding Alt4, considerations could be discussed separately for the different options for wideband operation with subband LBT, as well as for CA with LBT per carrier. The discussion could start once RAN1 (and RAN4) reaches a better understanding of the wideband operation options to be supported.

Regarding Alt5, a number of companies supported that PDSCH-to-HARQ-timing-indicator in one DCI can indicate multiple timings for HARQ A/N feedback reporting.

Proposed agreement:

- Support that a state of PDSCH-to-HARQ-timing-indicator can indicate multiple timings for HARQ A/N feedback reporting

### Dynamic HARQ codebook

With respect to proposals related to Alt1, a number of detailed proposals were provided on how to determine the dynamic HARQ codebook.

| Huawei | DCI is used to trigger the feedback for PDSCH(s) from earlier COT(s). Feedback is triggered for one or more PDSCH sets. In order to indicate the accumulated unacknowledged PDSCH(s), both the counter DAI and total DAI should be extended. PDSCH set is defined as the PDSCH(s) for which the HARQ-ACK information is originally indicated to be carried in a same PUCCH. |
| --- | --- |
| Mediatek | A Group Indicator (GI) field (e.g. 2 bits) in the DCI scheduling each PDSCH is used to dynamically group PDSCHs. A Request (RQ) field (1 bit) is used to trigger the UE to report HARQ-ACK feedback, in another DCI triggering PDSCH. Both C-DAI and T-DAI should be accumulated for PDSCHs in a same group to determine a correct HARQ-ACK codebook. |
| Qualcomm | Define all ACKs that correspond to the same PUCCH resource by virtue of the PDSCH-to-ACK delay field pointing to that PUCCH resource to belong to a particular ACK-Feedback-Group Introduce mechanisms for UE to derive the following fields from the DCI. - Current ACK-Feedback-Group index (e.g. 2 bits if 4 HARQ feedback groups configured) - Indicator for additional ACK-Feedback-Group(s) to include along with the current ACK feedback (e.g. bitmap, index of another ACK-Feedback-Group) (e.g. 2 or 3 bits) - A new Ack-Feedback Group Indicator (NGI) bit for each ACK-Feedback-Group (e.g. 2 or 4 bits) The DAI counter field in DL grants that request including of ACK feedback for additional ACK-Feedback-Group(s) should count the DAI of those additional ACK-Feedback-Group(s) as well. Total: 9 or 6 additional bits (without counting changes to DAI) assuming 4 HARQ feedback groups. |
| Ericsson | Enlarge DAI field (to be able to count more missed DCIs) Introduce a toggle bit (NFI) in a DCI to indicate whether the expected HARQ-ACK feedback(s) from the UE was correctly received by the gNB or not: - Not toggled: the HARQ-ACK occasion(s) since the bit was last toggled was not received. The UE will automatically include the none received HARQ-ACK bits in the next reporting occasion - Toggled: the HARQ-ACK occasion(s) since the bit was last toggled was received by the gNB |
| Nokia | Alt1a by incrementing DAI field: UE identifies that dynamic codebook (determined by DAI) is extended across COTs by the K1 value indicating that HARQ feedback time is determined later. In the next COT, the transmission for the pending HARQ feedback is triggered by the incremented [total] DAI. UE continues with the DAI based codebook determination, with the actual transmission time and resource determined by K1 and PRI indicated in the next COT |
| Intel | In addition to DAI, introduce signalling to link the next PDSCHs of the next COT (e.g. ACK-NACK group index), and 1-bit HARQ-ACK reset indicator. If HARQ-ACK reset indicator is not toggled, the UE has to transmit both old HARQ-ACKs and new HARQ-ACK in the new PUCCH resource. |
| Samsung | For Alt1a, HARQ-ACK codebook index and HARQ-ACK codebook set to report should be added in DCI to ensure the aligned understanding of which HARQ-ACK codebooks are triggered to retransmit. C-DAI and T-DAI should be enhanced to count accumulative PDCCHs within the HARQ-ACK codebook set to alleviate HARQ-ACK codebook confusion, e.g. with one additional bit field for T-DAI counting for all PDCCHs of previous HARQ-ACK codebook. For approaches to provide multiple pre-configured/pre-indicated transmission opportunities in time-domain (Alt5), the HARQ-ACK codebook design needs enhancement to avoid any potential ambiguity. One common HARQ-ACK codebook or always aggregated HARQ-ACK codebook within UL slot set for multiple opportunities in time domain can be considered to avoid HARQ-ACK codebook ambiguity. |
| VIVO | Use HARQ process group based bitmap to trigger HARQ feedback for a group of HARQ processes (e.g. 4 groups) using a DCI. HARQ-ACK codebook based on the number of configured HARQ processes can be used when there is a need to combine multiple types of HARQ-ACK feedbacks, or there may be ambiguity between gNB and UE once legacy HARQ-ACK codebook would be used. |
| ZTE | Enhanced DAI mechanisms should be considered to indicate HARQ feedback of PDSCH(s) including the accumulated unacknowledged PDSCH(s) due to LBT failures |
| Fujitsu | Counter DAI is incremented sequentially from the preceding COT to align the understanding of HARQ codebook size between the gNB and the UE. FFS to support HARQ feedback from multiple earlier COTs, which would require an additional DCI field, e.g. HARQ process number or window ID, to explicitly indicate the requested HARQ A/N. |
| Panasonic | dynamic codebook size determination should be supported for requested/triggered HARQ feedback from earlier COT(s) |
| NTT DOCOMO | Propose to not extend DAI field for NR-U in Rel-16. |
| Spreadtrum | Dynamic HARQ-ACK codebook enhancements should take into account the triggering DCI in NR-U. HARQ-ACK group based triggering should be supported and the group index should be contained in the triggering DCI. |
| LG | DCI can indicate a slot group ID (i.e., requested slot group ID) then UE can report HARQ-ACK feedback for the DL slots corresponding to the indicated slot group ID, and in this case, each of DL grant DCIs can indicate the slot group ID (i.e., current slot group ID) in which the scheduled DL slot belongs to. In this case, counter-DAI is to be accumulated for PDSCHs in a same slot group. |
| ZTE | Semi-static HARQ codebook size can be a predefined size or the size of COT |

Proposed observation: for triggering additional HARQ feedback of earlier PDSCHs, relying only on DAI is insufficient for resolving dynamic HARQ codebook ambiguities between gNB and UE for the case where HARQ feedback for those PDSCHs was already transmitted by the UE but not received by the gNB, because the UE would think that it has missed a number of DCIs scheduling PDSCH in the meantime.

Proposed agreement

- For Alt1, C-DAI and T-DAI should be able to indicate accumulating a first group of HARQ A/N feedback (corresponding to a PDSCH-to-HARQ-timing-indicator pointing to an earlier slot) with a second group of HARQ A/N feedback (corresponding to a PDSCH-to-HARQ-timing-indicator pointing to a later slot) in the same dynamic HARQ codebook.

### Semi-static HARQ codebook

Summary of proposals on semi-static HARQ codebook enhancements:

| Intel | Option 1: to form HARQ-ACK codebook base on configured PDSCH-to-HARQ-ACK timings as in Rel-15. For accommodating the PDSCHs of the previous COT, additional bits may need to be reserved and appended to the HARQ-ACK codebook. Option 2: to form the HARQ-ACK codebook based on HARQ processes. |
| --- | --- |
| Qualcomm | Introduce a new field in the DL grant that controls the max delay (i.e. extension of K1) considered in semi-static HARQ ACK codebook size determination, as well as a new field in the DL grant that indicates the carriers to consider for codebook size determination. |
| VIVO | HARQ process based bitmap |
| Nokia | All HARQ processes |
| OPPO | Semi-static HARQ codebook size can restricted to the size of the COT |
| Samsung | For triggered mechanism, dynamic extension of K1 can be introduced Use semi-static HARQ codebook in case of preconfigured/pre-indicated multiple opportunities |
| Sharp | With some enhancement, e.g. a HARQ-ACK for a given PDSCH can be mapped to multiple codebooks which are reported at different timings. |
| Spreadtrum | NR-U semi-static HARQ-ACK codebook consists of 2 sub-codebooks, the first sub-codebook contains the HARQ-ACK information of the PDSCHs in the former COT while the second sub-codebook contains the HARQ-ACK information of the PDSCHs in the current COT. - For the first sub-codebook, the number of occasions for candidate PDSCH receptions or the number of slots for candidate PDSCH receptions can be configured by RRC signalling. - The second sub-codebook is the NR R15 semi-static HARQ-ACK codebook. |
| LG | Option 1: HARQ-ACK codebook based on the configured K1 values (as in Rel-15) with adjustment of the max delay. Option 2: HARQ-ACK codebook based on the configured HARQ processes. |

One question to be answered is whether and how semi-static HARQ codebook handles the PDSCH that is scheduled without valid PDSCH-to-HARQ-ACK timing.

Proposed agreement

- A field in the DL grant can control the max delay (i.e. extension of K1) considered in semi-static HARQ ACK codebook size determination.

## Scheduling enhancements

### PUSCH scheduling

A proposal related to handling the longest COT was made to extend the number of bits in the UL PUSCH time domain resource assignment field to allow scheduling PUSCH with the longest COT.

Proposed agreement

- Increase the number of bits in the UL PUSCH time domain resource assignment field to allow scheduling a PUSCH at the end of the longest COT where the scheduling PDCCH is received at the start of the COT.

[DCM] As we commented in 2.1.1, this proposal should also be unnecessary or low priority since it is just an optimization for specific COT situation while multiple DL-UL switching within gNB-initiated COT will be allowed (i.e., no need to put PUSCH at the end of COT).

### Multi-TTI PUSCH scheduling

TR38.889: scheduling multiple TTIs for PUSCH each using a separate UL grant in the same PDCCH monitoring occasion is identified as beneficial. Scheduling multiple TTIs for PUSCH, i.e., scheduling multiple TBs with different HARQ process IDs over multiple slots, using a single UL grant, is identified as beneficial and should be supported in NR-U.

Agreement from RAN1#95: the previous agreement on multi-TTI scheduling implies that NR-U should at least support scheduling multiple TBs with different HARQ process IDs in multiple slots using a single UL grant.

First focusing on the case of a single UL grant scheduling multiple TTIs for PUSCH, companies views on single vs. multiple TBs are diverse. A majority of companies would prefer to focus on the design of scheduling multiple TTIs for PUSCH with different TBs:

- Multiple PUSCHs with different TBs: Ericsson, Panasonic, Huawei, Spreadtrum, Samsung, ZTE, DOCOMO, LG, MediaTek, Lenovo/Motorola Mobility

- Multiple PUSCHs with same TB: Qualcomm, OPPO

Several companies mentioned that only time-contiguous transmissions of PUSCHs need to be supported when scheduling using a single UL grant. No company proposed to support non-time-contiguous transmissions.

Proposed agreement:

- Multi-TTI scheduling by single DCI is restricted to only a set of consecutive TTIs where PUSCHs are continuous in time

Aspects to be further addressed:

- For multi-TTI PUSCH scheduling, can FeLAA design be a starting point for designing a single DCI scheduling PUSCHs over multiple TTIs schedules with multiple TBs?

- Whether to introduce enhancements to Rel-15 NR for scheduling PUSCHs over multiple TTIs with the same TB using a single DCI. Rel-15 NR only allows this with repetitions of the same TB.

There were also proposals to support scheduling mini-slots in the initial portion of the multi-TTI PUSCH.

Some companies started providing details of corresponding DCI design. One company proposed supporting separate UL grants in the same PDCCH monitoring occasion for scheduling PUSCHs with different TBs over multiple TTIs.

### Cross-carrier scheduling

Nokia proposed that for CA, the licensed band scheduling carrier may use lower subcarrier spacing than the unlicensed band scheduled carrier. Support for this type of cross-carrier scheduling should be allowed for NR-U, once the solution is defined in the Rel-16 CA/DC enhancements work item. It is proposed to come back to this proposal once progress has been made in the CA/DC enhancements work item.

Proposed agreement:

- Cross-carrier scheduling from a licensed band with smaller subcarrier spacing will be supported by NR-U, details to be based on the design from the Rel-16 CA/DC enhancements work item.

### Cross-carrier HARQ re-transmission

A majority of companies proposed not to consider allowing cross-carrier HARQ re-transmissions.

Proposed agreement:

- Cross-carrier HARQ re-transmissions will not be discussed for NR-U in Rel-16.

### 2-stage UL grant

2-stage UL grant is supported in LTE eLAA. Several companies proposed defining a 2-stage UL grant for NR-U, to allow a UE to be ready to transmit the PUSCH right after receiving the second grant.

Proposed agreement:

- 2-stage UL grant is supported for NR-U

[DCM] Although 2-stage UL grant is supported in LTE-LAA, we should carefully consider the necessity of 2-stage grant in NR-U. The UE processing time for preparing PUSCH after receiving UL grant could be much shorter in NR compared with that in LTE. So, scheduling PUSCH within the same gNB-initiated COT containing corresponding UL grant would be possible in almost cases.

# References

RP-182767	TR38.889 V1.1.0 (2018-12)

RP-182878	New WID on NR-based Access to Unlicensed Spectrum

R1-1900062	HARQ enhancement in NR unlicensed	Huawei, HiSilicon

R1-1900102	Discussion on scheduling and HARQ for NR-U	ZTE, Sanechips

R1-1900112	Discussion on HARQ operation for NR-U	vivo

R1-1900188	Enhancements to HARQ for NR-U operation	MediaTek Inc.

R1-1900241	Potential HARQ enhancements for NR-U	Panasonic Corporation

R1-1900245	Scheduling/HARQ enhancements for NR-U	Fujitsu

R1-1900264	On NR-U HARQ scheduling and feedback enhancements	Nokia, Nokia Shanghai Bell

R1-1900274	HARQ enhancements for NR-U	OPPO

R1-1900365	Consideration on HARQ enhancement for NR-U	sony

R1-1900396	HARQ-ACK enhancement for NR-U	Lenovo, Motorola Mobility

R1-1900456	Design of HARQ procedures for NR-based access to unlicensed spectrum	AT&T

R1-1900472	Enhancements to HARQ for NR-unlicensed	Intel Corporation

R1-1900608	HARQ procedure for NR-U	LG Electronics

R1-1900706	Discussion on HARQ enhancements in NR-U	Spreadtrum Communications

R1-1900788	HARQ enhancements for NR-U 	InterDigital, Inc.

R1-1900827	HARQ enhancement for NR unlicensed operation	Sharp

R1-1900875	Enhancements to Scheduling and HARQ Operation for NR-U	Qualcomm Incorporated

R1-1900955	HARQ enhancement for NR-U	NTT DOCOMO, INC.

R1-1901000	Potential HARQ enhancements for NR-U	Ericsson

R1-1901033	HARQ enhancement for NR-U	Samsung

R1-1901101	HARQ enhancement for NR-U	Beijing Xiaomi Mobile Software

R1-1901124	Discussions on HARQ enhancements in NR-U	CAICT