3GPP TSG-RAN WG2#101	Tdoc R2-1802354

Athens, Greece, 26th February – 2nd March 2018	(Revision of R2-1800321)

Agenda Item:	10.4.1.8.1

Source:	Ericsson

Title:	Establishment causes for NR

Document for:	Discussion, Decision

# Introduction

This contribution is an update of R2-1800321 [1].

In the LS received in [2] CT1 asks RAN2, among others, for information related to establishment cause and call type:

Question 1:	Is there a need for NAS to provide AS with the establishment cause when NAS makes a request to AS for access? If yes, what are these establishment causes and will there be more establishment causes than what is there for RRC establishment cause in E-UTRAN?

Question 2:	CT1 assumes that the call type will not be needed for NG-RAN access. Can RAN2 please confirm?

At the SA1#80 meeting, a CR [4] on requirements for unified access control for 5G to TS 22.261 [5] was agreed and SA1 sent a reply LS [3] to RAN2 to take the provided information into account.

This contribution further elaborates on the information included in a connection request and in particular aspects of access category, access identity, call type and establishment cause in NR. The establishment causes that should be defined for NR are also discussed.

# Discussion

## Use of establishment cause and call type in LTE

In E-UTRAN and over RRC, establishment causes are used in RRC Connection Request or Resume Request to indicate the reason for establishment. The main purpose of this is to allow differentiation by the network, dependent on cause. What RAN needs to know is how important it is to admit that UE at that point to allocate resources between RA and Initial Context Setup. For example, the network may choose to treat certain establishment causes with higher priority than others. Lower priority accesses may be rejected in case of congestion.

The UE NAS will, when requesting (or requesting to resume) a signaling connection establishment to lower layers, provide an RRC establishment cause and a call type. The RRC establishment cause is then included in Msg3 and the call type is used for purposes of access control.

For RRC Connection Request and RRC Connection Resume Request, the establishment/resume causes specified for E-UTRA are: Emergency, High Priority Access, mt-Access, mo-Signalling, mo-Data, delayTolerantAccess-v1020, mo-VoiceCall-v1280, spare (For NB, it is mt-Access, mo-Signalling, mo-Data, mo-ExceptionData, delayTolerantAccess-v1330, spare3, spare2, spare1)

The call types, (3GPP TS 24.301) are: "originating signalling", “emergency calls”, "originating MMTEL voice" , "originating MMTEL video", "originating SMSoIP", "originating SMS", "terminating calls", "originating calls", "mobile originating CS fallback".

The call types are input from NAS to RRC to perform barring checks as described in 36.331. The call types and the establishment causes seems to be largely the same, but there is no real 1-1 mapping (for example, delayTolerantAccess is not a call-type but EAB “call type” serves same purpose). The purpose of them are however both to allow for a differentiated handling, through either prevention or prioritization of an access or resource request. For NR, there seems to be opportunities to align these two mechanisms.

## Access Identities and Access Categories in Unified Access Control

In the agreed SA1 CR [4] on unified access control, different access identities and access categories are defined as can be seen in Table 1 and Table 2 respectively.

Table 1 Access Identities, from [4]

| Access Identity number | UE configuration |
| --- | --- |
| 0 | UE is not configured with any parameters from this table |
| 1 (NOTE 1) | UE is configured for Multimedia Priority Service (MPS). |
| 2 (NOTE 2) | UE is configured for Mission Critical Service (MCS). |
| 3-10 | Reserved for future use |
| 11 (NOTE 3) | Access Class 11 is configured in the UE. |
| 12 (NOTE 3) | Access Class 12 is configured in the UE. |
| 13 (NOTE 3) | Access Class 13 is configured in the UE. |
| 14 (NOTE 3) | Access Class 14 is configured in the UE. |
| 15 (NOTE 3) | Access Class 15 is configured in the UE. |
| NOTE 1: Access Identity 1 is used to provide overrides according to the subscription information in UEs configured for MPS. The subscription information defines whether an overide applies to UEs within one of the following categories: a) UEs that are configured for MPS; b) UEs that are configured for MPS and are in the PLMN listed as most preferred PLMN of the country where the UE is roaming in the operator-defined PLMN selector list or in their HPLMN or in a PLMN that is equivalent to their HPLMN; c) UEs that are configured for MPS and are in their HPLMN or in a PLMN that is equivalent to it. NOTE 2: Access Identity 2 is used to provide overrides according to the subscription information in UEs configured for MCS. The subscription information defines whether an overide applies to UEs within one of the following categories: a) UEs that are configured for MCS; b) UEs that are configured for MCS and are in the PLMN listed as most preferred PLMN of the country where the UE is roaming in the operator-defined PLMN selector list or in their HPLMN or in a PLMN that is equivalent to their HPLMN; c) UEs that are configured for MCS and are in their HPLMN or in a PLMN that is equivalent to it. NOTE 3: Access Identities 11 and 15 are valid in Home PLMN only if the EHPLMN list is not present or in any EHPLMN. Access Identities 12, 13 and 14 are valid in Home PLMN and visited PLMNs of home country only. For this purpose the home country is defined as the country of the MCC part of the IMSI. |  |

Table 2 Access Categories, from [4]

| Access Category number | Conditions related to UE | Type of access attempt |
| --- | --- | --- |
| 0 | All | MO signalling resulting from paging |
| 1 (NOTE 1) | UE is configured for delay tolerant service and subject to access control for Access Category 1, which is judged based on relation of UE’s HPLMN and the selected PLMN. | All except for Emergency |
| 2 | All | Emergency |
| 3 | All except for the conditions in Access Category 1. | MO signalling resulting from other than paging |
| 4 | All except for the conditions in Access Category 1. | MMTEL voice |
| 5 | All except for the conditions in Access Category 1. | MMTEL video |
| 6 | All except for the conditions in Access Category 1. | SMS |
| 7 | All except for the conditions in Access Category 1. | MO data that do not belong to any other Access Categories |
| 8-31 |  | Reserved standardized Access Categories |
| 32-63 (NOTE 2) | All | Based on operator classification |
| NOTE 1: The barring parameter for Access Category 1 is accompanied with information that define whether Access Category applies to UEs within one of the following categories: a) UEs that are configured for delay tolerant service; b) UEs that are configured for delay tolerant service and are neither in their HPLMN nor in a PLMN that is equivalent to it; c) UEs that are configured for delay tolerant service and are neither in the PLMN listed as most preferred PLMN of the country where the UE is roaming in the operator-defined PLMN selector list on the SIM/USIM, nor in their HPLMN nor in a PLMN that is equivalent to their HPLMN. NOTE 2: When there are an Access Category based on operator classification and a standardized Access Category to both of which an access attempt can be categorized, and the standardized Access Category is neither 0 nor 2, the UE applies the Access Category based on operator classification. When there are an Access Category based on operator classification and a standardized Access Category to both of which an access attempt can be categorized, and the standardized Access Category is 0 or 2, the UE applies the standardized Access Category. |  |  |

The above list of access categories in Table 2 seems to include aspects similar to establishment cause and call type, at least for the standardized categories. We can make the following observation:

Establishment causes and call types may be a subset of what can be used to determine new unified access category

Considering that the access category determination, i.e., what events on higher layers that should map to what access categories, is controlled by NAS and signalled to RRC, we think that at least the call type can be directly replaced with the access category. This would also allow that future services, slices or whatever distinctions desired can be applied without changing the framework, by simply changing the rules for what access category to use.

For an NR UE, a request from NAS including new access category removes the need of a call type indication.

The establishment cause value is, as mentioned above, also indicated by higher layers in LTE.

According to TS 24.301, for a single NAS event and call type, there could be different mappings to establishment causes. For example, for NAS event Tracking Area Update, there could be one and the same call type “originating signalling” but the establishment cause can differ. See below extract from 3GPP TS 24.301 on Tracking Area update:

| If the UE is allowed to use exception data reporting (see the ExceptionDataReportingAllowed leaf of the NAS configuration MO in 3GPP TS 24.368 [15A] or the USIM file EFNASCONFIG in 3GPP TS 31.102 [17]) and there is a pending request from upper layers to transmit user data related to an exceptional event, the RRC establishment cause shall be set to MO exception data. | "originating signalling" |
| --- | --- |
| If the UE is requesting resources for V2X communication over PC5, the RRC establishment cause shall be set to MO signalling. (See Note 1) | "originating signalling" |

We believe that many of these rules can be referred to an access category rather than specified with specific mappings between NAS events, call types and establishment causes. One reason why this would be attractive would be that the input priority indication for transmitting an access (barring, i.e., priority to use the PRACH resources) would be the same as the priority indication for allocating resources for further signaling (SRB). For example, the use of not blocking an access (use of PRACH) is questionable if the request will anyway be rejected after Msg3 and vice versa. If the same input (access category) is used both as barring input and for establishment cause in the RRC message (Msg3) to the network, then a consistent handling can be supported. Including an access category in Msg3 would be more future-proof in the sense that operators easier can incorporate new rules and priorities for access (both to prevent access and to allow resources for SRB), simply by defining how to configure and map, e.g., NAS events to access categories. Both the barring and the network treatment when an access request is received, can then be aligned and fully configurable by the operator through access category configuration.

A framework where access category is replacing both call type and establishment cause will be more future-proof and flexible

With all the above it seems reasonable to consider access category to be used as input for both access barring and admission/response to Msg3.

RAN2 should consider access category both for access barring purposes and for setting the establishment cause.

Continuing, we note that at least the following topics should be addressed:

- When multiple access categories may apply for a given access request

- Msg3 size

- How to set the establishment cause when accessing 5GC via E-UTRA

- Dependency on CT1 progess

These topics are further discussed below.

## When multiple access categories may apply for a given access request

Several access attempts can be performed in parallel - e.g. MMTEL voice call can be established in parallel with SMSoIP application sending an SMS. In other words, when requesting a NAS signalling connection, there can be several ongoing, already authorized, access attempts of possibly different access categories.

When an RRC connection establishment is triggered by NAS, multiple services and applications may be running and consequently more than one access attempt (and thus possibly more than one access category) may apply.

The UE needs to perform the unified access control barring check whether the access attempt is authorized for each of those access attempts separately. In order to deal with these aspects of access control, we think that the following principles are to be used when NAS requests to setup a connection (also discussed in [6]):

- At the moment when NAS requests setup of NAS signalling connection, RRC can assume that all the barring checks were already done and passed.

- NAS shall determine the “most applicable access category”. How this is done should be specified by CT1.

- NAS layer provides this single access category to RRC when requesting a NAS signalling connection.

NAS layer provides a single access category, for the purpose to set an establishment cause, to RRC when requesting a NAS signalling connection.

## Impact on Msg3 size due to establishment cause

Establishment cause is currently, in LTE, only a 3-bit field, i.e., less than 10% of the total size of Msg3. If a new unified access category should be included in Msg3 instead of establishment cause, more bits would be required. The SA1 CR [4] defines 64 access categories (32 standardized and 32 operator-defined).

As we propose above, NAS provides a single access category to RRC when requesting a NAS signalling connection. RRC then uses this single access category when setting the RRC establishment cause.

A straightforward way to set the establishment cause is to set it to the actual value of the access category. This would however result in that at least six bits would be needed in msg3 (if also operator-defined categories are possible to use as establishment cause). Looking at the E-UTRA establishment causes, the “High Priority Access” establishment cause is used by a UE with access class 11-15. To allow for the same information in NR it should then be possible to indicate e.g. that the UE has an access identity <> 0 (per Table 1), in the establishment request. To cater for that information the size of the establishment cause may then increase even further.

If access category/identity should replace establishment cause for NR, more bits are needed in Msg3.

An alternative way is to specify a mapping from access category and access identity to establishment cause. This may reduce the amount of bits needed in msg3.

If the RRC layer sets the establishment cause based on a specified mapping from the access category and access identity, the number of bits needed in msg3 can be reduced.

## Mapping from access category/identity to establishment cause

As discussed in section 2.2 it would be beneficial to have a direct mapping from the access category to the establishment cause since both access control (barring check) and (Msg3) admission control then would be based on precisely the same information and since the direct mapping means that it is easier to incorporate new rules and priorities also for admission control. The drawback is then however that the size of the establishment cause/Msg3 would become larger since 6 bits are needed to cater just for the access category. Since the access identities are not mapped towards the access categories in the stage-1 requirements (in Table 2) there is then also a need to indicate that a request is done by e.g. a UE with Access Identity <> 0 (per Table 1). At least one additional bit would therefore be needed to indicate whether the access request is for a UE with access identity <> 0.

In order to enable a mapping from both the Access Category and the Access Identity/-ies of the UE into an establishment cause, and to limit the size of the establishment cause in Msg3, it is therefore proposed to have a mapping from UAC access category and access identity to establishment cause in NR. With the unified access control being applicable also for E-UTRA connected to 5GC a similar mapping should be introduced also for E-UTRA connected to 5GC. Any dependencies on radio access technology should preferably be part of the Access Stratum (AS) and the mapping from the access category received from NAS to the RRC establishment cause should therefore be performed in the AS level (in RRC). The access identity should then also be included in the mapping by RRC, at least to indicate that the request is for a UE with access identity <> 0. In case the access attempt is mapped to an operator-defined access category by NAS, it will not be possible for the RRC layer to map it to an establishment cause since it is not aware of the definition of the different operator-defined access categories. In that case the NAS should therefore provide the AS/RRC layer with the corresponding standardized access category for the access, which then is used by the AS/RRC layer for mapping to establishment cause.

For NR, a mapping should be performed in the RRC layer from a standardized access category and access identity/-ies provided from NAS.

In case an access attempt is mapped to an operator-defined access category (for access control/barring check), NAS should provide a standardized access category to the AS/RRC layer, for the purpose to set the establishment cause.

For NR, specify a mapping from standardized access categories and access identities to NR establishment causes in TS 38.331

We propose to use the same principle for E-UTRA as we propose above for NR, i.e. that NAS provides a single access category when requesting setup of the connection, which is mapped to an E-UTRA establishment cause in the RRC layer. The E-UTRA RRC layer would then set the establishment cause based on the provided standardized access category value, just as proposed for NR above. Since one of the existing E-UTRA establishment causes needs to be set, an E-UTRA specific mapping would be needed for E-UTRA connected to 5GC. It is proposed to include such a mapping in TS 36.331.

When accessing 5GC via E-UTRA, the E-UTRA RRC layer should set the establishment cause based on a specified mapping from the standardized access category and access identity/-ies, provided by NAS. In case the access attempt is mapped to an operator-defined access category, NAS should then provide a standardized access category to the AS/RRC layer, for the purpose to set the establishment cause.

Include a mapping from UAC standardized access categories and access identities to E-UTRA establishment causes, for access to 5GC via E-UTRA, in TS 36.331.

## Establishment causes in NR

As discussed in section 2.1 the establishment cause field in the RRCConnectionRequest message in E-UTRA consists of 7 different code points, which are mapped to a 3 bit field. There is then a single spare value.

To be able to indicate the standardized access categories that have been defined so far by SA1 (per Table 2) and the case of high priority accesses, i.e. by UEs with an Access Identity <> 0, at an access request the establishment causes in NR should consists of the values in Table 3 below. It can be noted that this then corresponds to 9 different code points, which thus is not possible to indicate with 3 bits.

Table 3 RRC establishment causes in NR, based on the defined standardized access categories and access by UEs with Access Identity <> 0.

| Standardized access category | Type of access attempt | RRC establishment cause in NR |
| --- | --- | --- |
| 0 | MO signalling resulting from paging | MT Access |
| 1 | UE is configured for delay tolerant service | Delay Tolerant Access |
| 2 | Emergency | Emergency |
| 3 | MO signalling resulting from other than paging | MO Signalling |
| 4 | MMTEL voice | MO Voice Call |
| 5 | MMTEL video | MO Video call |
| 6 | SMS | MO SMS |
| 7 | MO data that do not belong to any other Access Categories | MO Data |
| Access Identity <> 0 | All | High Priority Access |

For the case where a UE in RRC_INACTIVE needs to perform a resume request with the purpose to perform a RAN Notification Area Update, it was further agreed at the RAN2#98 meeting that “Connection resume message will include information that can at least indicate RAN area update. Inclusion of information to enable access control is not precluded.”. An establishment cause value for RNA update should thus also be included in NR.Even if some standardized access categories are mapped to the same establishment cause to save bits, as for SMS and MMTEL video in LTE, it can also be noted that SA1 has reserved space for 24 access categories in their requirements (per Table 2). It is thus very likely that there will be a need to introduce further establishment causes in future releases. It can then also be noted that to increase the size of the (Msg3) establishment cause field in a later release can be rather complex.

By increasing the size of the establishment cause field in Msg3 to 4 bits it is thus possible to separate the different standardized access categories as well as a high priority access in NR. It will then also be possible to extend the number of establishment causes in later releases when needed, e.g. as further standardized access categories are introduced. It is therefore proposed that the establishment cause field in Msg3 has a size of 4 bits in NR, and that the following establishment causes are defined in NR Rel-15 “MT Access”, “Delay Tolerant Access”, “Emergency”, “MO Signalling”, “MO Voice Call”, “MO Video call”, “MO SMS”, “MO Data”, “High Priority Access” and “RNA Update”.

The establishment cause field in Msg3 has a size of 4 bits in NR. The following establishment causes should be defined in NR Rel-15 “MT Access”, “Delay Tolerant Access”, “Emergency”, “MO Signalling”, “MO Voice Call”, “MO Video call”, “MO SMS”, “MO Data”, “High Priority Access” and “RNA Update”.

# Conclusion

In section 2 we made the following observations:

Observation 1	Establishment causes and call types may be a subset of what can be used to determine new unified access category

Observation 2	For an NR UE, a request from NAS including new access category removes the need of a call type indication.

Observation 3	A framework where access category is replacing both call type and establishment cause will be more future-proof and flexible

Observation 4	When an RRC connection establishment is triggered by NAS, multiple services and applications may be running and consequently more than one access attempt (and thus possibly more than one access category) may apply.

Observation 5	If access category/identity should replace establishment cause for NR, more bits are needed in Msg3.

Observation 6	If the RRC layer sets the establishment cause based on a specified mapping from the access category and access identity, the number of bits needed in msg3 can be reduced.

Based on the discussion in section 2 we propose the following:

Proposal 1	RAN2 should consider access category both for access barring purposes and for setting the establishment cause.

Proposal 2	NAS layer provides a single access category, for the purpose to set an establishment cause, to RRC when requesting a NAS signalling connection.

Proposal 3	For NR, a mapping should be performed in the RRC layer from a standardized access category and access identity/-ies provided from NAS.

Proposal 4	In case an access attempt is mapped to an operator-defined access category (for access control/barring check), NAS should provide a standardized access category to the AS/RRC layer, for the purpose to set the establishment cause.

Proposal 5	For NR, specify a mapping from standardized access categories and access identities to NR establishment causes in TS 38.331

Proposal 6	When accessing 5GC via E-UTRA, the E-UTRA RRC layer should set the establishment cause based on a specified mapping from the standardized access category and access identity/-ies, provided by NAS. In case the access attempt is mapped to an operator-defined access category, NAS should then provide a standardized access category to the AS/RRC layer, for the purpose to set the establishment cause.

Proposal 7	Include a mapping from UAC standardized access categories and access identities to E-UTRA establishment causes, for access to 5GC via E-UTRA, in TS 36.331.

Proposal 8	The establishment cause field in Msg3 has a size of 4 bits in NR. The following establishment causes should be defined in NR Rel-15 “MT Access”, “Delay Tolerant Access”, “Emergency”, “MO Signalling”, “MO Voice Call”, “MO Video call”, “MO SMS”, “MO Data”, “High Priority Access” and “RNA Update”.

# References

R2-1800321, Establishment causes for NR, Ericsson, RAN2 WG2 NR AH#1801, Vancouver, Canada, 22nd - 26th January 2018

R2-1801642 / C1-180708, CT1's chosen solution for unified access control, 3GPP CT WG1 #108, Gothenburg, Sweden, January 22-26, 2018

S1-174624, Reply LS on unified access control for 5G NR, 3GPP SA1

S1-174619, Clarification on unified access control requirements, NTT DOCOMO INC., U.S. Department of Commerce, FirstNet, Vencore Labs, Qualcomm Incorporated, LG Electronics Inc., Nokia, Nokia Shanghai Bell, AT&T, Intel, Ericsson, 3GPP TSG-SA WG1 Meeting #80, Reno, Nevada, USA, 27 November – 1 December 2017

3GPP TS 22.261, Service requirements for the 5G system, v15.2.0, 2017-09

R2-1802350, AS-NAS functional division for Unified Access Control, Ericsson, RAN2 WG2#101, Athens, Greece, 26th February – 2nd March 2018

R2-1802351, RRC-initiated access attempts in RRC_IDLE and RRC_INACTIVE, Ericsson, RAN2 WG2#101, Athens, Greece, 26th February – 2nd March 2018