3GPP TSG-RAN WG2 #101	Tdoc R2-1802685

Athens, Greece, 26th February – 2nd March 2018

Agenda Item:	10.4.1.3.5

Source:	Ericsson

Title:	RRC UE processing time for RRC reconfiguration in 38.331

Document for:	Discussion, Decision

# Introduction

In RAN2#100, the following agreements were made regarding RRC processing times for EN-DC.

Agreements

1	A joint processing time requirement is defined to LTE reconfiguration message with embedded NR RRC part.

2	The processing time requirement for NR RRC received over SRB3 is defined in 38.331.

3	The processing time requirements only apply to the case where the UE receives one RRC message at a time.

4	The processing time definition and unit of LTE RRC is applied also for LTE RRC with embedded NR RRC.

5	Following aspects should be considered when deciding RRC processing times for LTE RRCConnectionReconfiguration with embedded NR RRC:

- LTE and NR ASN.1 decoding and encoding

- Configuration time including coordination between LTE and NR part of UE

- Encoding of LTE and NR complete message

- Increased complexity due to introduction of new features and RRC IEs in LTE-A and NR

- UE processing capability improvements since R8

6	The following two subcases are defined for RRCConnectionReconfiguration including NR RRC:

- RRC connection re-configuration (NR measurement configuration)

- RRC connection re-configuration (NR SCG establish/mod/release)

The processing times for RRC connection reconfiguration with embedded NR RRC were agreed in RAN2#NR-AH1801 in Vancouver, to be included in 36.331. In this paper, we continue the discussion on processing time values in 38.331. This is needed for NR RRC reconfiguration messages transmitted over SRB3 in EN-DC operation, and need thus to be included in 38.331 for the December drop.

# Discussion

In [1], we discuss the NR RRC processing time for standalone NR operation, based on ITU and 3GPP latency requirements for state transition from a power saving state to active. The outcome of that discussion is that a processing time of 3 ms is needed, in order to support both ITU and 3GPP requirements for 7 and 2 OFDM symbols at 15 kHz subcarrier spacing.

A processing time of 3 ms for NR RRC is needed in order to meet the ITU and 3GPP requirement for CP latency.

As there are no specific requirements for the NR RRC reconfiguration procedure, we suggest using the same UE processing time for NR RRC reconfiguration received over SRB3 in EN-DC operation.

Processing time in UE for RRC reconfiguration in 38.331 is 3 ms.

# Conclusion

Based on the discussion in section 2 we propose the following:

Proposal 1	Processing time in UE for RRC reconfiguration in 38.331 is 3 ms.

# TP for 38.331

START OF CHANGES

11 Processing delay requirements for RRC procedures

The UE performance requirements for RRC procedures are specified in the following tables. The performance requirement is expressed as the time in [ms] from the end of reception of the network -> UE message on the UE physical layer up to when the UE shall be ready for the reception of uplink grant for the UE -> network response message with no access delay other than the TTI-alignment (e.g. excluding delays caused by scheduling, the random access procedure or physical layer synchronisation).

Figure 11.2-1: Illustration of RRC procedure delay

Table 11.2-1: UE performance requirements for RRC procedures for UEs

| Procedure title: | Network -> UE | UE -> Network | Value [ms] | Notes |
| --- | --- | --- | --- | --- |
| RRC Connection Control Procedures |  |  |  |  |
| RRC reconfiguration | RRCReconfiguration | RRCReconfigurationComplete | 3 |  |

END OF CHANGES

# References

R2-1802686, “RRC UE processing time for Standalone NR”, Ericsson