3GPP TSG-RAN WG2 #101	Tdoc R2-1803533

Athens, Greece, 26th February – 2nd March 2018

Agenda Item:	10.4.3.1

Source:	Ericsson

Title:	Summary of email discussion #23 for 38.331 ASN.1 review part 5

Document for:	Discussion, Decision

# Introduction

This contribution gives a summary for the following email discussion:

[NR-AH1801#23][NR] 38.331 ASN.1 review part 5 - Bearer config and high level procedures (Ericsson)

-	Detailed instructions to be provided by RRC rapporteur and/or chairman

Intended outcome: Merged CR to 38.331 submitted to next meeting

Deadline:  Tuesday 2018-02-20

The discussion was split to following parts:

Part 1. Scope, 2. References, 3. Definitions 4. General

Part 2 Procedures:  5.1. General, 5.2 System info (only EN-DC related aspects)

Part 3.  Procedures:  5.3.5.1-5.3.5.5 RRC Connection Reconfiguration

Part 4.   Procedures: 5.3.5.6 Radio bearer configuration, 5.3.5.8 security key update

Part 5.  5.3.5.9 Reconfiguration failure, 5.3.5.10 EN-DC release, 5.3.10 Radio link failure related actions, 5.7.3 SCG Failure  information

Part 6.  PDU (ASN.1): 6.1 General, 6.2 RRC messages

Part 7.  PDU (ASN.1) 6.3:

- Following IEs: CellGroupConfig, DRB-Identity, MeasResultsSCG-Failure, RadioBearerConfig,   RLF-TimersAndConstants,  SecurityAlgorithmConfig,   ServCellIndex, SRB-Identity

Part 8.   Subclause 6.4 RRC multiplicity and type constraint values

Part 9.   Subclause 7.1 Timers, 7.2 Counters, 7.3. Constants

Part 10.  Subclause Protocol Data Unit abstract syntax

Part 11   Subclause 10. Generic error handling

Part 12  Subclause Processing delay, A annex

# Discussion

## Issues identified in the email discussion

Part 1. Scope, 2. References, 3. Definitions 4. General

- Minor comments, no need to discuss them

- Related contribution: R2-1803531 PSCell and SCell terminology (RIL F008)

  - Coordinator proposal is to discuss R2-1803531 offline

Part 2 Procedures:  5.1. General, 5.2 System info (only EN-DC related aspects)

- Mostly minor comments

- Main issue is SI change for PSCell (F303):

  - F303: Class2: There is SCell release (specified in 5.3.5.5.8) and SCell addition (specified in 5.3.5.5.9) procedure but we have concern to extend to PSCell considering there is no PSCell release and PSCell addition procedure.

It seems that PSCell is released via SCG release procedure (specified in 5.3.5.4) and PSCell is added via reconfiguration with sync (specified in 5.3.5.5.2).

We are not sure which option is better:

  - Option 1: capture that: “Upon change of relevant SI, PSCell is released via SCG release procedure and then is added via reconfiguration with sync.

  - Option 2: add an FFS that: Upon change of relevant SI, FFS for PSCell.

- It should be noticed that

  - in ASN.1, SI for PSCell can be changed only with the reconfiguration with synch

  - SCG cannot be released. There is only EN-DC release bit in TS36.331

- Coordinator proposes to discuss this online

- Meanwhile, it should be also checked if RadioResourceConfigCommon should be optional

Part 3.  Procedures:  5.3.5.1-5.3.5.5 RRC Connection Reconfiguration

- Mostly minor comments

- M120:

  - Mediatek: In LTE, we have a section 5.5.6.1 – Action upon handover and re-establishment to handle the measurement behaviour upon handover. In NR, we should have a similar section 5.5.x.x – Action upon reconfiguration with sync to handle the measurement behaviour upon reconfiguration with sync. The section is missing in the measurement clauses.

  - Ericsson: our understanding that section has not been adopted from LTE in NR, as that optimization has not been agreed to be supported in NR.

  - Contributions R2-1802428 and R2-1803321 to be discussed online

Part 4.   Procedures: 5.3.5.6 Radio bearer configuration, 5.3.5.8 security key update

- Minor comments

- SH502: DRB ID mapping to eps-BearerIdentity can be discussed offline (as there is similar discussion for 36.331)

Part 5.

- All Comments are related to in 5.7.3 (SCG Failure information)

  - Q041: There is no means for UE to differentiate whether the current EN-DC configuration is synchronous deployment or not. RAN2 should further study how to address this issue.

    - Coordinator: This topic can be discussed online

  - C112/C121: Online discussion based on R2-1802620

Part 6.  PDU (ASN.1): 6.1 General, 6.2 RRC messages

- Minor comments, some comments on SIB1 can be addressed later

Part 7.  PDU (ASN.1) 6.3:

- CellGroupConfig C101:

  - When srb3-ToRelease is set to true, the RLC Bearer for SRB3 should be released

  - Can be discussed offline if there is need to capture network restrictions how to configure bearer configurations and RLC bearer configurations together

- CellGroupConfig ZTE301:

  - the relationship between rach-ConfigDedicated and BWP should be clarified. One possibility is to add a restriction somewhere that dedicated random access resources should be configured within the initial UL BWP

  - Needs to be discussed. However, not clear if there is input document on this.

- CellGroupConfig S033:

  - RLM signalling is agreed to be in SpCell configuration. However RLM-RS is configured per-BWP. Therefore, the parameter location can be splitted into SpCellConfig IE for common BWP and the other for per-BWP. The only IS/OOS decision threshold which is the parameter except RLM-RS can be located in SpCellConfig.

  - Coordinator: Discuss based on R2-1802483

- MeasResultSCG-Failure E382:

  - In RAN2 AH 1801 we have the agreement reagarding NR MOs:

    - 1: Within an MO, the SSB location is always indicated with GSCN with no additional offset.

    - FFS Whether the subcarrier offset is also required. To be checked with RAN1

    - 2: For MO with CSI-RS, an NR-ARFCN is used to indicate a frequency reference. Location of CSI-RS is relative to this reference

    - Thus, the measurement results included in the MeasResultSCG has to updated to reflect this (as they currently include only to the ARFCN)

  - Discuss based on R2-1802641 online

- RadioBearerConfig

  - C102 can be confirmed offline

  - Q039 and Q040 on security keys can be discussed offline or in a breakout session

- RLFTimersAndConstants

  - C105: R2-1802619

Part  8.   Subclause 6.4 RRC multiplicity and type constraint values

- E359 added by Ericsson/Coordinator:

All FFS should be discussed and closed in RAN2#101

Can be discussed based on R2-1803553     We suggest that proposals 1-2 are discussed online and rest of the proposals offline/breakout session in the context of the actual applicable CR

- D505 can be discussed and confirmed offline

Part  9.  Subclause 7.1 Timers, 7.2 Counters, 7.3. Constants

- In timers, minor comments which are addressed. Coordinator suggests to focus normative procedures first to check if anything is missing and after that correct informative annex.

Part  10.   Subclause Protocol Data Unit abstract syntax

- No comments received

Part  11   Subclause 10. Generic error handling

- Minor comments, no need for online discussion

Part   12   Subclause Processing delay, A annex

- To discuss online: Exxx: Class3: Reconsider this requirement. For larger structures it is perfectly OK but having to create IEs for all smaller occurrences is actually worse than using the traditional structure  Allow using the old CHOICE release/setup for smaller structures

## Other papers related to the scope of email discussion

Other papers related to old RIL issues relevant to this email discussion:

- In R2-1803103, it is proposed to discuss more high level structure of NR RRCREconfiguration message. Especially, it was proposed to add MCG config back, even it was just removed in this email discussion.

- In R2-1803104, it is proposed to align procedural specification with the ASN.1 structure. The discussion was postponed from RAN2 Ad hoc and not discussed in this email discussion.