3GPP TSG-RAN WG2 #101	R2-1803746

26th February – 2nd March 2018

Athens, Greece

Source:                    	NTT DOCOMO, INC.

Title:  	[D325] Sub-carrier spacing for serving cell/carrier and BWP

Document for:        	Discussion and decision

Agenda Item:         	10.4.3.2	ASN.1 issue document

- Introduction

In ServingCellConfigCommon, there is the following FFS issue.

-- Subcarrier spacing for SIB1, Msg.2/4 for initial access and SI-messages.

-- Values 15, and 30 kHz are applicable for carrier frequencies <6GHz; Values 60 and 120 kHz are applicable for carrier frequencies >6GHz

-- FFS: This must be one of the SCSs defined already inside FrequencyInfoDL. Consider flagging one of those as ”common” instead of this field.

subcarrierSpacingCommon				SubcarrierSpacing,

As can be seen in the current ASN.1 structure (see Annex A), sub-carrier spacing (SCS) is configured at several places as listed below:

-	ServingCellConfigCommon (SCS for common and SSB);

-	SCS-SpecificCarrier in FrequencyInfoDL/UL;

-	TDD-UL-DL-ConfigCommon/Common2;

-	Downlink/UplinkBWP-Common.

This paper attempts to discuss whether all of the SCS parameters are needed or some of them can be removed by referring to the other field.

- Discussion

Figure 2-1 illustrates an example of how PRB is indexed and BWP-Common is configured. By referring to Point A obtained in FrequencyInfoDL/UL, the list of SCS-SpecificCarrier is set for each SCS to index PRB grid. On top of that, DL/UL-BWP(-Common) is configure on a certain SCS-SpecificCarrier. Whilst SCS is configured inside DL/UL-BWP, there must be a corresponding SCS-SpecificCarrier configured in FrequencyInfoDL/UL. If DL/UL-BWP is linked to the corresponding SCS-SpecificCarrier, SCS is not needed inside DL/UL-BWP. This can be done by assigning ID for each SCS-SpecificCarrier. The ID of SCS-SpecificCarrier is included in DL/UL-BWP, instead of SCS. Alternatively, the ID of SCS-SpecificCarrier can be assigned implicity, e.g. by the order of entry in scs-SpecificCarriers. The followings are proposed:

Proposal 1:	ID is assigned for each SCS-SpecificCarrier (explicitly or implicitly). Downlink/UplinkBWP-Common includes the ID of each SCS-Specific carrier on which BWP is to be configured.

Proposal 2:		If proposal 1 is agreed, subcarrierSpacing is removed from the BWP IE.

The next question is whether SCS common and SCS for SSB are necessary in ServingCellConfigCommon. SCS for SSB has to be present as it can be different from the physical channels and signals. In contrast, the need of common SCS here is worthwhile discussing. The common SCS is the SCS for SIB1, Msg.2/4 for initial access and SI-messages. Even for this purposes, these messages are delivered on the “common” BWP configured in ServingCellConfigCommon. Thus, the common SCS also belongs to one of the scs-SpecificCarriers. In that sense, the SCS-SpecificCarrier for SIB1, Msg.2/4 for initial access and SI-Messages can be flagged one of the scs-SpecificCarriers as suggested by the email discussion rapporteur [NR-AH1801#19]. The following is proposed:

Proposal 3:	SCS for SIB1, Msg.2/4 for initial access and SI-messages is flagged to one of the SCS specific carriers included in ServingCellConfigCommon.

Proposal 4:		If proposal 3 is agreed, subcarrierSpecingCommon is removed from ServingCellConfigCommon.

Figure 2-1:	PRB indexing for NR

- Summary and proposal

In summary, the followings were proposed:

Proposal 1:	ID is assigned for each SCS-SpecificCarrier (explicitly or implicitly). Downlink/UplinkBWP-Common includes the ID of each SCS-Specific carrier on which BWP is to be configured.

Proposal 2:		If proposal 1 is agreed, subcarrierSpacing is removed from the BWP IE.

Proposal 3:	SCS for SIB1, Msg.2/4 for initial access and SI-messages is flagged to one of the SCS specific carriers included in ServingCellConfigCommon.

Proposal 4:		If proposal 3 is agreed, subcarrierSpecingCommon is removed from ServingCellConfigCommon.

TP to implement these proposals into 38.331 is provided in section 5.

- References

[1] TS 38.331 v15.0.1 “Radio Resource Control (RRC) Protocol specification,”

- TP to 38.331

The First Change

BWP information element

-- Generic parameters used in Uplink- and Downlink bandwidth parts

BWP ::= 					SEQUENCE {

-- Frequency domain location and bandwidth of this bandwidth part defined commonly in a table (FFS_Section). The location is given as

-- distance (in number of PRBs) in relation to the lowest usable subcarrier defined by the SCS-SpecificCarrier

-- with the same subcarrier spacing as this BWP.

-- Corresponds to L1 parameter 'DL-BWP-loc'. (see 38.211, section FFS_Section).

-- In case of TDD, a BWP-pair (UL BWP and DL BWP with the same bwp-Id) must have the same location (see 38.211, section REF)

-- FFS_Value: RAN1 seems to discuss the final range.

locationAndBandwidth			INTEGER (1..65536),

-- Subcarrier spacing to be used in this BWP. It is applied to at least PDCCH, PDSCH and corresponding DMRS.

-- The values provided here are converted into a subcarrier spacing as indicated in 38.211, Table 4.2-1.

scs-SpecificCarrierId			SCS-SpecificCarrierId,

-- Indicates whether to use the extended cyclic prefix for this bandwidth part. If not set, the UE uses the normal cyclic prefix.

-- Normal CP is supported for all numerologies and slot formats. Extended CP is supported only for 60 kHz subcarrier spacing.

-- (see 38.211, section 4.2.2)

cyclicPrefix					ENUMERATED { extended }														OPTIONAL	-- Need R

The Next Change

ServingCellConfigCommon information element

-- ASN1START

-- TAG-SERVING-CELL-CONFIG-COMMON-START

ServingCellConfigCommon ::=		SEQUENCE {

physCellId							PhysCellId													OPTIONAL, -- Cond HOAndServCellAdd,

frequencyInfoDL						FrequencyInfoDL												OPTIONAL, -- Cond InterFreqHOAndServCellAdd

-- The initial downlink BWP configuration for a SpCell (PCell of MCG or SCG).

-- FFS: Discuss and then clarify in condition which serving cells have an initial BWP

initialDownlinkBWP		DownlinkBWP-Common								OPTIONAL,	-- Cond FFS

-- FFS: Possibly remove the condition on uplinkConfigCommon or replace by ”UL”. Note that the entire ServingCellConfigCommon can

-- only be sent when upon reconfiguration with sync and upon PSCell/SCell addition.

uplinkConfigCommon					UplinkConfigCommon											OPTIONAL, -- Cond ReconfWithSyncAndSCellAdd

supplementaryUplinkConfig

UplinkConfigCommon

}																								OPTIONAL, -- Cond SUL

<< skip unrelevant part >>

-- TAG-SERVING-CELL-CONFIG-COMMON-STOP

-- ASN1STOP

The Next Change

SCS-SpecificCarrier ::=			SEQUENCE {

scs-SpecificCarrierId				SCS-SpecificCarrierId,

-- Offset in frequency domain between Point A (lowest subcarrier of common RB 0) and the lowest usable subcarrier on this carrier

-- in number of PRBs (using the subcarrierSpacing defined for this carrier). The maximum value corresponds to 275*8-1.

-- Corresponds to L1 parameter 'offset-pointA-low-scs' (see 38.211, section FFS_Section)

offsetToCarrier				INTEGER (0..2199)		,

-- Subcarrier spacing of this carrier. It is used to convert the offsetToCarrier into an actual frequency.

-- Corresponds to L1 parameter 'ref-scs' (see 38.211, section FFS_Section)

subcarrierSpacing					SubcarrierSpacing																	,

-- FFS_Description (see 38.211, section FFS_Section)

-- Subcarrier spacing for SIB1, Msg.2/4 for initial access and SI-messages.

scs-Common							ENUMERATED {true}						OPTIONAL,	-- Need R

k0									FFS_Value																			OPTIONAL,

-- Width of this carrier in number of PRBs (using the subcarrierSpacing defined for this carrier)

-- Corresponds to L1 parameter 'BW' (see 38.211, section FFS_Section)

carrierBandwidth					INTEGER (1..maxNrofPhysicalResourceBlocks)											,

...

}

–	SCS-SpecificCarrierId

The SCS-SpecificCarrierId identifies a set of SCS specific parameters defined in SCS-SpecificCarrier.

-- ASN1START

-- TAG-SCS-SPECIFICCARRIERID-START

SCS-SpecificCarrierId ::=				INTEGER (0..FFS_Value)

-- TAG-SCS-SPECIFICCARRIERID-STOP

-- ASN1STOP

End of Changes

Annex A:	ASN.1 structure from SevingCellConfig to BWP