3GPP TSG-RAN WG2 #102	R2-1807051

Busan, S. Korea, 21st – 25th May 2018

Agenda Item:	10.2.6

Source:	Ericsson

Title:	Open issues on SCG measurement results for NE-DC and NGEN-DC

Document for:	Discussion, Decision

# 1	Introduction

In this paper, we will discuss the encoding of the measurement results for the SCG measurement results for NE-DC and NGEN-DC. A text proposal for TS 38.331 is provided also as an example.

# 2	Discussion

In the RAN2 #98 meeting, the following was agreed regarding the configuration of the MCG and SCG measurements for EN-DC:

3	For MCG and SCG, measurements (objects/ID/reportConfigs) can be configured independently by LTE RRC (inter-RAT measurement on NR) and NR RRC (intra-NR measurements on serving and non serving frequencies). (noting that for the objects will be configured consistently as described by agreement 2)

Also, for those procedures where the MN does not need to understand coming from the SN, the RRC Transfer procedure is used to exchange RRC messages between the MN and the UE via the SN (split SRB) and to provide NR measurement reports from the UE to the SN.

According to this, the main reasons to have distributed measurement configurations were different mobility management of LTE and NR and, to some extent, also latency aspects. Thus, supporting both SRB3 for the SN node (NR) and allowing the SN to configure the measurement separately (without involving the MN) can speed up the measurements and measurement configurations for the SN. The SRB3 may allow faster transmission than the corresponding LTE SRB. Also, the backhaul link between MN and SN may be congested which could negatively affect both the measurement reporting and new measurement configurations. However, even if the argument for having lower latency does not hold when we have NE-DC, the other arguments are still valid for the other MR-DC options. For this reason, in [1] we already proposed that the same principle of EN-DC could be applied to NE-DC and NGEN-DC case.

As in EN-DC, it is beneficial also for NE-DC and NGEN-DC that the MN and SN can construct the measurement configuration and receive the measurement reports separately.

Therefore, in order to be aligned with the EN-DC principles and to have a harmonization from the specification point of view, it would make sense also for NE-DC and NGEN-DC to specify for those options the measurements exchange between MN and SN into the inter-node messages in NR specification (i.e., TS 38.331). Thus, we propose:

Measurement results for NE-DC and NGEN-DC are transferred through inter-node RRC messages specified in NR (i.e. 38.331)

However, having a look at the current 3GPP TS 38.331, we can notice that within the CG-ConfigInfo-IEs, the field candidateCellInfoListMN and candidateCellInforListSN refers to an information element (i.e., CandidateCellInfoList) that contain measurements results encoded only in NR format.

CG-ConfigInfo-IEs ::=		SEQUENCE {

ue-CapabilityInfo			OCTET STRING (CONTAINING UE-CapabilityRAT-ContainerList)		OPTIONAL,-- Cond SN-Addition

candidateCellInfoListMN			CandidateCellInfoList								OPTIONAL,

candidateCellInfoListSN			OCTET STRING (CONTAINING CandidateCellInfoList)			OPTIONAL,

measResultCellListSFTD			MeasResultCellListSFTD							OPTIONAL,

scgFailureInfo					SEQUENCE {

failureType						ENUMERATED { t310-Expiry, randomAccessProblem,

rlc-MaxNumRetx, scg-ChangeFailure,

scg-reconfigFailure,

srb3-IntegrityFailure},

measResultSCG					OCTET STRING (CONTAINING MeasResultSCG-Failure)

}																		OPTIONAL,

configRestrictInfo			ConfigRestrictInfoSCG									OPTIONAL,

drx-InfoMCG					DRX-Info									OPTIONAL,

measConfigMN				MeasConfigMN								OPTIONAL,

sourceConfigSCG				OCTET STRING (CONTAINING RRCReconfiguration)			OPTIONAL,

scg-RB-Config             	OCTET STRING (CONTAINING RadioBearerConfig)        				OPTIONAL,

mcg-RB-Config				OCTET STRING (CONTAINING RadioBearerConfig)				OPTIONAL,

nonCriticalExtension		SEQUENCE {}										OPTIONAL

}

Even if this still hold for the case of NGEN-DC, when considering NE-DC it would make sense to have measurements results coming from the SCG encoded in LTE format (since in this case the SN is an LTE RAT). Thus, we propose:

In NGEN-DC, measurements results coming from the SCG are encoded in NR format, as in EN-DC.

In NE-DC, the measurements results coming from the SCG are encoded in LTE format.

# Conclusion

In the previous sections we made the following observations:

Observation 1	As in EN-DC, it is beneficial also for NE-DC and NGEN-DC that the MN and SN can construct the measurement configuration and receive the measurement reports separately.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Measurement results for NE-DC and NGEN-DC are transferred through inter-node RRC messages specified in NR (i.e. 38.331)

Proposal 2	In NGEN-DC, measurements results coming from the SCG are encoded in NR format, as in EN-DC.

Proposal 3	In NE-DC, the measurements results coming from the SCG are encoded in LTE format.

# References

R2-1805451, NE-DC measurement principles, Ericsson, RAN2 #101bis, April 2018.

# Appendix (Example for 38.331)

### 10.2.2	Message definitions

#### –	CG-ConfigEUTRA

This message is used to transfer the SCG radio configuration as generated by the SeNB.

Direction: Secondary eNB to master gNB or eNB.

CG-ConfigEUTRA message

-- ASN1START

-- TAG-CG-CONFIG-EUTRA-START

CG-ConfigEUTRA ::=					SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE{

cg-ConfigEUTRA					CG-ConfigEUTRA-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

CG-ConfigEUTRA-IEs ::=			SEQUENCE {

cg-CellGroupConfig					OCTET STRING (CONTAINING RRCConnectionReconfiguration-v15x0-IEs)			OPTIONAL,

cg-RB-Config						OCTET STRING (CONTAINING RadioBearerConfig)									OPTIONAL,

configRestrictModReq				ConfigRestrictModReqSCG														OPTIONAL,

drx-InfoSCG							DRX-Info																	OPTIONAL,

candidateCellInfoListSN-EUTRA		OCTET STRING (CONTAINING CandidateCellInfoList-r10)							OPTIONAL,

measConfigSN-EUTRA					MeasConfigSN-EUTRA															OPTIONAL,

selectedBandCombinationNR			BandCombinationIndex														OPTIONAL,

nonCriticalExtension				SEQUENCE {}																	OPTIONAL

}

MeasConfigSN-EUTRA ::= SEQUENCE {

measuredFrequencies				SEQUENCE (SIZE (1..maxMeasFreqsMN))	OF EUTRA-FreqInfo		OPTIONAL,

...

}

EUTRA-FreqInfo ::= SEQUENCE {

measuredFrequency 					ARFCN-ValueEUTRA											OPTIONAL,

...

}

BandCombinationIndex ::= INTEGER (1..maxBandComb)

-- TAG-CG-CONFIG-EUTRA-STOP

-- ASN1STOP

#### –	CG-ConfigInfo

This message is used by master eNB or gNB to request the SgNB to perform certain actions e.g. to establish, modify or release an SCG. The message may include additional information e.g. to assist the SgNB to set the SCG configuration.It can also be used by a CU to request a DU to perform certain actions, e.g. to establish, modify or release an MCG or SCG.

Direction: Master eNB or gNB to secondary gNB, alternatively CU to DU.

CG-ConfigInfo message

-- ASN1START

-- TAG-CG-CONFIG-INFO-START

CG-ConfigInfo ::=				SEQUENCE {

criticalExtensions				CHOICE {

c1								CHOICE{

cg-ConfigInfo				CG-ConfigInfo-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture		SEQUENCE {}

}

}

CG-ConfigInfo-IEs ::=		SEQUENCE {

ue-CapabilityInfo			OCTET STRING (CONTAINING UE-CapabilityRAT-ContainerList)		OPTIONAL,-- Cond SN-Addition

candidateCellInfoListMN			CandidateCellInfoList									OPTIONAL,

candidateCellInfoListSN			OCTET STRING (CONTAINING CandidateCellInfoList)			OPTIONAL,

lte-candidateCellInfoListMN		CandidateCellInfoList-r10								OPTIONAL,

lte-candidateCellInfoListSN		OCTET STRING (CONTAINING CandidateCellInfoList-r10)		OPTIONAL,

measResultCellListSFTD			MeasResultCellListSFTD									OPTIONAL,

scgFailureInfo					SEQUENCE {

failureType						ENUMERATED { t310-Expiry, randomAccessProblem,

rlc-MaxNumRetx, scg-ChangeFailure,

scg-reconfigFailure,

srb3-IntegrityFailure},

measResultSCG					OCTET STRING (CONTAINING MeasResultSCG-Failure)

}																						OPTIONAL,

configRestrictInfo			ConfigRestrictInfoSCG										OPTIONAL,

drx-InfoMCG					DRX-Info													OPTIONAL,

measConfigMN				MeasConfigMN												OPTIONAL,

sourceConfigSCG				OCTET STRING (CONTAINING RRCReconfiguration)				OPTIONAL,

scg-RB-Config             	OCTET STRING (CONTAINING RadioBearerConfig)        			OPTIONAL,

mcg-RB-Config				OCTET STRING (CONTAINING RadioBearerConfig)					OPTIONAL,

nonCriticalExtension		SEQUENCE {}													OPTIONAL

}

ConfigRestrictInfoSCG ::=		SEQUENCE {

allowedBC-ListMRDC				BandCombinationIndexList								OPTIONAL,

allowedBPC-ListMRDC				BPC-IndexList											OPTIONAL,

powerCoordination-FR1				SEQUENCE {

p-maxNR							P-Max												OPTIONAL,

p-maxEUTRA						P-Max												OPTIONAL

}																						OPTIONAL,

servCellIndexRangeSCG			SEQUENCE {

lowBound						ServCellIndex,

upBound							ServCellIndex

}																						OPTIONAL,   -- Cond SN-Addition

maxMeasFreqsSCG-NR					INTEGER(1..maxMeasFreqsMN)								OPTIONAL,

...

}

BandCombinationIndexList ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombinationIndex

BPC-IndexList ::= SEQUENCE (SIZE (1..maxBasebandProcComb)) OF BPC-Index

BPC-Index ::= INTEGER (1..maxBasebandProcComb)

DRX-Info ::=					SEQUENCE {

drx-LongCycleStartOffset		CHOICE {

ms10							INTEGER(0..9),

ms20							INTEGER(0..19),

ms32							INTEGER(0..31),

ms40							INTEGER(0..39),

ms60							INTEGER(0..59),

ms64							INTEGER(0..63),

ms70							INTEGER(0..69),

ms80							INTEGER(0..79),

ms128							INTEGER(0..127),

ms160							INTEGER(0..159),

ms256							INTEGER(0..255),

ms320							INTEGER(0..319),

ms512							INTEGER(0..511),

ms640							INTEGER(0..639),

ms1024							INTEGER(0..1023),

ms1280							INTEGER(0..1279),

ms2048							INTEGER(0..2047),

ms2560							INTEGER(0..2559),

ms5120							INTEGER(0..5119),

ms10240							INTEGER(0..10239)

},

shortDRX							SEQUENCE {

drx-ShortCycle						ENUMERATED	{

ms2, ms3, ms4, ms5, ms6, ms7, ms8, ms10, ms14, ms16, ms20, ms30, ms32,

ms35, ms40, ms64, ms80, ms128, ms160, ms256, ms320, ms512, ms640, spare9,

spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 },

drx-ShortCycleTimer					INTEGER (1..16)

}																					OPTIONAL

}

MeasConfigMN ::= SEQUENCE {

measuredFrequenciesMN				SEQUENCE (SIZE (1..maxMeasFreqsMN))	OF NR-FreqInfo		OPTIONAL,

measGapConfigFR1					GapConfig												OPTIONAL,

gapPurpose							ENUMERATED {perUE, perFR1}								OPTIONAL,

...

}

-- TAG-CG-CONFIG-INFO-STOP

-- ASN1STOP

CG-ConfigInfo field descriptions

allowedBandCombinationListMRDC A list of indices referring to band combinations in MR-DC capabilities from which SN is allowed to select an NR band combination. Each entry refers to a band combination numbered according to supportedBandCombination in the UE-MRDC-Capability. All MR-DC band combinations indicated by this field comprise the same LTE band combination.

allowedBasebandProcessingCombinationListMRDC Indicates a list of NR BPCs the SN is allowed to configure. Each entry refers to a NR baseband processing combination numbered according to supportedBasebandProcessingCombination in the UE-NR-Capability.

candidateCellInfoListMN, candidateCellInfoListSN Contains information regarding cells that the master node or the source node suggests the target gNB to consider configuring. Including CSI-RS measurement results in candidateCellInfoListMN is not supported in this version of the specification.

lte-candidateCellInfoListMN, lte-candidateCellInfoListSN Contains information regarding cells that the master node or the source node suggests the target eNB to consider configuring.

maxMeasFreqsSCG-NR Indicates the maximum number of allowed NR frequencies SCG should configure for measurements.

mcg-RB-Config Contains the IE RadioBearerConfig of the MN, used to support delta configuration for bearer type change between MN terminated to SN terminated bearer and SN change.

p-maxEUTRA Indicates the maximum power for EUTRA (see TS 36.104 [XX]) the UE can use in LTE MCG.

p-maxNR Indicates the maximum power for NR (see TS 38.104 [12]) the UE can use in NR SCG.

powerCoordination-FR1 Indicates the maximum power that the UE can use in FR1.

scg-RB-Config Contains the IE RadioBearerConfig of the SN, used to support delta configuration e.g. during SN change. This field is absent when master eNB uses full configuration option.

sourceConfigSCG Includes the current dedicated SCG configuration in the same format as the RRCReconfiguration message, i.e. not only CellGroupConfig but also e.g. measConfig. This field is absent when master eNB uses full configuration option.

ConfigRestrictInfo Includes fields for which SgNB is explictly indicated to observe a configuration restriction.

servCellIndexRangeSCG Range of serving cell indices that SN is allowed to configure for SCG serving cells.

| Conditional Presence | Explanation |
| --- | --- |
| SN-Addition | The field is mandatory present upon SN addition. |