3GPP TSG-RAN WG2 Meeting #102	R2-1807887

Busan, South Korea, 21st – 25th May 2018

Agenda item:	9.18.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Text Proposal to 36.300: Flight path information

WID/SID:	LTE_Aerial-Core - Release 15

Document for:	Discussion and Decision

# 1	Introduction

During the 3GPP RAN2#101bis meeting it has been agreed to provide the UAV flight path information via RRC signalling. The following agreement may serve as a reference:

4 The flight path information provided from UE to eNB through RRC is supported. The trigger condition and content of the information is FFS.

The details have been discussed via [101bis#57][LTE/UAV] Flight path information e-mail thread. This paper provides the text proposal, which can be easily converted into the regular CR to TS 36.300, introducing flight path information. It is based on the CR 1139, endorsed in R2-1806527.

# Annex A: Text Proposal to TS 36.300

CHANGE START

### 23.X.5	Flight path information

The UE with Aerial subscription can be configured to provide the eNB the flight path information. Flight path information may include: 3D heading direction, the geographical coordinates of the destination, the geographical coordinates and associated time of visit for multiple waypoints between the take-off location and the destination location.

Flight path information can be provided to the target eNB during the Handover Preparation.

END OF CHANGES