3GPP TSG-RAN2 102                                                                                      R2-1808830

Busan, South Korea, May 21 – May 25, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 0101 | rev | 1 | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on BWP Switching Upon Initiation of Random Access Procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-05-24 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For RA procedure initiated on SCell (e.g. SCell X), RAR is received on SpCell. So, - If RA resources are not configured for the active UL BWP of SCell X, UE needs to switch the UL BWP to initial UL BWP of SCell X. There is no need to switch the DL BWP of SCell X. - If RA resources are configured for the active UL BWP of SCell X, the switching of DL BWP of SCell X based on linkage between UL BWP and DL BWP of SCell X is not needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | If RA resources are not configured in active UL BWP, switching to initial DL BWP is applied only for RA procedure initiated on SpCell. If RA resources are configured in active UL BWP, switching to DL BWP linked to UL BWP is applied only for RA procedure initiated on SpCell. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CA aspects are not specified for BWP switching upon initiation of RA procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.15 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | None |  |  |  |  |  |  |  |  |

<Start of modification>

## 5.15	Bandwidth Part (BWP) operation

In addition to clause 12 of TS 38.213 [6], this subclause specifies requirements on BWP operation.

A Serving Cell may be configured with one or multiple BWPs, and the maximum number of BWP per Serving Cell is specified in TS 38.213 [6].

The BWP switching for a Serving Cell is used to activate an inactive BWP and deactivate an active BWP at a time. The BWP switching is controlled by the PDCCH indicating a downlink assignment or an uplink grant, by the bandwidthPartInactivityTimer, or by the MAC entity itself upon initiation of Random Access procedure. Upon addition of SpCell or activation of an SCell, one BWP is initially active without receiving PDCCH indicating a downlink assignment or an uplink grant. The active BWP for a Serving Cell is indicated by either RRC or PDCCH (as specified in TS 38.213 [6]). For unpaired spectrum, a DL BWP is paired with a UL BWP, and BWP switching is common for both UL and DL.

On the active BWP for each activated Serving Cell configured with a BWP, the MAC entity shall apply normal operations including:

1>	transmit on UL-SCH;

1>	transmit on RACH;

1>	monitor the PDCCH;

1>	transmit PUCCH;

1>	transmit SRS;

1>	receive DL-SCH;

1>	(re-)initialize any suspended configured uplink grants of configured grant Type 1 according to the stored configuration, if any, and to start in the symbol according to rules in subclause 5.8.2.

On the inactive BWP for each activated Serving Cell configured with a BWP, the MAC entity shall:

1>	not transmit on UL-SCH;

1>	not transmit on RACH;

1>	not monitor the PDCCH;

1>	not transmit PUCCH;

1>	not transmit SRS;

1>	not receive DL-SCH;

1>	clear any configured downlink assignment and configured uplink grant of configured grant Type 2;

1>	suspend any configured uplink grant of configured Type 1.

Upon initiation of the Random Access procedure on a Serving Cell, the MAC entity shall for this Serving Cell:

1>	if PRACH occasions are

not configured for the active UL BWP:

2>	switch the active UL BWP to BWP indicated by initialUplinkBWP;

2>	if the Serving Cell is a SpCell:

3>switch the active DL BWP to BWP indicated by initialDownlinkBWP;

1> else:

2>	if the Serving Cell is a SpCell:

3> if the active DL BWP does not have the same bwp-Id as the active UL BWP:

4> switch the active DL BWP to the DL BWP with the same bwp-Id as the active UL BWP;

1>	perform the Random Access procedure on the active DL BWP of SpCell and active UL BWP of this Serving Cell .

If the MAC entity receives a PDCCH for BWP switching of a serving cell, the MAC entity shall:

1>	if there is no ongoing Random Access procedure associated with this Serving Cell; or

1>	if the ongoing Random Access procedure associated with this Serving Cell is successfully completed upon reception of this PDCCH addressed to C-RNTI (as specified in subclauses 5.1.4 and 5.1.5):

2>	perform BWP switching to a BWP indicated by the PDCCH.

If the MAC entity receives a PDCCH for BWP switching while a Random Access procedure is ongoing in the MAC entity, it is up to UE implementation whether to switch BWP or ignore the PDCCH for BWP switching, except for the PDCCH reception for BWP switching addressed to the C-RNTI for successful Random Access procedure completion (as specified in subclauses 5.1.4 and 5.1.5) in which case the UE shall perform BWP switching to a BWP indicated by the PDCCH. Upon reception of the PDCCH for BWP switching other than successful contention resolution, if the MAC entity decides to perform BWP switching, the MAC entity shall stop the ongoing Random Access procedure and initiate a Random Access procedure on the new activated BWP; if the MAC decides to ignore the PDCCH for BWP switching, the MAC entity shall continue with the ongoing Random Access procedure on the active BWP.

If the bwp-InactivityTimer is configured, the MAC entity shall for each activated Serving Cell:

1>	if the default-DL-BWP is configured, and the active DL BWP is not the BWP indicated by the default-DL-BWP; or

1>	if the default-DL-BWP is not configured, and the active DL BWP is not the initial BWP:

2>	if a PDCCH addressed to C-RNTI or CS-RNTI indicating downlink assignment or uplink grant is received on the active BWP; or

2>	if a MAC PDU is transmitted in a configured uplink grant or received in a configured downlink assignment:

3>	if there is no ongoing random access procedure associated with this Serving Cell; or

3>	if the ongoing Random Access procedure associated with this Serving Cell is successfully completed upon reception of this PDCCH addressed to C-RNTI (as specified in subclauses 5.1.4 and 5.1.5):

4>	start or restart the bandwidthPartInactivityTimer associated with the active DL BWP.

2>	if a PDCCH for BWP switching is received on the active DL BWP, and the MAC entity switches the active BWP:

3>	start or restart the bwp-InactivityTimer associated with the active DL BWP.

2>	if Random Access procedure is initiated on this Serving Cell:

3>	stop the bwp-InactivityTimer associated with the active DL BWP of this Serving Cell.

3>	if the Serving Cell is SCell (other than PSCell):

4>	stop the bwp-InactivityTimer associated with the active DL BWP of SpCell, if running.

2>	if the bwp-InactivityTimer associated with the active DL BWP expires:

3>	if the default-DL-BWP is configured:

4>	perform BWP switching to a BWP indicated by the default-DL-BWP.

3>	else:

4>	perform BWP switching to the initial DL BWP.

<End of modification>