- 3GPP TSG RAN WG2 Meeting #102              			             	 				    R2-1808877
Busan, Republic of Korea, May 21 – 25, 2018

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | 3392 | rev | 1 | Current version: | 13.9.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Handling of Pmax for PC2 and uplink intra-band contiguous CA capable UEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KDDI, Ericsson, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13 |  |  |  |  |  |  | Date: |  | 2018-05-25 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | In RAN4#86, RAN4 agreed R4-1802502(36.101CR-4942), which specifies that in case PC2 and uplink intra-band contiguous CA capable UE, if UE receives Pmax in SCell, then that applies both to Scell and Pcell, instead of using Pmax in Pcell. However this UE behavior is not clearly captured in the current TS36.331, so clarification is needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The uplink intra-band contiguous CA capable UE behavior in PC2 operation is specified as follows. - If Pmax is provided in RadioResourceConfigCommonSCell by the NW in dedicated signaling, then UE applies given Pmax to both Scell and Pcell. Impact Analysis Impacted functionality: Maximum transmission power determination Inter-operability: - If the network is implemented according to the CR and the UE is not, the HPUEs in uplink intra-band contiguous CA operation would transmit with larger UL power than intended - If the UE is implemented according to the CR and the network is not, the UE in uplink intra-band contiguous CA operation will be using lower (default) power than what the NW intends the UE to use. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Inconsistency between RAN4 and RAN2 specification would happen. - Operators can not control UL power of High power class UEs (HPUEs) in uplink intra-band contiguous CA operation |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Beginning of changes

### 6.3.2	Radio resource control information elements

---------------------------------------------------Unmodified parts omitted--------------------------------------------------------

#### RadioResourceConfigCommon

The IE RadioResourceConfigCommonSIB and IE RadioResourceConfigCommon are used to specify common radio resource configurations in the system information and in the mobility control information, respectively, e.g., the random access parameters and the static physical layer parameters.

RadioResourceConfigCommon information element

-- ASN1START

RadioResourceConfigCommonSIB ::=	SEQUENCE {

rach-ConfigCommon			RACH-ConfigCommon,

bcch-Config			BCCH-Config,

pcch-Config			PCCH-Config,

prach-Config			PRACH-ConfigSIB,

pdsch-ConfigCommon			PDSCH-ConfigCommon,

pusch-ConfigCommon			PUSCH-ConfigCommon,

pucch-ConfigCommon			PUCCH-ConfigCommon,

soundingRS-UL-ConfigCommon		SoundingRS-UL-ConfigCommon,

uplinkPowerControlCommon		UplinkPowerControlCommon,

ul-CyclicPrefixLength		UL-CyclicPrefixLength,

...,

[[	uplinkPowerControlCommon-v1020	UplinkPowerControlCommon-v1020	OPTIONAL	-- Need OR

]],

[[	rach-ConfigCommon-v1250		RACH-ConfigCommon-v1250		OPTIONAL	-- Need OR

]],

[[	pusch-ConfigCommon-v1270	PUSCH-ConfigCommon-v1270		OPTIONAL	-- Need OR

]],

[[	bcch-Config-v1310		BCCH-Config-v1310			OPTIONAL,	-- Need OR

pcch-Config-v1310		PCCH-Config-v1310			OPTIONAL,	-- Need OR

freqHoppingParameters-r13	FreqHoppingParameters-r13		OPTIONAL,	-- Need OR

pdsch-ConfigCommon-v1310	PDSCH-ConfigCommon-v1310		OPTIONAL,	-- Need OR

pusch-ConfigCommon-v1310	PUSCH-ConfigCommon-v1310		OPTIONAL,	-- Need OR

prach-ConfigCommon-v1310	PRACH-ConfigSIB-v1310		OPTIONAL,	-- Need OR

pucch-ConfigCommon-v1310	PUCCH-ConfigCommon-v1310		OPTIONAL	-- Need OR

]]

}

RadioResourceConfigCommon ::=	SEQUENCE {

rach-ConfigCommon			RACH-ConfigCommon			OPTIONAL,	-- Need ON

prach-Config			PRACH-Config,

pdsch-ConfigCommon			PDSCH-ConfigCommon			OPTIONAL,	-- Need ON

pusch-ConfigCommon			PUSCH-ConfigCommon,

phich-Config			PHICH-Config			OPTIONAL,	-- Need ON

pucch-ConfigCommon			PUCCH-ConfigCommon			OPTIONAL,	-- Need ON

soundingRS-UL-ConfigCommon		SoundingRS-UL-ConfigCommon		OPTIONAL,	-- Need ON

uplinkPowerControlCommon		UplinkPowerControlCommon		OPTIONAL,	-- Need ON

antennaInfoCommon			AntennaInfoCommon			OPTIONAL,	-- Need ON

p-Max				P-Max				OPTIONAL,	-- Need OP

tdd-Config				TDD-Config				OPTIONAL,	-- Cond TDD

ul-CyclicPrefixLength		UL-CyclicPrefixLength,

...,

[[	uplinkPowerControlCommon-v1020	UplinkPowerControlCommon-v1020	OPTIONAL	-- Need ON

]],

[[	tdd-Config-v1130		TDD-Config-v1130			OPTIONAL	-- Cond TDD3

]],

[[	pusch-ConfigCommon-v1270	PUSCH-ConfigCommon-v1270		OPTIONAL	-- Need OR

]],

[[

prach-Config-v1310		PRACH-Config-v1310			OPTIONAL,	-- Need ON

freqHoppingParameters-r13	FreqHoppingParameters-r13		OPTIONAL,	-- Need ON

pdsch-ConfigCommon-v1310	PDSCH-ConfigCommon-v1310		OPTIONAL,	-- Need ON

pucch-ConfigCommon-v1310	PUCCH-ConfigCommon-v1310		OPTIONAL,	-- Need ON

pusch-ConfigCommon-v1310	PUSCH-ConfigCommon-v1310		OPTIONAL,	-- Need ON

uplinkPowerControlCommon-v1310	UplinkPowerControlCommon-v1310	OPTIONAL	-- Need ON

]]

}

RadioResourceConfigCommonPSCell-r12 ::=	SEQUENCE {

basicFields-r12			RadioResourceConfigCommonSCell-r10,

pucch-ConfigCommon-r12		PUCCH-ConfigCommon,

rach-ConfigCommon-r12		RACH-ConfigCommon,

uplinkPowerControlCommonPSCell-r12	UplinkPowerControlCommonPSCell-r12,

...,

[[	uplinkPowerControlCommonPSCell-v1310

UplinkPowerControlCommon-v1310	OPTIONAL	-- Need ON

]]

}

RadioResourceConfigCommonPSCell-v12f0 ::=	SEQUENCE {

basicFields-v12f0			RadioResourceConfigCommonSCell-v10l0

}

RadioResourceConfigCommonSCell-r10 ::=	SEQUENCE {

-- DL configuration as well as configuration applicable for DL and UL

nonUL-Configuration-r10			SEQUENCE {

-- 1: Cell characteristics

dl-Bandwidth-r10			ENUMERATED {n6, n15, n25, n50, n75, n100},

-- 2: Physical configuration, general

antennaInfoCommon-r10			AntennaInfoCommon,

mbsfn-SubframeConfigList-r10		MBSFN-SubframeConfigList	OPTIONAL,	-- Need OR

-- 3: Physical configuration, control

phich-Config-r10			PHICH-Config,

-- 4: Physical configuration, physical channels

pdsch-ConfigCommon-r10			PDSCH-ConfigCommon,

tdd-Config-r10				TDD-Config			OPTIONAL	-- Cond TDDSCell

},

-- UL configuration

ul-Configuration-r10		SEQUENCE {

ul-FreqInfo-r10			SEQUENCE {

ul-CarrierFreq-r10			ARFCN-ValueEUTRA		OPTIONAL,	-- Need OP

ul-Bandwidth-r10			ENUMERATED {n6, n15,

n25, n50, n75, n100}	OPTIONAL,	-- Need OP

additionalSpectrumEmissionSCell-r10	AdditionalSpectrumEmission

},

p-Max-r10				P-Max				OPTIONAL,	-- Need OP

uplinkPowerControlCommonSCell-r10	UplinkPowerControlCommonSCell-r10,

-- A special version of IE UplinkPowerControlCommon may be introduced

-- 3: Physical configuration, control

soundingRS-UL-ConfigCommon-r10	SoundingRS-UL-ConfigCommon,

ul-CyclicPrefixLength-r10		UL-CyclicPrefixLength,

-- 4: Physical configuration, physical channels

prach-ConfigSCell-r10			PRACH-ConfigSCell-r10	OPTIONAL,	-- Cond TDD-OR-NoR11

pusch-ConfigCommon-r10		PUSCH-ConfigCommon

}									OPTIONAL,	-- Need OR

...,

[[	ul-CarrierFreq-v1090		ARFCN-ValueEUTRA-v9e0		OPTIONAL	-- Need OP

]],

[[	rach-ConfigCommonSCell-r11		RACH-ConfigCommonSCell-r11	OPTIONAL,	-- Cond ULSCell

prach-ConfigSCell-r11		PRACH-Config			OPTIONAL,	-- Cond UL

tdd-Config-v1130			TDD-Config-v1130		OPTIONAL,	-- Cond TDD2

uplinkPowerControlCommonSCell-v1130

UplinkPowerControlCommonSCell-v1130		OPTIONAL	-- Cond UL

]],

[[	pusch-ConfigCommon-v1270	PUSCH-ConfigCommon-v1270		OPTIONAL	-- Need OR

]],

[[	pucch-ConfigCommon-r13		PUCCH-ConfigCommon	OPTIONAL,	-- Cond UL

uplinkPowerControlCommonSCell-v1310

UplinkPowerControlCommonSCell-v1310	OPTIONAL	-- Cond UL

]]

}

RadioResourceConfigCommonSCell-v10l0 ::=	SEQUENCE {

-- UL configuration

ul-Configuration-v10l0		SEQUENCE {

additionalSpectrumEmissionSCell-v10l0	AdditionalSpectrumEmission-v10l0

}

}

BCCH-Config ::=			SEQUENCE {

modificationPeriodCoeff		ENUMERATED {n2, n4, n8, n16}

}

BCCH-Config-v1310 ::=		SEQUENCE {

modificationPeriodCoeff-v1310	ENUMERATED {n64}

}

FreqHoppingParameters-r13 ::=	SEQUENCE {

dummy		ENUMERATED {nb2, nb4}		OPTIONAL,

dummy2		CHOICE {

interval-FDD-r13		ENUMERATED {int1, int2, int4, int8},

interval-TDD-r13		ENUMERATED {int1, int5, int10, int20}

}								OPTIONAL,

dummy3		CHOICE {

interval-FDD-r13		ENUMERATED {int2, int4, int8, int16},

interval-TDD-r13		ENUMERATED { int5, int10, int20, int40}

}											OPTIONAL,

interval-ULHoppingConfigCommonModeA-r13	CHOICE {

interval-FDD-r13		ENUMERATED {int1, int2, int4, int8},

interval-TDD-r13		ENUMERATED {int1, int5, int10, int20}

}								OPTIONAL,	-- Cond MP-A

interval-ULHoppingConfigCommonModeB-r13	CHOICE {

interval-FDD-r13		ENUMERATED {int2, int4, int8, int16},

interval-TDD-r13		ENUMERATED { int5, int10, int20, int40}

}								OPTIONAL,	-- Cond MP-B

dummy4		INTEGER (1..maxAvailNarrowBands-r13)

OPTIONAL

}

PCCH-Config ::=			SEQUENCE {

defaultPagingCycle			ENUMERATED {

rf32, rf64, rf128, rf256},

nB					ENUMERATED {

fourT, twoT, oneT, halfT, quarterT, oneEighthT,

oneSixteenthT, oneThirtySecondT}

}

PCCH-Config-v1310 ::=		SEQUENCE {

paging-narrowBands-r13		INTEGER (1..maxAvailNarrowBands-r13),

mpdcch-NumRepetition-Paging-r13	ENUMERATED {r1, r2, r4, r8, r16, r32, r64, r128, r256},

nB-v1310				ENUMERATED {one64thT, one128thT, one256thT}

OPTIONAL	-- Need OR

}

UL-CyclicPrefixLength ::=		ENUMERATED {len1, len2}

-- ASN1STOP

RadioResourceConfigCommon field descriptions

additionalSpectrumEmissionSCell The UE requirements related to additionalSpectrumEmissionSCell are defined in TS 36.101 [42]. E-UTRAN configures the same value in additionalSpectrumEmissionSCell for all SCell(s) of the same band with UL configured. The additionalSpectrumEmissionSCell is applicable for all serving cells (including PCell) of the same band with UL configured.

defaultPagingCycle Default paging cycle, used to derive 'T' in TS 36.304 [4]. Value rf32 corresponds to 32 radio frames, rf64 corresponds to 64 radio frames and so on.

dummy This field is not used in the specification. If received it shall be ignored by the UE.

interval-DLHoppingConfigCommonModeX Number of consecutive absolute subframes over which MPDCCH or PDSCH for CE mode X stays at the same narrowband before hopping to another narrowband. For interval-FDD, int1 corresponds to 1 subframe, int2 corresponds to 2 subframes, and so on. For interval-TDD, int1 corresponds to 1 subframe, int5 corresponds to 5 subframes, and so on.

interval-ULHoppingConfigCommonModeX Number of consecutive absolute subframes over which PUCCH or PUSCH for CE mode X stays at the same narrowband before hopping to another narrowband. For interval-FDD, int1 corresponds to 1 subframe, int2 corresponds to 2 subframes, and so on. For interval-TDD, int1 corresponds to 1 subframe, int5 corresponds to 5 subframes, and so on.

modificationPeriodCoeff Actual modification period, expressed in number of radio frames= modificationPeriodCoeff * defaultPagingCycle. n2 corresponds to value 2, n4 corresponds to value 4, n8 corresponds to value 8, n16 corresponds to value 16, and n64 corresponds to value 64.

mpdcch-NumRepetition-Paging Maximum number of repetitions for MPDCCH common search space (CSS) for paging, see TS 36.211 [21].

mpdcch-pdsch-HoppingOffset Parameter: , see TS 36.211 [21, 6.4.1].

mpdcch-pdsch-HoppingNB The number of narrowbands for MPDCCH/PDSCH frequency hopping. Value nb2 corresponds to 2 narrowbands and value nb4 corresponds to 4 narrowbands.

nB Parameter: nB is used as one of parameters to derive the Paging Frame and Paging Occasion according to TS 36.304 [4]. Value in multiples of 'T' as defined in TS 36.304 [4]. A value of fourT corresponds to 4 * T, a value of twoT corresponds to 2 * T and so on. In case nB-v1310 is signalled, the UE shall ignore nB (i.e. without suffix). EUTRAN configures nB-v1310 only in the BR version of SI message.

paging-narrowBands Number of narrowbands used for paging, see TS 36.304 [4], TS 36.212 [22] and TS 36.213 [23].

p-Max Pmax to be used in the target cell. If absent the UE applies the maximum power of the default power class for the band used in the target cell, defined according to 36.101 [42]. In case the UE is configured with uplink intra-band contiguous CA and the UE indicates ue-CA-PowerClass-N in that band combination, then the p-Max in RadioResourceConfigCommonSCell for that SCell, if present, also applies for that band combination whenever that SCell is activated.

ul-Bandwidth Parameter: transmission bandwidth configuration, NRB, in uplink, see TS 36.101 [42, table 5.6-1]. Value n6 corresponds to 6 resource blocks, n15 to 15 resource blocks and so on. If for FDD this parameter is absent, the uplink bandwidth is equal to the downlink bandwidth. For TDD this parameter is absent and it is equal to the downlink bandwidth.

ul-CarrierFreq For FDD: If absent, the (default) value determined from the default TX-RX frequency separation defined in TS 36.101 [42, table 5.7.3-1] applies. For TDD: This parameter is absent and it is equal to the downlink frequency.

ul-CyclicPrefixLength Parameter: Uplink cyclic prefix length see TS 36.211 [21, 5.2.1] where len1 corresponds to normal cyclic prefix and len2 corresponds to extended cyclic prefix.

End of changes