3GPP TSG-RAN WG2 Meeting #105	R2-1900634

Athens, Greece, 25 Feb - 01 Mar 2019

Agenda item:	11.7.2.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Analysis of synchronization solutions proposed by SA2

WID/SID:	FS_NR_IIOT - Release 16

Document for:	Discussion and Decision

# 1	Introduction

This contribution analyses TSN synchronization solutions proposed by SA2 in TR23.374 [1] against RAN2 impacts as requested by SA2 in [2]. At first, in section 2, requirements are interpreted to identify key issues to be considered when analysing proposals in the second section. These interpretations may not be easily measurable but are presented to further illustrate given requirements.

# 2	Interpretation of requirements

This section brings up several points which stem from the requirements captured in paragraphs 5.6 and 6.2 of TS 22.104 [3] and other aspects which need to be considered when designing an efficient synchronization solution for TSN support over 5G/NR system:

- Both IEEE1588v2 (PTP) and IEEE802.1AS (gPTP) are listed in [3], where the first has been understood as a functional requirement for 5GS. These two require different implementations and may have also different impact to 5GRAN depending on the solution. Intended usage of these should be further clarified by SA2, e.g. is either one of those selected. Note that fronthaul transport (IEE802.1CM-2018) is based on IEEE1588v2. This analysis assumes that support for only IEEE1588v2 is required for functional point of view.

- Working clock grandmasters are typically inexpensive and in typical installations are prone to extreme industrial environmental conditions, e.g. fast temperature changes, vibration and EMC issues, e.g., welding machines.  According to related study [7] and whitepaper [8] working clock performance can be characterized with frequency drifts up to 3ppm/s and 5ppm/min and a frequency error up to 100ppm.  Transferring such unstable clock through 5GS system with 1µs synchronicity may have impact amount of signalling and may have impact to performance, compared to telecom class of clocks. So, robustness of solutions to mitigate these errors found in working clock grandmasters should be included among evaluation criteria.

- It is expected that working clock domain including grand master clock may be also mobile as depicted in Fig D1-1 in [3]. Thus, the master clock may appear also in UE side. This is important aspect to be considered, when wireless operation is one of the main benefits that 5G will be providing for industrial verticals.

- In order to ensure optimal timing for TSN data transfer through 5GRAN e.g., for SPS resources, it would be beneficial that gNB is kept aware of TSN time domain. Thus, there is a trade-off on RAN transparency for external clocks, which should be also considered.

- There are also other possible use cases than TSN, which will need precise time information delivered via Uu.  Thus, synchronization solution, especially for global time, should not rely on coexistence of TSN type service.

- Reliability and availability aspects of precise time should be considered, e.g. by support for independent and redundant time domains.

- Grand master(s) used by 5GRAN may locate independently of UPF, thus RAN and UPF may be considered as individual entities for PTP synchronization point of view. This may enable shorter paths with less hops from GM, i.e. better accuracy.

# 3	Analysis

There are 7 solution options documented in the TR, which are shortly summarized below:

Solution #11 Option 1: Transport of 802.1AS messages over the 5G system to convey timing to the UE.

This solution option has the challenge that 802.1AS messages must be transported via the 5G system with predictable latency (as is done via wired 802.1AS compliant nodes currently). This may be difficult for wireless systems due to the variability of latency over a wireless link.

Observation 1: Solution #11 Option 1 requires packet jitter control in RAN, which has a big impact on scheduler and would require significant amount of work in RAN.

Solution #11 Option 2:  5G RAN conveying timing to the UE that acts as boundary master clocks towards connected TSN device via 5G specific signalling via 5G broadcast/5G unicast RRC. The 5G RAN indicates time associated with a specific point e.g. start of frame boundary. UE acts as a boundary master clock for each clock domain towards N60 and may transmit PTP Sync messages with configured period. In case of multiple clock domains, overhead of option scales linearly in the total number of clock domains. However, as shown in [4], the current SI message size limits are sufficient to deliver timing information for 33 clock domains as currently required. gNB may also be able to choose between broadcast and unicast timing information delivery depending on the number of UEs requiring a specific clock domain information to increase radio resources efficiency. Unicast signalling may also be used for clocks which require more frequent time reference information delivery (e.g. for poor TSN GM quality clocks). This solution also requires gNB to be (g)PTP capable, but that is an implementation option, has no impact on standards and can also be supported in many cases, e.g. in fronthaul network. Furthermore, as indicated in section 2 and as concluded in [5], knowledge of TSN time is also useful in RAN, e.g. for efficient traffic scheduling.

Observation 2: In Solution #11 Option 2 the overhead of the delivered time information increases ~linearly with the number of signalled domains in case broadcast signalling is used. Existing SI message size limits are sufficient to carry information for 33 clock domains as required by SA1.

Observation 3: Unicast RRC signalling may be used to improve radio resources efficiency, especially when delivering timing information for multiple clock domains or for clocks of poor quality.

Observation 4: Solution #11 Option 2 requires gNB to be (g)PTP capable, but that has not impact on RAN specifications. Knowledge of TSN time is useful for RAN, e.g. for more deterministic scheduling of TSN traffic.

Solution #11 Option 3: This solution is relying on a time-aware relay implemented with 5G blackbox model as described by Solution#8 in [1]. In such an implementation, the 5G Core Network can be kept untouched, therefore there will be minimal impact on the 5G system nodes.  The translator/adaptor function located at the edge of 5G system, can take care all 802.1AS related functions.  For example, the (g)PTP support, time stamping, BMCA can be all implemented in the translator. This solution requires only 5G GM timing to be delivered from the gNB to all the UEs, but additionally (g)PTP packets need to be delivered to the UEs with unicast UP transmissions.

Observation 5: Solution #11 Option 3 requires 5G GM timing to be delivered from gNB to UEs using RRC/SIB signalling and additionally (g)PTP packets need to be delivered to the UEs with unicast UP transmissions.

Solution #11 Option 4: Multiple time domains merged into one domain using 5G clock. In this option a single clock domain is enough and a suitable one could be provided by the 5G system itself (in fact, it normally must operate synchronous with an internationally recognized standard such as GPS). Working clocks grand masters that are expected to be PTP compliant must accept better quality GM, i.e., 5Gclock as per BMCA rules.

Observation 6: Solution #11 Option 4 is an extension of Option 2 where multiple clock domains can be supported by delivering a single clock information (5G GM) from gNB to UEs using RRC/SIB signalling. On the other hand, it requires working clocks in TSN networks to accept 5G GM as a Master clock, which may be an issue, but is outside of RAN2 expertise.

Solution #17: Deterministic Delay QoS Class for Time Synchronization Support of 3GPP Network. Providing deterministic delay in TSN time scale means that both UPF and UE must be first synchronized with TSN GM to mitigate e.g. frequency error between 5G GM and TSN GM. In this solution, depending on the design choices, the RAN may be required to ensure deterministic and sometimes even symmetric delay for packets over Uu interface. This has substantial impact on RAN, especially for scheduling, and feasibility of this solution is currently unclear.

Observation 7: A feasibility of Solution #17 from RAN perspective is unclear. Even if feasible, which cannot be at the moment guaranteed, the impacts on RAN are deemed very high.

Solution #19: Time Synchronization between UE and TSN. Solution exploits GTP and PDCP protocols for carrying timestamps in N3 and Uu interfaces, respectively. As a pre-condition UE, RAN and UPF needs to be time synchronized with 5G GM. This means that SIB/RRC signalling of fine 5G GM needs to be specified with fine granularity (similarly as in previous solutions), but on top of that GTP and PDCP layer timestamping needs to be introduced. This has an additional complexity and impact on both RAN network and Uu interface. This solution relies also on sending gPTP packets with unicast UP delivery, which even though feasible, will depend on message size, message frequency SINR geometry of the cell as indicated by RAN1 in [6]. Such mechanism is expected to scale worse than those based on broadcast signalling of timing information.

Observation 8: Solution #19, on top of SIB/RRC time information signalling, but introduces additional complexity by requiring time stamps to be added by gNB on PDCP layer in DL direction and by UE on PDCP layer as well as gNB on GTP layer in UL direction to all gPTP packets.

Observation 9: Synchronization solutions based on time information delivery using UP unicast delivery as expected to scale worse than those based on broadcast signalling of timing information.

Solution #28: TSN Time Synchronization Considering Multiple Clock Domains. 5GS acts as transparent clock with independent internal clock achieving common concept of time between UEs and UPF as well as among different UEs. In this way, the one-way measurement and control of the E2E delay are made possible. Based on the internal synchronization, the 5GS transparently pass the external PTP message through and makes proper correction of the PTP header's "correctionField" with the known residence time. Solution #28 can be realized using either introducing new QoS class to achieve a fixed residence time of the packets (similar to Solution #17) or by introducing time stamps for UP packets (similar to Solution #19). This solution will therefore suffer from same drawbacks as mentioned for Solution #17 and Solution #19 in observations 6, 7 and 8.

Observation 10: Observation 6 is applicable to Solution #28 option X.1, Observation 7 is applicable to Solution #28 option X.1 and Observation 8 is applicable to Solution #28 option X.1 and option X.2.

# 4 	Summary

The following observations are made within the document with respect to different synchronization solutions from [1]:

Observation 1: Solution #11 Option 1 requires packet jitter control in RAN, which has a big impact on scheduler and would require significant amount of work in RAN.

Observation 2: In Solution #11 Option 2 the overhead of the delivered time information increases ~linearly with the number of signalled domains in case broadcast signalling is used. Existing SI message size limits are sufficient to carry information for 33 clock domains as required by SA1.

Observation 3: Unicast RRC signalling may be used to improve radio resources efficiency, especially when delivering timing information for multiple clock domains or for clocks of poor quality.

Observation 4: Solution #11 Option 2 requires gNB to be (g)PTP capable, but that has not impact on RAN specifications. Knowledge of TSN time is useful for RAN, e.g. for more deterministic scheduling of TSN traffic.

Observation 5: Solution #11 Option 3 requires 5G GM timing to be delivered from gNB to UEs using RRC/SIB signalling and additionally (g)PTP packets need to be delivered to the UEs with unicast UP transmissions.

Observation 6: Solution #11 Option 4 is an extension of Option 2 where multiple clock domains can be supported by delivering a single clock information (5G GM) from gNB to UEs using RRC/SIB signalling. On the other hand, it requires working clocks in TSN networks to accept 5G GM as a Master clock, which may be an issue, but is outside of RAN2 expertise.

Observation 7: A feasibility of Solution #17 from RAN perspective is unclear. Even if feasible, which cannot be at the moment guaranteed, the impacts on RAN are deemed very high.

Observation 8: Solution #19, on top of SIB/RRC time information signalling, but introduces additional complexity by requiring time stamps to be added by gNB on PDCP layer in DL direction and by UE on PDCP layer as well as gNB on GTP layer in UL direction to all gPTP packets.

Observation 9: Synchronization solutions based on time information delivery using UP unicast delivery as expected to scale worse than those based on broadcast signalling of timing information.

Observation 10: Observation 6 is applicable to Solution #28 option X.1, Observation 7 is applicable to Solution #28 option X.1 and Observation 8 is applicable to Solution #28 option X.1 and option X.2.

Table 1 summarizes the impacts of the solutions from RAN perspective.

Table 1. Summary of feasibility and impacts of different synchronization solutions from RAN perspective

|  | Main characteristic | Feasibility | RAN impact |
| --- | --- | --- | --- |
| Solution #11 Option 1 | Relies on transport of 802.1AS messages over the 5G system to convey timing to the UE | Unclear | - Requires packet jitter control in RAN - Significant impact on RAN scheduler - gNB is not aware of working clock |
| Solution #11 Option 2 | Relies on broadcast/unicast RRC signalling of time information. Multiple domains are supported by adding additional clock information in RRC signalling | Feasible | - Requires extension of SIB9 - can support 33 clock domains as per SA1 requirement, but overhead increases with the number of signaled domains. - requires PTP support in gNB, (but it may already exist as part of fronthaul network) |
| Solution #11 Option 3 | Relies on broadcast/unicast RRC signalling of time information and additional per UE (g)PTP messages delivery via UP unicast. | Feasible | - Requires extension of SIB9 - can support 33 clock domains as per SA1 requirement - Worse scalability on Uu interface is expected compared to solutions relying on broadcast signalling of time information. - gNB is not aware of working clock. |
| Solution #11 Option 4 | Relies on broadcast/unicast RRC signalling of time information. Only a single clock domain needs to be provided with RRC signalling, even for multiple clock domains support | Feasible | - Requires extension of SIB9 - Can merge several clock domains without additional overhead on Uu interface or additional complexity in RAN. - gNB is not aware of working clock |
| Solution #17 | Relies on deterministic delay QoS Class for time synchronization. | Unclear | - Requires extension of SIB9 - Additionally, requires RAN capability to ensure deterministic delay, feasibility of which is unclear - Even if feasible, high impact on the RAN scheduler is expected - Worse scalability on Uu interface is expected compared to solutions relying on broadcast signalling of time information. - gNB is not aware of working clock. |
| Solution #19 | Solution exploits GTP and PDCP protocols for carrying timestamps in N3 and Uu interfaces, respectively | Feasible | - Requires extension of SIB9 - Requires addition of time stamping capability to PDCP and GTP layers - Worse scalability on Uu interface is expected compared to solutions relying on broadcast signalling of time information. - gNB is not aware of working clock. |
| Solution #28 option X.1 | Relies on deterministic delay QoS Class for time synchronization. | Unclear | Same as for Solution #17 |
| Solution #28 option X.2 | Relies on introducing time stamps for UP packets. | Not clear how time stamping is realized | Same as for Solution #19 |

Based on the analysis it is proposed to indicate to SA2 that from RAN2 perspective Solution #11 Option 2 and Solution #11 Option 4 are preferred while feasibility of Solution #11 Option 1, Solution #17 and Solution #28 is unclear from RAN perspective. Even though Solution #11 Option 4 seems to have least impact to RAN, it should be noted that there are use cases with specific wide area time domains which would not be applicable for Solution #11 Option 4.

Proposal 1: RAN2 should indicate to SA2 that Solution #11 Option 2 and Solution #11 Option 4 are preferred from RAN2 perspective.

Proposal 2: Include Table 1 in reply LS to SA2 to justify RAN2 recommendation.

# References

[1]		3GPP TR 23.274, Study on enhancement of 5GS for Vertical and LAN Services

[2]		S2-1813392, LS on RAN Impact analysis due to TSN, SA2

[3]		3GPP TS 22.104, “Service requirements for cyber-physical control applications in vertical domains”

[4]		R2-1900633, Support for Clock Synchronization Service, Nokia, Nokia Shanghai Bell

[5]	R2-1900635, Summary of e-mail discussion on TSN traffic patterns (with TP), Nokia, Nokia Shanghai Bell

[6] 	R1-1901442, Reply LS on RAN Impact analysis due to TSN, RAN1

[7]	Schriegel and Jasperneite, “Investigation of Industrial Environmental Influences on Clock Sources and their Effect on the Synchronization Accuracy of IEEE 1588”, in 2007 IEEE International Symposium on Precision Clock Synchronization for Measurement, Control and Communication, 2007

[8]	Goetz, Hoga and Weber, “Dual Time Scale in Factory & Energy Automation”, White Paper about Industrial Requirements and Concepts at Time Synchronization in IEEE 802.1ASbt Industrial Requirements working group, 2012