3GPP TSG-RAN WG2#105	R2-1900684

Athens, Greece, 25th February – 1st March 2019

Agenda Item:	10.5.7

Source:	Ericsson

Title:	Correction to RLC bearer release

Document for:	Discussion, Decision

# Introduction

In EN-DC, the concept of RLC bearer was introduced for the NR side, where the lower layer (RLC and logical channel) were configured with a new IE; RLC-BearerConfig which was then associated with a particular radio bearer. This contribution discusses remaining issues with the release of these RLC bearers.

# Discussion

In MR-DC, it is possible to configure split bearers, where two RLC entities are serving the same PDCP entity. In LTE, this was configured explicitly in the DRB configuration, with the field drb-Type set to split which configures a single PDCP entity and two RLC entities.

In NR-DC, the split bearer is instead configured implicitly, where the PDCP entity is configured with the RadioBearerConfig while the RLC entity is configured with the RLC-BearerConfig. The RLC-BearerConfig in turn contain an indication (servedRadioBearer) which radio bearer it is associated with. If split bearers are configured, the configurations will contain an RLC-BearerConfig with the same servedRadioBearer in both the masterCellGroup and the secondaryCellGroup IEs. In the procedures in TS 38.331, the logical channels are then associated with a specific PDCP entity:

5.3.5.5.4	RLC bearer addition/modification

For each RLC-BearerConfig received in the rlc-BearerToAddModList IE the UE shall:

1>	if the UE's current configuration contains a RLC bearer with the received logicalChannelIdentity:

<<skipped parts>>

1>	else (a logical channel with the given logicalChannelIdentity was not configured before):

<<skipped parts>>

2>	associate this logical channel with the PDCP entity identified by servedRadioBearer.

NR-DC is configured by configuring an RLC bearers in both masterCellGroup and secondaryCellGroup serving the same radio bearer.

For NE-DC, the SCG RLC bearer is configured with an E-UTRA RRCConnectionReconfiguration message embedded in the NR RRCReconfiguration message. The RLC entity is configured per radio bearer and is associated with the PDCP entity in the RB with the same radio bearer identity (srb-Identity or drb-Identity).

5.3.10.3a1a	DC specific SRB addition or reconfiguration (NE-DC, HRLLC)

The UE shall:

1>	for each srb-Identity value included in the srb-ToAddModListSCG that is not part of the current UE E-UTRA SCG configuration (i.e. SCG SRB RLC bearer establishment):

<< skipped parts>>

2> else (i.e. the UE is configured with NE-DC):

3>	associate the SCG RLC bearer and DCCH logical channel with the NR PDCP entity, i.e. as configured by NR see TS 38.331 [82], identified with the same srb-Identity within the current UE configuration;

5.3.10.3ax	DRB addition or reconfiguration for NE-DC

The UE shall:

1>	for each drb-Identity value included in drb-ToAddModListSCG:

2>	if drb-Identity value is not part of the current UE configuration (SCG RLC bearer establishment):

3>	establish an SCG RLC entity or entities and an SCG DTCH logical channel in accordance with the rlc-ConfigSCG, logicalChannelIdentitySCG and logicalChannelConfigSCG included in drb-ToAddModListSCG;

3>	associate the SCG RLC bearer and DTCH logical channel with the NR PDCP entity, i.e. as configured by NR see TS 38.331 [82], identified with the same drb-Identity within the current UE configuration;

NE-DC is configured by configuring an RLC bearers in both masterCellGroup and the E-UTRA configurations serving the same radio bearer.

The same principle applies in NR carrier aggregation, For CA, the masterCellGroup (or secondaryCellGroup) will contain two rlc-BearerConfig (with different logicalChannelId) with the same servedRadioBearer.

In NR, CA duplication is configured by including two RLC bearers with different logicalChannelId in masterCellGroup (or secondaryCellGroup) of the same radio bearer.

Since the UE need to use one of the RLC bearers as the primary path in UL, the NR PDCP-Config contain these configurations in the moreThanOneRLC field:

OPTIONAL,   -- Cond DRB

moreThanOneRLC          SEQUENCE {

primaryPath             SEQUENCE {

cellGroup               CellGroupId                                                     OPTIONAL,   -- Need R

logicalChannel          LogicalChannelIdentity                                          OPTIONAL    -- Need R

},

ul-DataSplitThreshold   UL-DataSplitThreshold                                               OPTIONAL, -- Cond SplitBearer

pdcp-Duplication            BOOLEAN                                                         OPTIONAL    -- Need R

}                                                                                               OPTIONAL, -- Cond MoreThanOneRLC

An RLC bearer can be released either explicitly, using the RLC-BearerToReleaseList or implicitly using MR-DC release or SCellsToReleaseList. If a PDCP entity is configured in CA or DC with a primary path on the RLC bearer which is released, the UE will not be able to use the bearer until the PDCP entity has been reconfigured. In case the RLC bearer is explicitly released, it is possible to also reconfigure the associated PDCP entity by including the PDCP-Config in the same RRCReconfiguration message. However, if the PDCP configuration shall remain the same with regards to all other configurations, the network would have to include an empty PDCP-Config only to release the moreThanOneRLC field. Similarly, if MR-DC is released, the entire SCG configuration is released (either NR or E-UTRA SCG) and it is not necessarily possible to reconfigure the PDCP-entity at that time.

For instance, during initiation of RRC Resume, the UE will perform an MR-DC release, however, the target node may apply delta configurations of the PDCP configurations assuming that the UE doesn’t have an SCG RLC associated with the PDCP entity.

In current specifications, it is unclear what happens if the network doesn’t explicitly release the moreThanOneRLC field in PDCP-Config if one of the RLC entities are released.

To avoid any ambiguity, the UE should release the moreThanOneRLC field in any PDCP configuration, whenever the second RLC entity is released, either in NR-DC, NE-DC, or CA duplication.

If an RLC bearer is released, the UE shall also reconfigure its PDCP entity to release the moreThanOneRLC field

We have provided a CR capturing this behaviour in R2-1900685 [1].

Agree to the CR in R2-1900685

# Conclusion

In section 2 we made the following observations:

Observation 1	NR-DC is configured by configuring an RLC bearers in both masterCellGroup and secondaryCellGroup serving the same radio bearer.

Observation 2	NE-DC is configured by configuring an RLC bearers in both masterCellGroup and the E-UTRA configurations serving the same radio bearer.

Observation 3	In NR, CA duplication is configured by including two RLC bearers with different logicalChannelId in masterCellGroup (or secondaryCellGroup) of the same radio bearer.

Observation 4	In current specifications, it is unclear what happens if the network doesn’t explicitly release the moreThanOneRLC field in PDCP-Config if one of the RLC entities are released.

Based on the discussion in section 2 we propose the following:

Proposal 1	If an RLC bearer is released, the UE shall also reconfigure its PDCP entity to release the moreThanOneRLC field

Proposal 2	Agree to the CR in R2-1900685

# References

R2-190685, Correction to RLC bearer release, Ericsson, 3GPP WG2#105, Athens, Greece, 25th February – 1st March 2019