# 3GPP TSG-RAN WG2 Meeting #105	    					     R2-1901955

# Greece, Athens, 25 Feb - 1 Mar 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.355 | CR | 235 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 36.355 on NB-IoT OTDOA positioning accuracy enhancement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15 |  |  |  |  |  |  | Date: |  | 2019-2-12 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to the current specification, the OTDOA-NeighbourCellInfoListNB provided by E-SMLC is cell-specific. With the cell-specific neighbour cells, the UEs in the same cell from different places may have different RSRP values on these neighour cells. The worst case is all the measured RSRP values of neighbour cells are very poor and the corresponding positioning accuracy is also unsatisfying. To ensure the positioning accuracy, it is better for E-SMLC to configure the neighbour cells with higher measured RSRP values to UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Enable the E-SMLC provide an UE-specific neighbour cell list to the UE. Enable the UE to provide its latest measurement results to the E-SMLC to assist determine the UE-specific OTDOA measurable neighbour cell list. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some of the NB-IoT UEs would have wildly inaccurate locations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR … CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of change

#### 6.5.1.3	OTDOA Assistance Data Request

#### –	OTDOA-RequestAssistanceData

The IE OTDOA-RequestAssistanceData is used by the target device to request assistance data from a location server.

-- ASN1START

# OTDOA-RequestAssistanceData ::= SEQUENCE {

physCellId			INTEGER (0..503),

...,

[[

adType-r14			BIT STRING { prs (0), nprs (1) } (SIZE (1..8))		OPTIONAL

]],

[[

nrPhysCellId-r15	INTEGER (0..1007)									OPTIONAL

]]

[[

otdoaNeighbourMeasurementInformation-NB-r15		OTDOA-NeighbourMeasurementInformation-NB-r15			OPTIONAL

]]

}

-- ASN1STOP

OTDOA-RequestAssistanceData field descriptions

physCellId This field specifies the E-UTRA physical cell identity of the current primary cell of the target device.

adType This field specifies the assistance data requested. This is represented by a bit string, with a one-value at the bit position means the particular assistance data is requested; a zero-value means not requested. Bit 0 indicates that PRS assistance data are requested, bit 1 indicates that NPRS assistance data are requested.

nrPhysCellId This field specifies the NR physical cell identity of the current primary cell of the target device. If this field is present, the target device sets the physCellId to an arbitrary value which shall be ignored by the location server.

#### 6.5.1.4	OTDOA Assistance Data Request Elements

#### –	OTDOA-NeighbourMeasurementInformation-NB

The IE OTDOA-NeighbourMeasurementInformation-NB is used by the target device to provide neighbour cell measurements to the location server. The neighbour cell measurements are used to assist the location server to determine the optimal OTDOA assistance data, and are provided in the IE OTDOA- RequestAssistanceData.

-- ASN1START

# OTDOA-NeighbourMeasurementInformation-NB-r15 ::= SEQUENCE {

physCellId							INTEGER (0..503),

otdoaNeighbourMeasurementList-r15	OTDOA-NeighbourMeasurementList-NB-r15,

...

}

# OTDOA-NeighbourMeasurementList-NB-r15 ::= SEQUENCE (SIZE(1..24)) OF NeighbourMeasurementElement-NB-r15

# NeighbourMeasurementElement-NB-r15 ::= SEQUENCE {

physCellIdNeighbour			INTEGER (0..503),

cellGlobalIdNeighbour		ECGI					OPTIONAL,

earfcnNeighbour				ARFCN-ValueEUTRA		OPTIONAL,

measResultNeighbour			SEQUENCE {

rsrpResult						INTEGER(0..97),

rsrqResult						INTEGER(-30..46),

rssiResult						INTEGER(0..76),

},

...

}

-- ASN1STOP

OTDOA-NeighbourMeasurementInformation-NB field descriptions

physCellId This field specifies the E-UTRA physical cell identity of the current primary cell of the target device.

neighbourMeasurementList This list contains the measurements for neighbour cells.

physCellIdNeighbour This field specifies the physical cell identity of the neighbour cell for which the measurements are provided.

cellGlobalIdNeighbour This field specifies the ECGI, the globally unique identity of a cell in E-UTRA, of the neighbour cell for which the measurements are provided.

earfcnNeighbour This field specifies the EARFCN of the neighbour cell used for the measurements.

#### 6.5.1.5	OTDOA Location Information

[…]

#### 6.5.1.6	OTDOA Location Information Elements

[…]

#### 6.5.1.7	OTDOA Location Information Request

[…]

#### 6.5.1.8	OTDOA Capability Information

[…]

#### 6.5.1.9	OTDOA Capability Information Request

[…]

#### 6.5.1.10	OTDOA Error Elements

[…]

End of change