3GPP TSG-RAN WG2 Meeting #105bis	R2-1904106

Xi’an, China, 8-12 Apr 2019

Agenda item:	11.8.2.3

Source: 	Huawei, HiSilicon

Title: 	Considerations on broadcasting of location assistance data

Document for:	Discussion and Decision

Introduction

In the RAN #83 meeting, a NR positioning WID [RP-190752] was approved, in which there are the following RAN2+RAN3 objectives.

RAN2 + RAN3 centric objectives - Resolve open aspects on architecture and proceed with normative work for the following items [RAN2/RAN3]: - Location of the transmission measurement function - Define signaling for broadcast assistance data delivery [RAN2/RAN3/SA2/SA3]

In this contribution, we discuss the signalling for broadcasting of location assistance data in NR.

Discussion

By broadcasting assistance data, the signalling overhead of transmitting assistance data could be reduced and it is beneficial for UE to obtain assistance data in idle mode. In LTE, broadcasting assistance data has been supported and assistance data is broadcasted as SI message. The system information structure for broadcasting assistance data in LTE is like the following figure.

The IE AssistanceDataSIBelement is defined to carry assistance data and it includes the following IEs.

- valueTag, indicate to the target device any changes in the broadcast assistance data content. The valueTag is incremented by one, by the location server, every time a modified assistance data content is provided.

- expirationTime, indicates how long the broadcast assistance data content is valid. It is specified as UTC time and indicates when the broadcast assistance data content will expire.

- cipheringKeyData, indicates that the assistanceDataElement octet string is ciphered.

- segmentationInfo, indicates that the assistanceDataElement is one of many segments.

- assistanceDataElement OCTET STRING, depends on the posSibType.

The following table shows the assistance data elements defined in 36.355[1] could be broadcasted:

|  | posSibType [12] | assistanceDataElement |
| --- | --- | --- |
| GNSS Common Assistance Data (clause 6.5.2.2) | posSibType1-1 | GNSS-ReferenceTime |
|  | posSibType1-2 | GNSS-ReferenceLocation |
|  | posSibType1-3 | GNSS-IonosphericModel |
|  | posSibType1-4 | GNSS-EarthOrientationParameters |
|  | posSibType1-5 | GNSS-RTK-ReferenceStationInfo |
|  | posSibType1-6 | GNSS-RTK-CommonObservationInfo |
|  | posSibType1-7 | GNSS-RTK-AuxiliaryStationData |
| GNSS Generic Assistance Data (clause 6.5.2.2) | posSibType2-1 | GNSS-TimeModelList |
|  | posSibType2-2 | GNSS-DifferentialCorrections |
|  | posSibType2-3 | GNSS-NavigationModel |
|  | posSibType2-4 | GNSS-RealTimeIntegrity |
|  | posSibType2-5 | GNSS-DataBitAssistance |
|  | posSibType2-6 | GNSS-AcquisitionAssistance |
|  | posSibType2-7 | GNSS-Almanac |
|  | posSibType2-8 | GNSS-UTC-Model |
|  | posSibType2-9 | GNSS-AuxiliaryInformation |
|  | posSibType2-10 | BDS-DifferentialCorrections |
|  | posSibType2-11 | BDS-GridModelParameter |
|  | posSibType2-12 | GNSS-RTK-Observations |
|  | posSibType2-13 | GLO-RTK-BiasInformation |
|  | posSibType2-14 | GNSS-RTK-MAC-CorrectionDifferences |
|  | posSibType2-15 | GNSS-RTK-Residuals |
|  | posSibType2-16 | GNSS-RTK-FKP-Gradients |
|  | posSibType2-17 | GNSS-SSR-OrbitCorrections |
|  | posSibType2-18 | GNSS-SSR-ClockCorrections |
|  | posSibType2-19 | GNSS-SSR-CodeBias |
| OTDOA Assistance Data (clause 7.4.2) | posSibType3-1 | OTDOA-UE-Assisted |

So far, the system information structure still could work in NR. Therefore, the system information structure for broadcasting assistance data in LTE could be as baseline in NR.

Proposal 1: the system information structure for broadcasting assistance data in LTE could be as baseline in NR.

Different from LTE, on-demand SI mechanism, value tag per SIB, and area ID are introduced in NR. On-demand SI mechanism still be necessary for positioning SIBs from the perspective of saving the broadcasting resource overhead. The area ID also is beneficial to avoid UE re-reading positioning SIBs when serving cell changes. However, value tag per SIB is not necessary for positioning SIBs due to the fact that there has been a valueTage indicator in the IE AssistanceDataSIBelement. Therefore, SI-BroadcastcastStatus, SI-requestConfig, and areaScope indicator should be included in the scheduling information for positioning SIBs but valueTag is not necessary to be included.

Proposal 2: SI-BroadcastcastStatus, SI-requestConfig, and areaScope indicator should be included in the scheduling information for positioning SIBs but valueTag is not necessary to be included.

Conclusion

In this contribution, we discussed the signalling for broadcasting of location assistance data in NR, and we get the following observation and proposals:

Proposal 1: the system information structure for broadcasting assistance data in LTE could be as baseline in NR.

Proposal 2: SI-BroadcastcastStatus, SI-requestConfig, and areaScope indicator should be included in the scheduling information for positioning SIBs but valueTag is not necessary to be included.

References

[1] 3GPP TS 36.355, "Evolved Universal Terrestrial Radio Access (E-UTRA); LTE Positioning Protocol (LPP)".