3GPP TSG-RAN WG2#106	R2-1907549

Reno, USA, 13th – 17th May 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | DraftCR | rev | - | Current version: | 15.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Measurements for SCell failure report |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | n.a. (draft CR to be merged into rapporteur's CR when agreed) |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-Core |  |  |  |  |  |  | Date: |  | 2019-05-02 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | When RLC failure is detected on a logical channel restricted to SCell(s) in CA duplication, the UE reports the failure to the network using the FailureInformation message. The report currently only includes information of the logical channel identity, cell group and failure type. Even though the RLC failure is a protocol error as such, it is often triggered by poor radio conditions on the SCell. Therefore, in order to enable the network to make a good decision, whether to just release the affected logical channel, release the SCell or replace it with other SCell(s), the availabity of latest measurements by the UE is essential. By including also the latest available measurements in the report, the network has better information based on which the correct action can be taken. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The UE includes in the FailureInformation message the latest available measurements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Network may not have sufficient information to reconfigure UE after reported RLC failure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2 Message definitions |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

#### –	FailureInformation

The FailureInformation message is used to inform the network about a failure detected by the UE.

Signalling radio bearer: SRB1 or SRB3

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to network

FailureInformation message

FailureInformation ::=         SEQUENCE {

criticalExtensions                  CHOICE {

failureInformation             FailureInformation-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

FailureInformation-IEs ::=.        SEQUENCE {

failureInfoRLC-Bearer						FailureInfoRLC-Bearer   OPTIONAL,

lateNonCriticalExtension        OCTET STRING                        OPTIONAL,

nonCriticalExtension            FailureInformation-v16xy-IEs        OPTIONAL

}

FailureInformation-v16xy-IEs ::=    SEQUENCE {

measResultsNR                       MeasResultNR              OPTIONAL,

measResultsEUTRA                    MeasResultEUTRA           OPTIONAL,

nonCriticalExtension				SEQUENCE {}               OPTIONAL

}

FailureInfoRLC-Bearer ::=					SEQUENCE {

cellGroupId							CellGroupId,

logicalChannelIdentity				LogicalChannelIdentity,

failureType							ENUMERATED {duplication, spare3, spare2, spare1}

}

-- TAG-FAILUREINFORMATION-STOP

-- ASN1STOP

END OF CHANGES