3GPP TSG-RAN2 Meeting #106	R2- 1907939

# Reno, U.S.A., May 13th – May 17th, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.304 | CR | 0131 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of dedicated priority handling from RRC_INACTIVE to RRC_IDLE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-05-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the meeting RAN2#102, it was agreed that when a UE in RRC_INACTIVE moves to RRC_IDLE without state transition to RRC_CONNECTED, the UE shall delete priorities provided by dedicated signalling. In TS 38.304 : The UE shall delete priorities provided by dedicated signalling when: - the UE enters a different RRC state; - the optional validity time of dedicated priorities (T320) expires; or - a PLMN selection is performed on request by NAS. However, current statement seems to have ambiguity on existing procedure – When the UE is in RRC_CONNECTED and receives RRCRelease with configured cellReselectionPriorities. The UE is still in RRC_CONNECTED when it stores cellReselectionPriorities, and then transits to RRC_INACTIVE or RRC_IDLE. According the current specification state in TS 38.304 above, as the UE enters a different RRC state, the UE shall delete the dedicated priorities. |
| Summary of change: |  | The UE discards the cell reselection priority information provided by the cellReselectionPriorities, only when the UE enters RRC_IDLE from RRC_INACTIVE. Other state transition cases, such as entering RRC_CONNECTED from RRC_IDLE or RRC_INACTIVE, are already state in 38.331, by stopping T320. Impact analysis Impacted 5G architecture options: Standalone, NE-DC, NR-DC – For the cases UE can camp on the cell with inactive state in NR. Impacted functionality: Cell reselection Inter-operability: - If the network is implemented according to this CR but the UE is not, there is no impact. - If the UE is implemented according to this CR but the network is not, there is no impact. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | When the UE enters RRC_IDLE from RRC_INACTIVE state while cellReselectionPriorities is configured, the stored cellReselectionPriorities information is still remaining. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | S5.2.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of changes

5.2.4.1	Reselection priorities handling

Absolute priorities of different NR frequencies or inter-RAT frequencies may be provided to the UE in the system information, in the RRCRelease message, or by inheriting from another RAT at inter-RAT cell (re)selection. In the case of system information, an NR frequency or inter-RAT frequency may be listed without providing a priority (i.e. the field cellReselectionPriority is absent for that frequency). If priorities are provided in dedicated signalling, the UE shall ignore all the priorities provided in system information. If UE is in camped on any cell state, UE shall only apply the priorities provided by system information from current cell, and the UE preserves priorities provided by dedicated signalling and deprioritisationReq received in RRCRelease unless specified otherwise. When the UE in camped normally state, has only dedicated priorities other than for the current frequency, the UE shall consider the current frequency to be the lowest priority frequency (i.e. lower than any of the network configured values).

The UE shall only perform cell reselection evaluation for NR frequencies and inter-RAT frequencies that are given in system information and for which the UE has a priority provided.

In case UE receives RRCRelease with deprioritisationReq, UE shall consider current frequency and stored frequencies due to the previously received RRCRelease with deprioritisationReq or all the frequencies of NR to be the lowest priority frequency (i.e. lower than any of the network configured values) while T325 is running irrespective of camped RAT. The UE shall delete the stored deprioritisation request(s) when a PLMN selection is performed on request by NAS (TS 23.122 [9]).

NOTE:	UE should search for a higher priority layer for cell reselection as soon as possible after the change of priority. The minimum related performance requirements specified in TS 38.133 [8] are still applicable.

The UE shall delete priorities provided by dedicated signalling when:

-	the UE enters RRC_IDLE state from RRC_INACTIVE state; or

-	the optional validity time of dedicated priorities (T320) expires; or

-	a PLMN selection is performed on request by NAS (TS 23.122 [9]).

NOTE 2:	Equal priorities between RATs are not supported.

The UE shall not consider any black listed cells as candidate for cell reselection.

The UE shall inherit the priorities provided by dedicated signalling and the remaining validity time (i.e. T320 in NR and E-UTRA), if configured, at inter-RAT cell (re)selection.

NOTE 3:	The network may assign dedicated cell reselection priorities for frequencies not configured by system information.

End of changes