3GPP TSG-RAN WG2 Meeting #107                                                            	    R2-1908907

Prague, Czech Republic, 26th August – 30th August 2019            Revision of R2-1906921

Source:	CATT

Title:	Fallback Mechanism for Fast Recovery

Agenda Item:	11.10.5

Document for:	Discussion and Decision

1	Introduction

Fast recovery solutions were discussed at previous meetings. The applicability of fast recovery for different failure cases was also discussed. It was agreed to have SCG failure –like procedure to recover from the MCG failure. The fast recovery will be used for MCG leg RLF.

In case the fast recovery procedure is unable to regain the MCG connection, there should be a way of the UE to regain connection to the network. In this contribution, we discuss what solutions could be used to regain the connection to the network if fast recovery is failed.

2. Discussion

With MCG failure, the MN could take actions to reconfigure MCG, change PCell, handover to another cell depending on the failure case which triggered the fast recovery. The fast recovery is applied upon the MCG leg RLF. To resolve MCG leg RLF, the network may change PCell or handover to another cell.

One of the most likely actions from the network upon the reception of MCG failure information is to handover the UE to another cell (PCell change or another gNB). For successful handover, the target cell should be able to support the UE. There is a possibility of the target is unable to accept the UE. The fast recovery may fail. It was agreed that “upon receiving the MCG failure indication, the MN sends reconfiguration with sync or RRC Release to the UE via SRB1”.

Even though, RRC Release was agreed as a possible network action for MCG failure (only to be used when all other actions are exhausted), release of MCG is not an optimal action for MCG failure recovery given that release of MCG results in sending the UE to Idle mode.

Proposal 1: RRC release in response to MCG failure is not an optimal action and it should only be used by the network when all other actions are exhausted.

Some companies proposed to have a guard timer for the MCG failure message. At the guard timer expiry, the UE initiates the RRC reestablishment procedure. The guard timer, in our view, addresses a scenario where the network is not able to take an action upon the reception of MCG failure information. We don’t see it is a realistic to introduce a UE based mechanism to handle network failure. However we see that the network could guide the UE to perform RRC re-establishment, in case the RRC reconfiguration is not possible as a response to MCG failure.

The conventional RRC re-establishment procedure involves cell (re)selection to identify a suitable cell. Cell (re)-selection adds latency to the recovery procedure. Also, the UE has sent the MCG failure information to the network via the SN as the SN link is good. Considering the latency of cell (re)selection and also the SN is found to be good quality link for the UE, the UE should be guided to re-establish the connection at the SN, in case of the MN has exhausted of actions to resolve MCG failure.

The conventional RRC re-establishment also involves the UE transmitting RACH on the selected cell. Given that the UE is already communicating with the SN, the step involving RACH access to the selected cell could be avoided. Note that RRC re-establishment procedure in NR supports the fallback to cell setup in order to mitigate another RACH access hence fast cell setup in case of Re-establishment failure.

Considering that the UE is communicating with the SN, a fallback mechanism for fast recovery failure should be aimed at re-establishment of the UE connection at the SN.

Proposal 2: In case that the network is not able to reconfigure the connection, the network should inform the UE to re-establish the connection at the SN as a response to MCG failure indication.

The following was agreed for fast recovery solution.

- when MCG failure occurs, UE follows SCG failure-like procedure. Thus, UE does not trigger RRC connection re-establishment. The UE triggers MCG failure information procedure in which the MCGFailureInformation message is transmitted to the network using SRB1S.

A fallback solution realising Proposal 2 is shown in Figure 1.

Figure 1: Fallback mechanism in case of fast recovery failure.

MCG failure indication is sent to the MN via the SN over split SRB1S. It is considered that the SN cannot comprehend the MCGFailureInformation sent by the UE in step 1. At step 2a, the MN is aware of fast recovery failure. The MN informs the SN of unsuccessful fast recovery together with the necessary UE context required for RRC re-establishment at the SN. The SN performs the admission control and access the UE as re-establishment at the SN. The SN sends the RRC re-establishment message to the UE in response to the MCGFailureInformation sent at step 1. The UE sends the RRC re-establishment complete message.

Proposal 3: RAN2 is requested to discuss fallback solution shown in Figure 1, where RRC Re-establishment message is sent in response to MCGFailureInfromation.

To enable RRC re-establishment, conventional RRCRe-establishmentRequest message includes the ue-Identity and ReestablishmentCause. ReestablishmentCause can be derived from the MCGFailureInformation and the ue-Identity information is known by the MN. As the UE is in communication with the SN, the SN may not need to verify the UE identity. The MN can generate the message in step 2b including ReestablishmentCause and ue-Identity [FFS] based on MCGFailureInformation and known UE information at the MN. If required, the ue-Identity information could be signalled in MCGFailureInformation itself. In which case, the MN simply forwards the MCGFailureInformation message to the SN.

Proposal 4: If fallback mechanism to re-establish the RRC connection at the SN is considered, the ReestablishmentCause and the ue-Identity [FFS] should be provided to the SN by the MN over Xn interface.

Proposal 5: The MN can simply forward the MCGFailureInformation message to the SN for the UE re-establishment at the SN.

3	Conclusion

In this contribution, we have discussed the necessity for a fallback mechanism in case of fast recovery failure. The following proposals were made.

Proposal 1: RRC release in response to MCG failure is not an optimal action and it should only be used by the network when all other actions are exhausted.

Proposal 2: In case that the network is not able to reconfigure the connection, the network should inform the UE to re-establish the connection at the SN as a response to MCG failure indication.

Proposal 3: RAN2 is requested to discuss fallback solution shown in Figure 1, where RRC Re-establishment message is sent in response to MCGFailureInfromation.

Proposal 4: If fallback mechanism to re-establish the RRC connection at the SN is considered, the ReestablishmentCause and the ue-Identity [FFS] should be provided to the SN by the MN over Xn interface.

Proposal 5: The MN can simply forward the MCGFailureInformation message to the SN for the UE re-establishment at the SN.