3GPP TSG-RAN WG2 Meeting #107	R2-1910060

Prague, Czech Republic, 26th – 30th August 2019

Title:	DRAFT LS on the aperiodic DL quality report in connected mode

Response to:

Release:	Release 16

Work Item:	LTE_eMTC5-Core

Source:	Huawei, HiSilicon [TSG RAN WG2]

To:	TSG-RAN WG1, TSG-RAN WG4

Cc:

Contact Person:

Name:	Yulong Shi

E-mail Address:	shiyulong5@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

1. Overall Description:

RAN2 has discussed the aperiodic DL quality report in connected mode. There are several options in RAN2 discussion for trigger signaling and enable signaling:

Option 1: There is an explicit signaling to enable the feature at the beginning, UE reports the DL quality report after receiving a MAC CE trigger.

Option 2: There is an explicit signaling to enable the feature at the beginning, UE reports the DL quality report after receiving a DCI trigger.

Option 3: There is an explicit signaling to enable the feature at the beginning, UE reports the DL quality report after receiving an UL grant with large enough TBS.

In option 1, option 2 and option 3, it is assumed that UE can either perform the measurement once the feature is enabled occasionally or perform the measurement after receiving the trigger (by explicit signaling or implicit trigger).

Option 4: There is no explicit signaling to enable the feature at the beginning, UE reports the DL quality report after receiving a MAC CE trigger.

Option 5: There is no explicit signaling to enable the feature at the beginning, UE reports the DL quality report after receiving a DCI trigger.

In both option 4 and option 5, it is assumed that UE perform the measurement after receiving the trigger signaling.

RAN2 would like to ask RAN1 and RAN4 the following questions:

If MAC CE or DCI or TBS of the UL grant is used to trigger the aperiodic DL quality report in connected mode, whether UE has enough time to perform measurement and when the UE performs the measurement, in the following cases:

Case 1: There is an explicit signaling to enable the feature at the beginning;

Case 2: There is no explicit signaling to enable the feature at the beginning.

2. Actions:

To RAN1 and RAN4:

RAN2 respectfully asks RAN1 and RAN4 the following questions:

If MAC CE or DCI or TBS of the UL grant is used to trigger the aperiodic DL quality report in connected mode, whether UE has enough time to perform measurement and when the UE performs the measurement, in the following cases:

Case 1: There is an explicit signaling to enable the feature at the beginning;

Case 2: There is no explicit signaling to enable the feature at the beginning.

3. Date of Next TSG-RAN WG2 Meetings:

3GPPRAN2#107bis	14 - 18 October 2019	Chongqing	China

3GPPRAN2#108	18 – 22 November 2019	Reno		USA