3GPP TSG RAN WG2 #107	R2-1910894

Prague, Czechia, August 26-30, 2019

Agenda item:	11.4.2

Source: 	Qualcomm Incorporated

Title: 	RLC packet format in NR V2X

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss RLC PDU format to be used for NR V2X sidelink communication, and related parameters.

# Discussion

Regarding the NR V2X sidelink communication, we only consider support for UM and AM mode in this paper. The TM mode is used for transporting SLSS signals in PSBCH channel, and there are no RLC functions involved and no RLC header needed for TM mode.

Regarding the RLC PDU format, we think the NR Uu format shall be basically reused. There is no need to remove or add any parameters for SL-specific reasons.

Proposal 1. AMD PDU, UMD PDU formats and status PDU formats for NR Uu are reused for NR V2X SL.

It is worth noting that NR Uu support several different types of RLC header formats, due to different SN size. For example, In Uu RLC AM, both 12-bit SN size and 18-bit SN size are allowed. Also, in Uu RLC UM mode, 6-bit SN size and 12-bit SN size are allowed.

It has been agreed by RAN2 that UM mode is used for groupcast, broadcast and unicast, For Sidelink broadcast and multicast, the choice of SN size shall be fixed in RAN2 specification. To avoid complexity, we need to select a single SN length to be used. In NR user plane, the need of SN field in NR RLC layer is significantly reduced because RLC layer concatenation has been removed from RLC functions, and concatenation is done in MAC layer instead. As a result, the SN field is only used if there is a segmented SDU included.

For RLC AM mode, which is only used for SL unicast, there is a PC5-RRC procedure agreed to support a negotiation/configuration of this parameter, Thus, some flexibility of SN length can be supported.

Proposal 3	SN lengths for sidelink RLC AM are 12-bit and 18-bit, configurable during PC5-RRC procedure.

# Conclusion

In this contribution, we discuss the RLC PDU format for NR V2X. The following is proposed:

Proposal 1	 AMD PDU, UMD PDU formats and status PDU formats for NR Uu are reused for NR V2X SL.

Proposal 2	SN lengths for sidelink RLC AM are 12-bit and 18-bit, configurable during PC5-RRC procedure.

# References