3GPP TSG-RAN WG2 #107	 	     			R2-1911346

Prague, Czech, 26th – 30th August, 2019

Agenda item:	   11.1.5 (NR_IAB-Core)

Source:	LG Electronics Inc.

Title: 	Access barring for IAB-nodes

Document for:	Discussion and Decision

1.	Introduction

During the initial access procedure, a UE performs cell search and system information acquisition. Before performing random access to a cell, the UE checks cellBarred field of the acquired MIB in order to know whether the cell is barred or not. The UE will attempt a random access to the cell if cellBarred is set to notBarred. RAN2 may also have common understanding that MT on IAB-node performs initial access as UE, as described in TR 38.874.

### 7.2.1	IAB-node initial access (Stage 1)

In case of SA deployments, initial IAB-node discovery by the MT (Stage 1) follows the same Rel-15 initial access procedure as a UE, including cell search based on the same SSBs available for access UEs, SI acquisition, and random access, in order to initially set up a connection to a parent IAB-node or an IAB-donor.

In case of an NSA deployment (from an access UE perspective), when the IAB-node MT performs initial access on the NR carrier, it follows the same Stage 1 initial access as in SA deployments (from an access UE perspective). The SSB/RMSI periodicity assumed by the MTs for initial access may be longer than 20ms assumed by Rel-15 UEs, and a single value is to be selected from the following candidate values: 20ms, 40ms, 80ms, 160ms.

IAB-nodes determine whether to attempt random access based on cellBarred as UE which means that the following two cases only occur: 1) both UE and IAB-node are not allowed and 2) both UE and IAB-node are allowed.

In this document, we are discussing if it is not necessary to consider other cases: 3) UE is not allowed but IAB-node is allowed and 4) UE is allowed but IAB-node is not allowed.

2.	Discussion

According to TR 38.874, an IAB-node can operate in Stand-Alone (SA) mode or in Non-Stand-Alone (NSA) mode. A UE connecting to an IAB-node may choose a different operation mode than the IAB-node. The UE may further connect to a different type of core network than the IAB-node it is connected to. In Figure 1, option A is that both UE and IAB-node operate in SA mode with NGC, option B is that UE operates in NSA mode with EPC while IAB-node operates in SA mode with NGC, and option C is that both UE and IAB-node operate in NSA mode with EPC.

For option B, UE must not perform a random access to an NR cell, while IAB-node can perform a random access to the NR cell. In other words, the above third case “UE is not allowed but IAB-node can be allowed” can happen.

Figure 1: Examples for SA mode with NGC and/or NSA mode with EPC

Observation 1: For option B that UE operates in NSA mode with EPC while IAB-node operates in SA mode with NGC, UE is not allowed but IAB-node can be allowed.

Lastly, we are checking if the above fourth case “UE is allowed but IAB-node is not allowed” exists in IAB networks. RAN2 decided to only support HBH ARQ in Rel-16. HBH ARQ based IAB networks may have to restrict hop count due to PDCP parameters, as shown in Table 8.2.3-1 of TR 38.874.

Table 8.2.3-1: Observations for end-to-end and hop-by-hop ARQ

| Metric | Hop-by-hop RLC ARQ | End-to-end RLC ARQ |
| --- | --- | --- |
| Forwarding latency | Potentially higher as packets have to pass through RLC-state machine on each hop. | Potentially lower as packets do not go through the RLC state machine on intermediate IAB-nodes. |
| Latency due to retransmission | Independent of number of hops | Increases with number of hops |
| Capacity | Packet loss requires retransmission only on one link. Avoids redundant retransmission of packets over links where the packet has already been successfully transmitted. | Packet loss may imply retransmission on multiple links, including those where the packet was already successfully transmitted. |
| Hop count limitation due to RLC parameters | Hop count is not affected by max window size. | Hop count may be limited by the end-to-end RLC latency due to max window size. |
| Hop count limitation due to PCDP parameters | Hop count may be limited by increasing disorder of PDCP PDUs over sequential RLC ARQ hops. This may increase probability to exceed max PDCP window size. | Hop count does not impact disorder of PDCP PDUs due to RLC ARQ. |

Therefore, some IAB-nodes must not accommodate other IAB-nodes as its child node in order to limit hop count.  In that case, the IAB-nodes would only allow UE’s access.

Observation 2: IAB-node’s access may not be allowed in order to limit hop count.

Based on these observations, we think that access barring for IAB-nodes should be designed in the light of all the cases: 1) both UE and IAB-node are not allowed, 2) both UE and IAB-node are allowed, 3) UE is not allowed but IAB-node is allowed and 4) UE is allowed but IAB-node is not allowed.

We think that the access barring for IAB-nodes can be supported using existing access barring mechanisms. For Case 3, existing indicator “cellReservedForOperatorUse” can be used so that IAB-nodes assigned to a specific access identity (e.g., 11 or 15) only attempt random access. For Case 4, IAB-nodes are able to receive some information on accessible cells, and determine whether to attempt access based on the information.

Proposal 1: Existing access barring mechanisms are applied to IAB-nodes.

3.	Proposal

In this document, we present our view on access barring for IAB-nodes. We have following proposal:

Proposal 1: Existing access barring mechanisms are applied to IAB-nodes.