3GPP RAN WG2 Meeting #107	R2-1911697

Prague, Czech, 26 – 30 August, 2019

# Title:	[DRAFT] LS on misalignment in Counter Check Procedure

# Response to:	-

# Release:	Rel-15

# Work Item:	NR_newRAT-Core

Source:	Samsung [to be RAN2]

To:	SA3

Cc:	-

Contact Person:

Name:	Mangesh Abhimanyu Ingale

E-mail Address:	m.ingale@samsung.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

RAN2 discussed the misalignment between TS 33.501 and TS 38.331 regarding the UE behaviour for the counter check procedure. According to TS 38.331, if the counter check procedure is initiated by the network then the UE responds with the COUNT values for each established DRB regardless of integrity protection is activated or not for a DRB (refer section 5.3.6.3 in TS 38.331). RAN2 identified the UE behaviour specified in TS 38.331 is not aligned with following text from section 6.13 of TS 33.501, which states:

“Whenever user plane integrity protection is activated and used on a DRB, the procedure defined in this clause shall not be used for that particular DRB.”

RAN2 decided not to change the UE behaviour specified in TS 38.331 at this stage in Rel-15.

2. Actions:

To SA3:

ACTION:	RAN2 respectfully ask SA3 to take the above into consideration in their specification work.

3. Date of Next RAN2 Meetings:

RAN2 Meeting #107bis 	14 – 18 Oct 2019	Chongqing, China

RAN2 Meeting #108	18 – 22 Nov 2019	Reno, USA