3GPP TSG-RAN WG2 Meeting#107bis			 		        R2- 1912792

Chongqing, China, 14th - 18th October 2019

Agenda item:	6.4.2

Source:		Intel Corporation

Title:		Need for SL RLC re-establishment

Document for:	Discussion

Introduction

During the previous RAN2 meeting (#107), RLC support for sidelink was discussed based on the corresponding email discussion and the following agreements were made related to Sidelink RLC [1]:

Agreements on SL RLC:

1:     NR SL TM RLC entity is configured to submit/receive RLC PDUs to/from SBCCH.

2:    RLC functionalities defined for NR Uu are reused for SL RLC design. FFS on the need of RLC reestablishment.

3:    For NR SL unicast, RLC TX side and RX side establishment/release is triggered by upper layer request. FFS the case for RLC TX side re-establishment.

4:    For NR SL groupcast/broadcast, RLC TX side establishment/release is triggered by upper layer request. FFS the case for RLC TX side re-establishment. RLC RX side establishment is triggered by the reception of first PDU where there is not yet a corresponding receiving RLC entity. FFS the case for RLC RX side re-establishment. RLC RX side release is up to UE implementation.

5:     The following procedures defined in NR Uu RLC are reused for NR SL RLC:

- Data transfer procedures (incl. TM/UM/AM data transfer)

- ARQ procedures (incl. retransmission, polling, status reporting)

- SDU discard procedures

- Data volume calculation

- Handling of unknown, unforeseen and erroneous protocol data

In this contribution, we aim to address the FFS aspects primarily related to RLC re-establishment.

Discussion

  - Sidelink RLC re-establishment

### In the last meeting, it was agreed that all the RLC procedures defined in NR Uu would be re-utilized for NR Sidelink with FFS on RLC re-establishment aspect. In EN-DC architecture, RLC entity is re-established during the following scenarios:

- Security key change (due to node change) for a bearer

- Bearer type change (e.g. SCG to SCG split)

- Handover

### Since these mechanisms are not expected to be supported in Sidelink, we can assume that support of re-establishment of RLC entity is potentially not necessary. Furthermore, in NR Uu, the RLC re-establishment is supported by the network using RLC configuration while performing RRC reconfiguration and also performed at the UE during T300 expiry, resumption of RRC connection, etc.

    - Groupcast/broadcast scenario

### As RLC re-establishment can be considered to be applicable for specific sidelink radio bearer(s), it can be considered as configurable SL radio bearer parameter. For SL groupcast/broadcast, it has already been agreed that the network configured sidelink radio bearer parameters should relate only to TX. Therefore, a dynamic re-establishment configuration indication cannot be sent from the network to the RX entities in a groupcast/broadcast scenario (assuming it were even possible). Furthermore, the SLRB parameters requiring TX/RX alignment are agreed to be fixed in the specification. It is not possible to set the RLC re-establishment configuration as a fixed parameter. Therefore, we think that it is not possible to configure the RX entities in groupcast/broadcast scenario for RLC re-establishment.

### Proposal 1. 	RAN2 to agree that for NR SL groupcast/broadcast cases, the RLC RX side re-establishment is not supported.

### As already discussed, the above mechanisms such as handover are not applicable for groupcast/broadcast situations. Moreover, as the SL groupcast/broadcast TX UE cannot communicate RLC re-establishment configuration with the RX entities for alignment, it is generally not feasible to re-establish TX side entity either.

### Proposal 2.	RAN2 to agree that for NR SL groupcast/broadcast cases, the RLC TX side re-establishment is not supported.

    - Unicast scenario

### When a V2X UE is involved in unicast communication, the TX side can receive network configuration and it can share it with the RX side over PC5-RRC message. However, it is evident that the network cannot support RLC reestablishment configuration when UE is not in RRC_CONNECTED state for sidelink. The V2X UE will end up with different behaviour for when in idle or out-of-coverage situations compared to in-coverage/connected state for the same trigger if we rely on network configuration for RLC re-establishment. Therefore, to provide common UE behaviour, we prefer to not support network configuration based RLC re-establishment for all the SL unicast cases.

### Proposal 3.	RAN2 to agree that for NR SL unicast, network-configured RLC TX side re-establishment is not supported.

### Although the triggers of bearer type change, handover are still not applicable for unicast case compared to Uu, there can be other triggers such as:

- Security key change potentially due to source ID change (pending SA3 input)

- Timer expiry for PC5-RRC connection establishment (if applicable)

### While the security solution is being finalized in SA3, there might be link update procedure defined for unicast wherein security (root) key may get updated when the source layer-2 ID changes. Source ID of V2X UE may be changed to avoid tracking of the UE. When this change happens, it may be necessary to re-establish PDCP entity and potentially RLC entity. We need to check with SA3 if this is a valid concern. Also, depending on whether PC5-RRC connection establishment procedure is defined, the RLC re-establishment may be necessary to reset the RLC entity without tearing down the SLRBs between V2X UEs involved in unicast communication. Therefore, it may be beneficial to support UE-triggered RLC re-establishment for SL unicast.

### Proposal 4.  	RAN2 to make a working assumption that for NR SL unicast, UE-triggered RLC TX/RX side re-establishment without network involvement is supported, pending SA3 input.

Conclusion

In this contribution, we provide considerations on RLC re-establishment in V2X and have the following proposals:

### Proposal 1. 	RAN2 to agree that for NR SL groupcast/broadcast cases, the RLC RX side re-establishment is not supported.

### Proposal 2.	RAN2 to agree that for NR SL groupcast/broadcast cases, the RLC TX side re-establishment is not supported.

### Proposal 3.	RAN2 to agree that for NR SL unicast, network-configured RLC TX side re-establishment is not supported.

### Proposal 4.  	RAN2 to make a working assumption that for NR SL unicast, UE-triggered RLC TX/RX side re-establishment without network involvement is supported, pending SA3 input.

References

[1] RAN2#107: NR V2X Chairman notes