3GPP TSG-RAN WG2 Meeting #107bis	R2-1913651

Chongqing, China, 14th - 18th October 2019	Revision of R2-1911107

Agenda item:	6.2.3.1

Source:	Charter Communications

Title:	Paging Monitoring in NR-U

WID/SID:	NR-U WID - Release 16

Document for:	Discussion and Decision

# 1	Introduction

In NR-Unlicensed WID [1], paging was identified as a subject that requires enhancements compared to the baseline operation in licensed bands. Specifically, the WID lists the following:

-	Paging: specify required NR modifications to enhance paging opportunities by increasing time-domain paging occasions or paging monitoring occasions while taking UE power consumption into account.

In RAN2#105 meeting [2], the following agreement was reached:

- A UE can be configured for an additional number of monitoring occasions at or after or before (FFS) its calculated PO (when the paging message can be transmitted). FFS dynamic extension. FFS dynamic termination

In RAN2#106 meeting [3], the following agreements related to paging were reached:

- The UE should also stop monitoring paging for the PO even if it does not decode a P-RNTI if it can detect that the gNB had access to the channel at the PDCCH monitoring occasion. FFS if there are additional detection methods to detection of PRNTI and what those are.

- We extend paging monitoring by extending PDCCH occasions for a PO

In RAN2#107 meeting [4], the following agreements related to paging were reached:

- PO consists of ‘S × X’ PDCCH monitoring occasions for paging, where 'S' is the number of actual transmitted SSBs and X is the number of PDCCH monitoring occasions per SSB.

- Parameter 'X' is signaled in paging configuration (i.e. pcch-Config). Parameter Name FFS

In this contribution, we express our views regarding potential issues of NR-U paging process and possible enhancements.

# 2	Discussion

## 2.1 Motivation for early notification of paging

Paging is a fundamental aspect of NR operation, and a UE is required to monitor paging occasions according to its configuration. While such design guarantees a minimum delay in reaching UEs (for delivering control or data content), it’d also mean during a majority of paging occasions there may not be any paging message, hence affecting UEs’ power consumption.

Above observation is applicable to operation in both licensed and unlicensed bands, however for NR-U operation there is additional reasons that a UE may not find any paging message during a paging occasion. Particularly, if during channel access by gNB (to transmit paging messages) LBT fails, UEs will not find any paging message and would have to continue checking for paging messages subsequently. To address this problem, a UE should be allowed to extend or stop its paging monitoring depending on whether it has detected a gNB-initiated COT or not. Particularly, to enhance UE power consumption, monitoring an extended PO should be subject to lack of a gNB-initiated COT (e.g. unsuccessful PDCCH detection or low likelihood of presence of DM-RS) in the assigned PO. Similarly, once a gNB-initiated COT is detected, the UE may stop monitoring remaining of the extended PO.

Proposal 1: To enhance UE power consumption, monitoring an extended PO should be subject to lack of a gNB-initiated COT in the assigned PO. Also, once a gNB-initiated COT is detected, the UE may stop monitoring remaining POs of an extended paging opportunity.

Above proposal helps a UE to manage its paging monitoring based of detection of a gNB-initiated COT or lack of thereof. However, there is still a large percentage of paging occasions (where the gNB successfully performs LBT and is able to transmit but) lacks any paging message, simply due to the fact that the UE may have low activity. To address this issue, one solution would be for a UE, in Idle/Inactive mode that is about to perform detection in a PO, to first detect the start of a gNB-initiated COT. If the gNB has failed to initiate a COT, due to LBT failure, the UE can skip further processing until it detects a gNB-initiated COT. Furthermore, based on RAN1 agreements, a gNB-initiated COT would carry a COT format indicator within which information about presence of any paging message in the current COT (and maybe in next few COTs) may be carried. The benefit of such design is that in majority of cases (where there is no paging message in a PO), a UE in Idle/Inactive mode need not perform any further processing beyond COT format indicator. This behaviour also reduces UE processing when time-domain POs are increased.

We suggest RAN2 to consider early notification of an upcoming paging (within current or next COTs) by first detecting a gNB-initiated COT and retrieving information within COT format indicator, which indicates whether any paging message is carried within COT or not. This helps UEs to skip monitoring POs when there no paging message is expected.

Proposal 2: RAN2 to consider early notification of an upcoming paging by first detecting a gNB-initiated COT and retrieving information within COT format indicator, which indicates whether any paging message is carried within COT or not.

Proposal 3: Inform RAN1 of allocation of bit-space within COT format indicator to indicate presence of paging message(s) within current or next COTs.

## 2.1 Additional paging opportunities

Increasing the paging occasions enhances the chance of a successful channel access for transmission of a paging message by a gNB. The additional time-domain POs should be spaced apart enough in time so that an LBT failure on one PO would not lead, with high likelihood, to another LBT failure in the next PO. The time-separation and the likelihood of LBT failure on consecutive POs depends on the congestion level of the unlicensed channel. Therefore, the additional POs should be adjusted by the gNB, depending of the channel congestion level among other things. Given above, increasing POs within a paging frame (PF) is less likely to enhance the chance of a successful LBT in the subsequent POs within a PF.

From the perspective of a UE in Idle/Inactive state, more time-domain paging occasions per DRX cycle would be required so that a UE would be able to receive the page. While increasing time-domain POs enhances the chance of successful transmission, a UE would be required to check for paging in more POs. Compared to baseline licensed operation, this increases the power consumption of a UE in Idle/Inactive state. One might suggest that once a UE receives a page, the UE may skip monitoring the remaining POs, hence avoiding unnecessary PDCCH/PDSCH detection. This indeed helps and was agreed in the RAN2#105bis [3] meeting:

- As a starting point: If UE receives on PDCCH addressed to P-RNTI in a PDCCH monitoring occasion for paging corresponding to an SSB in a PO, UE is not required to monitor subsequent PDCCH monitoring occasions corresponding to that SSB in that PO

One should also consider the case where LBT fails in the first X POs until LBT is successful at the next PO. A UE would have to monitor X+1 POs until it can detect a PDCCH addressed to P-RNTI in a PDCCH monitoring occasion. Therefore, increasing time-domain paging occasions increases the power consumptions, especially in a dense unlicensed channel where LBT failure is frequent.

Consider a situation where a gNB is able to perform LBT successfully and transmit during the first PO. Considering the maximum number of UEs possible to be paged within a paging message, it may happen that the number of paged UEs exceed the limit. Therefore, we suggest to study whether multiple POs associated with a baseline PF can carry different paging messages (i.e. paging various UEs).

Proposal 4: FFS whether multiple POs associated with a baseline PF can carry different paging messages.

# 3	Conclusions

This contribution proposes the following regarding NR-U paging enhancements:

Proposal 1: To enhance UE power consumption, monitoring an extended PO should be subject to lack of a gNB-initiated COT in the assigned PO. Also, once a gNB-initiated COT is detected, the UE may stop monitoring remaining POs of an extended paging opportunity.

Proposal 2: RAN2 to consider early notification of an upcoming paging by first detecting a gNB-initiated COT and retrieving information within COT format indicator, which indicates whether any paging message is carried within COT or not.

Proposal 3: Inform RAN1 of allocation of bit-space within COT format indicator to indicate presence of paging message(s) within current or next COTs.

Proposal 4: FFS whether multiple POs associated with a baseline PF can carry different paging messages.

# References

RP-182878, “New WID on NR-based Access to Unlicensed Spectrum,” Qualcomm, RAN#82, December 2018

RAN2 #105 Chairman notes, Athens, Greece, RAN2#105, February-March 2019

RAN2 #106 Chairman notes, Reno, USA, RAN2#106, May 2019

RAN2 #107 Chairman notes, Prague, Czech Republic, RAN2#107, August 2019