3GPP TSG-RAN2 Meeting #107bis 	R2-1913971

# Chongqing, China,  14 – 18 October 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | 4138 | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Handover in (NG)EN-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Google Inc., Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-10-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | A UE suspends a SCG DRB due to SCG failure. The current text prevents the network from initiating handover for the UE with a suspended DRB in SCG. - the mobilityControlInfo is included only when AS-security has been activated, and SRB2 with at least one DRB are setup and not suspended; If a handover condition is triggered, e.g., by measurement reports from the UE which has a suspended DRB in SCG, the network is not able to initiate handover for the UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The mobilityControlInfo is included only when AS-security has been activated, SRB2 is setup and not suspended, and at least one DRB is setup. Impact analysis Impacted 5G architecture options: (NG)EN-DC Impacted functionality: Handover Inter-operability: If the UE is implemented according to the CR but the network is not, there is no inter-operability issue. However, the network is not able to initiate handover for the UE which has a suspended DRB in SCG. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The network is not able to initiate handover for the UE which has a suspended DRB in SCG. The UE with a suspended SCG cannot be handed over. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 5.3.5	RRC connection reconfiguration

#### 5.3.5.1	General

Figure 5.3.5.1-1: RRC connection reconfiguration, successful

Figure 5.3.5.1-2: RRC connection reconfiguration, failure

The purpose of this procedure is to modify an RRC connection, e.g. to establish/ modify/ release RBs, to perform handover, to setup/ modify/ release measurements, to add/ modify/ release SCells. As part of the procedure, NAS dedicated information may be transferred from E-UTRAN to the UE.

#### 5.3.5.2	Initiation

E-UTRAN may initiate the RRC connection reconfiguration procedure to a UE in RRC_CONNECTED. E-UTRAN applies the procedure as follows:

-	the mobilityControlInfo is included only when AS-security has been activated, SRB2 is setup and not suspended, and at least one DRB is setup;

-	the establishment of RBs (other than SRB1, that is established during RRC connection establishment) is included only when AS security has been activated;

-	the addition of SCells is performed only when AS security has been activated;