3GPP TSG-RAN WG2 Meeting #108		R2-1914695

Reno, USA, 18th - 22th November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.321 | CR | 1460 | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Running CR for even further mobility enhancement in E-UTRAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_feMob-Core |  |  |  |  |  | Date: |  |  | 2019-11-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To capture agreements for even further mobility enhancement in E-UTRAN into MAC specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of even further mobility enhancement in E-UTRAN. This CR captures the MAC aspects of feMob and it is based on the RAN2 agreements made so far. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Even further mobility enhancement in E-UTRAN is missing in MAC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 4.2.1, 5.1.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of change

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AUL	Autonomous Uplink

BL	Bandwidth reduced Low complexity

BR	Bandwidth Reduced

BSR	Buffer Status Report

C-RNTI	Cell RNTI

CBR	Channel Busy Ratio

CC-RNTI	Common Control RNTI

CG	Cell Group

CQI	Channel Quality Indicator

CRI	CSI-RS Resource Indicator

CSI	Channel State Information

DAPS		Dual Active Protocol Stack

DRB	Data Radio Bearer

EDT	Early Data Transmission

eIMTA	Enhanced Interference Management and Traffic Adaptation

eIMTA-RNTI	Enhanced Interference Management and Traffic Adaptation - RNTI

E-UTRA	Evolved UMTS Terrestrial Radio Access

E-UTRAN	Evolved UMTS Terrestrial Radio Access Network

G-RNTI	Group RNTI

H-SFN	Hyper SFN

MAC	Medium Access Control

MCG	Master Cell Group

M-RNTI	MBMS RNTI

MPDCCH	MTC Physical Downlink Control Channel

LCG	Logical Channel Group

NB-IoT	Narrow Band Internet of Things

NPDCCH	Narrowband Physical Downlink Control Channel

NPDSCH	Narrowband Physical Downlink Shared channel

NPRACH	Narrowband Physical Random Access Control Channel

NPUSCH	Narrowband Physical Uplink Shared channel

PCell	Primary Cell

PSCell	Primary Secondary Cell

PHR	Power Headroom Report

PMI	Precoding Matrix Index

PPPP	ProSe Per-Packet Priority

P-RNTI	Paging RNTI

ProSe	Proximity-based Services

pTAG	Primary Timing Advance Group

PTI	Precoding Type Indicator

RA-RNTI	Random Access RNTI

RAI	Release Assistance Indication

RI	Rank Indicator

RN	Relay Node

RNTI	Radio Network Temporary Identifier

SCell	Secondary Cell

SC-FDM	Single-Carrier Frequency Division Multiplexing

SCG	Secondary Cell Group

SCI	Sidelink Control Information

SC-N-RNTI	Single Cell Notification RNTI

SC-PTM	Single Cell Point to Multipoint

SC-RNTI	Single Cell RNTI

SI-RNTI	System Information RNTI

SL	Sidelink

SL-RNTI	Sidelink RNTI

SL-V-RNTI	Sidelink V2X RNTI

SR	Scheduling Request

SRS	Sounding Reference Symbols

SRS-TPC-RNTI	Sounding Reference Symbols-Transmit Power Control-RNTI

SpCell	Special Cell

sTAG	Secondary Timing Advance Group

sTTI	Slot or subslot TTI

TAG	Timing Advance Group

TB	Transport Block

TPC-PUCCH-RNTI	Transmit Power Control-Physical Uplink Control Channel-RNTI

TPC-PUSCH-RNTI	Transmit Power Control-Physical Uplink Shared Channel-RNTI

V2X	Vehicle-to-Everything

Next change

4.2.1	MAC Entities

E-UTRA defines two MAC entities; one in the UE and one in the E-UTRAN. These MAC entities handle the following transport channels:

-	Broadcast Channel (BCH);

-	Downlink Shared Channel(s) (DL-SCH);

-	Paging Channel (PCH);

-	Uplink Shared Channel(s) (UL-SCH);

-	Random Access Channel(s) (RACH);

-	Multicast Channel(s) (MCH);

-	Sidelink Broadcast Channel (SL-BCH);

-	Sidelink Discovery Channel (SL-DCH);

-	Sidelink Shared Channel (SL-SCH).

The exact functions performed by the MAC entities are different in the UE from those performed in the E-UTRAN.

The RN includes both types of MAC entities; one type for communication with UEs and one type for communication with the E-UTRAN.

In Dual Connectivity, two MAC entities are configured in the UE: one for the MCG and one for the SCG. In DAPS handover, two MAC entities are configured in the UE: one MAC entity for the source cell and one MAC entity for the target cell. Each MAC entity is configured by RRC with a serving cell supporting PUCCH transmission and contention based Random Access. In this specification, the term SpCell refers to such cell, whereas the term SCell refers to other serving cells. The term SpCell either refers to the PCell of the MCG or the PSCell of the SCG depending on if the MAC entity is associated to the MCG or the SCG, respectively. A Timing Advance Group containing the SpCell of a MAC entity is referred to as pTAG, whereas the term sTAG refers to other TAGs.

The functions of the different MAC entities in the UE operate independently if not otherwise indicated. The timers and paramenters used in each MAC entity are configured independently if not otherwise indicated. The Serving Cells, C-RNTI, radio bearers, logical channels, upper and lower layer entities, LCGs, and HARQ entities considered by each MAC entity refer to those mapped to that MAC entity if not otherwise indicated.

If the MAC entity is configured with one or more SCells, there are multiple DL-SCH and there may be multiple UL-SCH and RACH per MAC entity; one DL-SCH, one UL-SCH, and one RACH on the SpCell, one DL-SCH, zero or one UL-SCH and zero or one RACH for each SCell.

The physical layer may perform a listen-before-talk procedure, according to which transmissions are not performed if the channel is identified as being occupied or the physical layer may monitor for PUSCH trigger, as specified in TS 36.213 [2], according to which transmissions are not performed if PUSCH trigger B is not received. In both cases a MAC entity considers the transmission to have been performed anyway, unless stated otherwise.

Figure 4.2.1-1 illustrates one possible structure for the UE side MAC entity when SCG is not configured and for each MAC entity when the DAPS handover is configured, and it should not restrict implementation.

Editor’s Note: FFS which functions will be supported by the source and target MAC entity in DAPS HO.

Figure 4.2.1-1: MAC structure overview, UE side

Figure 4.2.1-2 illustrates one possible structure for the UE side MAC entities when MCG and SCG are configured, and it should not restrict implementation. MBMS reception and SC-PTM reception are excluded from this figure for simplicity.

Figure 4.2.1-2: MAC structure overview with two MAC entities, UE side

Figure 4.2.1-3 illustrates one possible structure for the UE side MAC entity when sidelink is configured, and it should not restrict implementation.

Figure 4.2.1-3: MAC structure overview for sidelink, UE side

Next change

5.1.6	Completion of the Random Access procedure

At completion of the Random Access procedure, the MAC entity shall:

-	discard explicitly signalled ra-PreambleIndex and ra-PRACH-MaskIndex, if any;

-	flush the HARQ buffer used for transmission of the MAC PDU in the Msg3 buffer;

-	If the Random Access Procedure is successfully completed by the target MAC entity when the DAPS handover is configured, the target MAC entity indicates the uplink data switching to the upper layers.

Editor’s Note: The terminology of “uplink data switching” indication is to be aligned with the PDCP specification. RACHless applicability can be discussed after procedure has progressed more.

In addition, the RN shall resume the suspended RN subframe configuration, if any.

End of change

# Annex – RAN2 agreements

RAN2#107 meeting:

Agreements

Reconfirm the following understanding on DAPS

1	For DAPS DL transmission/reception operation:

•	The source eNB and the target eNB perform header compression, ciphering and add PDCP header separately;

•	UE performs deciphering and header decompression for the DL PDCP SDUs received from the source eNB and target eNB separately; stores those PDCP SDUs in the common PDCP reception buffer and performs PDCP reordering; and then delivers the PDCP SDUs to upper layers in ascending order.

2	single UL new PUSCH data transmission as baseline and UE switches UL data transmission (new and unacknowledged PDCP SDUs) to target gNB upon reception of the first UL grant for data transmission from the target gNB after RA procedure towards the target gNB is successfully completed.

- As described in single UL new data transmission solution: For the DL data transmission, the UE continues to provide HARQ ACK/NACK, other CSI kind of feedback, ARQ ACK/NACK to the source eNB before release of the source cell connection.

FFS whether UL HARQ retransmissions continue

FFS whether RoHC feedback is needed

4	We do not restrict UP specifications without clear reason (e.g. BSR, PHR, etc.)

Agreements

- UE shall be able to send UL PUSCH user plane data to source eNB until the point when the message including RRC Connection Reconfiguration Complete has been successfully transmitted to target eNB.

- Rel-15 PDCP duplication via DC (from HRLLC WID) is not supported in combination with DAPS during handover.

3	For UL transmission operation during DAPS based HO.

•	UE maintains PDCP SN for UL PDCP PDUs in the common SN allocation function throughout the handover procedure;

•	Performs header compression and ciphering for the UL PDCP SDUs based on the destination of the PDU (source or target eNB);

•	Adds PDCP header and submits the PDCP date PDU to the lower layers associated to the destination of the PDU (source or target eNB);

•	FFS on whether security and ROHC are modelled as separate functions or not.

RAN2#107bis meeting:

Agreements for LTE and NR

1 	UE switches the UL PDCP data transmission upon successful RACH procedure (Msg2 for CFRA or Msg4 for CBRA).

2	The UE keeps the UL HARQ (re)transmission of the source link after UL data transmission switching to the target eNB.

3	When an uplink grant indicating the HARQ new transmission is received in the source link after UL data switching, the UE is expected to perform the corresponding UL transmission accordingly.

4	During Rel-16 RUDI handover, the UE only supports two links (i.e. the source MCG link and the target MCG link).

Agreements for LTE

- RACHless applicability can be discused after procedure has progressed more.

Agreements for NR

- FFS if Msg.B for 2-step RACH works the same.

Agreements for NR

UL new data transmission switching:

13	The indication to switch the UL new data transmission and will be specified in MAC.

14	After UL new data transmission switching, data available for transmission/the PDCP data volume is indicated to the MAC entity associated to the target eNB/gNB. UE starts retransmission of packets from the earliest unacknowedged SDU of source cell.

FFS how this is done in specification.

FFS if something different is needed for LTE than NR

15	After UL new data transmission switching, the size of the PDCP control PDUs containing the ROHC feedback to the source cell is indicated to the MAC entity associated to the source eNB/gNB as data available for transmission/the PDCP data volume.

Agreements

1	RAN2 adopts DAPS HO as the feature name used in all running CRs and LSs.

Agreements for NR

- We do not support TDM pattern.

- We leave it up to network implementation how to coordinate UL scheduling.

- For single UL transmission, we will not specify rules how UE handles which link to transmit if UL should be sent to both source and target.