3GPP TSG-RAN2 Meeting #108	R2-1915295

Reno, USA, 18-22 November 2019

Agenda Item:	6.11.3 UE assistance

Source: 	Ericsson

Title:  	UE Assistance Information for minimum K0 and K2

Document for:	Discussion and Decision

# Introduction

In this contribution UE assistance for preferred minimum K values is discussed further.

# Background

RAN1 indicated in an LS [1] that it has discussed cross-slot scheduling, and for higher layer signalling of suggested minimum applicable values for K0/K2 agreed:

UE higher layer signalling (detailed mechanisms up to RAN2) of suggested minimum applicable values for K0/K2 (one for each) for applying cross-slot scheduling is supported:

- For each of the all possible SCSs, the values are reported separately

- For same-carrier scheduling, each suggested value is in the range from 1 to

  - 15kHz/30kHz SCS: [2-4] slots

  - 60kHz/120kHz SCS: [4-8] slots

- FFS how to apply the values to the cross-carrier scheduling case in terms of minimum applicable value

# Discussion

Based on the RAN1 agreements it is proposed to introduce “suggested minimum applicable values for K0/K2” via UE assistance information:

Proposal 1: Introduce UE suggested minimum applicable values for K0/K2 (one for each) via UE assistance.

Proposal 2: The NW actions, if any, after receiving UE preference for minimum K values, is up to NW implementation and is not specified.

Proposal 3: A prohibit timer is configured when UE is configured to report preferred minimum K values.

In our understanding the preferred minimum K value signalling is motivated by different UE implementations for which the optimal power savings depend on the K value that is configured. For that reason we propose to introduce a “one-shot” solution, i.e. UE assistance is signalled after configuration only once. This can be realized by configuring an infinite prohibit timer or in the procedure rules that the UE shall only sent it once upon configuration (and not in case its preference has changed):

Proposal 4: UE assistance signalling for preferred minimum K values is “one-shot”.

RAN1 has working assumptions for the preferred minimum values the UE can signal for same-carrier scheduling, and those value for cross-carrier scheduling are still FFS:

Proposal 5: RAN2 to wait for RAN1 agreements on the value range for the preferred minimum K values.

# Summary

RAN2 is kindly asked to discuss UE assistance for preferred minimum K values:

Proposal 1: Introduce UE suggested minimum applicable values for K0/K2 (one for each) via UE assistance.

Proposal 2: The NW actions, if any, after receiving UE preference for minimum K values, is up to NW implementation and is not specified.

Proposal 3: A prohibit timer is configured when UE is configured to report preferred minimum K values.

Proposal 4: UE assistance signalling for preferred minimum K values is “one-shot”.

Proposal 5: RAN2 to wait for RAN1 agreements on the value range for the preferred minimum K values.

# References

- R1-1911586, LS on UE higher layer signalling for cross-slot scheduling, LS out, To: RAN2, RAN1#98bis