3GPP TSG-RAN2 Meeting #108	R2-1915883

Reno, Nevada, USA, 18 - 22 November 2019

Agenda Item:	[7.1.8] Improvements for non-BL UEs

Source: 	Apple Inc

Title:  	Improvements for non-BL UEs camping in CE Mode

Document for:	Discussion and Decision

# Introduction

In RAN2#107 meeting, following agreement was made.

FFS if, from Rel-16, it should be possible for a non-BL UE that fullfills S criteria for normal coverage to camp in a “normal” cell, i.e. not standalone, in enhanced coverage.

This discussion will continue as part of the AI 12.1.8 Improvements for non-BL UEs.

# Background

In RAN2#107bis meeting, [1] was discussed, and it was suggested that non-BL UEs with S criterion in normal coverage fulfilled, if allowed to camp in coverage enhancement mode would benefit from power savings, which it might otherwise not be able to if camped in normal coverage. As part of the discussion it was indicated that any such power saving should be presented.

In this contribution, we provide our observations and propose that a non-BL UE that fullfills S criteria for normal coverage to camp in a “normal” cell in CE mode.

# Discussion

## Uplink and Downlink PRB allocation distribution

Based on data from field, it is observed that NW on an average allocates 6 PRBs or less over a 20 MHz carrier bandwidth for about 85% of time on UL and about 50% of time on DL. Given this statistical distribution, UEs can still satisfy all the service requirements in a majority of scenarios, if the same number of PRBs are allocated over a smaller carrier bandwidth (e.g. 1.4 MHz as is the case with CE mode). The additional benefit is that UEs has to monitor a smaller carrier bandwidth.

Observation 1: NW resource allocation over 6 or less PRBs seems to be the most preferred configuration in a majority of UL (85%) and DL (50%) deployments. Continuing with the same deployment over a smaller carrier bandwidth does not seem to affect any UE service requirements.

## Power Savings

To compare the power savings for UE between mPDCCH and PDCCH, the test profile included a typical background and foreground data usage, with periodic NW signalling (TAU every 54 min and IMS registration every 2 hours).

The following PDCCH sub states were considered

- PDCCH only – UE attempts PDCCH decoding but no DL or UL grant decoded

  - Within this, there are three different sub-states

    - PDCCH Alone – RF ON only for PDCCH symbols (i.e. 3 symbols) and decode PDCCH offline when mid to high SNR

    - PDCCH Early – RF ON until PDCCH decoding is done in mid to high SNR

    - PDCCH Normal – RF ON for all 14 symbols within one sub frame (low SNR case)

- mPDCCH only – UE attempts to decoding within the 1.4 MHz narrowband but no grant decoded

Based on field test data and lab simulation it was observed that a UE camped in BR mode compared to normal mode achieves 13% power savings. This is especially useful for power limited devices.

Observation 2: UEs camping in BR mode compared to normal mode on an average save about 13% of power, resulting in increased battery performance.

Proposal 1: The current (pre-REL16) CE mode behavior should be carried over for REL16, wherein a REL16 non-BL UE should be allowed to camp in a cell in BR mode, even if it satisfies the S criteria for normal coverage.

# Summary

Observation 1: NW resource allocation over 6 or less PRBs seems to be the most preferred configuration in a majority of UL (85%) and DL (50%) deployments. Continuing with the same deployment over a smaller carrier bandwidth will not affect any UE service requirements.

Observation 2: UEs camping in BR mode compared to normal mode on an average save about 13% of power, resulting in increased battery performance.

Proposal 1: The current (pre-REL16) CE mode behavior should be carried over for REL16, wherein a REL16 non-BL UE should be allowed to camp in a cell in BR mode, even if it satisfies the S criteria for normal coverage.

# References

- R2-1913572 Enhancements to idle mode mobility for non-BL UEs, Huawei, HiSilicon RAN2#107bis