| 3GPP TS 38.340 V0.2.2 (2020-02) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Backhaul Adaptation Protocol (BAP) specification (Release 16) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Abbreviations	7

4	General	7

4.1	Introduction	7

4.2	Architecture	7

4.2.1	BAP structure	7

4.2.2	BAP entities	7

4.3	Services	8

4.3.1	Services provided to upper layers	8

4.3.2	Services expected from lower layers	8

4.4	Functions	8

5	Procedures	8

5.1	BAP entity handling	8

5.1.1	BAP entity establishment	8

5.1.2	BAP entity release	8

5.2	Data transfer	9

5.2.1	Transmitting operation	9

5.2.1.1	General	9

5.2.1.2	Routing	9

5.2.1.3	Bearer mapping	10

5.2.1.4	Routing identity selection	10

5.2.2	Receiving operation	11

5.3	Flow control feedback	11

5.4	Backhaul RLF indication	11

5.4.1	Transmitting operation	11

5.4.2	Receiving operation	11

5.5	Handling of unknown, unforeseen, and erroneous protocol data	12

6	Protocol data units, formats, and parameters	12

6.1	Protocol data units	12

6.1.1	Data PDU	12

6.1.2	Control PDU	12

6.2	Formats	12

6.2.1	General	12

6.2.2	Data PDU	12

6.2.3	Control PDU	13

6.3	Parameters	13

6.3.1	General	13

6.3.2	DESTINATION	13

6.3.3	PATH	13

6.3.4	Data	13

6.3.5	R	13

6.3.6	D/C	13

Annex A (informative): Change history	14

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, certain modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

NOTE 1:	The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

NOTE 2:	The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

NOTE 3:	The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

NOTE 4:	The constructions "can" and "cannot" shall not to be used as substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

NOTE 5:	The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document provides description of the Backhaul Adaptation Protocol (BAP).

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.300: "NG Radio Access Network; Overall description".

[3]	3GPP TS 38.331: "NR Radio Resource Control (RRC); Protocol Specification".

[4]	3GPP TS 38.322: "NR Radio Link Control (RLC) protocol specification".

[5] 	3GPP TS 38.473: "NG-RAN F1 application protocol (F1AP) protocol specification".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Backhaul RLC channel: An RLC channel between two nodes, which is used to transport backhaul packets.

Ingress RLC channel: An RLC channel on which a packet is received by a node.

Egress RLC channel: An RLC channel on which a packet is transmitted by a node.

Ingress link: A radio link on which a packet is received by a node.

Egress link: A radio link on which a packet is transmitted by a node.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

# 4	General

## 4.1	Introduction

The present document describes the functionalities of BAP.

## 4.2	Architecture

### 4.2.1	BAP structure

Figure 4.2.1.1 represents one possible structure for the BAP sublayer; it should not restrict implementation. The figure is based on the radio interface protocol architecture defined in TS 38.300 [2].

Figure 4.2.1-1: BAP layer, structure view

The BAP sublayer is configured by upper layers TS 38.331 [3] and TS 38.473 [5].

### 4.2.2	BAP entities

On the IAB-node, the BAP sublayer contains one BAP entity at the MT function and a separate BAP entity at

the DU function. On the IAB-donor DU, the BAP sublayer contains only one BAP entity. Each BAP entity has a transmitting part and a receiving part.

The transmit part of the BAP entity has a corresponding receiving part of a BAP entity at the IAB node or IAB donor DU across the backhaul link.

Figure 4.2.2.1

shows one example of the functional view of the BAP sublayer . This functional view should not restrict implementation. The figure is based on the radio interface protocol architecture defined in TS 38.300 [2].

In the example of Figure 4.2.2.1, the receiving part on the BAP sublayer delivers BAP PDUs to the collocated transmitting part on the BAP sublayer. Alternatively, the receiving part may delivers BAP SDUs to the collocated transmitting part. When passing BAP SDUs, the receiving part removes the BAP header and the transmitting part adds the BAP header with the same BAP routing ID as carried on the BAP PDU header prior to removal. Passing BAP SDUs in this manner is therefore functionally equivalent to passing BAP PDUs, in implementation. The following specification therefore refers to the passing of BAP data Units.

Figure 4.2.2.1. Example of functional view of BAP sublayer

## 4.3	Services

### 4.3.1	Services provided to upper layers

The following services are provided by the BAP sublayer to upper layers:

-	data transfer;

### 4.3.2	Services expected from lower layers

- A BAP sublayer expects the following services from lower layers per RLC entity (for a detailed description see TS 38.322 [4]):

-	acknowledged data transfer service;

-	unacknowledged data transfer service.

## 4.4	Functions

The BAP sublayer supports the following functions:

-	Transfer of data;

-	Routing of packets to next hop;

-	Determination of BAP destination and path for packets from upper layers;

-	Determination of egress RLC channels for packets routed to next hop;

-	Differentiating traffic to be delivered to upper layers from traffic to be delivered to egress link;

-	Flow control feedback signalling;

-	Backhaul RLF indication;

## 4.5	Configurations

The configuration of the BAP sublayer includes:

-	The IAB-node’s BAP address via RRC.

-	Mapping from next hop BAP address to downstream egress link via F1AP.

- 	Mapping from next hop BAP address to upstream egress link via RRC.

-	Mapping from upper layer traffic to BAP routing ID in BAP header via F1AP and RRC.

-	The BAP routing entries via F1AP.

-    Mapping to egress RLC channels via F1AP and RRC.

Backhaul RLC channels are configured via RRC on the IAB-MT function of the IAB-node and via F1AP on the IAB-DU function on the IAB-node and DU function on the IAB-donor.

# 5	Procedures

## 5.1	BAP entity handling

### 5.1.1	BAP entity establishment

When upper layers request establishment of the BAP entity at the IAB-MT, the node shall:

-	establish a BAP entity;

-	follow the procedures in subclause 5.2.

### 5.1.2	BAP entity release

When upper layers request a BAP entity release, the node shall:

[-	discard all stored BAP SDUs and BAP PDUs ;]

-	release the BAP entity.

Editor’s Notes: FFS BAP has buffer in RX part and TX part. If there is no buffer at all, there is no need to specify the operation to discard the stored SDUs/PDUs.

## 5.2	Data transfer

### 5.2.1	Transmitting operation

#### 5.2.1.1	General

The transmitting part of the BAP entity on the IAB-MT can receive BAP SDUs from upper layers and BAP Data Units from the receiving part of the BAP entity on the IAB-DU of the same IAB node. The transmitting part of the BAP entity on the IAB-DU can receive BAP Data Units from the receiving part of the BAP entity on the IAB-MT of the same IAB node. The transmitting part of the BAP entity on the IAB-donor DU can receive BAP SDUs from upper layers.

Upon receiving a BAP data Unit from the receiving part of the associated BAP entity, the transmitting part of the BAP entity shall:

-	if the BAP header of the received BAP Data PDU was stripped off:

-	add the BAP header with the same value as it had on the received BAP Data PDU.

Upon receiving a BAP SDU from upper layers, the transmitting part of the BAP entity shall:

-	select a BAP address and a BAP path identity for this BAP SDU in accordance with sub-clause 5.2.1.4;

-	construct a BAP Data PDU by adding a BAP header to the BAP SDU, where the DESTINATION field is set to the selected BAP address and the PATH field is set to the selected BAP path identity, in accordance with sub-clause 6.2.2;

When the BAP entity has a BAP Data PDU to transmit, the transmitting part of the BAP entity shall:

-	perform routing to determine the egress link in accordance with sub-clause 5.2.1.2;

-	determine the egress RLC channel in accordance with sub-clause 5.2.1.3;

-	submit this BAP Data PDU to the selected egress RLC channel of the selected egress link.

NOTE:  	Data buffering on the transmitting part of the BAP sublayer,  e.g., until RLC-AM entity has received an acknowledgement, is up to implementation.  The transmitting part of BAP entity could reroute the BAP data, which has not been acknowledged by lower layer before the backhaul RLF, to the new path (once established and configured) in case of backhaul RLF.

#### 5.2.1.2	Routing

The BAP entity performs routing based on the BH Routing Configuration received via an F1AP BH ROUTING CONFIGURATION message as specified in TS 38.473 [5]. Each entry of the BH Routing Configuration contains a BAP Routing ID consisting of a BAP address and  [optionally] a BAP path identity, and a Next Hop BAP Address.

For a BAP Data PDU to be transmitted, BAP entity shall:

-	if the transmission is in upstream direction, and

- 	if there is no BH Routing Configuration configured in accordance with TS 38.473 [5] (e.g. during IAB-node integration phase):

-	select any egress link;

-	else if there is an entry in the BH Routing Configuration whose  BAP address is the same as the DESTINATION field, whose path identity is the same as the PATH field, and whose egress link corresponding to the Next Hop BAP Address  is available:

-	select the egress link corresponding to the Next Hop BAP Address of the entry;

NOTE:	An egress link is not considered to be available if the link is in RLF.

NOTE:	For each combination of a BAP address and a BAP path ID, there should be at most one entry in the BH Routing Configuration.

-	else if there is at least one entry in the BH Routing Configuration whose BAP address is the same as the DESTINATION field, and whose egress link corresponding to the Next Hop BAP Address is available:

- 	select an entry from the BH Routing Configuration  whose BAP address  is the same as the DESTINATION field, and whose egress link corresponding to the Next Hop BAP Address  is available;

-	select the egress link corresponding to the Next Hop BAP Address of the entry selected above;

Editor’s Notes: the exact field names and signalling organization are TBD.

Editor’s Notes: FFS on if and how to use priority.

5.2.1.3	Mapping to BH RLC Channel

For a BAP Data PDU received from the ingress logical channel, the BAP entity performs mapping to an egress logical channel based on a backhaul RLC channel mapping configuration, which is contained in [backhaulRLC-ChannelMappingConfigRLClayer] configured on the IAB-node.

Each entry of the [backhaulRLC-ChannelMappingConfigRLClayer] contains [egressLinkID], [ingressLinkID], [ingressBH-RLC-ID] and [egressBH-RLC-ID].

Editor’s Note: RAN3 to decide if separate mapping configurations are necessary for UL or DL.

For a BAP Data PDU received from an ingress RLC channel of an ingress link and for which the egress link has been selected:

-	select an entry from the [backhaulRLC-ChannelMappingConfigRLClayer], whose [ingressBH-RLC-ID] is the same as the [BH-RLC-ID] of the ingress RLC channel, whose [ingressLinkID] corresponds to the BAP Data PDU’s ingress link, and whose [egressLinkID] corresponds to the selected egress link;

-	select the egress RLC channel corresponding to [egressBH-RLC-ID] of the entry selected above;

For a BAP Data PDU received from upper layers at the IAB-node, the BAP entity performs mapping to an egress logical channel based on a backhaul RLC channel mapping configuration, which is contained in [backhaulRLC-ChannelMappingConfigUpperLayers] configured on the IAB-node.

Each entry of the [backhaulRLC-ChannelMappingConfigUpperLayers] contains [trafficTypeSpecifier], [egressLinkIdentity] and [egressBH-RLC-ID].

Editor’s Note: FFS [trafficTypeSpecifier] to be defined by RAN3.

For a BAP SDU received from upper layers at the IAB-node for transmission in upstream direction, whose egress link has been selected as specified in clause 5.2.1.4, the BAP sublayer shall:

-	if there is no [backhaulRLC-ChannelMappingConfigUpperLayers] configured in accordance with [TS 38.473, 5]:

-	select the egress RLC channel corresponding to [defaultRLC] in [TS 38.331, 3];

-	else:

-	for the BAP SDU encapsulating an F1-U packet with an destination IP address and a TEID:

-	select an entry from the [backhaulRLC-ChannelMappingConfigUL] with its [destination IP address and TEID] same as the destination IP address and TEID of this BAP SDU and its [egressLinkIdentity] corresponding to the selected egress link;

-	for the BAP SDU encapsulating an F1-C packet with an F1-C type:

-	select an entry from the [backhaulRLC-ChannelMappingConfigUL] with its [F1-C-Type] same as the F1-C type of this BAP SDU and its [egressLinkIdentity] corresponding to the selected egress link;

-	select the egress RLC channel corresponding to [egressBH-RLC-ID] of the entry selected above;

At the IAB-donor-DU, for a BAP SDU received from upper layers for transmission in downstream direction, whose egress link has been selected, the BAP sublayer shall:

-	for the BAP SDU encapsulating an IPv6 packet:

-	select an entry from the [backhaulRLC-ChannelMappingConfigDL] with its [egressLinkIdentity] corresponding to the selected egress link, and the entry fulfils the following conditions:

-	the IPv6 Flow Label of this BAP SDU matches [Ipv6-flow-label] if configured; and

-	the DSCP (i.e. the most significant 6 bits of the Traffic Class field) of this BAP SDU matches [DSCP] if configured; and

-	the Destination IP address of this BAP SDU matches [Dest-IP-address] if configured;

-	for the BAP SDU encapsulating an IPv4 packet:

-	select an entry from the [backhaulRLC-ChannelMappingConfigDL] with its [egressLinkIdentity] corresponding to the selected egress link, and the entry fulfils the following conditions:

-	the DSCP of this BAP SDU matches [DSCP] if configured; and

-	the Destination IP address of this BAP SDU matches [Dest-IP-address] if configured;

-	select the egress RLC channel corresponding to [egressBH-RLC-ID] of the entry selected above;

Editor’s Notes: the exact names and signalling organization are TBD.

Editor’s Notes: it is FFS how to handle non-F1 packets.

#### 5.2.1.4	Routing identity selection

ForAt the BAP entity at IAB-donor-DUnode, for a BAP SDU received from upper layers for transmission in upstream direction, the BAP sublayer performs mapping to a BAP address and BAP path ID, which are contained in [UL BH Information] configured on the IAB-node.

Each entry of the [UL BH Information] contains [trafficTypeSpecifier], [BAP-address] and [pathID].

Editor’s Note: RAN3 to decide if separate mapping configurations are necessary for UL or DL.

At the IAB-node, for a BAP SDU received from upper layers for transmission in upstream direction, the BAP sublayer shall:

-	if there is no UL BH Information configured in accordance with [TS 38.473, 5]:

-		select the BAP address and the BAP path identity as configured by [defaultRouting-ID] in [TS 38.331, 3];

-	else:

-	for the BAP SDU encapsulating an F1-U packet:

- FFS

-	for the BAP SDU encapsulating an F1-C packet:

- FFS

Editor’s Notes: The procedure of routing ID selection for F1-U, F1-C and non-F1 packets is pending until RAN3 agrees the related signaling.

For a BAP Data SDU received from IP layer at the IAB-donor DU, the BAP sublayer performs mapping to a BAP address, which is contained in [BAP-AddressConfigDL] configured on the IAB-donor DU via F1AP, and mapping to a BAP path identifier which is contained in [pathIdentityConfigDL] configured on the IAB-donor DU via F1AP.

Each entry of the [BAP-AddressConfigDL] contains [Dest-IP-address] and [BAP-address]. Each entry of the [pathIdentityConfigDL] contains [IPheaderSpecifier] and [path-ID].

At IAB-donor-DU, for a BAP SDU received from upper layers for transmission in downstream direction, the BAP sublayer shall:

-	for the BAP SDU encapsulating an IPv6 packet:

-	select an entry from the [pathIdentityConfigDL] which fulfils the following conditions:

-	the IPv6 Flow Label of this BAP SDU matches [Ipv6-flow-label] if configured; and

-	the DSCP (i.e. the most significant 6 bits of the Traffic Class field) of this BAP SDU matches [DSCP] if configured; and

-	the Destination IP address of this BAP SDU matches [Dest-IP-address] if configured;

-	for the BAP SDU encapsulating an IPv4 packet:

-	select an entry from the [pathIdentityConfigDL] which fulfils the following conditions:

-	the DSCP (i.e. the most significant 6 bits of the Traffic Class field) of this BAP SDU matches [DSCP] if configured; and

-	the Destination IP address of this BAP SDU matches [Dest-IP-address] if configured;

-	select the BAP path identity corresponding to [path-ID] of the entry selected above if any;

-	select the BAP address corresponding to [BAP-address] of the entry from the [BAP-AddressConfigDL], whose [Dest-IP-address] is same as the Destination IP address of this BAP SDU;

Editor’s Notes: the exact field names and signalling organization are TBD.

### 5.2.2	Receiving operation

Upon receiving a BAP Data PDU from lower layer (i.e. ingress RLC channel), the receiving part of the BAP entity shall:

-	if DESTINATION field of this BAP PDU is same as the BAP address of this node:

-	remove the BAP header of this BAP PDU and deliver the BAP SDU to upper layers;

-	else:

-	deliver the BAP Data Unit to the transmitting part of the associated BAP entity on the same node.

5.3	Flow control feedback

For an egress link, the transmitting part of the BAP entity at the IAB-MT shall:

-	when a flow control feedback is triggered when the buffer load exceeds the certain level, or

-	when a flow control poll is received:

-	construct a BAP Control PDU for flow control feedback in accordance with sub-clause 6.2.3;

-	submit this BAP Control PDU to lower layers;

Editors Notes: How to signal the poll is FFS.

Editor’s Notes: Details will be captured for those agreement: “We support O1 and O2, which one to use is configurable. 	R2 assumes that e.g. when the buffer load exceeds the certain level, the DL hop-by-hop flow control feedback should be triggered, the details of this trigger is left for implementation (in this Rel). We use Available or desired buffer size (absolute e.g. MB kB)”

Editors Notes: FFS which RLC channel the BAP sublayer submits the BAP control PDU for flow control feedback.

## 5.4	Backhaul RLF indication

### 5.4.1	Transmitting operation

When a backhaul RLF recovery failure is detected at the IAB-MT, for each egress link associated with the IAB-DU, the transmitting part of the BAP entity at the IAB-DU:

-	constructs a BAP Control PDU for backhaul RLF indication in accordance with sub-clause 6.2.3:

-	submits this BAP Control PDU to lower layers:

Editor’s Notes: FFS the content of the control PDU for backhaul RLF indication and whether a control PDU contains a PDU header including e.g., destination address and path ID.

Editor’s Notes: FFS which RLC channel the BAP sublayer submits the BAP control PDU for backhaul RLF indication.

### 5.4.2	Receiving operation

Upon receiving a BAP Control PDU for backhaul RLF indication from lower layer (i.e. ingress RLC channel), the receiving part of the BAP sublayer shall:

-	[indicate to upper layers that the backhaul RLF indication has been received for the ingress link where this BAP Control PDU is received.]

## 5.5	Handling of unknown, unforeseen, and erroneous protocol data

When a BAP PDU that contains reserved or invalid values or contains a BAP address which is not included in the configured BH routing information received, the BAP sublayer shall:

-	discard the received PDU.

# 6	Protocol data units, formats, and parameters

## 6.1	Protocol data units

### 6.1.1	Data PDU

The BAP Data PDU is used to convey one of the following in addition to the PDU header:

-	upper layer data;

### 6.1.2	Control PDU

The BAP Control PDU is used to convey one of the following [in addition to the PDU header]:

-	flow control feedback;

-	backhaul RLF indication;

Editor’s Notes: FFS whether a control PDU contains a PDU header including e.g., destination address and path ID.

## 6.2	Formats

### 6.2.1	General

A BAP PDU is a bit string that is byte aligned (i.e. multiple of 8 bits) in length. The formats of BAP PDUs are described in sub clause 6.2.2, 6.2.3 and their parameters are described in sub clause 6.3.

### 6.2.2	Data PDU

Figure 6.2.2-1 shows the format of the BAP Data PDU.

Figure 6.2.2-1: BAP Data PDU format

6.2.3	Control PDU

Editor’s Notes: Details are FFS

## 6.3	Parameters

### 6.3.1	General

If not otherwise mentioned in the definition of each field, the bits in the parameters shall be interpreted as follows: the left most bit string is the first and most significant and the right most bit is the last and least significant bit.

Unless otherwise mentioned, integers are encoded in standard binary encoding for unsigned integers. In all cases the bits appear ordered from MSB to LSB when read in the PDU.

### 6.3.2	DESTINATION

Length: 10 bits.

This field carries BAP address of destination of IAB node or IAB-donor.

### 6.3.3	PATH

Length: 10 bits.

This field carries BAP path identity.

### 6.3.4	Data

Length: Variable

This field carries the BAP SDU (i.e. IP packet).

### 6.3.5	R

Length: 1 bit

Reserved. In this version of the specification reserved bits shall be set to 0. Reserved bits shall be ignored by the receiver.

### 6.3.6	D/C

Length: 1 bit

This field indicates whether the corresponding BAP PDU is a BAP Data PDU or a BAP Control PDU.

Table 6.3.6-1: D/C field

| Bit | Description |
| --- | --- |
| 0 | BAP Control PDU |
| 1 | BAP Data PDU |

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2019.10 | RAN2#107bis | R2-1914006 |  |  |  | skeleton | 0.0.0 |
| 2019.10 | RAN2#107bis | R2-1914008 |  |  |  | Captured agreements made before RAN2#107bis | 0.1.0 |
| 2020.02 | RAN2#109e | R2-2000990 |  |  |  | Captured agreements made in RAN2#108 and RAN2#109e | 0.2.1 |