3GPP TSG-RAN WG2 Meeting #109 electronic	draft_R2-2xxx

24th Feb - 6 Mar 2020

Agenda Item:	6.1.1

Source: 	Huawei, HiSilicon

Title:	Open issue discussion for TS 38.304 and 36.304

Document for:	Discussion and Decision

# Introduction

This paper aims at capturing the summary of following email discussion

R2-2002166	Correction of TS 38.304 to introduce IAB	Huawei, HiSilicon	CR	Rel-16	38.304	15.6.0	0150	1	B	NR_IAB-Core

- [AT109e][016][IAB] Endorsed as baseline

R2-2002167	Correction of TS 36.304 to introduce IAB	Huawei, HiSilicon	CR	Rel-16	36.304	15.5.0	0784	1	B	NR_IAB-Core

- [AT109e][016][IAB] Endorsed as baseline

- [AT109e][016][IAB] Idle CRs 38304 36304 (Huawei)

Scope: Progress xx304 CRs

Part 1 (if needed)

Intended outcome: Endorsed CRs, revision with tdoc number

Deadline: Feb 26 0900 CET

Part 2:

Intended outcome: Address Open issues, take this meeting’s agreements into account, as they become available. Produce final agreed CR.

Deadlines: Mar 4, 5, 6 (see the schedule).

# Issues

- Issue 1: Description for Cell status and cell reservations

In running CR, companies have different understanding of the need of following highlight part.

Cell status and cell reservations are indicated in the MIB or SIB1 message as specified in TS 38.331 [3] by means of following fields: - cellBarred (IE type: "barred" or "not barred") Indicated in MIB message. In case of multiple PLMNs indicated in SIB1, this field is common for all PLMNs - cellReservedForOperatorUse (IE type: "reserved" or "not reserved") Indicated in SIB1 message. In case of multiple PLMNs indicated in SIB1, this field is specified per PLMN. - cellReservedForOtherUse (IE type: "true") Indicated in SIB1 message. In case of multiple PLMNs indicated in SIB1, this field is common for all PLMNs. NOTE: For IAB node, it ignores the cellBarred, cellReservedForOperatorUse and cellReservedForOtherUse as defined in TS 38.331 [3]. Editor Notes: This can be updated based on the possible agreements by discussion of “R2-2002058, Summary of 6.1.5.3: SI Broadcast, cell Restrictions/Reservation and Barring, Initial Access, and Connection Setup, Ericsson”. - iab-Support (IE type: "true") Indicated in SIB1 message. In case of multiple PLMNs indicated in SIB1, this field is specified per PLMN. [This field indicates if the cell is barred for IAB node or the cell does not support IAB node, or both. When this field is absent, the IAB node shall treat this cell as if cell status is barred] Editor Notes: The need of the text in bracket is FFS.

To reduce the ambiguity, rapporteur proposes to remove the text in bracket, since the field description in TS 3x.331 is enough to clarify the meaning of iab-Support.

Q1: Companies are asked if you are fine to remove the text in bracket. If not, please provide the comments.

| Companies | Yes or No | Comments |
| --- | --- | --- |
| CATT | Yes | Agree with rapporteur. No need for duplication in specs. We didn’t do this for IEs like cellBarred, … |
| ZTE | Yes |  |
| Ericsson | No | In our view, for a better understanding, the spec should provide some details, like in LTE 36.304. So, we see no strong motivation to change it from LTE 36.304. |
| Nokia | Yes | There is no need to repeat the text from 38.331. It is already captured: Cell status and cell reservations are indicated in the MIB or SIB1 message as specified in TS 38.331 [3]. In case of corrections, changes need to be applied to both specs unnecessarily. |

- Issue 2: IAB-MT orations after cell status "barred" is indicated

When cell status "barred" is indicated or to be treated as if the cell status is "barred", - The UE is not permitted to select/reselect this cell, not even for emergency calls. - The UE shall select another cell according to the following rule: - If the cell is to be treated as if the cell status is "barred" due to being unable to acquire the MIB: - the UE may exclude the barred cell as a candidate for cell selection/reselection for up to 300 seconds. - the UE may select another cell on the same frequency if the selection criteria are fulfilled. - else: - If operating as an IAB-MT and the cell is to be treated as if the cell status is "barred" due to iab-Support being absent in SIB1: - The IAB-MT may exclude the barred cell as a candidate for cell selection/reselection for up to 300 seconds; Editor Notes: FFS on the need of above text. - If the cell is to be treated as if the cell status is "barred" due to being unable to acquire the SIB1 or due to trackingAreaCode being absent in SIB1 as specified in TS 38.331 [3]: - The UE may exclude the barred cell as a candidate for cell selection/reselection for up to 300 seconds. - If the field intraFreqReselection in MIB message is set to "allowed", the UE may select another cell on the same frequency if re-selection criteria are fulfilled; - The UE shall exclude the barred cell as a candidate for cell selection/reselection for 300 seconds. - If the field intraFreqReselection in MIB message is set to "not allowed" the UE shall not re-select a cell on the same frequency as the barred cell; - The UE shall exclude the barred cell and the cells on the same frequency as a candidate for cell selection/reselection for 300 seconds.

This text was proposed by Nokia. Rapporteur quotes their explanation below:

“If we don’t have it, then by following the current 38.304 specifications, the IAB-MT may be easily stuck on the cell which does not provide support for IAB (as it will be the best cell according to the ranking). This is exactly the same reason why this is specified for normal UEs for cellBarred etc. – to avoid them being stuck on the barred cells. Please also note that the wording used is “IAB-MT may”, so the IAB-MT may actually check for the IAB indication status changes as often as it wants to avoid delay in IAB node integration.”

Rapporteur has sympathy to consider the proposal text, but has different interpretation for the text.

Without the added text, IAB-MT would perform normal UE behaviors, highlighted by green, since the cell is barred. That means IAB-MT “shall exclude the barred cell as a candidate for cell selection/reselection for 300 seconds”. But, with the added text, IAB-MT is allowed to re-consider and check the barred cell within 300 seconds.

Q2: Companies are asked if you think the highlighted text is helpful to be added.

| Companies | Needed? | Comments |
| --- | --- | --- |
| CATT | Yes | We think it is OK to add this. IAB is a network node and we think ‘MAY’ is OK as Nokia mentioned in the previous comments. The part with green highlight is for UE and it is ‘shall’, we do not want to touch those so we are ok with adding yellow highlight part to make the IAB MT behavior clearer. |
| ZTE | No | We think it is not necessary to add the highlighted text. For the cell selection and re-selection procedure description in 38.304, we think the description for UE applies to both normal UE and IAB node MT unless specified otherwise. So the text highlighted by green should also apply to IAB node MT. |
| Ericsson | No | The MT assesses if it should consider the cell as barred or not and then, the actions are already captured in the specification. So we do not think if anything extra is needed here. The LTE 36.304 specifications also followed the same approach to new IEs. |
| Nokia | Yes | The actions are specified for other reasons for treating cell as barred, but not for iab-Support indication. It should be clarified that a barred cell can be reconsidered for reselection/selection after 300s as the status may change. The text highlighted in green refers to indication in MIB, which we agreed will be ignored by the IAB-MT, so it does not seem to be applicable. If this is not deemed to be needed then we wonder why we would have similar text for other cell reservation indications. |

# Conclusion and proposals

TBD.