3GPP TSG-RAN WG2 Meeting #109 electronic	R2-200xxx

24th Feb - 6 Mar 2020

Agenda Item:	6.1.3

Source: 	Huawei, HiSilicon

Title:	[AT109e][021][IAB] BAP functionality

Document for:	Discussion and Decision

# Introduction

This paper aims at capturing the summary of following email discussion

- [AT109e][021][IAB] BAP functionality (Huawei)

Scope: Treat remaining parts to be treated from email discussion and from summary

Intended outcome: resolution to issues, solutions.

Deadline: Mar 3 1200 CET (from conclusions pow focus on easy agreements)

This document merges the remaining essential issues discussed in the two summaries:

R2-2000989	Summary of email discussion 108#51 on BAP open issue	Huawei	discussion

R2-2002055	Summary on BAP functionality in AI 6.1.3	Huawei, HiSilicon

# BAP general

- Issue 2.1: Whether BAP has transmission buffer

This issue is discussed by R2-2001562 (LG), proposing the BAP TX buffer, and also captured as FFS in draft BAP TS.

As to the BAP TS, there are two options to assume the BAP layer buffer:

Option 1: The specification assumes that BAP layer has transmission buffer;

Option 2: Transmission buffer at BAP layer is implementation, i.e. no need to specify the BAP buffer in the specification;

Based on this paper and previous discussion on this issue, rapporteur thinks it is the majority understanding that there is a transmission buffer at BAP but the buffer is managed by implementation, i.e. no specification impacts in Rel-16. I think this is also in line with this paper which only proposes that there is a buffer at BAP but without further specification impacts proposed. The rapporteur therefore proposes the following for RAN2 discussion:

Proposal 1: There may be a transmission buffer at BAP layer by implementation. R16 will not specify BAP buffer related operations in specifications.

Companies are asked if the above proposal is agreeable. If not, please provide the comments or any other compromised wording.

| Companies | Yes or No | Comments |
| --- | --- | --- |
| LG | No, but | We understand that there is a majority view on this, but would like to leave one comment on this. We are ok with not specify BAP buffer related operation. However, according to the agreements so far, a BSR only includes RLC data volume in IAB. With this, this BSR may not give sufficient scheduling information to the network in some cases, e.g., mobility handling with RLC reestablishment. So, we think that it would be good to have at least data volume calculation in BAP layer for BSR as in PDCP, i.e., Not want to specify detailed buffer operation. Proposal 1: There may be a transmission buffer at BAP layer by implementation. R16 will not specify BAP buffer related operations in specifications except data volume calculation. |
| Nokia |  | First sentence not needed, rephrase: “Proposal 1: R16 will not specify BAP buffer related operations in specifications.” Data volume in BAP can only be taken into account when the routing and BH RLC channel mapping has been done which in practice means that the data is already submitted to RLC layer. Thus, RLC data volume is enough. |
| ZTE | Yes |  |

# BAP bearer mapping

- Issue 3.1: The bearer mapping of non-F1 traffic

This issue was discussed in the email discussion [108#51], R2-2000989, which is copied here for you information

Q3.1: Do you agree to confirm that a specific routing ID and BH RLC channels can be configured for non-F1 traffic via F1AP and RRC (for bootstrapping)?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | Yes |  |
| QCOM | See comment | For non-bootstrapping case, RAN3 has already decided that a routing ID and BH RLC channel can be configured for non-F1 traffic. For bootstrapping, we already discussed in RAN2 that we would not configure a routing table, so there is no need for a traffic-type specific routing ID and BH RLC channel. Please let’s not discuss issues that have already been done! |
| KDDI | See comment | Same view as QCOM |
| CATT | Yes | We can follow RAN3 agreements for later stage. As captured in R2-1916641, during IAB-node integration, before F1AP is established, a default BH RLC channel and a default BAP routing ID are configured via RRC, which are used for all upper layer traffic. We see no issue given the current progress. |
| LG | Yes |  |
| Ericsson | Yes |  |
| Futurewei | Yes | Agree with the comment from CATT |
| OMESH | Yes |  |
| Nokia, Nokia Shanghai Bell | Yes | However, it seems we need only single BAP routing ID and BH RLC channel for all kinds of traffic to be used for bootstrapping. |
| ZTE | Yes | The BAP routing ID configured via RRC during bootstrapping could be used for non-F1 traffic. After IAB node integration, the F1AP based BAP routing ID configuration could be used for non-F1 traffic. |
| Huawei, HiSilicon | Yes |  |

Summary: Majorities have replied yes, but the suggested solutions may be different according to the comments. Some companies announced that RAN3 has already decided that a routing ID and BH RLC channel can be configured for non-F1 traffic (which is an agreement I cannot find. Need double check). Some others suggest that the default BH RLC channel and default BAP routing ID as configured by RRC are used for non-F1 traffic. Some further suggest that during bootstrapping the BAP routing ID and BH RLC channel as configured by RRC are used for non-F1 traffic, and after IAB node integration, F1AP configuration is used for non-F1 traffic.

Based on the inputs from companies, the following proposal is proposed:

Proposal 2: After bootstrapping, the specific routing ID and BH RLC channel as configured by F1AP are used for non-F1 traffic.

Companies are asked if the above proposal is agreeable. If not, please provide the comments or any other compromised wording.

| Companies | Yes or No | Comments |
| --- | --- | --- |
| Nokia | Yes |  |
| ZTE | Yes |  |

- Issue 3.2: The BH RLC channel to be used for BAP control PDU

This issue was discussed in the email discussion [108#51], R2-2000989, which is copied here for you information

Q7.3: Except the implicit way in Q4.3, how is the BH RLC channel determined to transmit for BAP control PDUs including flow control feedback and BH RLF indication?

Option-1: the BH RLC channel for control PDUs is configured by F1AP or RRC;

Option-2: BAP selects any BH RLC channel to transmit control PDUs.

| Company | Yes or No | Comments |
| --- | --- | --- |
| Samsung | Depends… | - Per BH RLC CH reporting: the BL RLC CH for transmitting control PDU is the one the flow control information refers to. - Per Routing ID reporting: any BH RLC CH can be selected as long as routing ID is included. - RLF reporting: any BH RLC CH can be used. |
| QCOM | Option 1 | Straightforward. Easy to implement. |
| CATT | Option 1 |  |
| LG | Option 1 |  |
| Ericsson | Option 2 |  |
| Futurewei | Option 1 |  |
| OMESH | Option 2 |  |
| Nokia, Nokia Shanghai Bell | Option 1 | To avoid usage of UM mode BH RLC channels. |
| ZTE | Option 2 |  |
| Huawei, HiSilicon | Option 1 |  |

Summary: Majorities prefer to configure the BH RLC channel for control PDUs transmission.

The following contributions have been submitted to R2#109e meeting on this topic:

| Contribution | Proposals |
| --- | --- |
| R2-2000745 Further Discussion on BAP Layer Signaling, Ericsson | Proposal 1 No dedicated/separate BH RLC channels are needed for inter-IAB nodes BAP control signaling, rather the configured BH RLC channels for other types of traffic can be used to carry inter-IAB nodes signaling. Proposal 2 IAB nodes can map BAP control signaling messages to any of the configured BH RLC channels. |
| R2-2001565 Configuration of BH RLC channel for control PDU transmission, LG Electronics Inc. | Proposal. The BH RLC channel configuration for BAP control PDU transmission is configured by F1-AP. |
| R2-2000271 Discussion on BAP control PDU vivo | Proposal 3: In a UL/DL BH link, a specific BH RLC channel is configured for BAP control PDU transmissions. |

Companies’ views based on email discussion and contributions:

Proponents of Option 1 (7): Qualcomm, CATT, LG, Futurewei, Nokia, Huawei, vivo

Proponents of Option 2 (4): Samsung (mostly on Option 2), Ericsson, OMESH, ZTE

Given the situation, the following proposed is made:

Proposal 3a: The BH RLC channel is assigned for control PDU transmission by F1AP or RRC.

Companies are asked if the above proposal is agreeable. If not, please provide the comments or any other compromised wording.

| Companies | Yes or No | Comments |
| --- | --- | --- |
| Ericsson |  | As explained in our tdoc R2-2000745, no dedicated/separate BH RLC channels are needed for control PDU, rather the configured BH RLC channels for other types of traffic (data PDU) can be used to carry control PDU. However, if some companies want to have dedicated or separate BH RLC channels for the control PDU then it should be optional. So, we suggest this should be optional, not mandatory. |
| Nokia | Yes, by F1AP | Propose to rephrase: “The BH RLC channel is assigned for control PDU transmission by F1AP or RRC.” to avoid mixing this with configuration of RLC parameters (RLC-config). |
| ZTE | No | The control PDU is not transmitted frequently. Any existing BH RLC channels could be used to deliver it. Which BH RLC channel to use can be up to IAB node implementation. It is not necessary to configure dedicated BH RLC channel for it. |

There are at least two companies (LG and previously CATT) who proposed that the BH RLC channel configuration for control PDU transmission should be configured by F1-AP only, and no company explicitly proposes otherwise. Therefore, the following proposal was made.

Proposal 3b: F1AP is used to assign the BH RLC channel for BAP control PDU transmission.

Companies are asked if the above proposal is agreeable. If not, please provide the comments or any other compromised wording.

| Companies | Yes or No | Comments |
| --- | --- | --- |
| Nokia | Yes | “F1AP is used to assign a BH RLC channel for BAP control PDU transmission.” |
| ZTE | No | Which BH RLC channel shall be used for BAP control PDU transmission could be up to implementation. |

Based on the comments received, one more proposal is added.

Companies are asked if the above proposal is agreeable. If not, please provide the comments or any other compromised wording.

| Companies | Yes or No | Comments |
| --- | --- | --- |
| Nokia | No | There should always be a BH RLC channel assigned to control-PDU transmission (otherwise we have to specify what happens when it is not assigned). Whether the channel is dedicated-purpose or not can be up to implementation. |
| ZTE | No | Which BH RLC channel shall be used for BAP control PDU transmission could be up to IAB node implementation. |

Based on the comments received from Nokia, proposal 3c could be aligned with both Ericsson and Nokia’s understanding, as proposal 3d.

Proposal 3d: Whether the separated BH RLC channel is configured, which is only used for control PDU transmission, is up to CU implementation.

# BAP based flow control

- Issue 4.1: Supporting of two types simultaneously enabled

This issue was discussed in the email discussion [108#51], R2-2000989, which is copied here for you information

- Option-1: a control PDU indicating BH RLC channel ID(s) and its desired buffer size; [Type1]

Fig. 4-1 example (only) of option-1 (only for one RLC channel)

- Option-2: a control PDU indicating routing ID(s) and its desired buffer size; [Type2]

Fig.4-2 example (only) for option-2 (only for routing ID)

Q4.2: According to the agreements, do you think that both option 1 and option 2 can be configured to an IAB node, and the IAB node can report two kinds of control PDUs for flow control feedback?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | Yes | We might as well allow this, since the two options will require different control PDU types, and therefore the receiving node will always know what kind of reporting its child node is using. |
| QCOM | Yes | Since per-hop flow control has only limited benefit in the present release and may create significant overhead, it would be desirable to have either of these two solutions made configurable. |
| CATT | Yes | We see no need for any limitation on configuration. |
| LG | Yes | It’s up to configuration by CU and no limitation is required. |
| Ericsson | Yes | It will depend on the MT capabilities and what the DU can support too. The MT cannot be configured with features the DU does not support. |
| Futurewei | Yes | The CU should be able to configure an IAB node MT to report either or both flow control PDU types |
| OMESH | Yes |  |
| Nokia, Nokia Shanghai Bell | See comment. | This seems to be ruled out by our agreement in the last meeting. However, in principle this could be allowed depending on the capabilities of the IAB node. |
| ZTE | Yes |  |
| Huawei, HiSilicon | Yes |  |

Summary: Seems all agree that both these two options can be configured to an IAB node.

Proposal 4: Flow control feedback per BH RLC channel and flow control feedback per routing ID can be simultaneously configured to an IAB node.

Companies are asked if the above proposal is agreeable. If not, please provide the comments or any other compromised wording.

| Companies | Yes or No | Comments |
| --- | --- | --- |
| Nokia | No | For simplicity we prefer that only one is configured at a time. Simplifies polling and control PDU design. |
| ZTE | Yes |  |

- Issue 4.2: Which type of flow control feedback to be reported and its configuration/enabling

We agreed two types of flow control feedback, including the flow control feedback per BH RLC channel and flow control feedback per routing ID. We call the child node as the polled IAB node and the parent node as the polling IAB node.

Based on your inputs to following questions, we may have following options to proceed.

Question A: Whether the polled IAB node should be configured/enabled with the type(s) of flow control to report.

Question B: Whether the flow control polling BAP control PDU indicates the type to be polled?

Question C: Whether the polling IAB node should be configured with the type(s) of flow control, which is enabled at the polled IAB node?

| Option | Question A | Question B | Question C | Example of how this option works |
| --- | --- | --- | --- | --- |
| Option 1 | Yes (configured) | No (no indication) | No (not configured) | Child node only reports the configured flow control type(s). If both the two types of flow control feedback are configured, upon receiving a poll, it is up to implementation which type(s) of flow control feedback is reported. It is up to parent node to determine whether to poll, since parent is not aware of which type will be reported from child. Parent node can ignore the type of report received, which is not the one it wants. |
| Option 2 | Yes (configured) | No (no indication) | Yes (configured) | Child node only reports the configured flow control type(s). If both the two types of flow control feedback are configured, upon receiving a poll, it is up to implementation which type(s) of flow control feedback is reported. It is up to parent node to determine whether to poll, since parent is aware of which type will be reported from child. Parent node can ignore the type of report received, which is not the one it wants. |
| Option 3 | Yes (configured) | Yes (with indication) | No (not configured) | Child node only reports the type which is configured and indicated in the polling control PDU. It is up to parent node to determine whether to poll, since parent is not aware of which type will be reported from child. |
| Option 4 | No (not configured) | Yes (with indication) | Yes (configured) | Child node only reports the type which is indicated in the polling control PDU. Parent node polls the type which is configured. If both types are configured, it is up to the parent node which type to poll. |
| Option 5 | Yes (configured) | Yes (with indication) | Yes (configured) | Child node only reports the type which is configured and indicated in the polling control PDU. Parent node polls the type which is configured. If both types are configured, it is up to the parent node which type to poll. |

Companies are asked about your preference on the above question A,B, C and the options. You can also indicate which option(s) is not acceptable. You may add more options if not covered by above 4 options.

| Companies | Question A: Yes or No? | Question B: Yes or No? | Question C: Yes or No? | Preferred option? | Comments |
| --- | --- | --- | --- | --- | --- |
| Huawei | Yes | No | No | Option 1 (or option 2 is also acceptable) | This is simplest way, with less specification impact and IAB node implementation is trustable. Option 4 is not feasible. It means the child node is not configured with the type to be used. For the case child node triggered reporting (e.g. based on buffer size threshold), the child node has to be configured/enabled by CU. |
| Nokia | Yes | Yes | Yes | Option 5 | See reply to previous proposal. If only one type is configured at a time, there is no ambiguity what polling means. Question C can be understood differently depending on whether or not it is assumed that CU shall configure all IAB nodes with the same feedback type(s). We have assumed yes. Thus we think a polling node (which can also be a polled node) needs only once feedback-type configuration, i.e. nothing specific to its child nodes. |
| ZTE | Yes | No | No | Option 1 |  |

- Issue 4.3: Which BH RLC channels to be reported

This issue is discussed by P5, P6 of R2-2001622 (Futurewei). RAN2 should discuss how to determine the reported BH RLC channels or BAP Routing IDs by polled IAB node.

Option 1: Upon receiving the polling, polled IAB node reports all the BH RLC channels (or all BAP Routing IDs) configured on its ingress interface.

Option 2: Upon receiving the polling, it is polled IAB node implementation to report which BH RLC channels (or BAP Routing IDs).

Companies are asked which option is agreeable. If not, please provide the comments or any other compromised wording.

| Companies | Option? | Comments |
| --- | --- | --- |
| Nokia | 2 | We note that the RLC channel / Routing ID to report could be implicit from a Data-PDU header if the poll was carried there. Option 1 could result in massive-sized feedback. |
| ZTE | 2 |  |

# Conclusion and proposals

TBD.