3GPP TSG-RAN WG2 Meeting #109 electronic	draft_R2- 2002157

24th Feb - 6 Mar 2020

Agenda Item:	6.1.3

Source: 	Huawei, HiSilicon

Title:	Proposals for approval on BAP functionality (Set I)

Document for:	Discussion and Decision

# Introduction

This paper aims at capturing the summary of following email discussion

- [AT109e][021][IAB] BAP functionality (Huawei)

Scope: Treat remaining parts to be treated from email discussion and from summary

Intended outcome: resolution to issues, solutions.

Deadline: Mar 3 1200 CET (from conclusions pow focus on easy agreements)

This document merges the remaining essential issues discussed in the two summaries:

R2-2000989	Summary of email discussion 108#51 on BAP open issue	Huawei	discussion

R2-2002055	Summary on BAP functionality in AI 6.1.3	Huawei, HiSilicon

# BAP bearer mapping

- Issue 3.1: The bearer mapping of non-F1 traffic

This issue was discussed in the email discussion [108#51], R2-2000989, which is copied here for you information

Q3.1: Do you agree to confirm that a specific routing ID and BH RLC channels can be configured for non-F1 traffic via F1AP and RRC (for bootstrapping)?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | Yes |  |
| QCOM | See comment | For non-bootstrapping case, RAN3 has already decided that a routing ID and BH RLC channel can be configured for non-F1 traffic. For bootstrapping, we already discussed in RAN2 that we would not configure a routing table, so there is no need for a traffic-type specific routing ID and BH RLC channel. Please let’s not discuss issues that have already been done! |
| KDDI | See comment | Same view as QCOM |
| CATT | Yes | We can follow RAN3 agreements for later stage. As captured in R2-1916641, during IAB-node integration, before F1AP is established, a default BH RLC channel and a default BAP routing ID are configured via RRC, which are used for all upper layer traffic. We see no issue given the current progress. |
| LG | Yes |  |
| Ericsson | Yes |  |
| Futurewei | Yes | Agree with the comment from CATT |
| OMESH | Yes |  |
| Nokia, Nokia Shanghai Bell | Yes | However, it seems we need only single BAP routing ID and BH RLC channel for all kinds of traffic to be used for bootstrapping. |
| ZTE | Yes | The BAP routing ID configured via RRC during bootstrapping could be used for non-F1 traffic. After IAB node integration, the F1AP based BAP routing ID configuration could be used for non-F1 traffic. |
| Huawei, HiSilicon | Yes |  |

Summary: Majorities have replied yes, but the suggested solutions may be different according to the comments. Some companies announced that RAN3 has already decided that a routing ID and BH RLC channel can be configured for non-F1 traffic (which is an agreement I cannot find. Need double check). Some others suggest that the default BH RLC channel and default BAP routing ID as configured by RRC are used for non-F1 traffic. Some further suggest that during bootstrapping the BAP routing ID and BH RLC channel as configured by RRC are used for non-F1 traffic, and after IAB node integration, F1AP configuration is used for non-F1 traffic.

Based on the inputs from companies, the following proposal is proposed:

Proposal 2: After bootstrapping, the specific routing ID and BH RLC channel as configured by F1AP are used for non-F1 traffic.

Companies are asked if the above proposal is agreeable. If not, please provide the comments or any other compromised wording.

| Companies | Yes or No | Comments |
| --- | --- | --- |
| Nokia | Yes |  |
| ZTE | Yes |  |
| QC | Yes | This is actually a RAN3 issue, or let’s say it this way, RAN3 is currently struggling to agree on F1AP configuration for non-F1 traffic. |
| Futurewei |  | I tend to agree with QC here. I think RAN2’s scope ends with providing appropriate configuration to support bootstrapping. If after bootstrapping RAN3 decides to provide a specific configuration for non-F1 traffic via F1AP, or to continue to reuse the default configuration, they can address this within their specifications. I think that RAN3 are addressing this issue in one of their comeback discussions. So we can wait for their discussion to conclude, and see if anything still needs to be addressed by RAN2. |

- Issue 3.2: The BH RLC channel to be used for BAP control PDU

This issue was discussed in the email discussion [108#51], R2-2000989, which is copied here for you information

Q7.3: Except the implicit way in Q4.3, how is the BH RLC channel determined to transmit for BAP control PDUs including flow control feedback and BH RLF indication?

Option-1: the BH RLC channel for control PDUs is configured by F1AP or RRC;

Option-2: BAP selects any BH RLC channel to transmit control PDUs.

| Company | Yes or No | Comments |
| --- | --- | --- |
| Samsung | Depends… | - Per BH RLC CH reporting: the BL RLC CH for transmitting control PDU is the one the flow control information refers to. - Per Routing ID reporting: any BH RLC CH can be selected as long as routing ID is included. - RLF reporting: any BH RLC CH can be used. |
| QCOM | Option 1 | Straightforward. Easy to implement. |
| CATT | Option 1 |  |
| LG | Option 1 |  |
| Ericsson | Option 2 |  |
| Futurewei | Option 1 |  |
| OMESH | Option 2 |  |
| Nokia, Nokia Shanghai Bell | Option 1 | To avoid usage of UM mode BH RLC channels. |
| ZTE | Option 2 |  |
| Huawei, HiSilicon | Option 1 |  |

Summary: Majorities prefer to configure the BH RLC channel for control PDUs transmission.

The following contributions have been submitted to R2#109e meeting on this topic:

| Contribution | Proposals |
| --- | --- |
| R2-2000745 Further Discussion on BAP Layer Signaling, Ericsson | Proposal 1 No dedicated/separate BH RLC channels are needed for inter-IAB nodes BAP control signaling, rather the configured BH RLC channels for other types of traffic can be used to carry inter-IAB nodes signaling. Proposal 2 IAB nodes can map BAP control signaling messages to any of the configured BH RLC channels. |
| R2-2001565 Configuration of BH RLC channel for control PDU transmission, LG Electronics Inc. | Proposal. The BH RLC channel configuration for BAP control PDU transmission is configured by F1-AP. |
| R2-2000271 Discussion on BAP control PDU vivo | Proposal 3: In a UL/DL BH link, a specific BH RLC channel is configured for BAP control PDU transmissions. |

Companies’ views based on email discussion and contributions:

Proponents of Option 1 (7): Qualcomm, CATT, LG, Futurewei, Nokia, Huawei, vivo

Proponents of Option 2 (4): Samsung (mostly on Option 2), Ericsson, OMESH, ZTE

Given the situation, the following proposed is made:

Companies are asked if the above proposal is agreeable. If not, please provide the comments or any other compromised wording.

| Companies | Yes or No | Comments |
| --- | --- | --- |
| Ericsson |  | As explained in our tdoc R2-2000745, no dedicated/separate BH RLC channels are needed for control PDU, rather the configured BH RLC channels for other types of traffic (data PDU) can be used to carry control PDU. However, if some companies want to have dedicated or separate BH RLC channels for the control PDU then it should be optional. So, we suggest this should be optional, not mandatory. |
| Nokia | Yes, by F1AP | Propose to rephrase: “The BH RLC channel is assigned for control PDU transmission by F1AP or RRC.” to avoid mixing this with configuration of RLC parameters (RLC-config). |
| ZTE | No | The control PDU is not transmitted frequently. Any existing BH RLC channels could be used to deliver it. Which BH RLC channel to use can be up to IAB node implementation. It is not necessary to configure dedicated BH RLC channel for it. |
| CATT |  | We are not against any of the previous proposals. A comment to P3b here. we once asked in a previous email whether it would be agreeable to go directly to F1 AP so that we do not need to ask F1AP or RRC. Now that P3a and 3b look like a two-step procedure to reach a conclusion. It is fine if this is considered more fluent in handling the topic. |
| Futurewei | Yes, but | We think configuring a specific BH RLC channel for BAP Control PDUs is cleaner, and appropriate QoS handling can be configured for this RLC channel. During the e-mail discussion some use cases (implicit RLC-based flow control feedback) were proposed for transmitting specific BH RLC PDUs over RLC channels that carry UP/CP traffic. However, it does not seem there was agreement to support such a proposal. Therefore, at least for Rel. 16, we do not yet see a specific use case to support this. However, we are also open to consider this if a particular use case warrants it (see for example response to issue 4.3 below) |
| Lenovo&MM | Yes | My understating is that F1AP or RRC configure one or more than one BH RLC channel for BAP control PDU (P3a). Then, F1AP assigns one of them for control PDU transmission (P3b). If my understanding is correct, we agree with P3a. But, for P3b, it is better for implementation to select one BH RLC channel for control transmission. |

There are at least two companies (LG and previously CATT) who proposed that the BH RLC channel configuration for control PDU transmission should be configured by F1-AP only, and no company explicitly proposes otherwise. Therefore, the following proposal was made.

Companies are asked if the above proposal is agreeable. If not, please provide the comments or any other compromised wording.

| Companies | Yes or No | Comments |
| --- | --- | --- |
| Nokia | Yes | “F1AP is used to assign a BH RLC channel for BAP control PDU transmission.” |
| ZTE | No | Which BH RLC channel shall be used for BAP control PDU transmission could be up to implementation. |
| Ericsson |  | F1-AP can be used to assign a BH RLC channel, but it should not be mandatory (rather optional) that the assigned BH RLC channel be used only for control PDU. In other words, this BH RLC channel can also be used for user traffic. |
| Futurewei | Yes, but | At the DU side, use of F1-AP is natural to assign the BH RLC channel for BAP control PDUs. How about for the child node’s MT? It seems companied do not believe that the receive side needs to be configured for this BH RLC channel? In other words, the BAP receiver may not know specifically which BH RLC channel carries BAP control PDUs, but this also seems to somehow imply that the BAP should monitor for control PDUs from all BH RLC channels. |
| Lenovo&MM | No | See comment for P3a. |

Based on the comments received, one more proposal is added.

Companies are asked if the above proposal is agreeable. If not, please provide the comments or any other compromised wording.

| Companies | Yes or No | Comments |
| --- | --- | --- |
| Nokia | No | There should always be a BH RLC channel assigned to control-PDU transmission (otherwise we have to specify what happens when it is not assigned). Whether the channel is dedicated-purpose or not can be up to implementation. |
| ZTE | No | Which BH RLC channel shall be used for BAP control PDU transmission could be up to IAB node implementation. |
| Futurewei | No strong opinion | The only advantage of mandating configuration of this control PDU BH RLC channel seems to be that we would then need to address how to deal with this case in the spec (as mentioned by Nokia above). On the other hand, the spec impact may be very minimal. Anyway, the difference between P3c and P3d is not so clear to us, and may well result in the same impact to the spec. So, we are fine to go with the majority view on this. |

Based on the comments received from Nokia, proposal 3c could be aligned with both Ericsson and Nokia’s understanding, as proposal 3d.

Companies are asked if the above proposal is agreeable. If not, please provide the comments or any other compromised wording.

| Companies | Yes or No | Comments |
| --- | --- | --- |
| CATT |  | we are not sure if 3d is needed. Is it just an implementation choice? |
| QC |  | There are too many proposals and it is not clear if they are mutually interdependent. I propose the following to cover all 3a,b,c,d: - Proposal 3: The CU may assign a BH RLC channel for BAP PDU transmission via F1AP. If not assigned by the CU, the IAB-node selects the BH RLC channel by implementation. |

Based on the companies’ view above to the proposal 3 sets, one combined proposal is summarized as following, and drop P3a, 3b, 3c, 3d.

Proposal 3e: The CU may assign a dedicated BH RLC channel for BAP control PDU transmission via F1AP. If not assigned by the CU, the IAB-node selects the BH RLC channel for BAP control PDU by implementation.

Companies are asked if the above proposal is agreeable. If not, please provide the comments or any other compromised wording.

| Companies | Yes or No | Comments |
| --- | --- | --- |
| Nokia | Yes without “dedicated” | The word “dedicated” should be removed to make this more generic and acceptable. “The CU may assign a BH RLC channel for BAP control PDU transmission via F1AP.” It is up to network implementation whether a dedicated BH RLC channel is assigned for this purpose or the BH RLC channel is also used for other traffic. |

# Conclusion and proposals

TBD.