3GPP TSG-RAN WG2 #109-e	draft R2-2002217

E-meeting, February 2020

Agenda item:	6.1.4

Source: 	Samsung (rapporteur)

Title: 	IAB MAC impacts: Proposals for discussion and approval (Set III)

Document for:	Discussion and decision

# Introduction

In this document – based on initial comments received and captured in R2-2002120 (in response to discussion [AT109e][022][IAB] kick-off at the start of the e-meeting), and the resulting initial sets of proposals (Set I, followed by Set II), discussed widely by RAN2 and submitted to Chair, and captured in R2-2002162 and R2-2002162 – the discussion rapporteur proposes in this document a further and final set of candidate proposals (Set III) for approval via email.

# Proposals for discussion and approval (Set III)

The rapporteur has re-organized (and in a small number of cases, reworded or dropped certain) proposals into several groups, keeping the same numbering as in the original discussion document (‘[AT109e][022][IAB] User Plane Aspects…’).

Final set of candidate proposals

- Proposal i: RAN2 to insert a NOTE in TS 38.321 to acknowledge the issue of “double counting” of the buffer data e.g. that the first BSR triggered after the transmission of a pre-emptive BSR may need to be adjusted by subtracting the data volume reported in the previously sent pre-emptive BSR.

| Company | Comments |
| --- | --- |
| ZTE | Disagree, We think it can be up to implementation without specifying it. |

- Pre-emptive BSR received by a node shall not trigger a pre-emptive BSR at the node in question.

| Company | Comments |
| --- | --- |
| ZTE | Agree |

- The T_delta MAC CE will not contain SCS.

| Company | Comments |
| --- | --- |
| ZTE | Agree |

- RAN2 to discuss the number of bits that are set aside for the T_delta index in the T_delta MAC CE (with the rest being reserved bits), using previously agreed calculation captured in MAC CR or the revised calculations in submissions to this meeting.

| Company | Comments |
| --- | --- |
| ZTE | Only if RAN2 decides to define the mapping between T delta index and actual value of T delta in RAN2 spec, it may be necessary to further discuss the number of bits used for the index. Otherwise, it can be up to RAN1 to decide it. As agreed in the TP(R2-1916538) of last RAN2 meeting, the FR information could be determined by the IAB node based on its backhaul link since the backhaul link between one parent and one child node is located within one frequency range. Then the IAB node could determine whether 32Tc or 64 Tc shall be used to calculate the T delta value. Nevertheless, we think both calculations works. |

- RAN2 to discuss where (in RAN2 or RAN1 specs) the mapping between the T_delta index and actual value of T_delta should be captured. If it is decided it should be captured in the RAN1 specs, RAN2 to discuss sending an LS to RAN1.

| Company | Comments |
| --- | --- |
| ZTE | Agree with this proposal. We slightly prefer to capture it in RAN2 to save time. |

- Proposal ii: RAN2 to decide whether the IAB-specific PRACH resource may be used by IAB nodes for any cases of RACH trigger including beam failure.

| Company | Comments |
| --- | --- |
| ZTE | Agree. In our opinion, if the IAB-specific RACH resource is configured, IAB-MT should anyway use this resource. |