3GPP TSG-RAN WG2 #109-e			draft R2-20xxxxx

E-meeting, 24 February – 06 March 2020

Title:	[Draft] LS to RAN1 on T_delta in IAB

Release:	Rel-16

Work Item:	NR_IAB-Core

Source:	Samsung (will be RAN2)

To:	RAN1

CC :	RAN4

Contact Person:

#### Name:	Milos TESANOVIC

E-mail Address:	m [dot] tesanovic [at] samsung [dot] com

1. Overall Description:

At the RAN2#108 meeting (Reno, November 2019) RAN2 introduced the Timing Delta (T_delta) MAC CE and the relevant LCID value used to identify this MAC CE, into the running CR to TS38.321 which introduces IAB into Rel-16.

The T_delta MAC CE will contain (in addition to a number of reserved bits) the so-called T_delta field. With this field, the MAC entity indicates the index value of T_delta used to control the amount of timing delta adjustment. The length of the T_delta field is 11 bits.

2. Actions:

At the ongoing RAN2#109e meeting, RAN2 discussed the T_delta MAC CE further. RAN2 confirmed that the T_delta MAC CE shall not contain SCS. Additionally, RAN2 discussed where (in RAN1 or RAN2 specifications) the mapping between the T_delta index and the actual value of T_delta should be captured. In light of this discussion, RAN2 has the following two items for RAN1’s attention and action:

- RAN2 would like to inform RAN1 of its preference to have this mapping captured in RAN1 specifications, and would like to respectfully ask RAN1 to take appropriate action (or notify RAN2 as a matter of priority if this RAN2 preference cannot be met by RAN1).

- As already mentioned, the size of the T_delta field (containing the index value of T_delta) is 11 bits. If RAN1 finds this inadequate, RAN2 would very much appreciate it if RAN1 could inform RAN2 at their earliest convenience and provide the number of bits required (if different from 11).

3. Date of Next TSG-RAN WG2 Meetings:

TSG-RAN WG2 Meeting #109-Bis	April 2019 		E-meeting

TSG-RAN WG2 Meeting #110	25 – 29 May 2020 	Athens, Greece