3GPP TSG-RAN WG2 Meeting #109 electronic	 R2-200xxxx

24 Feb – 6 Mar 2020

Agenda Item:	x.x.x.x

Source:	Rapporteur

Title:	Summary of [AT109e][058][TEI16] Downgraded configuration SRS antenna switching (Intel OPPO)

Document for:	Discussion, Decision

# Introduction

Triggered by the LS from RAN1 [8].

RAN1 discussed on a TEI proposal regarding downgrading configuration of SRS for antenna switching, and RAN1 made following agreements.

Agreement: Regarding TEI “Enable gNB to configure downgrading configuration of SRS for antenna switching” - Rel-16 UE capability design for SRS antenna switching in conjunction with the existing Rel-15 UE capability should allow UE to indicate support of one of the following combinations - {t1r1, t1r2} - {t1r1, t1r2, t1r4} - {t1r1, t1r2, t2r2, t2r4} - {t1r1, t2r2} - {t1r1, t2r2, t4r4} - {t1r1, t1r2, t2r2, t1r4, t2r4} - Note: Detailed signaling design is up to RAN2 - The TP in R1-1913445 is endorsed in principle - Note: RAN1 spec can be further updated depending on the final RAN2 signalling design

In this contribution, we discuss some left issues for down-graded configuration for SRS antenna switching, based on the contribution [1]-[2].

# Discussion

## Issue-1: How to address the issue of SRS pattern matching between R15 IE and R16 IE

As noted by both the proposal from OPPO [1][2] and Intel [3]-[7], there should be coupling of reporting via R16 IE on down-grading SRS Tx switching pattern and the legacy R15 IE on SRS Tx switching pattern. There are different approaches on how to do this:

Approach-1: As indicated in OPPO paper [1][2] and Intel paper (Alt-1) [4][5], on the one hand, the code-points for the pattern are used in 38.331, exactly as included in the incoming LS and implemented in RAN1 CR, as follows

BandParameters-v16xy ::=            SEQUENCE {

srs-TxSwitch                    SEQUENCE {

supportedSRS-TxPortSwitch-v16xy     ENUMERATED {t1r1-t1r2, t1r1-t1r2-t1r4, t1r1-t1r2-t2r2-t2r4, t1r1-t1r2-t2r2-t1r4-t2r4,

t1r1-t2r2, t1r1-t2r2-t4r4}

}                                                                              OPTIONAL

On the other hand, some restriction needs to be added into 38.306, to indicate the coupling between R15 IE and R16 IE.

Approach-1A, either as some text as indicated in OPPO [1][2]

-	supportedSRS-TxPortSwitch indicates SRS Tx port switching pattern supported by the UE, which is mandatory to report. The indicated UE antenna switching capability of ′xTyR′ corresponds to a UE, capable of SRS transmission on ′x′ antenna ports over total of ′y′ antennas, where ′y′ corresponds to all or subset of UE receive antennas, where 2T4R is two pairs of antennas.supportedSRS-TxPortSwitch-v16xy indicates downgrading SRS Tx port switching pattern supported by the UE, which is optional to report. If supportedSRS-TxPortSwitch-v16xy is reported, the pattern of highest order shall be the same as the pattern reported in supportedSRS-TxPortSwitch (e.g. if supportedSRS-TxPortSwitch indicates t1r4-t2r4, supportedSRS-TxPortSwitch-v16xy, if reported, shall only indicate t1r1-t1r2-t2r2-t1r4-t2r4);

Approach-1B, or as a table as indicated in Intel (Alt-1) [4][5]

If the UE indicates the support of downgrading configuration of SRS for antenna switching using supportedSRS-TxPortSwitch-r16, the UE shall report the values for this as below, based on what is reported in supportedSRS-TxPortSwitch

| supportedSRS-TxPortSwitch | supportedSRS-TxPortSwitch-r16 |
| --- | --- |
| t1r2 | t1r1-t1r2 |
| t1r4 | t1r1-t1r2-t1r4 |
| t2r4 | t1r1-t1r2-t2r2-t2r4 |
| t2r2 | t1r1-t2r2 |
| t4r4 | t1r1-t2r2-t4r4 |
| t1r4-t2r4 | t1r1-t1r2-t2r2-t1r4-t2r4 |

Approach-2: As indicated in Intel-paper (Alt-2) [6][7], to only capture a single code-point in 38.331

downgradedSRS-TxPortSwitch-r16       ENUMERATED {supported}					 OPTIONAL,

And in 38.306, to clarify the intended down-grading SRS Tx switching pattern

downgradedSRS-TxPortSwitch

Indicates whether the UE supports downgrading configuration of SRS for antenna switching according to the below table, based on the capability signalled using supportedSRS-TxPortSwitch. If UE reported ‘notSupported’ for supportedSRS-TxPortSwitch, the UE does not report this field.

| supportedSRS-TxPortSwitch | Downgraded configuration the UE supports |
| --- | --- |
| t1r2 | t1r1 |
| t1r4 | t1r1, t1r2 |
| t2r4 | t1r1, t1r2, t2r2 |
| t2r2 | t1r1 |
| t4r4 | t1r1, t2r2 |
| t1r4-t2r4 | t1r1, t1r2, t2r2 |

Question-1: Do companies agree to capture coupling between R15 IE and R16 IE on SRS Tx switching pattern?

- Yes – in this case, which Approach you prefer.

- No – In this case, please clarify the reason.

| Company | Yes/No (if Yes, which approach is preferred) | Comments |
| --- | --- | --- |
| Qualcomm Incorporated | Yes, Approach 2 |  |
| Huawei | Yes, Approach-1 | Indicating the down-grading capability explicitly is more forward compatible, e.g. if there would be two or more down-grading values. Regarding 1A or 1B, we don’t have strong view. |

## Issue-2: How to handle the txSwitchImpactToRx and txSwitchWithAnotherBand

Since SRS-TxSwitch relate to 3 sub-field, i.e., supportedSRS-TxPortSwitch which has been addressed in the incoming LS and the issue-1 above, it is FFS on how to interpret the other two impacts for the down-grading SRS Tx switching pattern reported via R16 IE.

Approach-1: As indicated in OPPO paper [1][2], nothing is introduced into 38.331, but add some text in 38.306 to clarify that the reported R15 field is applicable to down-grading configuration as well

-	txSwitchImpactToRx indicates the entry number of the first-listed band with UL in the band combination that affects this DL, which is mandatory to report. It applies to SRS Tx port switching pattern reported in both  supportedSRS-TxPortSwitch and supportedSRS-TxPortSwitch-v16xy.

-	txSwitchWithAnotherBand indicates the entry number of the first-listed band with UL in the band combination that switches together with this UL, which is optional to report. It applies to SRS Tx port switching pattern reported in both  supportedSRS-TxPortSwitch and supportedSRS-TxPortSwitch-v16xy.

Approach-2: As indicated in Intel paper [4]-[7], a code-point is introduced into 38.331, to indicate whether the impact would be the same as reported via R15 IEs or not, with [4] explaining the reasons on why the UE may not have switching impact in some cases and how the UE capability signalling can make use of this.

txSwitchImpacted-r16                ENUMERATED {sameSwitchingImpact}			 OPTIONAL

And description in 38.306 that

txSwitchImpacted

Indicates whether the UE has the same switching impact as reported using txSwitchImpactToRx and txSwitchWithAnotherBand when configured with downgrading configuration of SRS for antenna switching. If the field is absent, there is no switching interruption impact at the UE when configured with downgrading configuration of SRS for antenna switching.

Question-2: in order to clarify the capability of txSwitchImpactToRx and txSwitchWithAnotherBand for the down-grading configuration, which approach is preferred?

- Approach-1, As indicated in OPPO paper [1][2]

- Approach-2, As indicated in Intel paper [4]-[7]

- Other

| Company | which approach is preferred | Comments |
| --- | --- | --- |
| Qualcomm Incorporated |  | RAN1 LS is completely silent about the "UL / DL band switch together" capabilities, and so it seems that the current capabilities should apply by default. However we would like to hear more explanations from Intel as to why they think it is “either the original switching impact remains, or none”, i.e. Observation 2 in R2-2001275. |
| Huawei |  | In our understanding, the capability of impact may be different for the down-grading SRS Tx switching pattern, so we are doubt that txSwitchImpactToRx and txSwitchWithAnotherBand can be applied to original and down-grading SRS Tx switching pattern by default. Using 1bit txSwitchImpacted is too vague, but adding explicit “txSwitchImpactToRx and txSwitchWithAnotherBand” for all down-grading SRS Tx switching pattern leads to signalling overhead. It is acceptable to us if nothing is added, and also we would like to hear companies’ views. |

- xxx.

- xxx.

# Conclusion

Based on the discussion in section 2, we observe

Observation 1	xxx.

And thus we propose:

Proposal 1	xxx.

# Reference

- R2-2002066	Introduction of downgraded configurations for SRS antenna switching	OPPO	CR	Rel-16	38.331	15.8.0	1433	1	B	NR_newRAT-Core

- R2-2002067	Introduction of downgraded configurations for SRS antenna switching	OPPO	CR	Rel-16	38.306	15.8.0	0258	-	B	TEI16	Late

- R2-2001275	Downgrading configuration of SRS for antenna switching	Intel Corporation	discussion	Rel-16	TEI16

- R2-2001273	Downgrading configuration of SRS for antenna switching - Alt. 1	Intel Corporation	CR	Rel-16	38.331	15.8.0	1480	-	C	TEI16

- R2-2001274	Downgrading configuration of SRS for antenna switching - Alt. 1	Intel Corporation	CR	Rel-16	38.306	15.8.0	0251	-	C	TEI16

- R2-2001276	Downgrading configuration of SRS for antenna switching - Alt. 2	Intel Corporation	CR	Rel-16	38.331	15.8.0	1481	-	C	TEI16

- R2-2001277	Downgrading configuration of SRS for antenna switching - Alt. 2	Intel Corporation	CR	Rel-16	38.306	15.8.0	0252	-	C	TEI16

- R1-1913580, LS on NR Rel-16 TEI