3GPP TSG-RAN WG2 Meeting #109e			R2-2001683

Elbonia, 24 Feb – 6 Mar 2020

Title:	[DRAFT] LS on eMIMO RRC parameters

Release:	Rel-16

Work Item:	NR_eMIMO-Core

Atta

Source:	Ericsson (to be RAN2)

To:	RAN1

Contact Person:

#### Name:	Helka-Liina Määttänen

E-mail Address:	helka-liina.maattanen@ericsson.com

Attachments:	-

1. Overall Description:

In RAN2#109e, RAN2 discussed the RRC for eMIMO and reached the following agreements:

Agreements: 1. Given the above analysis we propose to keep the three modes for ULFPTX. Inform RAN1 of this decision and ask if this is fine for them 2. BDFactor is signalled per cell. Ask RAN1 for confirmation 3. Agree the current RRC running CR implementation i.e. have only rsrp-ThresholdSSBBFR which is used for beam selection for MAC CE and rename rsrp-ThresholdSSBBFR to rsrp-ThresholdBFR. (MAC CR needs to be aligned) 4. The current RRC running CR implementation for max number of detection resource limitation as show above. Agreements: 1. Agree to implement two LTE CRS pattern lists corresponding to each CORESETPoolIndex as indicated in above changes and merge the changes to the running RRC CR for NR eMIMO. Can reconsider this if we find an issue. 2. Agree the existing RepetitionSchemeConfig IE (i.e. SEQUENCE) in the running CR as baseline for repetition scheme configuration, with additional restriction in the field description. Also ask RAN1 for confirmation that fdm-tdm and slotBased are mutually exclusive 3. enableDefaultBeamPlForPUSCH0_0, enableDefaultBeamPlForPUCCH, enableDefaultBeamPlForSRS, and PLRS-update parameter are kept in the RRC for now. Can consider to remove them later if not really needed

According to the above agreements, three aspects needs further confirmation from RAN1.

a) UL Full Power Transmission

The ULFPTX seems to have three distinct modes of operation which is why RAN2 agreed to support ULFPTX by parameter ul-FullPowerTransmission in IE PUSCH-Config as follows in [1]:

ul-FullPowerTransmission-r16            ENUMERATED {fullpower, fullpowerMode1, fullpoweMode2}     OPTIONAL    -- Need R

ul-FullPowerTransmission

Configures the UE with UL full power transmission mode as specified in TS 38.xxxx.

It is understood that this would imply certain changes needed in TS 38.213.

Question 1.

RAN2 would like to inform RAN1 about the above agreement and ask RAN1 to indicate if they find issues with the agreement.

b) Parameter BDFactor (mTRP)

In the parameter excel in R1-1913674 the parameter BDFactor was indicated to be configured per serving cell. However, during the running RRC CR discussions, it was deduced that better place for this parameter is under IE PhysicalCellGroupConfig. This was because the parameter BDFactor(r) does not look like per cell property according to below RAN1 agreements:

- If higher layer index is configured per CORESET for the UE supporting multi-DCI based multi-TRP transmission, support the followings for the principles above:

  - is replaced with , where  is the number of configured DL serving cell(s) without multi-DCI based multi-TRP with active DL BWP with SCS , and  is the number of configured DL serving cell(s) with multi-DCI based multi-TRP with active DL BWP with SCS

  - The value range of r is [1, 2], and it depends on UE capability.

  - UE indicates pdcch-BlindDetectionCA when it is possible to configure A+B DL cells to the UE with A>= 0 DL serving cells without multi-DCI based multi-TRP and B >=0  DL serving cells with multi-DCI based multi-TRP such that A+r∙B>4

  - When UE does not provide pdcch-BlindDetectionCA, the value of N^cap_cells is a+r.b, where a is the number of configured DL serving cells without multi-DCI based multi-TRP, and b is the number of configured DL serving cells with multi-DCI based multi-TRP.

  - FFS: other conditions for UE capability reporting are applied to multi-DCI based multi-TRP transmission

- FFS: details on how to determine a DL serving cell configured with multi-DCI based multi-TRP transmission and associated value of r.

Question 2. RAN2 would like to ask RAN1 to inform RAN2 whether the parameter BDFactor should be per cell or per cell group(in IE PhysicalCellGroupConfig)?

c) Configuration of different repetition schemes

RAN2 has been discussion on whether the repetition schemes 2a/2b/3 (fdmSchemeA, fdmSchemeB and tdmScheme) and scheme 4 (slotBased) are mutually exclusive in all UE configuration options or not. According to the below table rows B and F these look mutually exclusive but according to rows 0/G/G’ it seems UE can in some cases be configured with both one of schemes 2a/2b/3 and scheme 4.

Agreement

Following TCI state and joint schemes are supported

Note:

- Condition 1: indicates at least one entry inpdsch-TimeDomainAllocationList containing URLLCRepNum (>1) in TDRA by DCI

- Condition 2: indicates one entry inpdsch-TimeDomainAllocationList having noURLLCRepNum by DCI, but at least one entry having URLLCRepNum

- Condition 4: None of entry in TDRA containsURLLCRepNum

Question 3. RAN2 asks RAN1 to inform RAN2 if repetition schemes 2a/2b/3 (fdmSchemeA, fdmSchemeB and tdmScheme) and scheme 4 (slotBased) are mutually exclusive in all UE configuration options, or to provide a reference which could be used in TS 38.331 for referencing about the configuration limitations for the repetition schemes.

d)

2. Actions:

To RAN1

ACTION: 	RAN2 respectfully asks RAN1 to take the above RAN2 agreements into account in their work and provide feedback on the aforementioned questions.

3. Date of Next TSG-RAN2 Meetings:

TSG-RAN2 Meeting #109bis 	20th – 24th April, 2020		Sapporo, Japan.

TSG-RAN2 Meeting #110	25th – 29th May 2020		Athens, Greece.

4. Reference

- R2-2002071, Introduction of MIMO enhancements, Ericsson, Elbonia, 24 Feb – 6 Mar 2020