3GPP TSG-RAN WG2 Meeting#109 electronic                         R2-200xxxx

24 Feb – 6 Mar 2020

Agenda Item:	6.16.3

Source: 	OPPO

Title:  	[AT109e][121][EMIMO] DL MAC CE design (Oppo)

Document for:	Discussion and Decision

# Introduction

- [AT109e][121][EMIMO] DL MAC CE design (Oppo)

Scope: Continue the discussion on DL MAC CE design aspects which are still open after the discussion on R2-2000660 as well as those listed in R2-2001551

Intended outcome:

    - Set of proposals with full consensus (aim to agree to those over email) to be reflected in the updated MAC CR

    - Set of proposals with almost full consensus and easy to agree

    - Set of open issues and proposals to postpone to next meeting

Final deadline (for companies' feedback):  Friday 2020-02-28 12:00 CET

Final deadline (for rapporteur's summary):  Monday 2020-03-02 12:00 CET

# Discussion

## MAC CEs regarding to multiple CCs/BWPs

### Issue#1: how to differentiate the granularity of received MAC CE

We have agreed to use option 2 (RRC configuration only approach) for designing the MAC CEs regarding multiple CCs/BWPs. And the legacy Rel-15 MAC CE format will be reused to update the configuration of all the CCs in the same CC list. The CC lists are configured by RRC as agreed in RAN1 and up to 2 CC lists will be configured.

During the email discussion[108#68] [4], companies share different views on whether R16 UE should support MAC CE with granularity of both per CC-list and per CC. And we have the following proposal for further discussion.

Proposal 12 If RRC configuration only approach is agreed, RAN 2 discuss whether R16 UE should support MAC CE with granularity of both per CC-list and per CC.

Q1: If CC list is configured by RRC, do you agree that UE should also support MAC CE for a CC in the list?

| Company | Answer (Y/N) | Comments |
| --- | --- | --- |
| CATT | See comments | We think it is not adequate to say a granularity of per CC or per CC list is not supported by any of the options below. They are supported. It is just how it is done. |
| Samsung | No | From our understanding, if we go to RRC only mechanism, the legacy MAC CE (per serving cell/BWP) can be reused, it means there are no need to say a granularity of per CC or per CC list. It just follow the RRC configuration and we think the main use case is per CC list. |
| Ericsson | no | But this is not about UE support as such. The idea is that one MAC CE, whether legacy or Rel-16 one, that indicates one CC is applied to both per CC and per CC list. If UE is configured with the list, the MAC CE changes something for all CCs in that list. If UE is not configured with the list it changes something only in the CC. |
| Huawei, HiSilicon | No, i.e. if a CC is in a list, only the MAC CE which applies to the whole CC list applies to this CC | Maybe "for a CC in the list" should be added at the end of Q1, as it is now the question is unclear |
| OPPO | No | If the CC indicated in the MAC CE is configured in a CC list, the MAC CE apply for all the CC in the list. |
| Qualcomm | Y/N | The question is unclear. If RRC configuration only approach is agreed, update the TCI state or spatial relation whether for individual cell or multiple CC depending on RRC configuration. That is if the cell list includes only one individual cell or this individual cell does not belong to any cell list, the MAC CE only update the TCI state or spatial relation for this individual cell. Otherwise TCI state or spatial relation is updated for all the cells configured in the cell list which includes the indicated cell within the MAC CE if cell list is configured by RRC. |
| vivo | No | The MAC CE should apply to all the CCs in the list. |

Q2: If answer to Q1 is yes, do you agree we should use new LCID(s) to differentiate the granularity of the received MAC CE, i.e. either per CC-list or per CC?

| Company | Answer (Y/N) | Comments |
| --- | --- | --- |

Q3: If answer to Q1 is no, do you agree the legacy MAC CE is understood as per CC-list MAC CE only if CC list is configured by RRC, i.e. if the CC indicated in the MAC CE belongs to a configured CC-list, this MAC CE is assumed as CC-list level MAC CE?

| Company | Answer (Y/N) | Comments |
| --- | --- | --- |
| CATT | Yes | As clearly pointed out by RAN1 agreement, UE expect no overlapped CC in multiple RRC-configured lists of CCs. Since a given CC will not be in different CC-list there is no need to indicate CC list in the MAC CE. Also, RAN1 agreement explicitly says the applied list is determined by the indicated CC in the MAC CE. Furthermore, it is RAN1’s agreement that the TCI-states for PDSCH/PDCCH, or the Spatial Relation Info is applied for all the BWPs in the indicated CCs. If a CC is not configured as part of a CC-list, the MAC CE applies to this single CC, while if it is configured as part of a CC-list, the MAC CE simply applies to that CC-list. With these, we say Yes to Q3. |
| Samsung | Yes but we want to revert the decision (go to RRC+MAC CE. | As we explained above Q1, the legacy MAC CE can activate/deactivate the TCI states per CC list-level which is configured by RRC. In this case, we cannot support the individual CC-level control for TCI-state, see the below example: - CC list #1: CC1, CC2, CC3, CC4 - CC list #2: CC5, CC6, CC7, CC8 - Not included in CC list: CC9, CC10 If UE receives the legacy MAC CE which have serving cell ID and BWP ID, UE applies the activation/deactivation TCI state for all CCs within the same configured CC list e.g. UE receives TCI state activation for CC1, then apply same TCI state activation for CC2, CC3, CC4. However, if NW want to deactivate TCI state only for CC3, there are no ways to deactivate the individual CC because all CC1, CC2, CC3, CC4 can be configured as group. If NW want to deactivate TCI state only for CC3, then RRC reconfiguration is needed i.e. CC3 should be removed from CC list #1, but it is not the efficient way. So, we think this RRC based solution is not support all functions. If we go to RRC+MAC CE solution, we introduce the new MAC CE only for the group-based (i.e. CC list) activation/deactivation, and the legacy MAC CE can be used for TCI state activation/deactivation for the individual CC control. |
| Ericsson | Yes | Our understanding is that it is not needed to support per CC operation if CC list is configured as there is no use case for that. However, if there is a lot of unclarity and different understandings then fine to consult RAN1. |
| Huawei, HiSilicon | Yes | We agree with the limitation described by Samsung but we think there is no need for something else. |
| OPPO | Yes |  |
| Qualcomm | - | Same view with the Samsung. The RRC + MAC CE approach is better than RRC only approach. If the RRC only approach is agreed, see our comments in Q1. |
| vivo | Yes |  |

### Issue#2: whether to consider multiple TRP case for MAC CEs regarding multiple CCs/BWPs

Current RAN 1 agreements for TCI States MAC CEs for multiple CCs/BWPs are applied for single TRP case, and based on the email discussion [108#68], majority companies think there is no need to consider multiple TRP case for MAC CEs for multiple CCs/BWPs since there is no RAN1 agreement on this.

Q4: Do you agree with the following proposal?

Proposal 14	 Multiple TRP case is not considered for MAC CEs regarding multiple CCs/BWPs, i.e. TCI States Activation/Deactivation for UE-specific PDSCH MAC CE, TCI State Indication for UE-specific PDCCH MAC CE

| Company | Answer (Y/N) | Comments |
| --- | --- | --- |
| CATT | Yes |  |
| Samsung | Yes |  |
| Ericsson | Question is unclear. Do you mean having TRP Ids in addition to CCs or are you referring to mPDCCH/sPDCCH operation. | The below RAN1 agreement concerns the CC list operation. The question is do we support CC list configuration and CC list operation for both mPDCCH and sPDCCH. When a TCI-state ID is activated for a CORESET by a MAC CE for a set of CCs/BWPs at least for the same band, where the applicable list of CCs is indicated by RRC signalling, the TCI-state ID is applied for the CORESET(s) with the same CORESET ID for all the BWPs in the indicated CCs. - Further signaling details are up to RAN2. - Whether to support the inter-band CA for this feature will be decided in RAN1#99. - Whether to indicate the applicable list of bands for the feature of single MAC-CE to activate the same PDCCH TCI state IDs for multiple CCs/BWPs is up to capability discussion. - FFS on the UE capability signaling details - Note: This at least applies to single TRP case. |
| OPPO | Yes | As agreed in RAN1, the CC list for TCI activation MAC CEs applies to single TRP case. There is no need to consider multiple TRP case without RAN1 input/agreement. |
| Qualcomm | Yes |  |
| vivo | Yes |  |

### Issue#3: SRS activation/deactivation MAC CE for multiple CCs/BWPs

RAN1 have reach the following agreement for designing the SRS activation/deactivation MAC CE for CC list:

Agreement

The following working assumption is confirmed with revision in red

(Working assumption #1 @RAN1#98bis) When a Spatial Relation Info is activated for a SP/AP SRS resource by a MAC CE for a set of CCs/BWPs at least for the same band, where the applicable list of CCs is indicated by RRC signalling, the Spatial Relation Info is applied for the SP/AP SRS resource(s) with the same SRS resource ID for all the BWPs in the indicated CCs.

Note: This at least applies to single TRP case.

Based on RAN1 agreements, it is not clear that whether the SRS activation/deactivation MAC CE for multiple CCs/BWPs is used to activate the spatial relation info for a SRS resource [3][12] or a SRS resource set [2]. We have the following questions to collect companies’ understanding.

Q5: Based on RAN1 agreements, do you think that the SRS activation/deactivation MAC CE for multiple CCs/BWPs activate the spatial relation info for single SRS resource (option1) or SRS resource set (option2)?

| Company | Preference (option 1/option 2) | Comments |
| --- | --- | --- |
| CATT | Option 2 |  |
| Samsung | Option 1 | It seems already clear from RAN1 agreements that SRS resource ID is used to activate/deactivate for all BWPs in the configured CC. |
| Ericsson |  | We can use the same SP/AP SRS MAC CE as what we will have for single CC case. If we start to introduce per SRS resource ID MAC CE the MAC CE should be able to indicate more than just one resource in the single MAC CE. |
| Huawei, HiSilicon | Option 1 | RAN1 agreement and spec are clear that spatial relation update is per resource. |
| OPPO | Option 1 | We prefer SRS resource level MAC CE since it is clear from RAN1 agreement. |
| Qualcomm | Option 2 | We don’t see RAN1 agreements limit to update per SRS source level in MAC CE. Even in Rel-15 SP SRS MAC CE, the SP SRS Resource Set field still can support update the spatial relation information per SRS resource level. If there are any misunderstanding, it’s better to send LS to RAN1 for further clarification. In our understanding, the legacy (SP SRS) or then new (AP SRS) update MAC CE can be reused here for multiple CCs/BWPs case. Hence, we prefer to option2. |
| vivo | Option 1 | The RAN1 agreements seems already saying that this is for a specific SRS resource. |

If the answer to Q5 is option 1, we understand a new MAC CE is required, currently the submitted contributions gives the two MAC CE format design for the SRS activation/deactivation for multiple CCs/BWPs:

- Solution 1: the example format is given below[12]

- Solution 2: the example format is given below[3]

Q6: Which solution do you prefer to design the new MAC CE for SRS activation/deactivation MAC CE for multiple CCs/BWPs?

| Company | Preference (solution1/ 2) | Comments |
| --- | --- | --- |
| Samsung | Eighter way is fine | We think there are no difference but we want to design MAC CEs with same ways with other MAC CEs. It means we can apply the same design rule for TCI state activation/deactivation MAC CE if RRC+MAC CE mechanism is accepted. |
| Ericsson |  | If we start to introduce per SRS resource ID MAC CE the MAC CE should be able to indicate more than just one resource in the single MAC CE. |
| Huawei, HiSilicon | solution1 |  |
| OPPO | Solution 2 |  |
| vivo |  | We agree with Ericsson that the MAC CE should be able to indicated more than one SRS resource IDs. |

Q7: If the answer to Q5 is option 2, do you agree to reuse SP and AP SRS Activation/Deactivation MAC CE to active the spatial relation of SP/AP SRS for multiple CCs/BWPs?

| Company | Answer (Y/N) | Comments |
| --- | --- | --- |
| CATT | Y | We prefer to make use of legacy design. This seems to be majority’s view in the email discussion #68. |
| Ericsson | Yes-see detail | It can be used for per CC list SP SRS operation. New MAC CE is needed for AP SRS MAC CE that can then be used for per CC and CC list. |
| Qualcomm | Y |  |

## MAC CEs regarding to mPDCCH mTRP

    - TCI State Indication for UE-specific PDCCH MAC CE

In RAN1 LS reply [1], RAN 1 has confirmed that the maximum total number of configured CORESETs per cell (across BWPs) is 16. The current MAC CE format for TCI State Indication for UE-specific PDCCH includes the field for CORESET ID with 4 bits and no further updates are required for this MAC CE format. One company proposes to reuse the existing MAC CE format for multi PDCCH-based TRP operation because the network can send the TCI states activation MAC CE from any TRP and the UE does not need to know from which TRP this comes from [5].

Q8. Do you agree the existing “TCI State Indication for UE-specific PDCCH MAC CE” can be used for multi PDCCH-based TRP transmission?

| Company | Answer (Y/N) | Comments |
| --- | --- | --- |
| CATT | maybe | This is technically possible. Let’s simply follow the majority here. |
| Samsung | Yes | According to RAN1 agreement, the maximum total number of configured CORESETs per cell (across BWPs) is 16. It means that the network properly configures the increased numbers of CORESETs for certain BWPs i.e. not for all BWPs in a cell. Then, no further updates are required for this MAC CE format, i.e. we can re-use the existing MAC CE format for multi PDCCH-based TRP operation because the network can send the TCI states activation MAC CE from any TRP and the UE does not need to know from which TRP this comes from. |
| Ericsson | yes | The MAC CE points TCI state per CORESET ID and has 4 bits that should be able to indicate 16 CORESETs unambiguously. So far we have not found issues here. |
| Huawei, HiSilicon | Yes | No need for further enhancement as the current MAC CE can work well. |
| OPPO | Yes |  |
| Qualcomm | - | We don’t see there is a clear RAN1 requirement on considering the enhancement for TCI state indication for PDCCH under mTRP case. |
| vivo | Yes |  |

    - TCI States Activation/Deactivation for UE-specific PDSCH

For TCI States Activation/Deactivation for UE-specific PDSCH MAC CE, the main concern from companies is that how to differentiate which TRP the PDSCH TCI state activation MAC CE applies to in case the network configure the TCI states for the PDSCH from another TRP. And the proposed solutions are listed below:

- Solution 1: Introduce the new MAC CE (new LCID) of mPDCCH-based MAC CE for TRP2 and use same format of the existing MAC CE (R15) of TCI States Activation/Deactivation for UE-specific PDSCH.[5]

- Solution 2: Enhance the Rel-15 TCI States Activation/Deactivation for UE-specific PDSCH MAC CE with reinterpretation of R field for Rel-16 TCI States Activation/Deactivation for UE-specific PDSCH MAC CE for multi-PDCCH multi-TRP. [7][11]

- Solution 3: The Enhanced TCI States Activation/Deactivation for UE-specific PDSCH MAC CE for single-PDCCH case can be applied for multi-PDCCH case. These two cases can be distinguished according to the configuration of CORESETPoolIndex. For example, if two values of CORESETPoolIndex are contained in the configured PDCCH-Config, it is the multi-PDCCH case. And TCI-state IDi,j denotes the TCI-state corresponding to TCI codepoint i for PDCCH associated with CORESETPoolIndex j-1; Otherwise, it is the single-PDCCH case, TCI-state IDi,j denotes the jth TCI-state corresponding to TCI codepoint i.[10]

- Solution 4: extend the Enhanced TCI States Activation/Deactivation for UE-specific PDSCH MAC CE for single-PDCCH case with CORESET pool indexes. If the E field is set to 0, the activated TCI states apply to a DCI which is sent via CORESET pool with CORESETPoolIndex 0. If the E field is set to 1, the activated TCI states apply to a DCI which is sent via CORESET pool with CORESETPoolIndex 1.[8]

Q9: Which MAC CE format do you prefer to use as a baseline for TCI state activation/deactivation for UE-specific PDSCH MAC CE for mPDCCH case.

- Option 1:The legacy format (R-15), i.e. solution1 & soulution2

- Option 2: The enhanced format (R-16), i.e. solution3 & soulution4

| Company | Preference (option 1/option 2) | Comments |
| --- | --- | --- |
| CATT | Option 1 |  |
| Samsung | Option 1 | It is easy to design and can be differentiated with Rel-16 MAC CE for single PDCCH-based TCI state activation/deactivation. |
| Ericsson | Option 2(Solution4) | Option 4 is the only option that allows single and multiPDCCH operation which is possible when TRPs schedule nonoverlapping (in frequency) PDSCH(s). For separating the mPDCCH and sPDCCH(in which case Option 1(soluition2) is the only option that actually separates the operation), we should consult RAN1 first. Feature Lead summary for Multi-TRP (R1-2001169): 1-6; The current spec allows to configure multi-PDCCH based and single-PDCCH based M-TRP transmission simultaneously to a UE. But there is no agreement for that. - Both [4] and [10] propose to specify in TS 38.214 that when multiple PDCCHs are configured, only one TCI state can be indicated in each PDCCH.; [4],[10]; High. |
| Huawei, HiSilicon | Option 2 | For simplicity and easy understanding, R16 MAC CE is used for M-TRP including M-PDCCH case and S-PDCCH case, and R15 MAC CE is used for S-TRP case. It is strange that R15 MAC CE is used for S-TRP case and M-PDCCH based M-TRP case, while R16 MAC CE is used for S-PDCCH based M-TRP case. Moreover, with R15 MAC CE, it needs to introduce a new LCID or occupying the reserved bit to differentiate the use case of the MAC CE. While, with Option 2 (solution 3), all these efforts can be avoided as the configuration of CORESETPoolIndex can be used to differentiate the use case. |
| OPPO | Option 1 |  |
| Qualcomm | Option 1 |  |
| vivo | Option 1 |  |

- Q10: If the legacy (R-15) MAC CE format is preferred for the multiple PDCCH case (option 1 for Q9), which option do you prefer to map the received TCI state activation/deactivation MAC CE to TRP?

- Option 1: Introduce a new LCID for TRP2 transmission(solution 1)

- Option 2: Use the R bit in legacy MAC CE for identifying which TRP it is related to(solution2)

| Company | Preference (option 1/option 2) | Comments |
| --- | --- | --- |
| CATT | Option 1 |  |
| Samsung | Option 1 or Option 2 | Have no strong view on this, seems no functional differences between both options. |
| Ericsson |  | IF RAN1 is ok to separate operation between sPDCCH and mPDCCH. Then, Option 2 achieves this but Option 1 does not as the for mPDCCH for TRP 0 one would use same MAC CE as for sPDCCH. However, if we do not support simultaneous operation we should consult RAN1 that this really is the intention. |
| OPPO | Option 1 |  |
| Qualcomm | Option 2 | Option 2 offers more flexibility without much complexity. The MAC CE can be used to activate/deactivate the TCI states for PDSCH from any TRP, which increases the operation flexibility. |
| vivo | Option 1 |  |

- Q11: If the enhanced MAC CE (R16) format is preferred for the multiple PDCCH case (option2 for Q9), which option do you prefer to map the received TCI state activation/deactivation MAC CE to TRP?

- Option 1: Use TCI state field directly to indicate the activation of TCI state for TRP1 and TRP2 transmission correspondingly(solution 3)

- Option 2: Use the R bit in the enhanced MAC CE(R16) to map the MAC CE to TRP transmission(solution 4)

| Company | Preference (option 1/option 2) | Comments |
| --- | --- | --- |
| Ericsson | Option2(Solution4) | Option 4 is the only option that allows single and multiPDCCH operation which is possible when TRPs schedule nonoverlapping (in frequency) PDSCH(s). For separating the mPDCCH and sPDCCH Feature Lead summary for Multi-TRP (R1-2001169): 1-6; The current spec allows to configure multi-PDCCH based and single-PDCCH based M-TRP transmission simultaneously to a UE. But there is no agreement for that. - Both [4] and [10] propose to specify in TS 38.214 that when multiple PDCCHs are configured, only one TCI state can be indicated in each PDCCH.; [4],[10]; High. |
| Huawei, HiSilicon | Option 1 | In Option 2, the R16 MAC CE can only activate TCI-state for one TRP. And it needs two MAC CE for TCI-state activation of the two TRPs. This is really low in efficiency as the R16 MAC CE is originally designed to activate TCI-states for two TRPs simultaneously. Moreover, with option 2, the reserved R bit is used to differentiate the TRP that the activated TCI-state is mapped to. While, with Option 1, the TCI-state of the two TRPs can be activated simultaneously with one MAC CE. Besides, the R bit can be saved as it adopts the configuration of CORESETPoolIndex for implicit indication. Such indication mechanism has already been adopted in RAN1 spec widely. |

# Conclusion

# Reference

- R2-2000096	Reply LS on multi PDCCH-based and single PDCCH-based multi-TRP operation (R1-1913463; contact: Huawei)	RAN1

- R2-2000385	MAC CEs regarding multiple CCs/BWPs	vivo

- R2-2000659	CC list-based SRS Activation  MAC CE	OPPO

- R2-2000660	Report of [108#68][NR eMIMO] Design of DL MAC CEs	OPPO

- R2-2000766	Enhancement of multiple PDCCH-based TRP transmission	Samsung

- R2-2000890	Views on eMIMO MAC CEs	CATT

- R2-2001034	Design of MIMO DL MAC CE	Qualcomm Incorporated

- R2-2001126	Remaining update for PDSCH TCI state MAC CE	Ericsson

- R2-2001128	New MAC CE for indicating spatial resource for PUCCH resources	Ericsson

- R2-2001196	MAC CE signalling for multi-beam enhancement	Huawei, HiSilicon

- R2-2001465	Considerations on TCI state MAC CE for mPDCCH mTRP transmission	ZTE Corporation, Sanechips

- R2-1914676	MAC CE signaling for multi-beam enhancement	Huawei, HiSilicon	discussion