3GPP TSG-RAN WG2 Meeting #109-e                           R2-200xxxx

- Electronic meeting, 24th February - 6th March, 2020

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.306 | CR | 1712 | rev | 4 | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of UE capabilities for further performance enhancement for LTE in high speed scenario in Rel-16 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC, Huawei, HiSilicon, NTTDOCOMO INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_high_speed_enh2 |  |  |  |  |  |  | Date: |  | 2019-10-03 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduce the UE capability of further enhanced measurement, the UE capability of SCell measurement enhancement, the UE capability of further enhanced UE demodulation for LTE high speed scenario in Rel-16. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. UE capability of further enhanced measurement is to indicate whether the UE supports further enhanced measurements on PCC and timing adjustments for LTE high speed scenario in Rel-16 (supported velocity is up to 500km/h). 2. UE capability of SCell measurement enhancement is to indicate whether the UE supports to apply the enhanced measurements requirements in R14 (the maximum supported velocity is 350km/h) to SCC with active SCells or deactivated SCells 3. UE capability of further enhanced UE demodulation is to indicate whether the UE supports further enhanced demodulation requirements to support 500km/h velocity in HST-SFN scenario for LTE HST-SFN scenario in Rel-16. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE capability of further enhanced measurement, the UE capability of SCell measurement enhancement, the UE capability of further enhanced UE demodulation LTE high speed scenario in Rel-16 are not defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.33.x (new), 4.3.33.y (new), 4.3.33.z (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS 36.331 CR 4095 |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## << Start of changes >>

- 4.3.33	High speed enhancement parameters

- 4.3.33.1	measurementEnhancements-r14

This field defines whether UE supports measurement enhancements in high speed scenario as specified in TS 36.133 [16].

- 4.3.33.2	demodulationEnhancements-r14

This field defines whether the UE supports advanced receiver in SFN scenario as specified in TS 36.101 [6].

- 4.3.33.3	prach-Enhancements-r14

This field defines whether the UE supports random access preambles generated from restricted set type B in high speed scenario as specified in TS 36.211 [17].

- 4.3.33.x	measurementEnhancements2-r16

This field defines whether UE supports further enhanced measurements on PCC and timing adjustments to support 500km/h velocity in HST-SFN scenario as specified in TS 36.133 [16]. A UE indicating support of measurementEnhancements2-r16 shall also indicate support of measurementEnhancements-r14.

- 4.3.33.y	demodulationEnhancements2-r16

This field defines whether the UE supports further enhanced demodulation requirements to support 500km/h velocity in HST-SFN scenario as specified in TS 36.101 [6]. A UE indicating support of demodulationEnhancements2-r16 shall also indicate support of demodulationEnhancements-r14.

- 4.3.33.z	measurementEnhancementsSCell-r16

This field defines whether the UE supports enhanced measurements on SCC to support 350km/h velocity with active SCells or deactivated SCells as specified in TS 36.133 [16].

## << End of change >>