Agenda Item:	7.2.3, 7.1.3

Source: 	Qualcomm Incorporated

Title:    	WUS: Finalise the signalling

Document for:		Discussion and decision

# Introduction

The purpose of this offline is to progress on the following agreements:

Agreements: - For NB-IoT, RAN2 agree signaling changes proposed in Table 5 as the baseline. - For NB-IoT, RAN2 assume the changes proposed in Table 7, 8 and 9 as the baseline for signalling group WUS information. - For NB-IoT, RAN2 use the changes proposed in Table 10 as the baseline. - For eMTC, RAN2 agree to use the changes proposed in Table 12 as the baseline. - For eMTC, RAN2 assume the changes proposed in Table 15, 16 and 17 as the baseline for signalling group WUS information.

For detailed description, reader is advised to look at [1].

This revision of the document incorporates agreements reached from offline [AT109e][305][NBIOT/EMTC] in R2-20001789 as shown below. The agreements/working assumptions/FFSes affecting signalling are hihglighted.

Agreements: - For eMTC and NB-IoT support the same paging probability range and granularity. - No special handling of WUS resource overlap is specified and UE use the WUS resource corresponding to its gap capability - Update stage 2 to explain group WUS in more detail using text proposed in R2-2000639 as starting point. - From RAN2 point of view paging escalation does not need to be mandated Working assumption: - For NB-IoT, if only one R16 WUS resource is configured and no Release 15 WUS resource is configured then R16 WUS resource is always in primary location - Support of Release 16 WUS is independent to support of Release 15 WUS - Define WUS group selection based on the formula defined in R2-2001472 FFS: - Code points for paging probability thresholds. - Mechanism to minimize false wake-up

# Discussion

All proposed changes for ASN.1 and field descriptions are shown in red.

## WUS group signalling for NB-IoT

The common WUS group signalling for NB-IoT is shown in Table 1 with the detail IEs shown in Table 3.

Table 1: R16 WUS common signaling for NB-IoT – ASN.1

RadioResourceConfigCommonSIB-NB-r13 ::= SEQUENCE { …. [[ nprach-Config-v1530 NPRACH-ConfigSIB-NB-v1530 OPTIONAL, -- Need OR dl-Gap-v1530 DL-GapConfig-NB-v1530 OPTIONAL, -- Cond TDD wus-Config-r15 WUS-Config-NB-r15 OPTIONAL -- Need OR ]], [[ nprach-Config-v1550 NPRACH-ConfigSIB-NB-v1550 OPTIONAL -- Cond TDD1 ]], [[ gwus-Config-r16 GWUS-Config-NB-r16 OPTIONAL -- Need OR ]] }

Table 2: R16 WUS signaling for NB-IoT – Field description

RadioResourceConfigCommonSIB-NB field descriptions

defaultPagingCycle Default paging cycle, used to derive 'T' in TS 36.304 [4]. Value rf128 corresponds to 128 radio frames, rf256 corresponds to 256 radio frames and so on.

dl-Gap Downlink transmission gap configuration for the anchor carrier. See TS 36.211 [21], clause 10.2.3.4. If the field is absent, there is no gap.

modificationPeriodCoeff Actual modification period, expressed in number of radio frames= modificationPeriodCoeff * defaultPagingCycle. n16 corresponds to value 16, n32 corresponds to value 32, and so on. The BCCH modification period should be larger or equal to 40.96s.

nB Parameter: nB is used as one of parameters to derive the Paging Frame and Paging Occasion according to TS 36.304 [4]. Value in multiples of 'T' as defined in TS 36.304 [4]. A value of fourT corresponds to 4 * T, a value of twoT corresponds to 2 * T and so on.

npdcch-NumRepetitionPaging Maximum number of repetitions for NPDCCH common search space (CSS) for paging, see TS 36.213 [23], clause 16.6.

wus-Config For FDD: WUS Configuration.

| Conditional presence | Explanation |
| --- | --- |
| EnhPowerControl | This field is optional present, Need OR, if PowerRampingParameters-NB-v1450 is included in SIB2-NB. Otherwise the field is not present. |
| TDD | The field is optionally present, Need OR, for TDD; otherwise the field is not present and the UE shall delete any existing value for this field. |
| TDD1 | The field is mandatory present for TDD; otherwise the field is not present and the UE shall delete any existing value for this field. |

No other changes are needed to RadioResourceConfigCommonSIB-NB and field descriptions. The presence of wus-Config-R16 is not conditional on presence of wus-Config-R15, and vice versa.

Question 1.	Do you support the ASN.1 coding shown in Table 1 as the baseline?

| Company | Yes/No | If no, suggest changes |
| --- | --- | --- |
| Huawei, HiSilicon | no | Fine with the structure but propose to use gwus-Config-r16 / GWUS-Config-NB-r16 as WUS and GWUS are different features. |
| Qualcomm |  | Ok to change |

Summary 1: TBD

Proposal 1:	TBD.

Table 3: R16 WUS-Config signaling for NB-IoT – ASN.1

-- ASN1START WUS-Config-NB-r15 ::= SEQUENCE { maxDurationFactor-r15 WUS-MaxDurationFactor-NB-r15, numPOs-r15 ENUMERATED {n1, n2, n4} DEFAULT n1, numDRX-CyclesRelaxed-r15 ENUMERATED {n1, n2, n4, n8}, timeOffsetDRX-r15 ENUMERATED {ms40, ms80, ms160, ms240}, timeOffset-eDRX-Short-r15 ENUMERATED {ms40, ms80, ms160, ms240}, timeOffset-eDRX-Long-r15 ENUMERATED {ms1000, ms2000} OPTIONAL, -- Need OP ... } GWUS-Config-NB-r16 ::= SEQUENCE { gwus-GroupAlternation-r16 ENUMERATED (true) OPTIONAL, -- Need OR gwus-CommonSequence-r16 ENUMERATED {legacyWUS, groupWUS} OPTIONAL, -- Need OR gwus-TimeParameters-r16 WUS-Config-NB-r15 OPTIONAL, -- Cond No-WUS-Config-r15 gwus-ResourceConfigDRX-r16 GWUS-ResourcePerGapConfig-NB-r16, gwus-ResourceConfig-eDRX-Short-r16 CHOICE { useDRX NULL, explicit-r16 GWUS-ResourcePerGapConfig-NB-r16 } OPTIONAL, -- Need OR gwus-ResourceConfig-eDRX-Long-r16 CHOICE { use-DRX-or-eDRX-Short-r16 NULL, explicit-r16 GWUS-ResourcePerGapConfig-NB-r16 } OPTIONAL, -- Need OR gwus-ProbThreshList -r16 GWUS-ProbThreshList-NB-r16 OPTIONAL -- Need OR } WUS-ConfigPerCarrier-NB-r15 ::= SEQUENCE { maxDurationFactor-r15 WUS-MaxDurationFactor-NB-r15 } WUS-MaxDurationFactor-NB-r15 ::= ENUMERATED {one128th, one64th, one32th, one16th, oneEighth, oneQuarter, oneHalf} GWUS-ResourcePerGapConfig-NB-r16 ::= SEQUENCE { gwus-ResourcePosition-NB-r16 ENUMERATED {primary, secondary}, gwus-NumGroupsList-r16 SEQUENCE (SIZE (1..maxGWUS-Resources-NB-r16)) OF GWUS-NumGroups-NB-r16 OPTIONAL, -- Need OP gwus-GroupsForServiceList-r16 SEQUENCE (SIZE (1..maxGWUS-ProbThresholds-NB-r16)) OF INTEGER (1..maxGWUS-Groups-1-NB-r16) OPTIONAL -- Need OR } GWUS-NumGroups-NB-r16 ::= ENUMERATED {n1, n2, n4, n8} GWUS-ProbThreshList-NB-r16 ::= SEQUENCE (SIZE (1..maxGWUS-ProbThresholds-NB-r16)) OF GWUS-Paging-ProbThresh-NB-r16 GWUS-Paging-ProbThresh-NB-r16 ::= ENUMERATED {TBD} -- Editor’s Note: following constants to be moved to appropriate section in final CR. maxGWUS-Groups-1-NB-r16 ::= 15 maxGWUS-Resources-NB-r16 ::= 2 maxGWUS-ProbThresholds-NB-r16 ::= 3

Table 4: R16 WUS-Config signaling for NB-IoT – Field description

WUS-Config-NB field descriptions

gwus-CommonSequence Indicates common WUS sequence is configured. Value legacyWUS indicates for the shared WUS resource the legacy WUS sequence, value groupWUS indicates for the shared WUS resource the group WUS sequence, see TS 36.211[21].

gwus-GroupAlternation Enables hopping between the two WUS resources for the gap type, see TS 36.304[4].

gWUS-GroupsForServiceList Number of WUS groups for each paging probability group, see TS 36.304 [4]. The first entry corresponds to the first probability group, second entry corresponds to the second paging probability group, and so on. Any WUS group from the list of WUS groups defined in the numWUS-GroupsPerResourceList that are not assigned to a probability group is part of the list used for UE ID based grouping only. Total number of WUS groups in this list cannot be more than total number of WUS groups in gwus-NumGroupsList. If this field is absent, paging probability based WUS group selection is not configured.

gwus-NumGroupsList List of WUS groups for each WUS resource, see TS 36.304 [4]. First entry corresponds to the first resource, the second entry corresponds to the second resource. gwus-NumGroupsList is mandatory present in gwus-ResourceConfigDRX. If gwus-NumGroupsList is not present in gwus-Resourceconfig-eDRX-Short-r16 then gwus-NumGroupsList from gwus-ResourceConfigDRX applies. If gwus-NumGroupsList is not present in gwus-ResourceConfig-eDRX-Long and gwus-NumGroupsList is present in gwus-ResourceConfig-eDRX-Short then gwus-NumGroupsList from gwus-ResourceConfig-eDRX-Short applies. If gwus-NumGroupsList is not present in gwus-ResourceConfig-eDRX-Long and gwus-NumGroupsList is not present in gwus-ResourceConfig-eDRX-Short then gwus-NumGroupsList from gwus-ResourceConfigDRX applies.

gwus-ProbThreshList Paging probability thresholds corresponding to the paging probability groups, see TS 36.304 [4]. If this field is absent, then paging probability based WUS group selection is not configured.

gwus-ResourceConfigDRX, gwus-ResourceConfig-eDRX-Short, gwus-ResourceConfig-eDRX-Long WUS resource configured for each gap type, see TS 36.304 [4]. If gwus-ResourceConfig-eDRX-Long is not present but timeOffset-eDRX-Long is present in GWUS-TimeParameters and gwus-ResourceConfig-eDRX-Short is present, gwus-ResourceConfig-eDRX-Short parameters apply for long eDRX group WUS resource . If gwus-ResourceConfig-eDRX-Long is not present but timeOffset-eDRX-Long is present in GWUS-TimeParameters and gwus-ResourceConfig-eDRX-Short is not present, gwus-ResourceConfigDRX parameters apply for long eDRX group WUS resource .

gwus-ResourcePosition Indicates the position of the WUS resource corresponding to the first entry in gwus-NumGroupsList-r16 Value primary indicates that the end of the WUS resource is defined by the timeoffset value for the corresponding gap type, value secondary indicates that the end of the WUS resource is immediately before the WUS resource configured by wus-Config-r15. E-UTRAN may only configure secondary when there is only one entry exists in gwus-NumGroupsList-r16 and wus-Config-r15 is present in SystemInformationBlockType2-NB. If two entries exist in gwus-NumGroupsList-r16, the position for the second WUS resource corresponds to value secondary.

numDRX-CyclesRelaxed Maximum number of consecutive DRX cycles during which the UE may use WUS for synchronisation and skip serving cell measurements, see TS 36.133 [16]. Value n1 corresponds to 1 DRX cycle, value n2 corresponds to 2 DRX cycles and so on.

numPOs Number of consecutive Paging Occasions (PO) mapped to one Wake Up Signal (WUS), applicable to UEs configured to use extended DRX, see TS 36.304 [4]. Value n1 corresponds to 1 PO and value n2 corresponds to 2 POs and so on.

timeOffsetDRX When DRX is used, non-zero gap from the end of the configured maximum WUS duration to the associated PO, see TS 36.304 [4], clause 7.4 and TS 36.211 [21]. In milliseconds. Value ms40 corresponds to 40ms, value ms80 corresponds to 80 ms and so on.

timeOffset-eDRX-Short When eDRX is used, the short non-zero gap from the end of the configured maximum WUS duration to the associated PO, see TS 36.304 [4], clause 7.4 and TS 36.211 [21]. In milliseconds. Value ms40 corresponds to 40ms, value ms80 corresponds to 80 ms and so on. E-UTRAN configures timeOffset-eDRX-Short to a value longer than or equal to timeOffsetDRX.

timeOffset-eDRX-Long When eDRX is used, the long non-zero gap from the end of the configured maximum WUS duration to the associated PO, see TS 36.304 [4], clause 7.4 and TS 36.211 [21]. In milliseconds. Value ms1000 corresponds to 1000 ms, value ms2000 corresponds to 2000 ms.

Table 5: R16 WUS-Config signaling for NB-IoT – conditional presence

| Conditional presence | Explanation |
| --- | --- |
| No-WUS-Config-r15 | The field is mandatory present if wus-Config-r15 is not present in SystemInformationBlockType2-NB; otherwise the field is not present, and the UE shall delete any existing value for this field. |

Question 2.	Do you support the ASN.1 coding and associated field description shown in Table 3, Table 4 and Table 5 as the baseline?

| Company | Yes/No | If No, then suggest changes |
| --- | --- | --- |
| Huawei, HiSilicon | No | ASN.1 comments: 1. In general, the prefix always uses small letter, so we think we should use ‘gwus-‘ instead of ‘gWUS’ in the parameter names. 2. gWUS-UE-GroupAlternation: why do we need ‘-UE’ in the parameter name, we suggest gwus-GroupAlternation-r16 3. gWUS-CommonWUS-Sequence: we think the parameter should be conditional, Mandatory present in case of shared resource with Rel-15 WUS for any gap type, absent otherwise 4. gWUS-TimeParameters-r16: assuming that WUS and GWUS are different features, the condition could just be ‘NoWUS’. 5. align the name of the parameters with Rel-15 WUS: gWUS-ResourceConfig-DRX-r16  gwus-ResourceConfigDRX-r16 gWUS-ResourceConfigShort-eDRX-r16  gwus-ResourceConfig-eDRX-Short-r16 gWUS-ResourceConfigLong-eDRX-r16  gwus-ResourceConfig-eDRX-Long-r16 6.gWUS-ResourceConfigShort-eDRX-r16/ gWUS-ResourceConfigLong-eDRX-r16: the CHOICE structure is not correct: gwus-ResourceConfig-eDRX-Short-r16 CHOICE { useDRX-r16 NULL, explicit-r16 WUS-ResourcePerGapConfig-NB-r16 } OPTIONAL, -- Need OR The use of a CHOICE introduces one extra bit signalling compared to the initial proposal. gwus-ResourceConfig-eDRX-Short-r16 WUS-ResourcePerGapConfig-NB-r16 OPTIONAL, -- Cond timeOffset If we go with the CHOICE structure, we need to revise the field description, i.e. no need to describe rules for delta configuration. 7. wus-ProbabilityThresholdList  gwus-ProbabilityThresholdList-r16 WUS-ProbabilityThresholdList-R16  GWUS-ProbabilityThresholdList-NB-r16 8. resourcePosition-NB-r16  gwus-ResourcePosition-r16 9. gWUS-NumGroupsList-r16: we still think optionality is not needed, this extra signalling flexibility (same number of resources, same number of groups but different probability thresholds) is not useful and make the specification more complex. 10. gWUS-GroupsForServiceList-r16: WUS-Groups-NB-r16-1  maxGWUS-Groups-1-NB-r16 11. Paging-ProbabilityThreshold-r16  GWUS- Paging-ProbabilityThreshold-NB-r16 we usually use ‘p’ for probability, e.g. p00. Now changed to TBD but for next time. 12. WUS-Groups-NB-r16 ::= 16  maxGWUS-Groups-1-NB-r16 ::= 15 13. could make the names a bit shorter, e.g. Probability  Prob, Threshold  Thresh Field description comments: 1. gWUS-CommonWUS-Sequence: The definition is very confusing. Assuming conditional presence as proposed above, we propose: If present, Indicates the common WUS sequence on the WUS resource shared with Rel-15 WUS resource, see TS 36.211[21] is configured. Value legacyWUS indicates the legacy WUS sequence for the shared WUS resource the common WUS resource is non-group WUS. Value groupWUS indicates the common WUS group sequencefor the shared WUS resource the common WUS resource is group WUS, see TS 36.211[21]. 2. gWUS-UE-GroupAlternation: we propose: Enables hopping between the two WUS resources configured in time domain for the gap type, see TS 36.304[4]. 3. gWUS-NumGroupsList List of WUS groups for each WUS resource, see TS 36.304 [4]. First entry indicates the number of WUS groups for corresponds to the first resource, the second entry indicates the number of WUS groups for corresponds to the second resource, and so on. gwusWUS-NumGroupsList shall be is mandatory present in gwusWUS -ResourceConfigDRX. (actually it may be better to have a condition) 4. gwusWUS-ResourceConfig-DRX, gwusWUS -ResourceConfigShort-eDRX, gwusWUS-ResourceConfigLong-eDRX: WUS resource configurationed for each gap type, see TS 36.304 [4]. Text to be revised if we keep the CHOICE structure (see ASN.1 comments above), the delta configuration rules are implicit by the CHOICE entries. If gWUS-ResourceConfigShort-eDRX is not present, gWUS-ResourceConfigDRX parameters apply for short eDRX group WUS resource configuration with short eDRX. If gWUS-ResourceConfigLong-eDRX is not present but timeOffset-eDRX-Long is present in GWUS-TimeParameters and gWUS-ResourceConfigShort-eDRX is present, gWUS-ResourceConfig-eDRX parameters apply for long eDRX group WUS resource configuration with long eDRX. If gWUS-ResourceConfigLong-eDRX is not present but timeOffset-eDRX-Long is present in GWUS-TimeParameters and gWUS-ResourceConfigShort-eDRX is not present, gWUS-ResourceConfig-DRX parameters apply for long eDRX group WUS resource configuration with long eDRX. 5. gwusWUS-GroupsForServiceList Number of WUS groups for each paging probability group, see TS 36.304 [4]. The first entry corresponds to the first probability group, the second entry corresponds to the second paging probability group, and so on. Any WUS group from the list of WUS groups definedconfigured in the numgwus-NumGroupsPerResourceList that areis not assigned to a probability group is considered to be part of the list of WUS groups used for UE ID based grouping only. Total number of WUS groups in this list cannot be more than total number of WUS groups in gwusWUS-NumGroupsList. If this field is absent, paging probability based WUS group selection is not used. 6. gwus-probabilityThresholdList Paging probability thresholds corresponding to the WUS paging probability groups, see TS 36.304 [4]. If this field is absent, then paging probability based WUS group selection is not used. 7. gwus-ResourcePosition The parameter is mandatory so always present so the description should be revised. Indicates the position of the WUS resource when only one entry exists corresponding to the first entry in gwusWUS-NumGroupsList-r16. If wus-Config-r15 is configured in RadioResourceConfigCommonSIB-NB, value secondary indicates the end of group WUS resource is immediately before the start of WUS resource configured by wus-Config-r15. Otherwise end of group WUS resource is defined by the applicable timeoffset value in WUS-Config-NB-r15 IE. Value primary indicates that the end of the WUS resource is defined by the timeoffset value in gwus-TimeParameters for the corresponding gap type, value secondary indicates that the end of the WUS resource is immediately before the WUS resource configured by wus-Config-r15. E-UTRAN only configures value secondary if wus-Config-r15 is present in systemInformationBlockType2-NB. 8. No-WUS-Config-r15: The field is mandatorily present if wus-Config-r15 is not present in RadioResourceConfigCommonSIB-NB-r13 systemInformationBlockType2-NB; otherwise the field is not present, and the UE shall delete any existing value for this field |
| Qualcomm |  | Feedback to points raised by HW for ASN.1. [3]: gWUS-CommonWUS-Sequence: This IE needs to cover the following states: - No common WUS configured in the cell. This is regardless of R16 WUS only configured or both R16 and R15 WUS are configured. The absence of gWUS-CommonWUS-Sequence covers this state. - Common WUS configured in the cell and use legacy sequence for WUS resource shared amongst rel 15 and r16. - Common WUS configured in the cell and group sequence for WUS resource shared amongst rel 15 and r16. Therefore, we think the coding for gWUS-CommonWUS-Sequence fulfills the requirements. [4] Condition for inclusion of gWUS-TimeParameters-r16. I think it is more clearer to use ‘No-WUS-Config-r15’ instead of ‘NoWUS’ because WUS is somewhat a generic term. Therefore prefer to keep No-WUS-Config-r15. [6]: For gwus-ResourceConfig-eDRX-Short-r16 IE needs to have the following: - No short eDRX WUS configured. This state is needed because RAN1 description for gWUS-numGroups-eDRX-short implies it is optional (see [3]). Note, the timeOffset-eDRX-Short is mandatory in WUS-Config-NB-r15/WUS-Config-r!5. - short eDRX WUS configured with same configuration as DRX - short eDRX WUS configured with explicit configuration. We think the proposed coding fulfills these three requirements. Ok to remove description for delta configuration for gWUS-numGroups-eDRX-short. [9]: We think it is not reasonable to restrict specification to support only same number of WUS groups for each gap type. We think the overhead to provide this flexibility is small. |

Summary 2: TBD.

Proposal 2:	TBD.

Table 6: R16 WUS-Config signaling in SIB22-NB for NB-IoT – ASN.1

-- ASN1START SystemInformationBlockType22-NB-r14 ::= SEQUENCE { dl-ConfigList-r14 DL-ConfigCommonList-NB-r14 OPTIONAL, -- Need OR ul-ConfigList-r14 UL-ConfigCommonList-NB-r14 OPTIONAL, -- Need OR …. DL-ConfigCommon-NB-r14 ::= SEQUENCE { dl-CarrierConfig-r14 DL-CarrierConfigCommon-NB-r14, pcch-Config-r14 PCCH-Config-NB-r14 OPTIONAL, -- Need OR ..., [[ wus-Config-r15 WUS-ConfigPerCarrier-NB-r15 OPTIONAL -- Cond WUS ]], [[ gwus-Config-r16 CHOICE { useWUS-r16 NULL, explicit-r16 GWUS-ConfigPerCarrier-NB-r15 } OPTIONAL -- Cond GWUS ]] }

Table 7: R16 WUS-Config signaling in SIB22-NB for NB-IoT – Field description

SystemInformationBlockType22-NB field descriptions

dl-CarrierConfig For FDD: Provides the configuration of the DL non-anchor carrier. For TDD: Provides the configuration of the non-anchor carrier.

dl-ConfigList, dl-ConfigListMixed For FDD: List of DL non-anchor carriers and associated configuration that can be used for paging and/or random access. E-UTRAN configures DL non-anchor carriers operating in mixed operation mode only in dl-ConfigListMixed and only a UE that supports mixed operation mode uses the carriers in dl-ConfigListMixed. A given carrier is either signalled in the dl-ConfigList or in dl-ConfigListMixed. If dl-ConfigListMixed is present and at least one of the carriers in dl-ConfigListMixed is configured for paging: - If pagingDistribution is present, the UE supporting mixed operation mode creates a combined list of DL carriers for paging by appending dl-ConfigListMixed to the dl-ConfigList while maintaining the order among dl-ConfigList and dl-ConfigListMixed; the total number of signalled DL non-anchor carriers cannot be more than maxNonAnchorCarriers-NB-r14. - If pagingDistribution is absent, the UE supporting mixed operation mode uses the list of DL carriers for paging provided in dl-ConfigListMixed and considers pagingWeightAnchor being set to w0, i.e. the anchor carrier is not used. Otherwise, the pagingDistribution field is not applicable and the UE shall ignore the value. For TDD: List of non-anchor carriers and associated configuration that can be used for paging and/or random access.

mixedOperationModeConfig For FDD: Provides the configuration of DL and UL non-anchor carriers that can be used for paging and random access by a UE that supports mixed operation mode. For TDD: This parameter is absent.

npdcch-NumRepetitionPaging Maximum number of repetitions for NPDCCH common search space (CSS) for paging, see TS 36.213 [23], clause 16.6. If the field is absent, the value of npdcch-NumRepetitionPaging configured in SystemInformationBlockType2-NB in IE pcch-Config applies.

nprach-Distribution Indicates which UL carriers a UE supporting mixed operation mode uses for random access as defined in description of ul-ConfigList, ul-ConfigListMixed.

nprach-ParametersList, nprach-ParametersList-EDT Configure NPRACH parameters for each NPRACH resource on one non-anchor UL carrier. Up to three NPRACH resources can be configured on one non-anchor UL carrier. Each NPRACH resource is associated with a different number of NPRACH repetitions. NPRACH resources in nprach-ParametersListEDT are used to initiate EDT. Each NPRACH resource is associated with a maximum TBS signalled in the corresponding entry of edt-TBS-InfoList in SystemInformationBlockType2-NB. E-UTRAN includes the same number of entries, and listed in the same order, as in nprach-ParametersList in SystemInformationBlockType2-NB.

nprach-ParametersListTDD For TDD: Configure NPRACH parameters for each NPRACH resource on one non-anchor UL carrier. Up to three NPRACH resources can be configured on one non-anchor UL carrier. Each NPRACH resource is associated with a different number of NPRACH repetitions. E-UTRAN includes the same number of entries in nprach-ParametersListTDD, and listed in the same order, as in nprach-ParametersListTDD in SystemInformationBlockType2-NB..

nprach-ProbabilityAnchor Configure the selection probability for the anchor carrier NPRACH resource, see TS 36.321 [6]. Value zero corresponds to a probability of 0, oneSixteenth corresponds to the probability of 1/16, oneFifteenth corresponds to the probability of 1/15, and so on. If the field is absent, the selection probability of the anchor carrier NPRACH resource is 1. All non-anchor carriers NPRACH resources have equal probability between them. If there is no NPRACH resource defined on the anchor carrier for one repetition level in nprach-ParametersList-EDT, (respectively nprach-ParametersListFmt2, nprach-ParametersListFmt2-EDT), the UE shall use the value 'zero' and ignore the signalled value of nprach-ProbabilityAnchor for this repetition level for the NPRACH resources defined by nprach-ParametersList-EDT (respectively nprach-ParametersListFmt2, nprach-ParametersListFmt2-EDT).

nprach-ProbabilityAnchorList Configures the selection probability for each NPRACH resource on the anchor carrier. E-UTRAN includes the same number of entries, and listed in the same order, as in nprach-ParametersList in SystemInformationBlockType2-NB.

pagingDistribution Indicates which DL carriers a UE supporting mixed operation mode monitors for paging as defined in description of dl-ConfigList, dl-ConfigListMixed.

pagingWeight Weight of the non-anchor paging carrier for uneven paging load distribution across the carriers. Value w1 corresponds to a relative weight of 1, w2 corresponds to a relative weight of 2, and so on. The paging load for a carrier 'i' is equal to w(i)/W where i is equal to 0 for the anchor carrier and equal to the index of the carrier in the dl-ConfigList / dl-ConfigListMixed for a non-anchor carrier, W is the sum of the weights of all paging carriers. To avoid correlation between paging carrier and paging occasion, the weights should be assigned such that: nB * W <= 16384.

pagingWeightAnchor Weight of the anchor carrier for uneven paging load distribution across the carriers. Value w1 corresponds to a relative weight of 1, w2 corresponds to a relative weight of 2, and so on. If the field is absent, the (default) value of w0 is applied, i.e. the anchor carrier is not used for paging.

pcch-Config Configure the PCCH parameters for the non-anchor DL carrier.

tdd-UL-DL-AlignmentOffset Indicates the offset between the UL carrier frequency center with respect to DL carrier frequency center for the non-anchor carrier.

ul-CarrierFreq For FDD: UL carrier frequency of the non-anchor carrier as defined in TS 36.101 [42], clause 5.7.3F. For TDD: This field is absent and the uplink carrier frequency is same as the downlink frequency.

ul-ConfigList, ul-ConfigListMixed For FDD: List of UL non-anchor carriers and associated configuration that can be used for random access. E-UTRAN configures UL non-anchor carriers operating in mixed operation mode only in ul-ConfigListMixed and only a UE that supports mixed operation mode uses the carriers in ul-ConfigListMixed. A given carrier is either signalled in the ul-ConfigList or in ul-ConfigListMixed. If ul-ConfigListMixed is present and at least one of the carriers in ul-ConfigListMixed is configured for random access: - If nprach-Distribution is present, the UE supporting mixed operation mode creates a combined list of UL carriers for random access by appending ul-ConfigListMixed to the ul-ConfigList while maintaining the order among both ul-ConfigList and ul-ConfigListMixed; the total number of signalled UL non-anchor carriers cannot be more than maxNonAnchorCarriers-NB-r14. - If nprach-Distribution is absent, the UE supporting mixed operation mode uses the list of UL carriers for random access provided in ul-ConfigListMixed and considers nprach-ProbabiliyAnchor being set to zero for each NPRACH resource, i.e. the anchor carrier is not used for random access. Otherwise, the nprach-Distribution field is not applicable and the UE shall ignore the value. For TDD: E-UTRAN configures ul-ConfigList-r15 and includes the same number of entries as in dl-ConfigList. The UL carrier frequency of the non-anchor carrier is same as the DL carrier frequency.

wus-ConfigPerCarrier For FDD: Carrier specific WUS Configuration. E-UTRAN only configures value explicit if wus-Config-r15 is not present for the carrier. If the field is absent, GWUS is not supported for the carrier.

Table 8: R16 WUS-Config signaling in SIB22-NB for NB-IoT – condition description

| Conditional presence | Explanation |
| --- | --- |
| dl-ConfigList | This field is optionally present, Need OR, if the field dl-ConfigList is present. Otherwise the field is not present. |
| EDT | The field is optionally present, Need OR, if edt-Parameters in SystemInformationBlockType2-NB is present; otherwise the field is not present and the UE shall delete any existing value for this field. |
| pcch-config | This field is optionally present, Need OP, if the field dl-ConfigList is present and at least one of the carriers in dl-ConfigList is configured for paging. Otherwise the field is not present and only the anchor carrier is used for paging. |
| nprach-config | This field is mandatory present, if the field ul-ConfigList is present and at least one of the carriers in ul-ConfigList is configured for random access. Otherwise the field is not present and only the anchor carrier is used for random access. |
| TDD | This field is optionally present, Need OR, for TDD. Otherwise the field is not present. |
| ul-ConfigList | This field is optionally present, Need OR, if the field ul-ConfigList is present. Otherwise the field is not present. |
| WUS | This field is mandatory present, if the field wus-Config is present in SystemInformationBlockType2-NB. Otherwise the field is not present, Need OR. |
| GWUS | This field is optionally present, Need OR, if gwus-Config-r16 is present in SystemInformationBlockType2-NB. Otherwise the field is not present. |

Question 3.	Do you support the ASN.1 coding and associated field description shown in Table 6, Table 7, and Table 8 as the baseline?

| Company | Yes/No | If No, then suggest changes |
| --- | --- | --- |
| Huawei, HiSilicon | no | 1. gwus-Config-r16 CHOICE { wus-r15useWUS-r16 NULL, wus-r16 explicit-r16 WUS-ConfigPerCarrier-NB-r15 } OPTIONAL -- Cond GWUS-R16 2. wus-ConfigPerCarrier  gwus-Config For FDD: Carrier specific GWUS Configuration. If wus-Config-r16 is present, then group WUS supported for the carrier. if both wus-Config-r15 and wus-Config-r16 is present then wus-Config-r16 shall be set to wus-r15 and the signalled wus-Config-r15 value applies to this carrier. if only wus-Config-r16 is present and it is set to wus-r15 then value defined in SystemInformationBlockType2-NB applies. If only wus-Config-r16 is present and it is set to wus-r16 then value explicitly signalled for this carrier applies. E-UTRAN only configures value explicit if wus-Config-r15 is not present for the carrier. If the field is absent, GWUS is not supported for the carrier. 3. WUS-R16  GWUS This field is optionally present, Need OR, if gwus-Config is present in SystemInformationBlockType2-NB. Otherwise the field is not present. |

Summary 3: TBD.

Proposal 3:	TBD.

## WUS group signalling for eMTC

Table 9: R16 WUS common signaling for eMTC– ASN.1

RadioResourceConfigCommonSIB ::= SEQUENCE { …. [[ wus-Config-v1560 WUS-Config-v1560 OPTIONAL -- Need OR ]], [[ gwus-Config-r16 GWUS-Config-r16 OPTIONAL -- Need OR ]] }

Table 10: R16 WUS signaling for eMTC – Field description

RadioResourceConfigCommon field descriptions

additionalSpectrumEmissionSCell The UE requirements related to additionalSpectrumEmissionSCell are defined in TS 36.101 [42]. E-UTRAN configures the same value in additionalSpectrumEmissionSCell for all SCell(s) of the same band with UL configured. The additionalSpectrumEmissionSCell is applicable for all serving cells (including PCell) of the same band with UL configured.

defaultPagingCycle Default paging cycle, used to derive 'T' in TS 36.304 [4]. Value rf32 corresponds to 32 radio frames, rf64 corresponds to 64 radio frames and so on.

dummy This field is not used in the specification. If received it shall be ignored by the UE.

harq-ReferenceConfig Indicates UL/ DL configuration used as the DL HARQ reference configuration for this serving cell. Value sa2 corresponds to Configuration2, sa4 to Configuration4 etc, as specified in TS 36.211 [21], table 4.2-2. E-UTRAN configures the same value for all serving cells residing on same frequency band.

highSpeedEnhancedMeasFlag If the field is present, the UE shall apply the high speed measurement enhancements as specified in TS 36.133 [16].

highSpeedEnhancedDemodulationFlag If the field is present, the UE shall apply the advanced receiver in SFN scenario as specified in TS 36.101 [6].

highSpeedMeasGapCE-ModeA If the field is present, the UE in CE mode A shall apply the measurement gap sharing table associated with high-velocity scenario for measurements, as specified in TS 36.133 [16].

interval-DLHoppingConfigCommonModeX Number of consecutive absolute subframes over which MPDCCH or PDSCH for CE mode X stays at the same narrowband before hopping to another narrowband. For interval-FDD, int1 corresponds to 1 subframe, int2 corresponds to 2 subframes, and so on. For interval-TDD, int1 corresponds to 1 subframe, int5 corresponds to 5 subframes, and so on.

interval-ULHoppingConfigCommonModeX Number of consecutive absolute subframes over which PUCCH or PUSCH for CE mode X stays at the same narrowband before hopping to another narrowband. For interval-FDD, int1 corresponds to 1 subframe, int2 corresponds to 2 subframes, and so on. For interval-TDD, int1 corresponds to 1 subframe, int5 corresponds to 5 subframes, and so on.

modificationPeriodCoeff Actual modification period, expressed in number of radio frames= modificationPeriodCoeff * defaultPagingCycle. n2 corresponds to value 2, n4 corresponds to value 4, n8 corresponds to value 8, n16 corresponds to value 16, and n64 corresponds to value 64.

mpdcch-NumRepetition-Paging Maximum number of repetitions for MPDCCH common search space (CSS) for paging, see TS 36.211 [21].

mpdcch-pdsch-HoppingOffset Parameter: , see TS 36.211 [21], clause 6.4.1.

mpdcch-pdsch-HoppingNB The number of narrowbands for MPDCCH/PDSCH frequency hopping. Value nb2 corresponds to 2 narrowbands and value nb4 corresponds to 4 narrowbands.

nB Parameter: nB is used as one of parameters to derive the Paging Frame and Paging Occasion according to TS 36.304 [4]. Value in multiples of 'T' as defined in TS 36.304 [4]. A value of fourT corresponds to 4 * T, a value of twoT corresponds to 2 * T and so on. In case nB-v1310 is signalled, the UE shall ignore nB (i.e. without suffix). EUTRAN configures nB-v1310 only in the BR version of SI message.

paging-narrowBands Number of narrowbands used for paging, see TS 36.304 [4], TS 36.212 [22] and TS 36.213 [23].

p-Max Pmax to be used in the target cell. If absent, for the band used in the target cell, the UE applies the maximum power according to its capability as specified in 36.101 [42], clause 6.2.2. In case the UE is configured with uplink intra-band contiguous CA and the UE indicates ue-CA-PowerClass-N in that band combination, then the p-Max in RadioResourceConfigCommonSCell for that SCell, if present, also applies for that band combination whenever that SCell is activated.

prach-ConfigSCell Indicates a PRACH configuration for an SCell. The field is not applicable for an LAA SCell in this release.

rach-ConfigCommonSCell Indicates a RACH configuration for an SCell. The field is not applicable for an LAA SCell in this release.

soundingRS-FlexibleTiming Indicates the SRS flexible timing (if configured) for aperiodic SRS triggered by DL grant. If the SRS transmission is collided with ACK/NACK, postpone once to the next configured SRS transmission opportunity.

ul-Bandwidth Parameter: transmission bandwidth configuration, NRB, in uplink, see TS 36.101 [42], table 5.6-1. Value n6 corresponds to 6 resource blocks, n15 to 15 resource blocks and so on. If for FDD this parameter is absent, the uplink bandwidth is equal to the downlink bandwidth. For TDD this parameter is absent and it is equal to the downlink bandwidth.

ul-CarrierFreq For FDD: If absent, the (default) value determined from the default TX-RX frequency separation defined in TS 36.101 [42], table 5.7.3-1, applies. For TDD: This parameter is absent and it is equal to the downlink frequency.

ul-CyclicPrefixLength Parameter: Uplink cyclic prefix length see TS 36.211 [21], clause 5.2.1, where len1 corresponds to normal cyclic prefix and len2 corresponds to extended cyclic prefix.

| Conditional presence | Explanation |
| --- | --- |
| EDT | The field is optionally present, Need OR, if edt-Parameters is present; otherwise the field is not present and the UE shall delete any existing value for this field. |
| MP-A | The field is mandatory present for CE mode A. Otherwise the field is optional, Need OR. |
| MP-B | The field is mandatory present for CE mode B. Otherwise the field is optional, Need OR. |
| TDD | The field is optional for TDD, Need ON; it is not present for FDD and the UE shall delete any existing value for this field. |
| TDD2 | If tdd-Config-r10 is present, the field is optional, Need OR. Otherwise the field is not present and the UE shall delete any existing value for this field. |
| TDD3 | If tdd-Config is present, the field is optional, Need OR. Otherwise the field is not present and the UE shall delete any existing value for this field. |
| TDD-OR-NoR11 | If prach-ConfigSCell-r11 is absent, the field is optional for TDD, Need OR. Otherwise the field is not present and the UE shall delete any existing value for this field. |
| TDDSCell | This field is mandatory present for TDD; it is not present for FDD and LAA SCell, and the UE shall delete any existing value for this field. |
| UL | If the SCell is part of the STAG or concerns the PSCell or PUCCH SCell and if ul-Configuration is included, the field is optional, Need OR. Otherwise the field is not present and the UE shall delete any existing value for this field. |
| ULSCell | For the PSCell (IE is included in RadioResourceConfigCommonPSCell) the field is absent. Otherwise, if the SCell is part of the STAG and if ul-Configuration is included, the field is optional, Need OR. Otherwise the field is not present and the UE shall delete any existing value for this field. |
| ULSRS | If ul-Configuration-r10 is absent, the field is optional, Need OR. Otherwise the field is not present and the UE shall delete any existing value for this field. |

Question 4.	Do you support the ASN.1 coding shown in Table 9 as the baseline?

| Company | Yes/No | If no, suggest changes |
| --- | --- | --- |

Summary 4: TBD.

Proposal 4: 	TBD.

Table 11: R16 WUS-Config signaling for eMTC – ASN.1

-- ASN1START WUS-Config-r15 ::= SEQUENCE { maxDurationFactor-r15 ENUMERATED {one32th, one16th, one8th, one4th}, numPOs-r15 ENUMERATED {n1, n2, n4, spare1} DEFAULT n1, freqLocation-r15 ENUMERATED {n0, n2, n4, spare1}, timeOffsetDRX-r15 ENUMERATED {ms40, ms80, ms160, ms240}, timeOffset-eDRX-Short-r15 ENUMERATED {ms40, ms80, ms160, ms240}, timeOffset-eDRX-Long-r15 ENUMERATED {ms1000, ms2000} OPTIONAL -- Need OP ... } GWUS-Config-r16 ::= SEQUENCE { gwus-GroupAlternation-r16 ENUMERATED (true) OPTIONAL, -- Need OR gwus-CommonWUS-Sequence-r16 ENUMERATED {LegacyWUS, GroupWUS} OPTIONAL, -- Need OR gwus-TimeParameters-r16 GWUS-Time-Parameters-r16 OPTIONAL, -- Cond NO-WUS-Config-r15 gwus-ResourceConfigDRX-r16 GWUS-ResourcePerGapConfig-r16, gwus-ResourceConfig-eDRX-Short-r16 CHOICE { useDRX-r16 NULL, explicit-r16 GWUS-ResourcePerGapConfig-r16 } OPTIONAL, -- Need OR gwus-ResourceConfig-eDRX-Long-r16 CHOICE { use-DRX-or-eDRX-Short-r16 NULL, explicit-r16 GWUS-ResourcePerGapConfig-r16 } OPTIONAL, -- Need OR gwus-ProbaThreshList-r16 GWUS-ProbThreshList-r16 OPTIONAL, -- Need OR gwus-GroupNarrowBandList-r16 SEQUENCE (SIZE (1..maxAvailNarrowBands-r13)) OF BOOLEAN OPTIONAL -- Need OR } WUS-Config-v1560 ::= SEQUENCE { powerBoost-r15 ENUMERATED {dB0, dB1dot8, dB3, dB4dot8} } GWUS-TimeParameters-r16 ::= SEQUENCE { maxDurationFactor-r15 ENUMERATED {one32th, one16th, one8th, one4th}, numPOs-r15 ENUMERATED {n1, n2, n4, spare1} DEFAULT n1, timeOffsetDRX-r15 ENUMERATED {ms40, ms80, ms160, ms240}, timeOffset-eDRX-Short-r15 ENUMERATED {ms40, ms80, ms160, ms240}, timeOffset-eDRX-Long-r15 ENUMERATED {ms1000, ms2000} OPTIONAL -- Need OP ... } GWUS-ResourcePerGapConfig-r16 ::= SEQUENCE { gwus-resourceMappingPattern-r16 GWUS-ResourceMappingPattern-r16, gwus-NumGroupsList-r16 SEQUENCE (SIZE (1..maxWUS-Resources-r16)) OF GWUS-NumGroups-r16 OPTIONAL, -- Need OP gwus-GroupsForServiceList-r16 SEQUENCE (SIZE (1..maxProbThresholds-r16)) OF INTEGER (1..maxGWUS-Groups-1-r16) OPTIONAL, -- Need OR } GWUS- ResourceMappingPattern-r16 ::= CHOICE { gwus-ResourcePattern-withLegacy-r16 ENUMERATED {RP_ID0, RP_ID1, RP_ID2, RP_ID3, RP_ID4, RP_ID5, RP_ID6 RP_ID7}, gwus-ResourcePattern-withoutLegacy-r16 SEQUENCE { gwus-freqLocation-r16 ENUMERATED {n0, n2}, gwus-resoucePattern-r16 ENUMERATED {RP_ID0, RP_ID2, RP_ID4, RP_ID6} } } GWUS-NumGroups-r16 ::= ENUMERATED {n1, n2, n4, n8} GWUS-ProbThreshList-r16 ::= SEQUENCE (SIZE (1..maxGWUS-ProbThresholds-r16)) OF GWUS-paging-ProbThresh-r16 GWUS-paging-ProbThresh-r16 ::= ENUMERATED {TBD} maxGWUS-Groups-1-r16 ::= 32 maxGWUS-Resources-r16 ::= 4 maxGWUS-ProbThresholds-r16 ::= 3

Table 12: R16 WUS-Config signaling for eMTC– Field description

RadioResourceConfigCommon field descriptions

additionalSpectrumEmissionSCell The UE requirements related to additionalSpectrumEmissionSCell are defined in TS 36.101 [42]. E-UTRAN configures the same value in additionalSpectrumEmissionSCell for all SCell(s) of the same band with UL configured. The additionalSpectrumEmissionSCell is applicable for all serving cells (including PCell) of the same band with UL configured.

defaultPagingCycle Default paging cycle, used to derive 'T' in TS 36.304 [4]. Value rf32 corresponds to 32 radio frames, rf64 corresponds to 64 radio frames and so on.

Dummy This field is not used in the specification. If received it shall be ignored by the UE.

gwus-CommonWUS-Sequence Indicates common WUS sequence is configured. Value legacyWUS indicates for the shared WUS resource the legacy WUS sequence. Value groupWUS indicates for the shared WUS resource the WUS group sequence , see TS 36.211[21].

gwus-GroupAlternation Enables hopping between the two or more WUS resources for the gap type, see TS 36.304[4].

gwus-GroupNarrowBandList List indicating which narrowbands support group WUS see TS 36.304 [4]. First entry in the list indicates WUS support for first narrowband, second entry in the list indicates WUS support for second narrowband, and so on. If this list is absent, then group WUS supported on all narrowbands.

gwus-GroupsForServiceList Number of WUS groups for each paging probability group see TS 36.304 [4]. The first entry is for the first probability group, second entry is for the second paging probability group, and so on. Any WUS groups from the list if WUS groups defined in the numWUS-GroupsPerResourceList that are not assigned to a probability group is considered to be part of the UE ID based group only list. If this field is absent, then paging probability based WUS group selection is not configured.

gwus-NumGroupsList List of WUS groups for each WUS resource see TS 36.304 [4]. First entry corresponds to the first resource, second entry corresponds to the second resource, and so on. gwus-NumGroupsList shall be present in gwus-ResourceConfigDRX. If gwus-NumGroupsList is not present in gwus-ResourceConfig-eDRX-Short then gwus-NumGroupsList from gwus-ResourceConfigDRX applies. If gwus-NumGroupsList is not present in gwus-ResourceConfig-eDRX-Long and gwus-NumGroupsList is present in gwus-ResourceConfig-eDRX-Short then gwus-NumGroupsList from gwus-ResourceConfig-eDRX-Short applies. If gwus-NumGroupsList is not present in gwus-ResourceConfig-eDRX-Long and gwus-NumGroupsList is not present in gwus-ResourceConfig-eDRX-Short then gwus-NumGroupsList from gwus-ResourceConfigDRX applies.

gwus-ProbThreshList Paging probability thresholds corresponding to the paging probability groups, see TS 36.304 [4]. If this field is absent, then paging probability based WUS group selection is not configured.

gwus-ResourceConfigDRX, gwus-ResourceConfig-eDRX-Short, gwus-ResourceConfig-eDRX-Long WUS resource configured for each gap type see TS 36.304 [4]. If gwus-ResourceConfig-eDRX-Long is not present but timeOffset-eDRX-Long is present and gwus-ResourceConfig-eDRX-Short is present, then gwus-ResourceConfig-eDRX-Short parameters apply for long eDRX group WUS resource. If gwus-ResourceConfig-eDRX-Long is not present but timeOffset-eDRX-Long is present and gwus-ResourceConfig-eDRX-Short is not present, then gwus-ResourceConfigDRX parameters apply for long eDRX group WUS resource.

gwus-ResourcePattern-withLegacy, gwus-ResourcePattern-withoutLegacy Identifies the group WUS resource mapping to time/frequency as defined in TS 36.304 [4]. If wus-Config-r15 is present in SystemInformationBlockType2, gwus-ResourcePattern-withLegacy-R16 is configured; otherwise gwus-ResourcePattern-withoutLegacy-R16 is configured. If gwus-ResourcePattern-withLegacy is configured, then frequency location of group WUS resource 0 is defined by freqLocation-r15. If gwus-ResourcePattern-withoutLegacy is configured, then frequency location of group WUS resource 0 is defined by gwus-freqLocation-r16.

harq-ReferenceConfig Indicates UL/ DL configuration used as the DL HARQ reference configuration for this serving cell. Value sa2 corresponds to Configuration2, sa4 to Configuration4 etc, as specified in TS 36.211 [21], table 4.2-2. E-UTRAN configures the same value for all serving cells residing on same frequency band.

highSpeedEnhancedMeasFlag If the field is present, the UE shall apply the high speed measurement enhancements as specified in TS 36.133 [16].

highSpeedEnhancedDemodulationFlag If the field is present, the UE shall apply the advanced receiver in SFN scenario as specified in TS 36.101 [6].

highSpeedMeasGapCE-ModeA If the field is present, the UE in CE mode A shall apply the measurement gap sharing table associated with high-velocity scenario for measurements, as specified in TS 36.133 [16].

interval-DLHoppingConfigCommonModeX Number of consecutive absolute subframes over which MPDCCH or PDSCH for CE mode X stays at the same narrowband before hopping to another narrowband. For interval-FDD, int1 corresponds to 1 subframe, int2 corresponds to 2 subframes, and so on. For interval-TDD, int1 corresponds to 1 subframe, int5 corresponds to 5 subframes, and so on.

interval-ULHoppingConfigCommonModeX Number of consecutive absolute subframes over which PUCCH or PUSCH for CE mode X stays at the same narrowband before hopping to another narrowband. For interval-FDD, int1 corresponds to 1 subframe, int2 corresponds to 2 subframes, and so on. For interval-TDD, int1 corresponds to 1 subframe, int5 corresponds to 5 subframes, and so on.

modificationPeriodCoeff Actual modification period, expressed in number of radio frames= modificationPeriodCoeff * defaultPagingCycle. n2 corresponds to value 2, n4 corresponds to value 4, n8 corresponds to value 8, n16 corresponds to value 16, and n64 corresponds to value 64.

mpdcch-NumRepetition-Paging Maximum number of repetitions for MPDCCH common search space (CSS) for paging, see TS 36.211 [21].

mpdcch-pdsch-HoppingOffset Parameter: , see TS 36.211 [21], clause 6.4.1.

mpdcch-pdsch-HoppingNB The number of narrowbands for MPDCCH/PDSCH frequency hopping. Value nb2 corresponds to 2 narrowbands and value nb4 corresponds to 4 narrowbands.

nB Parameter: nB is used as one of parameters to derive the Paging Frame and Paging Occasion according to TS 36.304 [4]. Value in multiples of 'T' as defined in TS 36.304 [4]. A value of fourT corresponds to 4 * T, a value of twoT corresponds to 2 * T and so on. In case nB-v1310 is signalled, the UE shall ignore nB (i.e. without suffix). EUTRAN configures nB-v1310 only in the BR version of SI message.

paging-narrowBands Number of narrowbands used for paging, see TS 36.304 [4], TS 36.212 [22] and TS 36.213 [23].

p-Max Pmax to be used in the target cell. If absent, for the band used in the target cell, the UE applies the maximum power according to its capability as specified in 36.101 [42], clause 6.2.2. In case the UE is configured with uplink intra-band contiguous CA and the UE indicates ue-CA-PowerClass-N in that band combination, then the p-Max in RadioResourceConfigCommonSCell for that SCell, if present, also applies for that band combination whenever that SCell is activated.

prach-ConfigSCell Indicates a PRACH configuration for an SCell. The field is not applicable for an LAA SCell in this release.

rach-ConfigCommonSCell Indicates a RACH configuration for an SCell. The field is not applicable for an LAA SCell in this release.

soundingRS-FlexibleTiming Indicates the SRS flexible timing (if configured) for aperiodic SRS triggered by DL grant. If the SRS transmission is collided with ACK/NACK, postpone once to the next configured SRS transmission opportunity.

ul-Bandwidth Parameter: transmission bandwidth configuration, NRB, in uplink, see TS 36.101 [42], table 5.6-1. Value n6 corresponds to 6 resource blocks, n15 to 15 resource blocks and so on. If for FDD this parameter is absent, the uplink bandwidth is equal to the downlink bandwidth. For TDD this parameter is absent and it is equal to the downlink bandwidth.

ul-CarrierFreq For FDD: If absent, the (default) value determined from the default TX-RX frequency separation defined in TS 36.101 [42], table 5.7.3-1, applies. For TDD: This parameter is absent and it is equal to the downlink frequency.

ul-CyclicPrefixLength Parameter: Uplink cyclic prefix length see TS 36.211 [21], clause 5.2.1, where len1 corresponds to normal cyclic prefix and len2 corresponds to extended cyclic prefix.

Table 13: R16 WUS-Config signaling for eMTC – conditional presence

| Conditional presence | Explanation |
| --- | --- |
| NO-WUS-Config-r15 | The field is mandatory present if wus-Config-r15 is not present in SystemInformationBlockType2; otherwise the field is not present, and the UE shall delete any existing value for this field. |

Question 5.	Do you support the ASN.1 coding and associated filed descriptions shown in Table 11, Table 12 and Table 13 as the baseline?

| Company | Yes/No | If no, suggest changes |
| --- | --- | --- |

Summary 5: 	TBD

Proposal 5:	TBD

- Summary

TBD

- References

- R2-2000306, Report of Email Discussion 108#94 Finalise the WUS signalling, RAN2#109-e, February 2020.

- R2-2001979, WUS: Progress the FFS from Email Discussion 108#94 and Summary, RAN2#109-e, February 2020.

- R1-1913673, Updated consolidated parameter list for Rel-16 LTE, RAN1#99, Reno, November 2019.