3GPP TSG-RAN WG2 Meeting #109-e	R2-200xxxx

Online, 24th February – 6th March 2020

Agenda Item:	7.2.1

Source: 	Huawei, HiSilicon

Title:	Open issues and comments on the 36.300 running CR for NB-IoT.

Document for:	Discussion and Decision

# Introduction

The latest version of the running CR for 36.300 was endorsed in [1].

An offline discussion has been set up to update the running CR for NB-IoT:

- [AT109e][313][NBIOT] R16 36.300 CR  (Huawei)

Status: Not Started

Scope: Discuss and review the CR

Intended outcome: Agreeable CR in R2-2001784

Deadline: 06-03-2020, 12:00 CET

This document collects comments on the running CR. For ease of handling, it is organised by sections.

# Discussion

## Sections 2 and 3 (common NB-IoT and eMTC)

Discussion Point 1: Companies to indicate any comments on section 2 and 3.

| Company | Comments |
| --- | --- |
| Huawei | section 3.1: remove the comments from E/// on use of stage 3 reference and keep stage 2 reference as used for EPC |
| Ericsson | OK to remove the comment referred to by HW, would be interested in the answer to that though |

## Section 4

Discussion Point 2: Companies to indicate any comments on section 4.

| Company | Comments |
| --- | --- |

## Section 7.1 (common NB-IoT and eMTC)

Discussion Point 3: Companies to indicate any comments on section 7.1

| Company | Comments |
| --- | --- |

## Section 7.2 (common NB-IoT and eMTC)

Discussion Point 4: Companies to indicate any comments on section 7.2

| Company | Comments |
| --- | --- |

## Section 7.3 (common NB-IoT and eMTC)

Discussion Point 5: Companies to indicate any comments on section 7.3

| Company | Comments |
| --- | --- |

## Section 7.3a (common NB-IoT and eMTC)

In section 7.3a..3, there are two Editor’s notes on whether to follow EDT or RRC_INACTIVE for resumption of DRBs. They will be removed and the text updated after handling of Report of [108#19] when to resume DRBs in UP optimization for 5GC. they don’t need to be commented here.

Discussion Point 6: Companies to indicate any comments on section 7.3a

| Company | Comments |
| --- | --- |
| Ericsson | 7.3a.3 on resumption for 5GS case, consider changing "re-establishes the AS security" to "reactivates AS security", or is the intention to implicitly refer to re-establishing PDCP as well, or? On the other hand, re-established is used in legacy text as well, so perhaps OK. |

## Section 7.3b (common NB-IoT and eMTC)

Discussion Point 7: Companies to indicate any comments on section 7.3b

| Company | Comments |
| --- | --- |
| Huawei | In section 7.3b.3, there is an Editor’s note that the description is based on the CR agreed in SA2 at SA2#135 and that official approval was needed before finialising the description in 36.300. The procedure has now been introduced in TS 23.502 December version, so the Editor’s note can be removed. |
| Ericsson | Under Figure 7.3b-2: "If enabled, the UE may…", maybe "If enabled in the cell, the UE may…" otherwise potential confusion who is enabling, as sentence continues with ", the UE". |

## Section 7.3x (common NB-IoT and eMTC)

Discussion Point 8: Companies to indicate any comments on section 7.3x

| Company | Comments |
| --- | --- |
| Ericsson | Question on the RAN3 additions, is it necessary to change to "may include" in the MT-EDT UP description? If so, then same changes should apply to CP as well, right? And further, then we should make additional changes on that "eNB may…" etc. Now it is not consistent, or what is the intention? |

## Section 7.3y (common NB-IoT and eMTC)

In section 7.3.y1, there is an Editor’s Note that it needs to be confirmed whether transmission in PUR is supported for 5GC. This was agreed on line in AI 7.2.10 and the Editor’s note will be removed. There is no need to comment here.

Discussion Point 9: Companies to indicate any comments on section 7.3y

| Company | Comments |
| --- | --- |
| Huawei | In section 7.3y.4, there is an Editor’s Note whether the (ng-)eNB authenticate the UE using the short resume MAC-I when receiving RRCConnectionResumeRequest message. The Editor’s note was added before RAN2 agreed to reuse the existing messages and before knowing if there will be a resumeID. We think that the Editor’s Note can now be removed. |
| Huawei | In section 7.3y.3, there is an Editor’s Note RAN2 to discuss if only one mechanism (TA in DCI or TA in MAC CE) should be allowed in Transmission using PUR for the CP solution. We think that the Editor’s Note can be removed. As a baseline both mechanisms are supported. |
| Ericsson | In CP and UP PUR NOTE 2: "is left to" -> "are left to" |

## Section 7.4

Discussion Point 10: Companies to indicate any comments on section 7.4

| Company | Comments |
| --- | --- |

## Section 8.1 (common NB-IoT and eMTC)

Discussion Point 11: Companies to indicate any comments on section 8.1.

| Company | Comments |
| --- | --- |

## Section 10 and 10.1.3

Discussion Point 12: Companies to indicate any comments on sections 10 and 10.3.

| Company | Comments |
| --- | --- |
| Huawei | In section 10.1.3, there is a reference to 36.304 [11] for the description of serving cell measurements on non-anchor paging carrier. We think the reference should be TS 36.133 [21]. |

## Section 10.1.4 (common NB-IoT and eMTC)

Discussion Point 13: Companies to indicate any comments on section 10.1.4

| Company | Comments |
| --- | --- |
| Huawei | there is an editor’s note ‘Editor’s Note: FFS whether the paging operation in the MME/AMF is aware of the use of GWUS’ we have asked SA2 to introduce MME awareness from Rel’-15, so the editors’ note to be removed. Should we assume the rel-15 mechanism as a baseline at least that the eNB informs the MME whether it supports GWUS ? In that case, we need to incorporate the same changes as in the rel16 CR for Rel-15 WUS and make it applicable to GWUS |

## Section 10.1.5 (common NB-IoT and eMTC)

Discussion Point 14: Companies to indicate any comments on section 10.1.5

| Company | Comments |
| --- | --- |

## Section 10.1.6

Discussion Point 15: Companies to indicate any comments on section 10.1.6

| Company | Comments |
| --- | --- |
| Huawei | in section 10.1.6, ‘/5GS’ needs to be added in ‘Except for a NB-IoT UE using only Control Plane CIoT EPS optimizations’ and ‘ng-’ in ‘is used by the selected eNB to request the MME/AMF’ |

## Section 10.2.x

Discussion Point 16: Companies to indicate any comments on section 10.2.x

| Company | Comments |
| --- | --- |

## Section 11

Discussion Point 17: Companies to indicate any comments on sections 11.0, 11.1.1 and 11.7.

| Company | Comments |
| --- | --- |
| Ericsson | 11.1.1: Perhaps include the new text in the list "For NB-IoT:"? Also wording "can be scheduled via single NPDCCH" is a bit strange considering rest of the section, perhaps something like "when multi-TB scheduling is configured, one instance of scheduling information can indicate scheduling of multiple downlink transmissions". Should the added text be split to 11.1.1 and 11.1.2? Considering we have also agreed enabling/disabling is separate for uplink and downlink. |

## Section 15.3 (common NB-IoT and eMTC)

Discussion Point 18: Companies to indicate any comments on sections 15.3

| Company | Comments |
| --- | --- |
| Ericsson | Similar comments for wording as above on the addition related to multi-TB. |

Section 16.1.x

Ericsson: Fine with QC wording suggestions. Text OK as baseline.

## Section 16.3 (common NB-IoT and eMTC)

Note: there is no changes in the section and the section will be removed from the CR if there is no comment.

Discussion Point 19: Companies to indicate any comments on sections 16.3.

| Company | Comments |
| --- | --- |
| Huawei | in 5GC, there is no configuration for the use of AS RAI. so we prefer to clarify. In 5GS or if EPS, if configured, aA NB-IoT UE or BL UE may be configured to send assistance information to the eNB to assist the eNB in connection release handling. |

## Section 22.3

Discussion Point 20: Companies to indicate any comments on sections 22.3.2a and 23.3.4x.

| Company | Comments |
| --- | --- |

## Section 22.4

Discussion Point 21: Companies to indicate any comments on sections 22.4.2.x. 22.4.3 and 22.4.5.

| Company | Comments |
| --- | --- |

## Section 23.13 (common NB-IoT and eMTC)

Note: there is no changes in the section and the section will be removed from the CR if there is no comment.

Discussion Point 22: Companies to indicate any comments on sections 23.13.1 and 23.13.2.

| Company | Comments |
| --- | --- |

## Section 24 (common NB-IoT and eMTC)

Discussion Point 23: Companies to indicate any comments on sections 24.1, 24.2, 24.3, 24.4 and 24.5 23.13.2.

| Company | Comments |
| --- | --- |

# 3 Conclusion

<To be completed after comments have been received>

# 4 References

- R2-1916567, “Introduction of additional enhancements for NB-IoT in TS 36.300”, Huawei, RAN2#108

- R2-1916361, “Introduction of Rel-16 eMTC enhancements”, Intel, RAN2#108