3GPP TSG-RAN WG2 Meeting #109 electronic	draftR2-2001663

24th Feb - 06th Mar 2020

Agenda Item:	8.3

Source: 		Session Chair (Ericsson)

Title: 			Report eMTC breakout session

Document for:	Approval

General

Recording of voice or video at meetings is not used in 3GPP. This applies also to this e-Meeting. At this e-Meeting, no specific actions are taken to prevent the recording of web conferences. Companies that have concerns related to recordings, if any, may express those by email in the main meeting organizational thread [AT109e][000]

Please see the following Tdocs for e-meeting guidance:

R2-2000008	Agenda for RAN2#109-e	Chairman	agenda

R2-2002046	RAN2 109-e Methods and Guidance	RAN2 chairman, RAN2 vice chairmen, session chairs	discussion

Access Tools

HTTP Upload Tool:

ETSI IT has created a facility on Inbox and Inbox/Drafts folders on the public 3GPP servers to allow delegates to upload their documents using a web browser (however Internet Explorer is not yet supported). Open your browser and navigate to your chosen folder – for example,

https://www.3gpp.org/ftp/tsg_ran/WG2_RL2/TSGR2_109_e/Inbox

Click the green button to log in using your EOL account. A panel will appear in the upper part of the screen and documents may be dragged and dropped onto this landing pad; this causes them to be uploaded to the folder.

Secure FTP:

Those e-delegates who prefer to use FTP-like access to our e-meeting Inbox & Draft folders but are concerned by their usernames and passwords being sent unencrypted over the internet, ETSI IT has fitted the server with FTPS (SSL) so delegates can connect from their favourite FTP client using the address: ftps.3gpp.org. Please enter your username and password when prompted.

Organizational

- Incoming LSs are noted by default. Contact companies should flag LSs that need to be replied from this meeting.

- Running CRs submitted before the meeting are endorsed as baseline and moved to offline email discussion.

- With a few exceptions, only email discussion reports and summary discussion papers will be treated during the e-meeting

- All organization emails and notes will be shared over the following email discussion throughout both meeting weeks:

- [AT109e][400][eMTC/NB-IoT] Organizational Emre’s session

Scope:

- Share plans for the e-meeting and make announcements

- Share status of email discussions

- Share meeting minutes and agreements for review and endorsement

Deadline: Friday, Mar 6th 12:00 CET

Webinar Tool

Use of Question Box:

I plan to use the question/chat box to copy and paste the proposals under discussion during revisions for proper wording for delegates experiencing delays or not able to see via screen sharing. During the web conference, delegates can use the question/chat box to suggest alternative wording when it is their turn to talk. Note that it seems it is not possible for other attendees to see the suggested wording, so the session chair has to share it once received from an attendee for everyone to see.

List and Status of Offline Email Discussions

NOTE: The official kick off date for these email discussions are Monday Feb 24th at 9:00 CET. The rapporteurs can share them on the reflector earlier, however companies are not required to participate before the official kick off date.

- [AT109e][400][eMTC/NB-IoT] Organizational Emre’s session

Scope:

- Share plans for the e-meeting and make announcements

- Share status of email discussions

- Share meeting minutes and agreements for review and endorsement

Deadline: Friday, Mar 6th 12:00 CET

Status: Started

- [AT109e][401][eMTC] Correction for relaxed monitoring for BL and CE UE (Ericsson)

Intended outcome: To check whether there is support, and, if so, agreeable CR. The outcome can be provided in R2-2001866

Deadline: Wednesday, Feb. 26th 18:00 CET

Schedule: Thursday, Feb. 27th 13:30 - 15:30 CET

Status: Closed

- [AT109e][402][eMTC] Correction to support of UP-EDT, CP-EDT, in eMTC TDD (Huawei)

Intended outcome: To check whether there is support, and, if so, agreeable CR. The outcome can be provided in R2-2001867

Deadline: Friday, Mar 6th, 12:00 CET

Status: Closed

- [AT109e][403][eMTC]  Update 36.300 running CR (Intel)

Scope: Update 36.300 running CR, i.e., address the open issues and include the agreements from this meeting.

Intended outcome: Agreeable CR in R2-2001868

Deadline: Friday, Mar 6th 12:00 CET

Status: Started

- [AT109e][404][eMTC]  Update 36.302 running CR (ZTE)

Scope: Update 36.302 running CR, i.e., address the open issues and include the agreements from this meeting.

Intended outcome: Agreeable CR in R2-2001869

Deadline: Friday, Mar 6th 12:00 CET

Status: Started

- [AT109e][405][eMTC]  Update 36.304 running CR (Nokia)

Scope: Update 36.304 running CR, i.e., address the open issues and include the agreements from this meeting.

Intended outcome: Agreeable CR in R2-2001870

Deadline: Friday, Mar 6th 12:00 CET

Status: Started

- [AT109e][406][eMTC]  Update 36.306 running CR (Huawei)

Scope: Update 36.306 running CR, i.e., address the open issues and include the agreements from this meeting.

Intended outcome: Agreeable CR in R2-2001871

Deadline: Friday, Mar 6th 12:00 CET

Status: Started

- [AT109e][407][eMTC]  Update 36.321 running CR (Ericsson)

Scope: Update 36.321 running CR, i.e., address the open issues and include the agreements from this meeting.

Intended outcome: Agreeable CR in R2-2001872

Deadline: Friday, Mar 6th 12:00 CET

Status: Started

- [AT109e][408][eMTC]  Update 36.331 running CR (Qualcomm)

Scope: Update 36.331 running CR, i.e., address the open issues and include the agreements from this meeting.

Intended outcome: Agreeable CR in R2-2001873

Deadline: Friday, Mar 6th 12:00 CET

Status: Started

- [AT109e][409][eMTC]  Update 38.300 running CR (Qualcomm)

Scope: Update 38.300 running CR, i.e., address the open issues and include the agreements from this meeting.

Intended outcome: Agreeable CR in R2-2001874

Deadline: Friday, Mar 6th 12:00 CET

Status: Started

- [AT109e][410][eMTC] CE Mode Threshold Adjustments for non-BL and BL UE (DoCoMo)

Intended outcome: To check whether there is support, and, if so, list of proposals that are easily agreeable, almost agreeable and need further discussion. The outcome can be provided in R2-2001875

Deadline: Wednesday, Feb. 26th 18:00 CET

Schedule: Thursday, Feb. 27th 13:30 - 15:30 CET

Status: Closed

- [AT109e][411][eMTC/NB-IoT]  MT-EDT: Open issues (Huawei)

Scope: Further discussion to address the remaining issues and identify potential agreements.

Intended outcome: Report with a list of proposals categorized as agreeable, need further discussion, postpone. The outcome can be provided in R2-2001876

Deadline: Tuesday, Mar 3rd 17:00 CET

Schedule: Wednesday, Mar 4th, 06:30 - 07:30 CET

Status: Closed

- [AT109e][412][eMTC/NB-IoT]  Scheduling multiple TBs: Open issues (Ericsson)

Scope: Further discussion on proposals 3, 4, 6, and 10 and identify potential agreements

Intended outcome: Report with a list of proposals categorized as agreeable, need further discussion, postpone. The outcome can be provided in R2-2001877

Deadline: Tuesday, Mar 3rd 17:00 CET

Schedule: Wednesday, Mar 4th, 06:30 - 07:30 CET

Status: Closed

- [AT109e][413][eMTC]  Quality report: Open issues (Qualcomm)

Scope: Further discussion on proposal 2 from R2-2000309 and proposals 2 and 4 from R2-2001863 and 10 and identify potential agreements

Intended outcome: Report with a list of proposals categorized as agreeable, need further discussion, postpone. The outcome can be provided in R2-2001878

Deadline: Tuesday, Mar 3rd 17:00 CET

Schedule: Wednesday, Mar 4th, 06:30 - 07:30 CET

Status: Closed

- [AT109e][414][eMTC]  MPDCH performance imp: Open issues (Qualcomm)

Scope: Further discussion on proposals 1 and 2 and identify potential agreements

Intended outcome: Report with a list of proposals categorized as agreeable, need further discussion, postpone. The outcome can be provided in R2-2001879

Deadline: Tuesday, Mar 3rd 17:00 CET

Schedule: Wednesday, Mar 4th, 06:30 - 07:30 CET

Status: Closed

- [AT109e][415][eMTC]  Idle Mode Mobility: Open issues (Intel)

Scope: Further discussion on proposals 1, 2, and 3, and identify potential agreements

Intended outcome: Report with a list of proposals categorized as agreeable, need further discussion, postpone. The outcome can be provided in R2-2001880

Deadline: Tuesday, Mar 3rd 17:00 CET

Schedule: Wednesday, Mar 4th, 06:30 - 07:30 CET

Status: Closed

- [AT109e][416][eMTC]  Stand-alone deployment: Open issues (Nokia)

Scope: Further discussion to address the remaining issues and identify potential agreements..

Intended outcome: Report with a list of proposals categorized as agreeable, need further discussion, postpone. The outcome can be provided in R2-2001881

Deadline: Tuesday, Mar 3rd 17:00 CET

Schedule: Wednesday, Mar 4th, 06:30 - 07:30 CET

Status: Closed

- [AT109e][417][eMTC]  Mobility enhancements: Open issues (Ericsson)

Scope: Further discussion to address the remaining issues and identify potential agreements.

Intended outcome: Report with a list of proposals categorized as agreeable, need further discussion, postpone. The outcome can be provided in R2-2001882

Deadline: Tuesday, Mar 3rd 17:00 CET

Schedule: Wednesday, Mar 4th, 06:30 - 07:30 CET

Status: Closed

- [AT109e][418][eMTC/NB-IoT]  Coexistence with NR: Open Issues (ZTE)

Scope: Further discussion to address the remaining issues and identify potential agreements.

Intended outcome: Report with a list of proposals categorized as agreeable, need further discussion, postpone. The outcome can be provided in R2-2001883

Deadline: Tuesday, Mar 3rd 17:00 CET

Schedule: Wednesday, Mar 4th, 06:30 - 07:30 CET

Status: Closed

- [AT109e][419][eMTC/NB-IoT]  Connection to 5GC: Open Issues (Huawei)

Scope: Further discussion to address the remaining issues, i.e., proposals S1-5, S2-2, S3-1, and identify potential agreements.

Intended outcome: Report with a list of proposals categorized as agreeable, need further discussion, postpone. The outcome can be provided in R2-2001885

Deadline: Tuesday, Mar 3rd 17:00 CET

Schedule: Wednesday, Mar 4th, 06:30 - 07:30 CET

Status: Closed

- 4	EUTRA corrections Rel-15 and earlier

See Appendix A for reference to Work items, work item codes and WIDs.

No documents should be submitted to 4. Please submit to 4.x

NOTE For R2 109e for R15 and earlier releases, only documents on important and urgent issues shall be submitted and treated. No text enhancements without behavioural or functional change.

- 4.2	eMTC corrections Rel-15 and earlier

Documents in this agenda item will be handled in a break out session. Common NB-IoT/eMTC parts treated jointly with 4.1.

This agenda item may not be treated during the e-meeting. No web conference is planned for this agenda item

R2-2000339	Correction for relaxed monitoring for BL and CE UE	Ericsson	CR	Rel-14	36.304	14.7.0	0778	-	F	LTE_feMTC-Core

- [AT109e][401][eMTC] Correction for relaxed monitoring for BL and CE UE (Ericsson)

Intended outcome: To check whether there is support, and, if so, agreeable CR. The outcome can be provided in R2-2001866

Deadline: Wednesday, Feb. 26th 18:00 CET

Schedule: Thursday, Feb. 27th 13:30 - 15:30 CET

R2-2001866	Correction for relaxed monitoring for BL and CE UE	Ericsson

- Rappoteur reports that there was not much support for the change and proposes not to pursue the CR this meeting.

- Not pursued

R2-2001062	Correction to support of UP-EDT, CP-EDT, in eMTC TDD	Huawei, HiSilicon	CR	Rel-15	36.306	15.7.0	1734	-	F	LTE_eMTC4-Core

- [AT109e][402][eMTC] Correction to support of UP-EDT, CP-EDT, in eMTC TDD (Huawei)

Intended outcome: To check whether there is support, and, if so, agreeable CR. The outcome can be provided in R2-2001867

Deadline: Wednesday, Feb. 26th 18:00 CET

Schedule: Thursday, Feb. 27th 13:30 - 15:30 CET

R2-2001867	Correction to support of UP-EDT, CP-EDT, in eMTC TDD	Huawei, HiSilicon

- Revised in R2-2001889 with the following change on the cover page, i.e., 2019-03-04 to 2020-03-05, and agreed unseen.

# 7	Rel-16 LTE Work Items

Documents in these agenda items will be handled in break out sessions

## 7.1	Additional MTC enhancements for LTE

(LTE_eMTC5-Core; leading WG: RAN1; REL-16; started: Jun 18; target; Mar 20; WID: RP-191356)

Time budget: 2.5 TU

Documents in this agenda item will be handled in a break out session

Some sub-items in 7.1 and 7.2 may be treated jointly.

### 7.1.1	Organisational

Including incoming LSs, rapporteur inputs, running CRs

The LSs are presented only if presenting companies flag them, otherwise they will be noted. If flagged, response LSs will be agreed by email discussion if RAN2 decides that they are needed

R2-2000056	LS on category M devices and NR (S2-1912608; contact: Qualcomm)	SA2	LS in	Rel-16	5G_CIoT	To:RAN2

- Noted.

R2-2000094	Reply LS on direct indication of ETWS/CMAS (R1-1913367; contact: Futurewei)	RAN1	LS in	Rel-16	LTE_eMTC5-Core	To:RAN2

- Noted

R2-2000387	RAN2 agreements for Rel-16 additional enhancements for NB-IoT and MTC	Document Rapporteur (BlackBerry)	WI summary	Rel-16	LTE_eMTC5-Core, NB_IOTenh3-Core

- Noted

- [109e#xx] [NB-IoT/eMTC R16] Update RAN2 agreements for Rel-16 additional enhancements for NB-IoT and eMTC (Blackberry)

Scope: Update RAN2 agreements for Rel-16 additional enhancements for NB-IoT and eMTC

Intended outcome: endorsed document in R2-2001886

Deadline: 1 week

R2-2001470	Report from eMTC/NB-IoT RRC CR coordination telco	Ericsson	discussion	Rel-16	LTE_eMTC5-Core, NB_IOTenh3-Core

- Noted

R2-2002211	Reply LS on extended NAS timers for CE in 5GS (C1-200717; contact: Ericsson)	CT1	LS in	Rel-16	LTE_eMTC5-Core, NB_IOTenh3-Core, 5G_CIoT	To:RAN2	Cc:RAN3, SA2

- Noted

R2-2002214	LS on suspend indication to the NAS (C1-201040; contact: Samsung)	CT1	LS in	Rel-16	5G_CIoT	To:RAN2	Cc:SA2

- Noted

Running CRs

R2-2001097	Introduction of Rel-16 eMTC enhancements	Intel Corporation	CR	Rel-16	36.300	16.0.0	1267	-	B	LTE_eMTC5-Core

- [AT109e][403][eMTC]  Update 36.300 running CR (Intel)

Scope: Update 36.300 running CR, i.e., address the open issues and include the agreements from this meeting.

Intended outcome: Agreeable CR in R2-2001868

Deadline: Friday, Mar 6th 12:00 CET

R2-2000501	Addressing Editor's Notes in 36.302 running CR for eMTC	ZTE Corporation, Sanechips	discussion	Rel-16	36.302	LTE_eMTC5-Core

R2-2001213	Running 36.302 CR for R16 eMTC	ZTE Corporation, Sanechips	draftCR	Rel-16	36.302	15.2.0	LTE_eMTC5-Core	Late

- [AT109e][404][eMTC]  Update 36.302 running CR (ZTE)

Scope: Update 36.302 running CR, i.e., address the open issues and include the agreements from this meeting.

Intended outcome: Agreeable CR in R2-2001869

Deadline: Friday, Mar 6th 12:00 CET

R2-2000558	Rapporteur Summary :Discussion on cell selection for non-BL UE	Nokia, Nokia Shanghai Bell	discussion

R2-2001167	Introduction of additional enhancements for eMTC in TS36.304	Nokia Solutions & Networks (I)	CR	Rel-16	36.304	15.5.0	0781	-	B	LTE_eMTC5-Core

- [AT109e][405][eMTC]  Update 36.304 running CR (Nokia)

Scope: Update 36.304 running CR, i.e., address the open issues and include the agreements from this meeting.

Intended outcome: Agreeable CR in R2-2001870

Deadline: Friday, Mar 6th 12:00 CET

R2-2001066	Open issues on the 36.306 running CR for eMTC	Huawei, HiSilicon	discussion	Rel-16	LTE_eMTC5-Core

R2-2001065	Introduction of additional enhancements for eMTC	Huawei, HiSilicon	CR	Rel-16	36.306	15.7.0	1735	-	B	LTE_eMTC5-Core

- [AT109e][406][eMTC]  Update 36.306 running CR (Huawei)

Scope: Update 36.306 running CR, i.e., address the open issues and include the agreements from this meeting.

Intended outcome: Agreeable CR in R2-2001871

Deadline: Friday, Mar 6th 12:00 CET

- [109e#xx] [eMTC R16] 36.306 CR (Huawei)

Scope: Capture the agreements from this meeting

Intended outcome: endorsed CR in R2-2001890

Deadline: next meeting

R2-2000976	Running CR on 36.321 for eMTC	Ericsson	CR	Rel-16	36.321	15.8.0	1465	-	B	LTE_eMTC5-Core

- [AT109e][407][eMTC]  Update 36.321 running CR (Ericsson)

Scope: Update 36.321 running CR, i.e., address the open issues and include the agreements from this meeting.

Intended outcome: Agreeable CR in R2-2001872

Deadline: Friday, Mar 6th 12:00 CET

R2-2000433	Introduction of Rel-16 eMTC enhancements	Qualcomm Incorporated	CR	Rel-16	36.331	15.8.0	4191	-	B	LTE_eMTC5-Core	R2-1916364

- [AT109e][408][eMTC]  Update 36.331 running CR (Qualcomm)

Scope: Update 36.331 running CR, i.e., address the open issues and include the agreements from this meeting.

Intended outcome: Agreeable CR in R2-2001873

Deadline: Friday, Mar 6th 12:00 CET

R2-2000305	Introduction of additional enhancements for eMTC	Qualcomm Incorporated	CR	Rel-16	38.300	16.0.0	0175	3	B	LTE_eMTC5-Core	R2-1916363

- [AT109e][409][eMTC]  Update 38.300 running CR (Qualcomm)

Scope: Update 38.300 running CR, i.e., address the open issues and include the agreements from this meeting.

Intended outcome: Agreeable CR in R2-2001874

Deadline: Friday, Mar 6th 12:00 CET

### 7.1.2	Mobile-terminated (MT) early data transmission (EDT)

MT Early Data transmission for MTC and NB-IoT is treated jointly under this AI.

This agenda item will utilize a summary document to facilitate treatment of topics during the e-meeting (decision to be made based on the submitted tdocs). This may lead to postponing some items to the next meeting. A web conference may be used for handling some of the discussions in this AI.

R2-2001861	Summary of contributions on mobile-terminated (MT) early data transmission (EDT)	BlackBerry UK Limited	discussion	Rel-16	LTE_eMTC5-Core, NB_IOTenh3-Core

Proposal: UE category information, i.e., Cat-M2 (Cat-NB2 for NB-IoT) is provided in the UE Radio Paging information container.

Proposal: Discuss the different alternative solutions based on e.g. the list below (which may not be exhaustive) and agree on a way forward.

Solution 1 Make MT-EDT (for Control Plane and User Plane) apply to FDD only for Rel-16 NB-IoT and eMTC.

Solution 2 Allow a Rel-16 TDD UE to support MO-EDT (for Control Plane and User Plane).

Solution 3 Make in the Rel-16, MT-EDT and MO-EDT (for CP and UP) apply to:

FDD only for NB-IoT;

FDD and TDD for eMTC.

Proposal: For MT-EDT for the user plane CIoT optimisation, lower layers are not configured for EDT.

Proposal: In 5.3.2.3, clarify that the mt-EDT is the one included in the UE’s paging record.

Proposal: If MSG4 sizes for MT-EDT based on the UE category is allowed, it should be clarified in the specification.

- Huawei wonders what the use would be with the proposal to add UE category information in the paging information container. Qualcomm explained that it would be beneficial to know to compare with the size of the message. Huawei thinks the configuration would be the default configuration and thus it may not work for a message with a larger size, i.e., that is suitable for a Cat-M2 or Cat-NB2 UE.

R2-2000179	Cat. M2/NB2 indication in UERadioPagingInformation	Qualcomm Incorporated	discussion	Rel-16	LTE_eMTC5-Core, NB_IOTenh4_LTE_eMTC6-Core

R2-2000397	Support of MT-EDT CIoT EPS optimisation (for CP and UP)	BlackBerry UK Limited	discussion	Rel-16	LTE_eMTC5-Core, NB_IOTenh3-Core

R2-2001197	Remaining FFSs for MT-EDT	ZTE Corporation, Sanechips	discussion	Rel-16	LTE_eMTC5-Core, NB_IOTenh3-Core

- [AT109e][411][eMTC/NB-IoT]  MT-EDT: Open issues (Huawei)

Scope: Further discussion to address the remaining issues and identify potential agreements.

Intended outcome: Report with a list of proposals categorized as agreeable, need further discussion, postpone. The outcome can be provided in R2-2001876

Deadline: Tuesday, Mar 3rd 17:00 CET

R2-2001876	MT-EDT: Open issues	Huawei	discussion	Rel-16	LTE_eMTC5-Core, NB_IOTenh3-Core

Proposal P1-1: Capture in stage 2 that the eNB uses the UE category included in the S1 paging message to trigger MT-EDT for message size beyond cat M1/NB1.

Proposal P2-1: In RRC section 5.3.3.3a, for UP MT-EDT, lower layers are not configured for EDT.

- QC thinks this proposal has already been captured in the latest version of 36.331.

Proposal P3-1: In RRC section 5.3.2.3, clarify that the mt-EDT is the one included in the UE’s paging record. Wording to be discussed with running RRC CRs.

Agreements

- UE category information, i.e., Cat-M2 (Cat-NB2 for NB-IoT), is provided in the UE Radio Paging information container. FFS how the use of UE category information is captured in the specifications.

- Capture in stage 2 that the eNB uses the UE category included in the S1 paging message to trigger MT-EDT for message size beyond Cat M1/NB1.

- In RRC section 5.3.3.3a, for UP MT-EDT, lower layers are not configured for EDT.

- In RRC section 5.3.2.3, clarify that the mt-EDT is the one included in the UE’s paging record.

### 7.1.3	UE-group wake-up signal (WUS)

UE-group wake-up signal (WUS) for MTC is treated jointly with NB-IoT under AI 7.2.3. Do not use this AI for any item that can be discussed jointly.

### 7.1.4	Transmission in preconfigured resources

Transmission in preconfigured resources for MTC is treated jointly with NB-IoT under AI 7.2.4. Do not use this AI for any item that can be discussed jointly.

### 7.1.5	Scheduling multiple DL/UL transport blocks

Scheduling multiple DL/UL transport blocks with or without DCI for SC-PTM and unicast

Scheduling multiple DL/UL transport blocks for MTC and NB-IoT is treated jointly under this AI.

This agenda item will utilize a summary document to facilitate treatment of topics during the e-meeting. This may lead to postponing some items to the next meeting. A web conference may be used for handling some of the discussions in this AI.

R2-2001862	Summary of contributions on scheduling multiple Dl/UL transport blocks	Ericsson	discussion	NB_IOTenh3-Core, LTE_eMTC5-Core

Proposal 1	For LTE-M the length of HARQ RTT timer is set to 7+k*N for bundled HARQ ACK, where k is equal to the number of HARQ ACK bundles.

Proposal 2	Remove Editor's note from RAN2 agreements document and clarify which agreements related to HARQ RTT timer length are for non-interleaved NB-IoT case.

Proposal 3	For unicast, multiple TBs scheduling is enabled separately for uplink and downlink for both LTE-M and NB-IoT.

Proposal 4	Discuss over email the details of unicast configuration for scheduling multiple TBs for 1) NB-IoT and 2) LTE-M.

Proposal 5	Explicit enabling indication for multiple TBs scheduling in SC-PTM is not needed.

Proposal 6	Discuss over email whether scheduling gap configuration is in SC-MTCH or in SIB20(-NB) for 1) NB-IoT and 2) LTE-M with details of configuration to be captured.

Proposal 7	For NB-IoT, support of multiTB-UL-r16 and multiTB-DL-r16 is conditional on support of two HARQ processes.

Proposal 8	For NB-IoT, introduce separate capabilities multiTB-UL-Interleaving-r16, and multiTB-DL-Interleaving-r16 conditional to support of multiTB-UL-r16 and multiTB-DL-r16 respectively.

Proposal 9	For NB-IoT, introduce a new capability multiTB-HARQ-ACK-Bundling-r16, conditional to support of multiTB-DL-Interleaving-r16.

Proposal 10	Discuss whether multiple TBs scheduling in multicast is optional without capability reporting.

Proposal 11	Postpone discussion or discuss over email whether drx-InactivityTimer for LTE-M needs changes when scheduling multiple TBs.

- Proposals 8 and 9 are to be discussed within the context of RAN1 feature list, i.e. once finalized, those will be captured in RAN2 specifications.

R2-2000395	HARQ RTT Timers in Rel-16 NB-IoT	BlackBerry UK Limited	discussion	Rel-16	NB_IOTenh3-Core

R2-2000644	Signalling aspect of multiple TBs scheduling for NB-IoT	Huawei, HiSilicon	discussion	Rel-16	NB_IOTenh3-Core, LTE_eMTC5-Core

R2-2000977	Scheduling enhancements for LTE-M and NB-IoT	Ericsson	discussion	NB_IOTenh3-Core, LTE_eMTC5-Core

R2-2002027	Summary of Scheduling multiple DL/UL transmission blocks	Ericsson	discussion	Rel-16	NB_IOTenh3-Core, LTE_eMTC5-Core

=> Withdrawn

- [AT109e][412][eMTC/NB-IoT]  Scheduling multiple TBs: Open issues (Ericsson)

Scope: Further discussion on proposals 3, 4, 6, and 10 and identify potential agreements

Intended outcome: Report with a list of proposals categorized as agreeable, need further discussion, postpone. The outcome can be provided in R2-2001877

Deadline: Tuesday, Mar 3rd 17:00 CET

R2-2001877	Summary of contributions on scheduling multiple Dl/UL transport blocks	Ericsson	discussion	NB_IOTenh3-Core, LTE_eMTC5-Core

Proposal 1	For unicast, multiple TBs scheduling is enabled separately for uplink and downlink for both LTE-M and NB-IoT.

Proposal 2	Capture TP for MultiTB-Config-NB in [2] in the running RRC CR for NB-IoT.

Proposal 3	RAN2 to further discuss whether scheduling gap configuration is in SCPTMConfiguration(-NB) (SC-MCCH) or in SIB20(-NB).

- Rapporteur thinks further discussion is needed.

Proposal 4	Multiple TBs scheduling in multicast is optional without capability reporting.

Agreements

- For LTE-M, the length of HARQ RTT timer is set to 7+k*N for bundled HARQ ACK, where k is equal to the number of HARQ ACK bundles.

- Remove Editor's note from the document that captures RAN2 agreements and clarify that those agreements are for non-interleaved NB-IoT case.

- Capture the following RAN1 agreement in RAN2 specifications: “For NB-IoT, support of multiTB-UL-r16 and multiTB-DL-r16 is conditional on support of two HARQ processes.”’

- For LTE-M and NB-IoT, multiple TBs scheduling is enabled separately for uplink and downlink for unicast.

- Capture TP for MultiTB-Config-NB in [2] in the running RRC CR for NB-IoT.

- For LTE-M and NB-IoT, multiple TBs scheduling in multicast is optional without capability reporting.

- For LTE-M and NB-IoT, the configuration for scheduling gap is in SCPTMConfiguration(-NB) (SC-MCCH).

### 7.1.6	Quality report in Msg3

Including outcome of the email discussion [108#72][eMTC] To finalize the 2 bit Quality report (Qualcomm)

This agenda item will utilize a summary document to facilitate treatment of topics during the e-meeting. A web conference or an offline discussion may be used for handling the outcome of the email discussion in this AI.

R2-2000309	Report of Email Discussion 108#72  To finalize the 2 bit Quality report	Qualcomm Incorporated	report

Proposal 1: 	RAN2 should support 2-bit CQI report in MSG3.

Proposal 2:	Support the optimal and less restrictive solution (i.e. using R and F2 bits) to send 2-bit CQI in MSG3.

R2-2001863	Summary of contributions on quality report in Msg3	Qualcomm Incorporated	discussion	Rel-16	LTE_eMTC5-Core

Proposal 1:	Discuss the email report and if agreement on solution 1 then endorse proposal [2]. P2.

Proposal 2:	[FFS] a configuration where 2-bit report is enabled and 4-bit report is disabled is not allowed.

Proposal 3:	Quality Report trigger in Connected Mode for eMTC is the same MAC CE as agreed for NB-IoT.

Proposal 4:	CQI reporting in MSG3 is configured per CE Level.

R2-2000310	Text proposal for 2-bit downlink channel quality reporting in MSG3 for eMTC	Qualcomm Incorporated	discussion	Rel-16	LTE_eMTC5-Core

R2-2001069	Remaining issue of DL quality report	Huawei, HiSilicon	discussion	Rel-16	LTE_eMTC5-Core

R2-2001235	Quality Report in eMTC Remaining Issues	Ericsson	discussion	Rel-16

- [AT109e][413][eMTC]  Quality report: Open issues (Qualcomm)

Scope: Further discussion on proposal 2 from R2-2000309 and proposals 2 and 4 from R2-2001863 and 10 and identify potential agreements

Intended outcome: Report with a list of proposals categorized as agreeable, need further discussion, postpone. The outcome can be provided in R2-2001878

Deadline: Tuesday, Mar 3rd 17:00 CET

R2-2001878	Report of Email Discussion 109e[413][eMTC] Quality report: Open issues	Qualcomm Incorporated	report

Proposal 1: Majority support solution 1.

- Ericsson still prefers not to have 2 bit CQI reporting but has sympathy with Huawei proposal.

- QC thinks option 1 is more flexible considering the freedom to use any LCID. Huawei disagrees.

Proposal 2: When 2-bit CQI reporting enabled in MSG3 then 8-bit reporting shall also be enabled.

Proposal 3: Majority do not support on per CE level enable/disable for CQI reporting.

Agreements

- RAN2 confirms that 2-bit CQI report in MSG3 is supported.

- Quality Report trigger in Connected Mode for eMTC is the same MAC CE as agreed for NB-IoT.

- When 2-bit CQI reporting enabled in MSG3 then 8-bit reporting shall also be enabled.

- Separate flags are not required to enable/disable CQI reporting per CE level.

### 7.1.7	MPDCCH performance improvement using CRS

This agenda item will not utilize a summary document to facilitate treatment of topics during the e-meeting. This may lead to postponing some items to the next meeting. A web conference may be used for handling some of the discussions in this AI.

R2-2000978	Stage-3 details for MPDCCH performance improvement	Ericsson	discussion	LTE_eMTC5-Core

Proposal 1	When transmitted in SIB2, CRS-ChEstMPDCCH-Config contains the power-ratio  applied in RRC_IDLE and in RRC_CONNECTED.

Proposal 2	When transmitted in RadioResourceConfigDedicated, CRS-ChEstMPDCCH-Config  contains the CRS localized mapping type applied in RRC_CONNECTED.

Proposal 3	Capture the text proposal in Section 2.2.

- [AT109e][414][eMTC]  MPDCCH performance imp: Open issues (Qualcomm)

Scope: Further discussion on proposals 1 and 2 and identify potential agreements

Intended outcome: Report with a list of proposals categorized as agreeable, need further discussion, postpone. The outcome can be provided in R2-2001879

Deadline: Tuesday, Mar 3rd 17:00 CET

R2-2001879	Report of [AT109e][414][eMTC]  MPDCCH performance imp: Open issues	Qualcomm	discussion	LTE_eMTC5-Core

Proposal 1	Power ratio can be included in dedicated signalling (in addition to SIB2).

Proposal 2	Localized mapping type is possible to be included only in dedicated signalling. Value “predefined” is DEFAULT.

Proposal 3	It should be possible to enable the feature for IDLE mode UEs but disable in CONNECTED mode.

Proposal 4	TP1 (with separate IEs CRS-ChEstMPDCCH-ConfigCommon-r16 and CRS-ChEstMPDCCH-ConfigDedicated-r16) is baseline for running CR.

Agreements

- Power ratio can be included in dedicated signalling (in addition to SIB2).

- Localized mapping type is possible to be included only in dedicated signalling. Value “predefined” is DEFAULT.

- It should be possible to enable the feature for IDLE mode UEs but disable in CONNECTED mode.

- TP1 in R2-2001879 (with separate IEs CRS-ChEstMPDCCH-ConfigCommon-r16 and CRS-ChEstMPDCCH-ConfigDedicated-r16) is baseline for running CR.

### 7.1.8	Improvements for non-BL UEs

CE mode A and B improvements for non-BL UEs among “enhancements to idle mode mobility”, “UE demodulation performance requirements for 2 RX antennas and full duplex FDD”, “Dual layer DL reception”, “Feedback based on CSI-RS”, “ETWS/CMAS in connected mode”

No documents should be submitted to AI 7.1.8. Please submit the documents to AI 7.1.8.x

#### 7.1.8.1	Idle Mode Mobility

This agenda item will utilize a summary document to facilitate treatment of topics during the e-meeting. This may lead to postponing some items to the next meeting. A web conference may be used for handling some of the discussions in this AI.

R2-2001864	Summary of contributions on idle mode mobility	Intel Corporation	discussion	Rel-16	LTE_eMTC5-Core

Proposal 1: Rel-16 non-BL UE is allowed to camp in a cell (non-standalone case) in enhanced coverage when S-criterion for normal coverage is fulfilled only if indicated by the network via broadcast signaling (FFS on whether the presence or absence of the broadcast indication enables the feature).

Proposal 2: If proposal 1 is agreed, FFS on whether the new broadcast indication described in proposal 1 is (a) applicable only for non-BL UE entering (i.e. (re)selecting) the cell or (b) all non-BL UEs in the cell.

Proposal 3: If proposal 1 is agreed, FFS on whether priority based cell reselection rather than ranking should be applied for non-BL UEs using enhanced coverage when S criteria for normal coverage is fulfilled.

R2-2000251	Clarification to idle mode mobility for non-BL UEs	THALES	discussion

R2-2000979	S-Criterion interpretation for non-BL UEs	Ericsson	discussion	LTE_eMTC5-Core

R2-2001067	Enhancements to idle mode mobility for non-BL UEs	Huawei, HiSilicon	discussion	Rel-16	LTE_eMTC5-Core

R2-2001098	Non-BL UE in normal and enhanced coverage	Intel Corporation	discussion	Rel-16	LTE_eMTC5-Core

- [AT109e][415][eMTC]  Idle Mode Mobility: Open issues (Intel)

Scope: Further discussion on proposals 1, 2, and 3, and identify potential agreements

Intended outcome: Report with a list of proposals categorized as agreeable, need further discussion, postpone. The outcome can be provided in R2-2001880

Deadline: Tuesday, Mar 3rd 17:00 CET

R2-2001880	Email discussion of Idle mode mobility for non-BL UE	Intel Corporation	discussion	Rel-16	LTE_eMTC5-Core

Proposal#1: RAN2 is requested to choose between the following 2 alternatives:

•	Alternative 1: Rel-16 specification is not updated to capture whether the Rel-16 non-BL UE are allowed or not to camp in a cell (non-standalone case) on enhanced coverage mode when S criteria for normal coverage is fulfilled (i.e. UE’s behavior is left up to UE implementation as for legacy non-BL UEs).

•	Alternative 2: A Rel-16 UE is allowed to camp in a cell (non-standalone case) on enhanced coverage mode when S criteria for normal coverage is fulfilled. Network control is needed.

Proposal#2: Postpone the following proposals to the next meeting if Proposal 1 Alternative 2 is agreed:

-	Proposal A: The network may indicate whether the fulfillment of the S-criterion for CE mode is required for all non-BL devices or whether it only applies for the non-BL UEs trying to newly camp (select or re-select) to said cell.

-	Proposal B: Absolute priority rather than ranking applies for non-BL UEs using enhanced coverage when S criteria for normal coverage is fulfilled.

- No consensus on any of the alternatives.

#### 7.1.8.2	ETWS/CMAS in connected mode

This agenda item will not utilize a summary document to facilitate treatment of topics during the e-meeting. This may lead to postponing some items to the next meeting. A web conference may be used for handling some of the discussions in this AI.

### 7.1.9	Stand-alone deployment

Enable the use of LTE control channel region for DL transmission (MPDCCH/PDSCH) to BL/CE UEs

This agenda item will utilize a summary document to facilitate treatment of topics during the e-meeting. This may lead to postponing some items to the next meeting. A web conference may be used for handling some of the discussions in this AI.

R2-2001865	Summary of contributions on stand-alone deployment	Nokia, Nokia Shanghai Bell	discussion	LTE_eMTC5-Core

Proposal 1: RAN1 further discuss and agree on the final word for running CR text relevant for cell selection behaviour in standalone cell.

Proposal 2: RAN2 further discuss and conclude whether additional changes needed related to cell reselection behaviour for non BL UE from /to standalone cell.

Proposal 3: (Potential RAN2 agreement):  In standalone deployment, if a UE considers itself to be in enhanced coverage with S criteria of normal coverage fulfilled, absolute priorities for cell reselection are used (i.e. UE does not switch to ranking as it would when in enhanced coverage due to S-criteria).

Proposal 4: RAN2 further discuss on proposal 1 of [4] related to changes to be captured for TS36.304

- Nokia supports proposal 3.

R2-2000980	Cell Reselection improvement for LTE-M Standalone cells	Ericsson	discussion	LTE_eMTC5-Core	R2-1915401

R2-2001070	Remaining issue on standalone deployment	Huawei, HiSilicon	discussion	Rel-16	LTE_eMTC5-Core

R2-2001127	Remaining details for standalone LTE-M deployment	Ericsson	discussion	LTE_eMTC5-Core	Late

- [AT109e][416][eMTC]  Stand-alone deployment: Open issues (Nokia)

Scope: Further discussion to address the remaining issues and identify potential agreements.

Intended outcome: Report with a list of proposals categorized as agreeable, need further discussion, postpone. The outcome can be provided in R2-2001881

Deadline: Tuesday, Mar 3rd 17:00 CET

R2-2001881	Standalone deployments – Discussion on remaining Issues	Nokia, Nokia Shanghai Bell	discussion	LTE_eMTC5-Core

Proposal 1	The verb in [] in the text proposal of running CR should be changed to “shall”

Proposal 2	In standalone deployment, if a UE considers itself to be in enhanced coverage with S criteria of normal coverage fulfilled, absolute priorities for cell reselection are used (i.e. UE does not switch to ranking as it would when in enhanced coverage due to S-criteria).

Proposal 3	P1 from [3] and P1 from [4] are not agreed.

Agreements

- In standalone deployment, if a UE considers itself to be in enhanced coverage with S criteria of normal coverage fulfilled, absolute priorities for cell reselection are used (i.e. UE does not switch to ranking as it would when in enhanced coverage due to S-criteria).

-

### 7.1.10	Mobility Enhancements

Improving the DL RSRP and, RSRQ measurement accuracy, through use of RSS, relaxation of RRM measurements for serving cell for UEs using WUS for at least low mobility UEs

Including outcome of the email discussion [108#73][eMTC] TPs for RSS (Ericsson)

A web conference or an offline discussion may be used for handling the outcome of the email discussion in this AI.

R2-2001242	Summary of [108#73] [eMTC] TPs for RSS (Ericsson)	Ericsson	discussion	Rel-16

- QC thinks further check is needed for the connected mode case due to the ongoing RAN1 discussion. Ericsson confirms.

- [AT109e][417][eMTC]  Mobility enhancements: Open issues (Ericsson)

Scope: Further discussion to address the remaining issues and identify potential agreements.

Intended outcome: Report with a list of proposals categorized as agreeable, need further discussion, postpone. The outcome can be provided in R2-2001882

Deadline: Tuesday, Mar 3rd 17:00 CET

R2-2001882	TPs for RSS	Ericsson	discussion	Rel-16

- We will have an email discussion after the meeting to finalize the TP.

- [109e#xx] [eMTC R16] TP for RSS (Ericsson)

Scope: Finalize the text proposal for RSS

Intended outcome: endorsed document in R2-2001887

Deadline: 1 week

### 7.1.11	Coexistence with NR

Study NR and LTE specifications to identify possible issues related to coexistence of MTC with NR

Coexistence with NR for MTC and NB-IoT is treated jointly under this AI during the e-meeting.

This agenda item will utilize a summary document to facilitate treatment of topics during the e-meeting. This may lead to postponing some items to the next meeting. A web conference may be used for handling some of the discussions in this AI.

R2-2002024	Summary of Coexistence with NR	ZTE	discussion	Rel-16	LTE_eMTC5-Core, NB_IOTenh3-Core

For NB-IoT:

Potential easy agreements:

Proposal 1-1: For NB-IoT, resource reservation configuration for NR coexistence is only provided via dedicated RRC signalling.

Proposal 1-2: For NB-IoT, resource reservation configuration for NR coexistence is provided in PhysicalConfigDedicated-NB.

Proposal 1-4: For FDD, two new Rel-16 IEs for DL and UL resource reservation configuration can be introduced.

Proposal 1-8: Introduce two UE capabilities nr-ResourceResvUL-r16 and nr-ResourceResvDL-r16 in PhyLayerParameters-NB-v16xy.

Might need further discussion:

Proposal 1-3: RAN2 needs to discuss where to define new Rel-16 IE (s) for providing resource reservation configuration, in PhysicalConfigDedicated-NB or in the extension of CarrierConfigDedicated-NB in PhysicalConfigDedicated-NB.

Proposal 1-5: For TDD, RAN2 needs to discuss whether two Rel-16 IEs for DL and UL resource reservation configuration need to be introduced for TDD. And whether DL and UL resource reservation configuration for FDD can be reused for TDD.

Proposal 1-6: RAN2 needs to choose one from the following alternatives for new IE structure:

Alt1: Separate and independent IE structures for providing FDD UL and FDD DL resource reservation configuration, moreover, same ASN.1 structure as for FDD UL can be used for TDD DL/UL (proposal in [7]) .

Alt2: Only one IE structure for providing FDD UL, FDD DL and TDD DL/UL resource reservation configuration. In the structure, only the symbol-level configuration parameter is differentiated for DL and UL (Option A in [8]).

Alt3: Separate and independent IE structures for providing UL and DL resource reservation configuration, for both TDD and FDD (Option B in [8]).

Proposal 1-7: RAN2 needs to discuss whether it needs to explicitly indicate the dependence between the value of Periodicity and value range of startPosition.

Proposal 1-9: RAN2 needs to discuss whether to introduce two additional UE capabilities for TDD. If not, whether the above UE capabilities in Proposal 1-8 can be applied to both FDD and TDD.

Proposal 1-10: Introduce two new items nr-ResourceResvUL-r16 and nr-ResourceResvDL-r16 in 36.306. RAN2 needs to discuss whether they are capabilities or IOT bits.

For eMTC:

Potential easy agreements:

Proposal 2-1: For eMTC, configurations related to resource reservation and DL subcarrier puncturing for NR coexistence are provided via dedicated RRC signalling.

Proposal 2-2: The configurations related to resource reservation and DL subcarrier puncturing for NR coexistence can be provided in PhysicalConfigDedicated.

Proposal 2-3: For FDD, two new IEs for separate and independent UL and DL resource reservation configuration can be introduced.

Proposal 2-5: A same IE structure, e.g., NR-ResourceReservationConfig-r16 can be defined for UL and DL resource reservation configuration, for both TDD and FDD.

Proposal 2-6: Separate configuration for DL subcarrier puncturing can be introduced for both FDD and TDD, e.g., ce-NR-PuncturedSubcarrierDL-r16 in PhysicalConfigDedicated.

Proposal 2-7: Introduce four UE capabilities for resource reservation in UL and DL, e.g., ce-ModeA-NR-ResourceResvUL-r16, ce-ModeA-NR-ResourceResvDL-r16, ce-ModeB-NR-ResourceResvUL-r16 and ce-ModeB-NR-ResourceResvDL-r16.

Proposal 2-8: Introduce two UE capabilities for DL subcarrier puncturing, e.g., ce-ModeA-NR-SubcarrierPuncturing-r16, and ce-ModeB-NR-SubcarrierPuncturing-r16.

Might need further discussion:

Proposal 2-4: For TDD, RAN2 needs to discuss whether two Rel-16 IEs for DL and UL resource reservation configuration need to be introduced. And whether DL and UL resource reservation configuration for FDD can be reused for TDD.

Proposal 2-9: RAN2 needs discuss whether two separate UE capability IEs for TDD and FDD need to be introduced.

Proposal 2-10: Introduce six new items ce-DL-resourceReservation-CE-ModeA-r16, ce-DL-resourceReservation-CE-ModeB-r16, ce-UL-resourceReservation-CE-ModeA-r16, ce-UL-resourceReservation-CE-ModeB-r16, ce-DL-subcarrierPuncturing CE-ModeA -r16 and ce-DL-subcarrierPuncturing CE-ModeB -r16 in 36.306. RAN2 needs to discuss whether they are capabilities or IOT bits.

- Huawei, Ericsson, Nokia, and ZTE think that dedicated RRC signalling is acceptable whereas QC thinks that this will not be efficient since ~200 bits need to be transmitted everytime UE is in connected mode.

- The discussion will continue in [AT109e][418][eMTC/NB-IoT].

R2-2000981	LTE-M coexistence with NR	Ericsson	discussion	LTE_eMTC5-Core

=> Revised in R2-2002062

R2-2002062	LTE-M coexistence with NR	Ericsson	discussion	LTE_eMTC5-Core

R2-2001068	Coexistence with NR for eMTC	Huawei, HiSilicon	discussion	Rel-16	LTE_eMTC5-Core

- [AT109e][418][eMTC/NB-IoT]  Coexistence with NR: Open Issues (ZTE)

Scope: Further discussion to address the remaining issues and identify potential agreements.

Intended outcome: Report with a list of proposals categorized as agreeable, need further discussion, postpone. The outcome can be provided in R2-2001883

Deadline: Tuesday, Mar 3rd 17:00 CET

R2-2001883	Summary of Coexistence with NR	ZTE	discussion	Rel-16	LTE_eMTC5-Core, NB_IOTenh3-Core

Potential easy agreements:

For NB-IoT:

Proposal 1-1: Resource reservation configuration for NR coexistence is only provided via dedicated RRC signalling.

Proposal 1-2: Resource reservation configuration for NR coexistence is provided in PhysicalConfigDedicated-NB.

Proposal 1-3: New Rel-16 IE (s) can be introduced in PhysicalConfigDedicated-NB for providing resource reservation configuration for NR coexistence.

Proposal 1-4: For FDD, two independent parameters in UL and DL are needed for providing UL and DL resource reservation configuration separately.

Proposal 1-5: For TDD, two independent parameters in UL and DL are needed for providing UL and DL resource reservation configuration separately.

Proposal 1-6: One new IE structure for resource reservation configuration for both DL and UL, and for both TDD and FDD are introduced.

Proposal 1-7: Independent definition for periodicity and start position parameters are introduced and the dependency between them can be clarified in the field description.

Proposal 1-8: Working assumption: Introduce two UE capabilities for handling resources reservation on DL or UL in PhyLayerParameters-NB-v16xy.

Proposal 1-9: Working assumption: Two UE capabilities for handling resources reservation on DL or UL in PhyLayerParameters-NB-v16xy can be applied to both FDD and TDD, e.g., with separate values for FDD or TDD.

Proposal 1-10: Working assumption: Two UE capabilities for handling resources reservation on DL or UL can be introduced into TS 36.306, e.g., section 4.3.4.

For eMTC:

Proposal 2-1: Configurations related to resource reservation and DL subcarrier puncturing for NR coexistence are provided via dedicated RRC signalling.

Proposal 2-2: The configurations related to resource reservation and DL subcarrier puncturing for NR coexistence can be provided in PhysicalConfigDedicated.

Proposal 2-3: For FDD, two independent parameters in UL and DL are needed for providing UL and DL resource reservation configuration separately.

Proposal 2-4: For TDD, two independent parameters in UL and DL are needed for providing UL and DL resource reservation configuration separately.

Proposal 2-5: One new IE structure for resource reservation configuration for both DL and UL, and for both TDD and FDD are introduced.

Proposal 2-6: A parameter for providing DL subcarrier puncturing configuration can be introduced in PhysicalConfigDedicated, which is for both FDD and TDD.

Proposal 2-7: Working assumption: Introduce four UE capabilities for handling resources reservation on UL and DL, and for CE mode A and CE mode B separately, in PhyLayerParameters-v16xy.

Proposal 2-8: Working assumption: Introduce two UE capabilities for handling DL subcarrier puncturing for CE mode A and CE mode B separately, in PhyLayerParameters-v16xy.

Proposal 2-9: Working assumption: Six UE capabilities mentioned in Proposal 2-7 and Proposal 2-8 for handling resources reservation or DL subcarrier puncturing can be applied to both FDD and TDD, e.g., with separate values for FDD or TDD.

Proposal 2-10: Working assumption: Six UE capabilities for handling resources reservation or DL subcarrier puncturing can be introduced into TS 36.306.

Candidates to postpone

For eMTC:

Proposal 2-1a: FFS for eMTC, whether and how configurations related to resource reservation and DL subcarrier puncturing for NR coexistence are also provided via SI, e.g, a new SIB.

- T-Mobile prefers this feature to be mandatory for NB-IoT and eMTC.

Agreements

For NB-IoT:

- Resource reservation configuration for NR coexistence is provided via dedicated RRC signaling.

- Resource reservation configuration for NR coexistence is provided in PhysicalConfigDedicated-NB.

- New Rel-16 IE (s) can be introduced in PhysicalConfigDedicated-NB for providing resource reservation configuration for NR coexistence.

- For FDD, two independent parameters in UL and DL are needed for providing UL and DL resource reservation configuration separately.

- For TDD, two independent parameters in UL and DL are needed for providing UL and DL resource reservation configuration separately.

- One new IE structure for resource reservation configuration for both DL and UL, and for both TDD and FDD are introduced.

- Independent definition for periodicity and start position parameters are introduced and the dependency between them can be clarified in the field description.

- Working assumption: Introduce two UE capabilities for handling resources reservation on DL or UL in PhyLayerParameters-NB-v16xy.

- Working assumption: Two UE capabilities for handling resources reservation on DL or UL in PhyLayerParameters-NB-v16xy can be applied to both FDD and TDD, e.g., with separate values for FDD or TDD.

- Working assumption: Two UE capabilities for handling resources reservation on DL or UL can be introduced into TS 36.306, e.g., section 4.3.4.

For eMTC:

- Configurations related to resource reservation and DL subcarrier puncturing for NR coexistence are provided via dedicated RRC signaling.

- The configurations related to resource reservation and DL subcarrier puncturing for NR coexistence can be provided in PhysicalConfigDedicated.

- For FDD, two independent parameters in UL and DL are needed for providing UL and DL resource reservation configuration separately.

- For TDD, two independent parameters in UL and DL are needed for providing UL and DL resource reservation configuration separately.

- One new IE structure for resource reservation configuration for both DL and UL, and for both TDD and FDD are introduced.

- A parameter for providing DL subcarrier puncturing configuration can be introduced in PhysicalConfigDedicated, which is for both FDD and TDD.

- Working assumption: Introduce four UE capabilities for handling resources reservation on UL and DL, and for CE mode A and CE mode B separately, in PhyLayerParameters-v16xy.

- Working assumption: Introduce two UE capabilities for handling DL subcarrier puncturing for CE mode A and CE mode B separately, in PhyLayerParameters-v16xy.

- Working assumption: Six UE capabilities mentioned in Proposal 2-7 and Proposal 2-8 for handling resources reservation or DL subcarrier puncturing can be applied to both FDD and TDD, e.g., with separate values for FDD or TDD.

- Working assumption: Six UE capabilities for handling resources reservation or DL subcarrier puncturing can be introduced into TS 36.306.

- Send a LS to RAN1 to check whether it should be possible to have a common configuration for all NB-IoT carriers for resource reservation for NR coexistence. For NB-IoT and eMTC whether it is possible to optimize signalling.

- [109e#xx] [NB-IoT/eMTC R16] LS to RAN1 on co-existence with NR (Qualcomm)

Scope: LS to RAN1 to check whether it should be possible to have a common configuration for all NB-IoT carriers for resource reservation for NR coexistence and for NB-IoT and eMTC whether it is possible to optimize signalling.

Intended outcome: approved LS in R2-2001888

Deadline: 1 week

### 7.1.12	Connection to 5GC (eDRX, EDT, UP optimisation, RRC_INACTIVE and other MTC specific topics)

Support of eDRX in CM-IDLE, UP optimisation, and EDT for MTC and NB-IoT are treated jointly under this AI.

This agenda item will utilize a summary document to facilitate treatment of topics during the e-meeting. This may lead to postponing some items to the next meeting. A web conference may be used for handling some of the discussions in this AI.

No documents should be submitted to AI 7.1.12. Please submit the documents to AI 7.1.12.x

R2-2002014	Summary of contributions for connection to 5GC (AI 7.1.12)	Huawei	discussion	Rel-16	NB_IOTenh3-Core, LTE_eMTC5-Core

Proposal S1-1: When idle mode eDRX is configured and TeDRX is larger than 5.12 s, eMTC UEs in RRC_IDLE only need to monitor the paging occasions during the PTW. The UE monitors the paging occasions according to the shortest of the cell default paging cycle and the UE specific DRX (if configured).

- QC thinks the first 3 proposals are already the legacy behaviour. Huawei agrees.

Proposal S1-2: When idle mode eDRX is configured and TeDRX is equal to 5.12 s, eMTC UEs in RRC_IDLE monitor the paging occasions according to TeDRX.

Proposal S1-3: When idle mode eDRX is not configured, eMTC UEs in RRC_IDLE monitor the paging occasions according to the shortest of the cell default paging cycle and the UE specific DRX (if configured).

- The proposals above are describing the legacy behaviour and companies agree that they are applicable to 5GC.

Proposal S1-4: When idle mode eDRX is not configured, eMTC UEs in RRC_INACTIVE monitor the paging occasions according to the shortest of the cell default paging cycle, the UE specific DRX (if configured), and the RAN paging cycle (if configured).

- QC thinks this is acceptable only if eDRX cycle values are the legacy values, i.e., excluding 5.12 sec and 10.24 sec. Huawei thinks this should be same with legacy.

Proposal S2-1: DRB resumption for EDT for eMTC UEs connected to 5GC follows the same principle as in EPC, i.e.:

drb-ContinueROHC is provided in RRCConnectionRelease message triggering the suspension in RRC_IDLE. The flag applies to all DRBs.

When resuming the DRBs for EDT, RRC procedure text triggers PDCP re-establishment and provides NR PDCP with the drb-ContinueROHC indication received in RRCConnectionRelease message.

Open items proposed to be further discussed in this meeting (from all sub-topics)

Proposal S1-5: Offline discussion on paging in RRC_INACTIVE for eMTC UEs configured with idle mode eDRX:

-	option 1: UE monitors paging occasions (POs) during CM-IDLE PTW according to the min {UE specific DRX cycle, default DRX cycle, RAN paging cycle} and monitor paging occasions outside CM-IDLE PTW according to RAN paging cycle.

-	option 2 :

o	If extended RAN paging cycles (i.e. 5.12, 10.24 Sec) are configured, UE monitors paging occasions according to RAN paging cycle

o	If extended RAN paging cycles (i.e. 5.12, 10.24 Sec) are not configured, UE monitors paging occasions according to the min {UE specific DRX cycle, default DRX cycle, RAN paging cycle}

Proposal S2-2: Offline discussion on remaining aspects related to DRB resumption, covering:

full configuration

particularities of NR PDCP

Proposal S3-1: Discuss the change in the e-mail discussion on the running eMTC 36.306 running CR or postpone to next meeting

Open items proposed to be postponed to next meeting (from all sub-topics)

Proposal S3-2: Postpone discussion on Early UE capability retrieval enhancements for eMTC/5GC.

- [AT109e][419][eMTC/NB-IoT]  Connection to 5GC: Open Issues (Huawei)

Scope: Further discussion to address the remaining issues, i.e., proposals S1-5, S2-2, S3-1, and identify potential agreements.

Intended outcome: Report with a list of proposals categorized as agreeable, need further discussion, postpone. The outcome can be provided in R2-2001885

Deadline: Tuesday, Mar 3rd 17:00 CET

R2-2001885	[AT109e][419][eMTC/NB-IoT]  Connection to 5GC: Open Issues	Huawei	discussion	Rel-16	NB_IOTenh3-Core, LTE_eMTC5-Core

Potential easy agreements

Proposal P1-1: Option 1 is feasible although it has some impact on UE power consumption.

Proposal P1-2: Option 2 is not feasible with the extended DRX cycle values in RRC_INACTIVE.

Proposal P1-3: When idle mode eDRX is configured, eMTC UEs in RRC_INACTIVE monitor the paging occasions (POs) during CM-IDLE PTW according to the min {UE specific DRX cycle, default DRX cycle, RAN paging cycle} and monitor paging occasions (POs) outside CM-IDLE PTW according to RAN paging cycle.

Proposal P2-2: PDCP Suspend is triggered at the time of suspension to RRC_IDLE for eMTC UEs connected to 5GC.

Proposal P2-3: DRB resumption for non-EDT for eMTC UEs connected to 5GC follows the same principle as in RRC_INACTIVE, i.e.:

-	When resuming the DRBs for non-EDT, RRC procedure text does not trigger PDCP re-establishment.

-	PDCP re-establishment and ROHC continuation for each DRB are triggered by the presence of the respective flags in RRCConnectionResume message as specified in TS 38.331 [82], clause 5.3.5.6;

Proposal P2-4: When resuming the RRC connection, the default RLC configuration and default (NR) PDCP configuration is applied to SRB1 for eMTC and NB-IoT UEs connected to 5GC.

Candidates to postpone

Proposal P2-1: Postpone discussion on full configuration with MO-EDT.

Proposal P2-5: Postpone discussion on SRB1 handling at fallback to RRC connection establishment procedure during RRC connection resumption.

Proposal P3-1: Postpone the discussion on clarification for LTE-M in TS 36.306.

#### 7.1.12.1	Paging in RRC_INACTIVE

This agenda item may utilize a summary document to facilitate treatment of topics during the e-meeting (decision to be made based on the submitted tdocs). This may lead to postponing some items to the next meeting. A web conference of an offline discussion may be used for handling some of the discussions in this AI.

R2-2000538	Page monitoring in RRC_INACTIVE state with short eDRX	Qualcomm India Pvt Ltd	discussion	Rel-16	LTE_eMTC5-Core

R2-2000645	Discussion on paging of RRC_INACTIVE for eMTC connected to 5GC	Huawei, HiSilicon, Ericsson	discussion	Rel-16	LTE_eMTC5-Core

R2-2001211	FFSs for supporting short eDRX in RRC_INACTIVE for eMTC in 5GC	ZTE Corporation, Sanechips	discussion	Rel-16	LTE_eMTC5-Core

#### 7.1.12.2	DRB resume in UP optimization

Including outcome of the email discussion [108#19][eMTC NB-IoT] When to resume DRBs in UP optimization for 5GC (Ericsson)

A web conference or an offline discussion may be used for handling the outcome of the email discussion in this AI.

R2-2000982	Report of [108#19] when to resume DRBs in UP optimization for 5GC	Ericsson	discussion	NB_IOTenh3-Core, LTE_eMTC5-Core

Proposal 1	DRBs are resumed upon receiving RRCConnectionResume in UP optimization when connected to 5GC.

R2-2000646	SRBs and DRBs handling for NB-IoT and eMTC connected to 5GC	Huawei, HiSilicon	discussion	Rel-16	NB_IOTenh3-Core, LTE_eMTC5-Core

#### 7.1.12.3	Other

This AI may not be treated during the e-meeting (decision to be made based on the submitted tdocs).

R2-2000311	Text proposal for 36.306 to align Cat M definition with LTE-M indicator	Qualcomm Incorporated	discussion	Rel-16	LTE_eMTC5-Core

R2-2000536	Early UE capability retrieval enhancements for eMTC/5GC	Qualcomm India Pvt Ltd	discussion	Rel-16	LTE_eMTC5-Core

Agreements

- DRBs are resumed upon receiving RRCConnectionResume in UP optimization when connected to 5GC.

- When idle mode eDRX is not configured, eMTC UEs in RRC_INACTIVE monitor the paging occasions according to the shortest of the cell default paging cycle, the UE specific DRX (if configured), and the RAN paging cycle (if configured).

- When idle mode eDRX is not configured, eMTC UEs in RRC_INACTIVE cannot be configured with values 5.12 sec and 10.24 sec

- DRB resumption for EDT for eMTC UEs connected to 5GC follows the same principle as in EPC, i.e.:

drb-ContinueROHC is provided in RRCConnectionRelease message triggering the suspension in RRC_IDLE. The flag applies to all DRBs.

When resuming the DRBs for EDT, RRC procedure text triggers PDCP re-establishment and provides NR PDCP with the drb-ContinueROHC indication received in RRCConnectionRelease message.

- When idle mode eDRX is configured, eMTC UEs in RRC_INACTIVE monitor the paging occasions (POs) during CM-IDLE PTW according to the min {UE specific DRX cycle, default DRX cycle, RAN paging cycle} and monitor paging occasions (POs) outside CM-IDLE PTW according to RAN paging cycle.

- PDCP Suspend is triggered at the time of suspension to RRC_IDLE for eMTC UEs connected to 5GC.

-  DRB resumption for non-EDT for eMTC UEs connected to 5GC follows the same principle as in RRC_INACTIVE, i.e.:

- When resuming the DRBs for non-EDT, RRC procedure text does not trigger PDCP re-establishment.

- PDCP re-establishment and ROHC continuation for each DRB are triggered by the presence of the respective flags in RRCConnectionResume message as specified in TS 38.331 [82], clause 5.3.5.6;

- When resuming the RRC connection, the default RLC configuration and default (NR) PDCP configuration is applied to SRB1 for eMTC and NB-IoT UEs connected to 5GC.

### 7.1.13	Other

This AI may not be treated during the e-meeting (decision to be made based on the submitted tdocs).

R2-2000515	CE Mode Threshold Adjustments for non-BL and BL UE	NTT DOCOMO INC.	discussion	Rel-16	R2-1914474

- Noted

- [AT109e][410][eMTC] CE Mode Threshold Adjustments for non-BL and BL UE (DoCoMo)

Intended outcome: To check whether there is support, and, if so, list of proposals that are easily agreeable, almost agreeable and need further discussion. The outcome can be provided in R2-2001875

Deadline: Wednesday, Feb. 26th 18:00 CET

Schedule: Thursday, Feb. 27th 13:30 - 15:30 CET

R2-2001884	CE Mode Threshold Adjustments for non-BL and BL UE	NTT DOCOMO INC.

- Rapporteur proposes to postpone the discussion.

R2-2001396	Draft Reply LS on category M devices and NR	LG Electronics UK	discussion	Rel-16