3GPP TSG-RAN WG2 Meeting #109 electronic

24 Feb – 6 Mar 2020

Agenda Item:	7.1.1

Source: 	ZTE (email discussion rapporteur)

Title:  	Discussion on remaining issues for running 36.302 CR for R16 eMTC

Document for:	Discussion and Decision

# Introduction

The endorsed version of the TS 36.302 running CR for eMTC has been provided in Error: Reference source not found. During the RAN2#109 e-meeting, the following email discussion has been assigned:

- [AT109e][404][eMTC]  Update 36.302 running CR (ZTE)

Scope: Update 36.302 running CR, i.e., address the open issues and include the agreements from this meeting.

Intended outcome: Agreeable CR in R2-2001869

Deadline: TBA

In this document, we will list the remaining issues and collect the comments from companies with the aim to resolve the editor’s notes in the running CR and achieve aggregable text proposals.

# Discussion

#1 Editor’s note for "Reception Type" Combination

At the end of “Table 8.2-2a: Downlink "Reception Type" Combinations for BL UEs and UEs in enhanced coverage”, there has an Editor’s note about whether the new-added "Reception Type" Combinations in 2.3 and 2.4 are aligned with RAN1 specification, e.g., the description related to Type0-MPDCCH common search space and MPDCCH UE-specific search space.

In [2], company gives some analysis based on RAN1 specification and has the Observation 1 that it’s impossible for the non-BL UE in CE to simultaneously receive “M” (e.g. MPDCCH in Type 0 CSS) and USS-MPDCCH. In other word, it’s possible for the non-BL UE in CE to monitor both Type 0 CSS and USS but it can only receive either “M” or other USS-MPDCCH. The company think in current running CR, it looks that non-BL UE in CE cannot receive both “M” and USS-MPDCCHs for scheduling DL but can receive both “M” and USS-MPDCCHs for scheduling UL. They think this contradicts the above Observation 1.

Q1: Companies are invited to provide their comments on whether the Downlink "Reception Type" Combinations for eMTC in the running CR are aligned with the RAN1 specification? If no, what’s the issues?

| Company | Comments |
| --- | --- |
| ZTE | We agree the Obervation 1 and think there is contradiction between the Downlink "Reception Type" Combinations for eMTC in the running CR with the Obervation 1. We think the right combinations should reflect the following considerations: - The eMTC UE can receive both (J or E or B) and (I or F) simultaneously in the UE-specific search space; - The R15 and earlier eMTC UE can receive only one between the two Reception Types C, D. The R16 eMTC UE can receive only one among the three Reception Types C, D and M in the Type 0 CSS; - The eMTC UE can receive either the (C or D or M), or the ((J or E or B) and (I or F)), but not both. |
| Sequans | Agree with ZTE |

Based on the analysis in [2], the company give the following text proposal to correct the Downlink "Reception Type" Combinations for the eMTC UE in RRC_CONNECTED:

Table 8.2-2a: Downlink "Reception Type" Combinations for BL UEs and UEs in enhanced coverage

| ………. |  |
| --- | --- |
| 2. RRC_CONNECTED |  |
| 2.1 All UEs | A or ((((J or E or B) + (I or F)) or C or D) + K) or G or H |
|  | Remarks: The combination for Random Access procedure is only required, related to G and H. |
| 2.2 UEs supporting FS2 | A or ((((J or E or B) + (I or F)) or C or D) + I + K) or G or H |
|  | Remarks: For TDD UL/DL configuration 0, two MPDCCHs can be received in the same subframe for UL-SCH in two different uplink subframes, which is only applicable for UEs configured with CE mode A with no repetitions. Remarks: The combination for Random Access procedure is only required, related to G and H. |
| 2.3 UEs supporting ETWS and CMAS | A or ((((J or E or B) + (I or F)) or C or D or M) + K) or G or H |
|  | Remarks: The combination for Random Access procedure is only required, related to G and H. Remarks: The combination only applies for non-BL UE in CE. |
| 2.4 ETWS and CMAS UEs supporting FS2 | A or ((((J or E or B) + (I or F)) or C or D or M) + I + K) or G or H |
|  | Remarks: For TDD UL/DL configuration 0, two MPDCCHs can be received in the same subframe for UL-SCH in two different uplink subframes, which is only applicable for UEs configured with CE mode A with no repetitions. Remarks: The combination for Random Access procedure is only required, related to G and H. Remarks: The combination only applies for non-BL UE in CE. |

Q2: If Companies agree there has issue for the Q1, they are further invited to provide their comments on the above text proposal, e.g., whether it is agreeable? If not, please the companies give their preferred text proposal.

| Company | Comments |
| --- | --- |
| ZTE | Agree the text proposal. |
| Sequans | Agree the text proposal, but not now. As discussed in the “phase 1” discussion during [108#22] (There isn’t any official tdoc for that discussion, right?), since this also introduces changes in legacy, it should come coupled with legacy CRs, otherwise we may end up with discrepancies between releases. Once such legacy CRs are agreed this can also be incorporated into the R16 running CR. |

#2 Editor’s note for PUR "Reception Type"

At the end of “Table 8.2-1a: Downlink "Reception Types" for BL UEs and UEs in enhanced coverage”, there has an Editor’s note about whether PUR related new "Reception Types" would be needed in Table 8.2-1a. During previous email discussion, there already have some companies to mention its necessity. Therefore, rapporteur think it’s straightforward to add new Reception Types for PUR.

In [2], the company give some analysis on PUR reception based on RAN1 and RAN2 agreements. They further give the following text proposal about adding four new reception types for eMTC UE in IDLE.

Table 8.2-1a: Downlink "Reception Types" for BL UEs and UEs in enhanced coverage

| "Reception Type" | Physical Channel(s) | Monitored RNTI | Associated Transport Channel |
| --- | --- | --- | --- |
| A | PBCH | N/A | BCH |
| B | MPDCCH (Note 1) | C-RNTI | N/A |
| C | MPDCCH | TPC-PUCCH-RNTI | N/A |
| …… |  |  |  |
| M | MPDCCH (Note 8) | SI-RNTI | N/A |
| N | MPDCCH | PUR-RNTI | DL-SCH |
| O | MPDCCH | PUR-RNTI | UL-SCH |
| P | PDSCH | PUR-RNTI | DL-SCH |
| Q | MPDCCH | PUR-RNTI | N/A |

Q3: Companies are invited to provide their comments on the above text proposal for new reception types for PUR, e.g., whether it is agreeable? If not, please the companies give their preference, e.g., how many new reception types are needed and what are they?

| Company | Comments |
| --- | --- |
| ZTE | Agree the text proposal. |
| Sequans | Agree |

With the new reception types for PUR, new reception type combinations for eMTC UE are also needed. In [2], company give some high level considerations for the new reception type combinations for PUR. Rapporteur also think before we discuss the details of new reception type combinations, it’s better to firstly figure out some main principles.

Q4: Companies are invited to provide their general considerations on new reception type combinations for PUR for eMTC UE in IDLE, e.g., whether they can agree (part of) the analysis in [2]? If not, what are their thinking?

| Company | Comments |
| --- | --- |
| ZTE | According to the current RAN1 and RAN2 agreements, we have the following considerations: - Due to only single HARQ process is supported and there exists timing between MPDCCH transmission and corresponding PDSCH transmission (e.g., PDSCH transmission can only be after several subframes from the last subframe in which the MPDCCH is transmitted), the reception type “N” for MPDCCH for DL and reception type “P” for PDSCH cannot be received simultaneously; - The reception type “N” for MPDCCH for DL and reception type “O” for MPDCCH for UL can be received simultaneously due to eMTC UE is capable of duplex. - The reception type “O” for MPDCCH for UL and reception type “P” for PDSCH can also be received simultaneously as they are independent of each other. - Reception Type "Q" and any other new reception type for PUR cannot be received simultaneously. - The reception types for PUR and other existing reception types in RRC_IDLE cannot be received simultaneously. |

In [2], the company give the following text proposal for the new reception type combinations for PUR:

Table 8.2-2a: Downlink "Reception Type" Combinations for BL UEs and UEs in enhanced coverage

|  | PCell |
| --- | --- |
| 1. RRC_IDLE |  |
| 1.1 All UEs | A or G or H |
| 1.2 UEs supporting SC-PTM | A or G or H or (D1 + H1) |
| 1.3 UEs supporting MWUS | A or G or H or L |
| 1.4 UEs supporting PUR | A or G or H or ((O + (N or P)) or Q) |
| 1.5 UEs supporting SC-PTM and PUR | A or G or H or (D1 + H1) or ((O + (N or P)) or Q) |
| 1.6 UEs supporting MWUS and PUR | A or G or H or L or ((O + (N or P)) or Q) |
| 2. RRC_CONNECTED |  |
| 2.1 All UEs | A or ((J or C or E or B) + (I or D or F) + K) or G or H |
| …… |  |

Q5: Companies are invited to provide their comments on the above text proposal for new reception type combinations for PUR, e.g., whether it is agreeable? If not, please the companies give their preference.

| Company | Comments |
| --- | --- |
| ZTE | Agree the text proposal. |
| Sequans | We can agree to this, but this should be aligned with the discussion in [AT109e][314]. In principle, we would prefer adding 1.4 only to not have multiple combinations every time something is added. To not cause too many issues, we can leave ETWS/CMAS + FS2 as a special case, as it also is in the non-BL/NB table (8.2-2). It also looks redundant to us there, but it’s better left alone for now. |

#3 Editor’s note for remarks

At the end of “Table 8.2-2a: Downlink "Reception Type" Combinations for BL UEs and UEs in enhanced coverage”, there has an Editor’s note about whether a formal Note, e.g., “Note 1: the remark applies to all UEs also applies to UEs with specific capabilities” needs to be introduced in order to avoid repeating some common Remarks. Such Editor’s note was added based on the suggestion from the running CR discussion.

In [2], the company gives the example text proposal to optimize the remarks in the table for Downlink "Reception Type" Combinations for eMTC UE.

Table 8.2-2a: Downlink "Reception Type" Combinations for BL UEs and UEs in enhanced coverage

| ……. |  |
| --- | --- |
| 2. RRC_CONNECTED |  |
| 2.1 All UEs | A or ((J or C or E or B) + (I or D or F) + K) or G or H |
|  | Remarks: The combination for Random Access procedure is only required, related to G and H. |
| 2.2 UEs supporting FS2 | A or ((J or C or E or B) + (I or D or F) + I + K) or G or H |
|  | Remarks: For TDD UL/DL configuration 0, two MPDCCHs can be received in the same subframe for UL-SCH in two different uplink subframes, which is only applicable for UEs configured with CE mode A with no repetitions. |
| 2.3 UEs supporting ETWS and CMAS | A or ((J or C or E or B or M) + (I or D or F) + K) or G or H |
|  | Remarks: The combination only applies for non-BL UE in CE. |
| 2.4 ETWS and CMAS UEs supporting FS2 | A or ((J or C or E or B or M) + (I or D or F) + I + K) or G or H |
| NOTE 1: The remark for certain kind UE also applies to the UEs which have the same capabilities. So it only needs to be mentioned when it first appears. |  |

Q6: Companies are invited to provide their comments on the above text proposal for optimizing the remarks in the table for Downlink "Reception Type" Combinations for eMTC UE, e.g., whether it is needed? Or whether it is agreeable? If not, please the companies give their preference.

| Company | Comments |
| --- | --- |
| ZTE | Agree the text proposal. |
| Sequans | Not agree. Originally (and in principle) we wanted such a NOTE, but it goes against the style of the rest of the spec, so prefer to keep it as is. |

Q7: Companies are invited to indicate whether there are any other issues for the running 36.302 CR for eMTC?

| Company | Comments |
| --- | --- |

# Summary

[TBD]

# References

- R2-2001213, Running 36.302 CR for R16 eMTC, 	ZTE Corporation, Sanechips, draftCR, 	Rel-16	, 36.302, 15.2.0, RAN2 #109 e-meeting

- R2-2000501, Addressing Editor's Notes in 36.302 running CR for eMTC, ZTE Corporation, Sanechips, discussion, RAN2 #109 e-meeting