3GPP TSG-RAN WG2 Meeting #109 electronic 	R2-2000xxx

24th February ~6th March 2020

Agenda Item:	6.2.2

Source:	OPPO

Title:	Summary of AI 6.2.2.3 and AI 6.2.2.4

Document for:	Discussion, Decision

# Introduction

This contribution is to summarize the proposed open issues from contributions in NR-U agenda item 6.2.2.3 and 6.2.2.4.

# Discussion

## Issues related to NR-U CG

Issue 1: Auto-retransmission on different CG configuration considering LCP restriction

The first issue is raised by [MTK, R2-2000841], as agreed in NR-U, UE can perform auto-retransmission on another CG configuration than the original one. It’s also agreed in IIoT that logical channel can be configured with CG restrictions, which means some of the logical channel may be configured with LCP restriction which results in not being allowed to be transmission in anther CG.

RAN2#107b agreements:

A single LCH can be map to multiple CG configurations.

Multiple LCHs can be map to a single CG configuration.

In this case, it’s not clear whether UE needs to consider the LCP restriction when performing auto-retransmission on another CG configuration than the original one. [R2-2000841] gives several options and proposes to solve this issue by considering the LCP restriction with the observation that it may increase UE complexity.

- Proposed option:  When retransmitting a TB on a different CG configuration than the original transmission, the UE checks that the LCH restrictions specified in 38.321 5.4.3.1 are not violated for the TB. If the TB includes data from a LCH that is not allowed to be transmitted on the second (retransmitting) CG, the TB is not retransmitted on that CG

Q1: Do you think UE needs to consider LCP restriction when performing auto-retransmission on a CG configuration different from the original one? If, yes, please provide your views on the solution above.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | No | It is not clear to us what is the use case of this scenario. We do not see the benefit for the network to configure different CGs with HARQ process ID sharing and, at the same time, different LCH restrictions on the CGs. Especially if the TBS is the same (which is the only situation in which this cross-CG retransmission is allowed), there seems to be no strong reason to set different LCH restrictions. |
| Huawei | N | the motivation why IIoT adopt this LCP restriction is that certain CG config is for certain services with specific QoS requirement, while this motivation does not hold for NRU. The sole intention to have multiple CG configs to combat the impacts from LBT failure. |
| Samsung | Yes | Ok to consider LCP but there should be no rebuilding |
| Lenovo | No | Not sure what is the motivation for NR-U for having such restrictions, e.g. LCH-to-CG restriction. This was introduced for I-IOT |
| Nokia | No | There is no LCP procedure for retransmission. Up to NW implementation to ensure same processes can only be shared among the CGs that are allowed for the same LCHs |
| LG | No | When multiple CG configurations are share the same TBS and the HARQ processes pool, We think that these have a same LCP restriction. It is up to network implementation. |
| Spreadtrum | No | Considering the channel uncertainty in unlicensed band and latency requirement of services, it is more important to retransmit the TB as soon as possible without considering LCP restriction. |
| OPPO | No | It’s can be up to network to ensure that the CGs with sharing HPID have the same LCP restrictions to be met. |
| InterDigital | No | LCP procedure is performed when constructing a TB for a new transmission. |
| ZTE | No | In IIOT, the intention introducing allowedCG-List is to distinguish traffic types with different reliability requirements. In NR-U, the motivation of introducing multiple configured grants is to provide multiple configured grant occasions without gap. Hence the restriction is not needed for NR-U. |
| Intel | Yes | UE should ensure that the TB size is the same and LCP restriction is not violated for the MAC SDU of the LCHs multiplexed in the TB |
| Futurewei | No | There is no strong use of applying LCH-to-CG restriction for different QoS requirements in NR-U. Network should be able to avoid sharing the same HARQ process across CG configurations with different LCH-to-CG restrictions. |
| MediaTek | Yes | Agree with Samsung that re-building (or re-running LCP) should not be supported. The intention is to check which CGs can be used for autonomous retransmissions other than the CG where the initial transmission was performed. Regarding the LCP restrictions, firstly, there is no reason to limit the LCH-to-CG restrictions to apply to IIoT only but not to NR-U. Secondly, there are already LCP restrictions in Rel-15 that apply to NR-U, such as PUSCH duration and CG Type 1. In principle, there is still a requirement that the CG where the retransmissions are performed should satisfy the LCP restrictions for the data in the TB. One option (already mentioned in Tdoc R2-2000841 and by some companies above) is to rely on the network configuration, making sure that retransmissions are only possible on “compatible” CGs (by using different HARQ PID space or different TBS if necessary). In our view, this will limit the network configuration. Hence, our proposal was to leave it to the UE to check if retransmissions are possible on a different CG without violating the LCP retsrictions. |
| Fujitsu | Yes | Ok to consider LCP. |
| Charter Communications | No | It’s valid to have the same LCP restrictions. However, the attributes of LCP restrictions show that it’d be an unlikely configuration to have e.g. two CGs with different allowedSCS-List or maxPUSCH-Duration. |
| vivo | Yes | UE should ensure the LCHs mapping on the CG not violate the LCP restriction when performing retransmission on another CG configuration, just in case NW configures different LCP restriction for different CG configurations. |
| Qualcomm | No | We should definitely not do rebuilding of the PDU. Agree with Ericsson that different restrictions for different CGs with same TBS is an unlikely configuration. Better to keep the legacy LCP behaviour of checking the restrictions upon PDU generation. |

Summary: 17 companies provided the views on Q1, in which 12 don't support consider LCP for cross CG config retransmission, only 5 companies think LCP restrictions should be taken into account.

- When UE performs auto-retransmission on a different CG configuration with the same TBS, it will not consider the LCP restriction. (12/17)

Issue 2: new timer considering cg-minDFIDelay

In [HUAWEI, R2-2000841], it proposes to introduce a new timer considering minimum duration configured by RRC, i.e., cg-minDFIDelay in below.

- cg-minDFIDelay

Indicates the minimum duration (in unit of symbols) from the ending symbol of the CG-PUSCH to the starting symbol of the DFI carrying HARQ-ACK for that PUSCH. UE assumes HARQ-ACK is valid only for PUSCH transmissions ending before n-cg-DFIDelay-r16, where n is the time corresponding to the beginning of the start symbol of the DFI (see TS 38.213 [13], clause 10.3).

According to the description, UE may consider the DFI received in the duration as invalid for the associated HARQ process. Given this, [R2-2000841] proposes to introduce a new timer, similar as HARQ RTT timer, to make the UE consider the feedback as invalid.

Q2: Do you think MAC needs to introduce a new timer considering cg-minDFIDelay, so that during the timer running, UE would consider DFI as invalid?.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | No | There seems to be no reason to make the cg-minDFIDelay visible at the MAC. It is the PHY layer that is responsible to check the cg-minDFIDelay upon reception of a DFI, before forwarding the ACK/NACK to the MAC. So MAC can just assume that HARQ feedbacks are always valid when passed by PHY. |
| Huawei | Y | DFI will indicate the ACK/NACK for all the UL HARQ processes. If this timer is not used, when UE receiving a DFI, it will always read the ACK/NACK for the corresponding HARQ process. Then, the RAN1 agreement on the restriction that the DFI is only valid for the TB transmitted before cg-minDFIDelay does not hold anymore |
| Samsung | No |  |
| Lenovo | No |  |
| Nokia | No | Already captured in PHY and PHY will only deliver valid ACK/NACK. |
| LG | No | We have a similar view as Ericsson. Without introducing a new timer, the PHY layer can receive HARQ feedback in consideration of CG-minDFIdelay. In other words, this can be transparent in the MAC layer. |
| Spreadtrum | No |  |
| OPPO | No |  |
| InterDigital | No |  |
| ZTE | No | In 38.213 g00, the description for the validation of HARQ information is as follows: For a PUSCH transmission configured by ConfiguredGrantConfig or for activated PUSCH transmissions configured by semiPersistentOnPUSCH, HARQ-ACK information for a transport block of a corresponding HARQ process number is valid if a first symbol of the PDCCH reception is after a last symbol of the PUSCH transmission, or of any repetition of the PUSCH transmission, by a number of symbols provided by cg-minDFIDelay-r16. According to the description, judgment for the validation of HARQ information is up to RAN1. Once the HARQ information is valid, the PHY layer may indicate to the MAC layer. Hence it is not necessary to introduce a new timer for the MAC layer. |
| Intel | No | Our understanding is that the parameter has no visibility to MAC. |
| Futurewei | Yes | HARQ procedure, in general, and the action upon receiving ACK/NAK, in particular, are captured in MAC specs. |
| MediaTek | No | We think that L1 should maintain this timer, and it should not be visible in MAC. L1 should validate a DFI and forward it to MAC only if it is valid. CGRT duration could implicitly include the duration of this timer, so there is no need to delay starting the CGRT. |
| Fujistu | No | Agree with Ericsson. It’s not an issue for MAC. |
| Charter Communications | No | Given this is already captured in PHY, then agree with others that no need for a change in MAC |
| vivo | No | Agree with Ericsson, PHY layer can discard the invalid ACK/NACK rather than forward it to MAC. |
| Qualcomm | No | Agree with others that this is not and should not be visible to MAC. |

Summary: 15 companies out of 17 does not support to introduce a new timer in MAC to consider the DFI as invalid for a certain HARQ process.

- MAC does not introduce a new timer to consider the DFI as invalid. (15/17)

Issue 3: repetitions across CGs with the same TBS?

Given auto-retransmission is supported across different CG configurations, it’s not clear whether repetitions across different CGs with the same TBS can be supported as discussed in [HUAWEI, R2-2000959] and [ERICSSON, R2-2001205].

Q3: Do you support repetitions across different CGs with the same TBS in NR-U?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | Yes | RAN2 has already agreed that the UE can retransmit a TB using a different CG resource than the one used for the initial TB. There seems to be no strong reason to disallow it for repetition. |
| Huawei | No | Should not be supported considering that RAN1 has already concluded on the discussion. If supported, details issues such as RV need to be discussed. |
| Samsung | No | In theory it might be possible, but we do not see the strong motivation to support the repetitions across different CGs. RAN2 should introduce the scenario "only after" having consensus. |
| Lenovo | No | Don’t see a strong need/motivation to introduce a new timer in MAC. |
| Nokia | No | A bundle should only be within the same CG |
| LG | No | Considering the lack of time for the RAN1 decision, this is not needed in this release. |
| Spreadtrum | No |  |
| OPPO | No |  |
| InterDigital | No | Same view as Nokia. |
| ZTE | No | In RAN1 #99, the agreement for repetition is as follows: Agreement: When the UE is configured with repK > 1, repetition of a TB is mapped within a configuration in the case when an UE is configured with multiple active configurations - The UE repeats the TB in the earliest consecutive transmission occasion candidates within the same configuration instead of consecutive slots - The UE may drop repetition transmissions that fall into a subsequent configured period. - The UE terminates the repetitions if an explicit feedback indicating ACK in the DFI is received for the HARQ process. According to the agreement above, the repetition across the different configured grants is not allowed. |
| Intel | No | We think this is an optimization |
| Futurewei | No | RV handling needs RAN1 progress, and there is no strong motivation to initiate this work in RAN2. |
| MediaTek | No | Because there is limited time in Rel-16, and potential complexity this could introduce, it is better not to change the baseline repetition behaviour in Rel-16. |
| Fujistu | No | Repetitions across different CGs seems to introduce additional impact to spec. Compared to retransmitting across different CGs, there is not much advantage if repetition across CGs is used. |
| Charter Communications | Yes | Repetition across multiple CG configurations is more efficient. |
| vivo | Yes | In the last RAN2#108 meeting, we had achieved the agreement that retransmissions can be done on different CG resources as long as they are with the same TBS with the same HARQ process. In our understanding, this does not preclude the repetition case. Therefore, we prefer to support it. |
| Qualcomm | No | This should be a RAN1 decision and too late to discuss this there. Also, gNB can’t detect whether they are retransmissions. |

Summary: 15 companies out of 17 does not support repetitions across multiple CG configurations.

- Repetitions across multiple CG configurations is not supported in this release. (15/17)

Issue 4: prioritization for pending HARQ process

This issue is raised in [OPPO, R2-2000417], in the current running CR, it says:

For configured uplink grants configured with cg-RetransmissionTimer, the UE implementation select an HARQ Process ID among the HARQ process IDs available for the configured grant configuration. The UE shall prioritize retransmissions before initial transmissions.

Since retransmissions could be retransmissions for pending HARQ processes and retransmissions due to CGRT expiry, it’s not clear whether UE should prioritize retransmissions for pending HARQ process, or the retransmissions due to CGRT expiry, when performing HPID selection. The issue scenario is illustrated as follows, e.g., upon selection HPID for the blue CG resources, there are retransmission HARQ process for HPID=0, and retransmissions for pending HPID=1 to n, it’s not clear which HPID the UE should choose.

Q4: Do you agree pending HARQ process should be prioritized over retransmissions due to CGRT expiry?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | No | This seems to be an optimization that further complicates MAC procedures. Additionally, from a technical perspective, there is no guarantee that a TB of a pending HARQ process is always older or newer than a TB for which CGRT expired. So, it is difficult to come up with a prioritization rule. Better to leave it to UE implementation. |
| Huawei | N | This is an optimization. |
| Samsung | No | Leave it to UE implementation |
| Lenovo | No | Agree with Samsung that this should be left to UE implementation |
| Nokia | No | Retransmission of HID 0 should be prioritized. HID 1 from air interface point of view is still a new transmission. And for the figure, there should not be multiple pending HID. When there is some process pending, the UE should not obtain new data for another process. |
| LG | No | There must be a clear reason for prioritization between retransmissions, but there is not enough time to consider this in this release. Thus, it is up to UE implementation. |
| Spreadtrum | No | This is an optimization. We think UE should prioritize the HARQ process according to the time line, i.e. the first retransmission HARQ process has the highest priority regardless of retransmission HARQ type. |
| OPPO | Yes | We may consider to prioritize the HARQ process due to CGRT expiry over retransmissions from pending HARQ process, otherwise, the HARQ process may not perform auto-retransmission before CGT expiry. |
| InterDigital | No | It can be left to UE implementation |
| ZTE | No | For retransmission on configured grant, there are four cases. In addition to these two cases mentioned above, retransmission also includes retransmission upon NACK, or LBT failure on configured grant transmission when CGT is running. For these four cases, HPID selection should be up to UE implementation. |
| Intel | No | It can be left to UE implementation between retransmissions. Between new transmission and retransmission, the RAN2 agreement should be follow in general (or can be reverted and leave it to UE implementation) |
| Futurewei | No | We don’t see clear need of prioritizing one over the other, especially for NR-U; it can be left to UE implementation. |
| MediaTek | No | Leave it to UE implementation. |
| Fujistu | No | Leave it to UE implementation. |
| Charter Communications | No | There are arguments for and against such prioritization. Better to be left to UE implementation. |
| vivo | No | In the previous RAN2 meeting, we had achieved the agreement that it is up to UE implementation how to prioritize among retransmissions on CGs. Therefore, we prefer to leave it to UE implementation. |
| Qualcomm | No | Up to UE implementation |

Summary: 16 companies out of 17 does not support repetitions across multiple CG configurations.

- It’s up to UE implementation on selecting retransmissions, no prioritization is introduced in this release. (16/17)

### Issues from [R2-2001343] related to NR-U CG

Issue in Running CR list: [Lenovo, R2-2000821] HARQ process configuration

In legacy R15, CG configuration contains nrofHARQ-Processes that, together with the formula in 5.4.1, gives the HPIDs to be used by CG. With NR-U, the formula is not valid and thus which HP IDs that are valid for a CG is not defined. There are mainly two options:

- Option1: reuse the nrofHARQ-Processes and HPID-offset (introduced in IIoT), UE is allowed to select HPID given by [harq-procID-offset, harq-procID-offset + 1, …, harq-procID-offset + nrofHARQ-Processes – 1];

- Option2: introduce a new parameter per CG configuration explicitly configures the set of allowed HARQ processes for a CG. For example, as already implemented in 38.331 running CR:

cg-HARQ-Processes-r16                  BIT STRING (SIZE (16)) OPTIONAL,   -- Need R

Q 2.1.1a: Which option above do you prefer for the HARQ process configuration?

| Company | Options | Comments |
| --- | --- | --- |
| Ericsson | 1 | Option 1 has been already introduced in IIOT. Having two different HARQ process ID configurations for IIOT and NR-U does not seem to have a strong technical justification. |
| Huawei | 1 | Any solution can work as long as the space for HARQ pid can be segregated. Option1 has been discussed and agreed upon in the IIoT session and we can reuse it for NRU |
| Samsung | Option 1 | Reuse IIOT |
| Lenovo | Option2 | Option 2 doesn’t require any changes in MAC specification and provides the highest flexibility fo the network. HARQ process handling for configured grants is quite different for IIOT and NR-U. Also the motivation for having multiple active CG configurations is different for IIOT and NR-U, therefore we don’t think it’s a good approach to simply reuse functionality from IIOT in NR-U. |
| Nokia |  | No strong opinion. For option 2, it does not really need 16 bits bitmap, 4 bits for start and 4 bits for end should be enough? So overhead wise they are same. |
| LG | Option 2 | We prefer Option 2. |
| Spreadtrum | Option 2 | Agree with Lenovo |
| OPPO | Option 1 | Both options can achieve the agreements we made in the first session meeting, we prefer to reuse existing parameters instead of introducing new one. |
| InterDigital | Option 2 | Same view as Lenovo |
| ZTE | Option 2 |  |
| Intel | Option 1 | We should align with IIoT and avoid introducing unnecessary new IE. |
| Futurewei | Option 1 | Unless there is significant performance benefit, consistent handling is preferred between IIoT an NR-U (and other features, if there is similar issue). |
| MediaTek | Option 1 | Re-use the same mechanism as IIoT. |
| Fujitsu | Option 1 | Reuse IIOT. |
| Charter Communications | Option 1 |  |
| vivo | 3 | We prefer to use the legacy R15 mechanism. In IIoT, different HARQ PID pools are configured to different CG configuration, in order to avoid HARQ process collision. However, things are different for NR-U, where UE can select HARQ PID by itself to avoid HARQ process collision. Configure multiple CGs with a common HARQ process pool is more efficient than configuring each CG with a separate HARQ process pool. In the latter case, it may happen that the PUSCH of one CG is dropped due to out of HARQ process while another CG has HARQ process which is not used. |
| Qualcomm | Option 2 | Option 2 provides more granularity and flexibility. Given that the offset is used differently in IIoT (HARQ ID selection formula), we don’t need to align. |

Summary: 9 companies out of 17 prefer to reuse the existing parameter, i.e., harq-procID-offset and nrofHARQ-Processes to indicate the HPID to be used for NR-U CG. 6 companies prefer to introduce a new parameter to explicitly indicate the HPID. 1 company is ok for either of them but think using bitmap as the new parameter would cause overhead from signaling point of view. One company prefer to configure a common HARQ process pool for multiple CG configurations.

- With the nrofHARQ-Processes and HPID-offset (introduced in IIoT), UE is allowed to select HPID given by [harq-procID-offset, harq-procID-offset + 1, …, harq-procID-offset + nrofHARQ-Processes – 1]. (9/17)

Issue in Running CR list: [Huawei, R2-2000959] [ERICSSON, R2-2001205] [LG, R2-2001442] stopping CGRT and CGT upon CG (re-)Activation Command reception

In current Running CR, UE stops CGRT and CGT for the “corresponding HARQ process” upon activation/re-activation command is received.

3>	else if PDCCH contents indicate configured grant Type 2 activation:

4>	trigger configured uplink grant confirmation;

4>	store the uplink grant for this Serving Cell and the associated HARQ information as configured uplink grant;

4>	initialise or re-initialise the configured uplink grant for this Serving Cell to start in the associated PUSCH duration and to recur according to rules in clause 5.8.2;

4>	stop the configuredGrantTimer for the corresponding HARQ process, if running;

4>	stop the cg-RetransmissionTimer for the correponding HARQ process, if running.

It’s being captured that in the common issues for running 321 CR (Proposal 7) that the corresponding HARQ process is selected based on UE implementation, which seems to be consensus.

However, (issue related to Proposal 9 in R2-2001343) it’s not clear whether CGRT and CGT associated with other HARQ process(es) (besides the selected one) should stop as well. The concern was if UE does not stop CGRT and CGT for a HARQ process which was used for transmission in this CG configuration, UE may end up performing auto-retransmission even if the TBS changed upon re-activation of this CG. Besides, some other companies think it may also bring issues if UE simply stop all CGRTs and CGTs for this CG configuration, because the HARQ process(es) can be shared by other CG configuration(s) or dynamic scheduling.

All in all, the proposed solutions are summarized as follows:

- Option1: UE stops CGRT and CGT (if running) only for the selected HARQ process;

- Option2: UE stops CGRT(s) and CGT(s) (if running) for the HARQ process(es) configured for this CG configuration but not used by other CG configuration(s), irrespective of the TBS in (re-)activation command;

- Option3: UE stops UE stops CGRT(s) and CGT(s) (if running) for the HARQ process(es) configured for this CG configuration but not used by other CG configuration(s), only when the TBS in (re-)activation command is changed than the previous one;

Q 2.1.1b: which options do you prefer regarding UE behavior upon CG (re-) activation command reception?

| Company | Options | Comments |
| --- | --- | --- |
| Ericsson | 1 | We are fine with option 1, however there is the issue that the NW does not know when the MAC CE confirmation will be transmitted. So in our view option 1 in this question goes together with option 2 in next question. |
| Huawei | 2 | The reason why the timer is stopped in R15 is because the configuration for the CG type 2 has changed. While if the configuration is not used by any CG transmission, there is no need to stop the retransmission-Timer for this HARQ process. |
| Samsung | Option 1 |  |
| Lenovo | Option 1 |  |
| Nokia | 1 | As legacy |
| LG | Option 1 | We prefer Option 1. However, even if Option 1 is agreed, there will still be at risk for transmission delay of the CG confirmation MAC CE when the UE performs autonomous transmission of all HARQ processes. This is related to the following question. |
| Spreadtrum | Option 1 |  |
| OPPO | Option 1 |  |
| InterDigital | Option 1 |  |
| ZTE | Option 1 |  |
| Intel | Option 1 | We are assuming that Option 1 means that UE will stops all CGRT and CGT (if running) only for the selected HARQ processes associated with the CG configuration that has been reactivated. |
| Futurewei | Option 2 | A change of CG configuration impacts the operation of HARQ processes configured to this CG. |
| MediaTek | Option 2 or 3 | We should stop autonomous retransmissions by stopping CGRT, if the TB size has changed. CGT can keep running to allow the network to send retransmission grants. Note that this is different from legacy since autonomous retransmissions were not specified before Rel-16. |
| Fujitsu | Option 1 |  |
| Charter Communications | Option 1 |  |
| vivo | 1 | Option1 is sufficient. For the auto retransmission on the re-activated CG with changed TBS, the NW can schedule retransmission with the dynamic grant for the HARQ processes associated with the reactivated CG, as a way to stop the auto-retransmission. |
| Qualcomm | Option 1 | The gNB can use dynamic grants for retransmissions if TBS changes |

Summary: 14 companies out of 17 prefer to option 1, i.e., UE stops the CGRT and CGT for the selected HARQ process upon reception of PDCCH indicating CG type 2 (re-)activation. It should be noted that as agreed in the first session of this meeting, it’s up to UE implementation to select the corresponding HARQ process. Thus, option 1 means when CG type 2 (re-)activation DCI is received, UE selects a HARQ process, and stops the CGRT and CGT associated with the selected HARQ process, if running.

- When CG type 2 (re-)activation DCI is received, UE implementation selects a HARQ process (as agreed), and stops the CGRT and CGT associated with the selected HARQ process, if running. (14/17)

Another issue related to confirmation MAC CE transmission as raised by Eri07 in [R2-2001343] which is also discussed in R2-2001442. The issue is  the release may be delayed (compared to legacy where first available CG or DG can be used to transmit the confirmation MAC CE) if the UE is busy with autonomous retransmissions for CGs and no dynamic grant is available.

There are in general two views on this potential issue:

- Option 1: No special handling for confirmation MAC CE transmission upon deactivation/(re-)activation;

- Option 2: UE prioritizes initial transmission over retransmissions when UE have triggered and not cancelled confirmation MAC CE upon deactivation/(re-)activation;

- Option 3: UE flushes the HARQ buffer of the selected HARQ process upon deactivation/(re-)activation.

Q 2.1.1c: which options do you prefer regarding confirmation MAC CE transmission upon deactivation/(re-)activation?

| Company | Options | Comments |
| --- | --- | --- |
| Ericsson | 2 | Without option 2, the UE may not transmit the MAC CE at the first PUSCH occasion, and it may instead prioritize retransmissions. This may lead to ambiguity at the gNB side which will not know if MAC CE was not sent because of DCI command not received, or because the UE prioritized retransmissions. |
| Huawei | 1 | The issue can be handled by the network implementation. The proposed option2 seems to be an optimization. |
| Samsung | Option 1 |  |
| Lenovo | Option1 |  |
| Nokia | 1 |  |
| LG | Option 3 | Regarding Ericsson’s comment, we have some sympathy. In order to avoid delay of the CG confirmation MAC CE, we have another option (R2-2001442). According to current running MAC CR, if the UE flushes HARQ buffer, the UE can perform new transmission. In other words, there is no ambiguity at the gNB when the UE transmits the CG confirmation MAC CE after UE flushes the HARQ buffer of the selected HARQ process regardless of whether CGT/CGRT is running. On the other hand, Opton2 should revisit the previous agreement that “the UE shall prioritize retransmissions before initial transmissions”. We want to keep this agreement. |
| Spreadtrum | Option 1 | This can be up to gNB implementation. |
| OPPO | Option 1 | No special handling |
| InterDigital | Option 1 |  |
| ZTE | Option 1 |  |
| Intel | Option 1 |  |
| Futurewei | Option 1 |  |
| MediaTek | Option 1 |  |
| Fujistu | Option 1 |  |
| Charter Communications | Option 1 |  |
| vivo | Option1 | It seems Option2/3 revert the achieved agreement. Hence, we prefer Option1. To solve the issue raised by Ericsson, we think we can leave it to NW implementation, e.g., a dynamic grant can be scheduled for the confirmation MAC CE transmission. |
| Qualcomm | Option 1 |  |

Summary: 15 companies out of 17 does not want to introduce special handling of the transmission delay for confirmation MAC CE due to the previous agreement that UE will prioritize the retransmission over new transmission.

- No special handling of the transmission delay for confirmation MAC CE due to the previous agreement that UE prioritizes the retransmission over new transmission. (15/17)

Two issues related to RV selection in [R2-2001343]:

The first one is what RV the UE shall use for autonomous CG transmission. During the discussions for running CR, different views are observed, one is that UE can select the RV based on UE implementation similar to LAA case. The other view that UE can follow the way defined for CG bundling case, i.e., a defined RV pattern. The options are categorized as follows:

- Option 1: The UE uses RV zero for the initial transmission, and leave it to UE implementation for selection the RV for auto-retransmission as for LAA;

- Option 2: The Rel-15 setting of RV in MAC 5.4.2.1: “For each transmission within a bundle of the configured uplink grant, the sequence of redundancy versions is determined according to clause 6.1.2.3 of TS 38.214 [7].” Shall be reused when cg-RetransmissionTimer is configured

Q 2.1.1d: which options do you prefer regarding RV selection for NR-U CG?

| Company | Options | Comments |
| --- | --- | --- |
| Ericsson | 1 | Same as feLAA |
| Huawei | 1 | Can be chosen by UE implementation. UCI includes RV |
| Samsung | Option 1 |  |
| Lenovo | Option 1 | Since UCI indicates RV there is no problem |
| Nokia | 1 |  |
| LG | Option1 | Considering the complexity, option 1 is sufficient. |
| Spreadtrum | Option1 |  |
| OPPO | Option1 |  |
| InterDigital | Opt. 1 | This can be left UE implementation for now. |
| ZTE | Opt 1 |  |
| Intel | Option 1 |  |
| Futurewei | Option 1 |  |
| MediaTek | Option 1 |  |
| Fujitsu | Option 1 | Same as eLAA. |
| Charter Communications | Option 1 |  |
| vivo | Option1 | Same as feLAA. |
| Qualcomm | Option 1 |  |

Summary: all companies agree the UE uses RV zero for the initial transmission, and leave it to UE implementation for selection the RV for auto-retransmission as for LAA.

- The UE uses RV zero for the initial transmission, and leave it to UE implementation for selection the RV for auto-retransmission as for feLAA. (17/17)

The other one is, as agreed in last meeting, when a single DCI is used to schedule multiple PUSCH, the UE may transmit a new TB on any HARQ process in the grant that have the same TBS and the NDIs indicate new transmissions.

Agreements

1	For multi-TTI UL grant, UE is allowed to map generated TB(s) internally to different HARQ processes in case of LBT failure(s), i.e. UE may transmit a TB pending for transmission in a HARQ process due to a failed LBT in a different HARQ process being associated with a PUSCH for which LBT was successful.  FFS how it is captured in the spec

Given the agreements, it seems straightforward that the RV should also be the same while UE do the moving TB due to LBT failure. In other words, we need to add some clarification on the spec i.e., “UE may transmit a new TB on any HARQ process in the grants that have the same TBS, the same RV and the NDIs indicate new transmission”, which seems a way forward.

But still, some other companies think this feature may casue some complexity to either UE or network side thus prefer to revert the original agreement. In general, two options are given:

Q 2.1.1d: Do you agree the RV issue can be solved by adding the highlighted “the same RV” for the moving TB agreement?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | No | That is more an optimization in our view, so reverting the original agreement is fine to simplify MAC specification. |
| Huawei | Yes | RV is indicated by the DCI of the multi-TTI UL grant. If the RV of the transmission does not match with the DCI indicate, decoding in the network side will fail. |
| Samsung | No |  |
| Lenovo | Yes | There is no technical reason to revert the previous agreement. RV issue can be simply resolved. |
| Nokia |  | The two options seem to be missing? Ok to revert since this is mainly for URLLC which will be considered in Rel-17. |
| LG | No | Considering complexity, we prefer to revert this agreement. |
| Spreadtrum | Yes | Agree with HW. |
| OPPO | Yes | If we don't revert the agreement, we are ok to add the highlighted part. |
| InterDigital | Yes | Adding this highlighted qualifier addresses the issue without having to revert agreements. |
| ZTE | No | We agree with Ericsson. |
| Intel | No | No further optimisation is done (i.e. revert the previous agreement) and the UL grant is seen as separate UL grant to the UE. |
| Futurewei | Yes | There is no need to revert an agreement, if a simple addition can fix the problem. |
| MediaTek | No | We are okay to revert the agreement and postpone it to Rel-17. |
| Fujistu | No | RV of the PUSCH for which LBT is successful is no need to be the same as that used for the pending TB. |
| Charter Communications | Yes | Given adding the highlighted text resolves this issue, suggest to keep the agreement. |
| vivo | No | We are fine to revert the achieved agreement as it helps to reduce the UE complexity (i.e. we do not need to capture anything specific for multi-TTI UL grant in MAC spec). |
| Qualcomm | Maybe | If we don’t revert the agreement, agree on the need to add the RV. Reverting is also fine due to the complexity of this agreement. Another option is to have a simple note that the processing of the multi-TTI grants is up to the UE implementation (same as done for CA grants since LTE). |

Summary: 7 companies out 17 agree the change to clarify the RV when UE do the moving TB. However, there are also 7 companies explicitly express the previous agreement regarding mapping generated TB internally to different HARQ processes should be reverted. 2 companies (Samsung, Fujistu) seems does not agree the change, i.e., to add “the same RV”. 1 company (Qualcom) suggests to leave it as UE implementation. Given there is no majority on the proposed change, we propose:

- RAN2 try to agree “For multi-TTI UL grant, UE is allowed to map generated TB(s) internally to different HARQ processes in case of LBT failure(s), i.e. UE may transmit a new TB on any HARQ process in the grants that have the same TBS, the same RV and the NDIs indicate new transmission.”  (7/17), otherwise we revert the agreement (7/17).

## Issues related to CAPC selections

In the current TS 38.300 running CR, those agreements were implemented as follows:

For uplink transmissions on configured grants, the gNB configures the UE for the CAPC to be used for SRB2 and DRBs. The padding BSR and recommended bit rate MAC CEs use the lowest priority CAPC (i.e. highest number in Table 5.X.2-1) while other MAC CEs use the highest priority CAPC. For uplink transmissions on configured grants, the UE shall select the CAPC as follows:

- highest priority CAPC of MAC CE(s) if only MAC CE(s) are included;

- CAPC of DCCH if DCCH SDU is included;

- lowest priority CAPC of the logical channel(s) with MAC SDU multiplexed in this MAC PDU otherwise.

Editor’s Note: It is FFS how the UE selects the CAPC when the UE has a MAC PDU including the MAC SDUs from both SRB1 and SRB2.

Issue 5: CAPC selection for a MAC PDU containing both data from DTCHs and MAC CEs?

As discussed in [Ericsson, R2-2001206] thinks it does not make sense UE still select the lowest priority CAPC when the lowest priority MAC CE is multiplexed in the MAC PDU, thus it proposes to exclude the lowest priority MAC CE when select CAPC for MAC PDU containing both data from DTCHs and MAC CEs.

However, It seems it's already clear in the TP that “lowest priority CAPC of the logical channel(s) with MAC SDU multiplexed in this MAC PDU otherwise” from the rapporteur’s point of view. Anyway, it would be good to confirm the understanding:

Q5: Do you think CAPC selection for a MAC PDU containing both data from DTCHs and MAC CEs exclude the LCHs associated with the low priority MAC CEs?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | Yes |  |
| Huawei | Yes |  |
| Samsung | Yes |  |
| Lenovo | Yes |  |
| Nokia | Yes |  |
| Spreadtrum | Yes | It is necessary to confirm the understanding as it is not clear in RAN2# 108 agreements. |
| OPPO | Yes | Actually, it’s already capture in the current running CR, i.e., the following: - lowest priority CAPC of the logical channel(s) with MAC SDU multiplexed in this MAC PDU otherwise. We don't need to further capture something in the spec with the Q5 answering yes. |
| InterDigital | Yes |  |
| ZTE | Yes |  |
| Intel | Yes |  |
| Futurewei | Yes |  |
| MediaTek | Yes |  |
| Fujistu | Yes |  |
| Charter Communications | Yes |  |
| vivo | Yes |  |
| Qualcomm | Yes | It is already clear in stage-2. |

Summary: all companies confirm “CAPC selection for a MAC PDU containing both data from DTCHs and MAC CEs exclude the LCHs associated with the low priority MAC CEs.”

- As already captured in the 38.300 running CR for NR-U, CAPC selection for a MAC PDU containing both data from DTCHs and MAC CEs exclude the LCHs associated with the low priority MAC CEs. (16/16)

Issue 6: UE CAPC selection applied to MSGA PUSCH, MSG3 PUSCH and other UL transmission case when CAT4 is indicated but CAPC is not 14ignalle explicitly

As indicated in [Intel, R2-2001094], RAN1 has the following agreements:

Agreement:

At least for LBE operation:

- For 15ignalled15 of LBT type & CP extension for both Fallback DL assignment and Fallback UL Grant, the following table is used:

| LBT Type | CP extension |
| --- | --- |
| Cat1 16 µs | C2*symbol length – 16 us – TA |
| Cat2 25 µs | C3*symbol length – 25 us – TA |
| Cat2 25 µs | C1*symbol length – 25 us |
| Cat4 | 0 |

  - CAPC is not indicated explicitly:

    - For the UL grants

      - The UE assumes CAPC=4 was used by the gNB to acquire the CO,

      - For UE initiated COTs (Cat4 case) the UE may select the CAPC by itself.

        - Note: The mapping between priority classes and traffic classes follows the same mechanism as defined for UL CG transmissions.

According to RAN1, there may be some cases that CAPC is not explicitly indicated, e.g., MSGA, MSG3 PUSCH etc. Thus it’s proposed to capture in the following TP:

- Option proposed: Extend the UE CAPC selection to MsgA PUSCH transmission, Msg3 PUSCH transmission and other UL transmission case where CAT4 is indicated but CAPC is not signaled explicitly:

TP proposed: For uplink transmissions in RRC_CONNECTEDon configured grants, the gNB configures the UE for the CAPC to be used for SRB2 and DRBs. For uplink transmissions on configured grants, Msg3 transmission (UE initiated COT case), MsgA PUSCH transmission and other UL transmission case where CAT4 is indicated but CAPC is not signaled explicitly, the UE shall select the CAPC of DCCH if DCCH SDU is transmitted and otherwise the lowest priority CAPC (i.e. highest number in Table 5.X.2-1) of the logical channel(s) with MAC SDU and of the MAC CE(s) multiplexed in this MAC PDU.

Q6: Do you agree to extend the UE CAPC selection to MsgA PUSCH transmission, Msg3 PUSCH transmission and other UL transmission case where CAT4 is indicated but CAPC is not signaled explicitly?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | Yes |  |
| Huawei | Yes |  |
| Samsung | Yes |  |
| Lenovo | Yes |  |
| Nokia | Yes |  |
| Spreadtrum | Yes |  |
| OPPO | Yes |  |
| InterDigital | Yes |  |
| ZTE | Yes | It seems similar discussion is also happening in email discussion 501 (Q 2.1.2?). Do we need an LS to RAN1 if we capture this in stage 2? |
| Intel | Yes |  |
| Futurewei | Yes |  |
| MediaTek | Yes |  |
| Fujistu | Yes |  |
| Charter Communications | Yes |  |
| vivo | Yes |  |

Summary: all companies agree to extend the UE CAPC selection to MsgA PUSCH transmission, Msg3 PUSCH transmission and other UL transmission case where CAT4 is indicated but CAPC is not signaled explicitly. ZTE mentions whether to send an LS to inform RAN1 about this decision, which seems ok.

- Extend the UE CAPC selection to MsgA PUSCH transmission, Msg3 PUSCH transmission and other UL transmission case where CAT4 is indicated but CAPC is not signaled explicitly. (15/15)

The following issues related to CAPC selection are already discussed in Running CR issue list.

Issue in Running CR list: [vivo, R2-2000149], [HUAWEI, R2-2000176], [NEC, R2-2001108] CAPC for a MAC PDU containing both SRB1 and SRB2

It’s not clear what’s the CAPC for a MAC PDU containing both SRB1 and SRB2, some proposals as follows:

- Option1: the CAPC of SRB1 is selected (i.e. the highest priority);

- Option2: the CAPC of SRB2 is selected

- Option3: UE implementation

Wait the progress of the following proposal in Running CR issue list.

The agreement from the session slot 1 is as follows:

- For UL, if SDUs from multiple DCCHs (i.e. SRB1 and SRB2) are multiplexed in a MAC PDU, the CAPC of the MAC PDU is the highest priority CAPC of the DCCHs.

Thus, the above option 1 is selected.

## Issues related to PHR

For the PHR reporting issue in NR-U, RAN1 sent back a LS indicated that RAN1 will not develop solutions on this issue and it’s up to RAN2 to decide whether it should be solved or not.

Based on the contributions as summarized below:

| R2-2000838, Lenovo | Proposal1: UE reports a virtual PHR for a serving cell for cases when the PHR MAC CE is transmitted on a CG PUSCH on an unlicensed cell. For the case that a serving cell is configured with two UL carriers, e.g. SUL/NUL, UE reports a virtual PHR for a predefined PHR type, e.g. PHR type 1. |
| --- | --- |
| R2-2000842, MTK | The PHR includes a timestamp (SFN and slot) of the time that the reported power headroom corresponds to. |
| R2-2000960, Huawei | Proposal 1: it is suggested RAN2 to develop a solution for PH confusion problem on NR-U CG.  Option 1: PHR MAC CE is not allowed to be transmitted on any NR-U CG.  Option 2: a timestamp should be included in each CG transmission.  Option 3: PH for both UL carriers are reported in the MAC CE. |
| R2-2001204, Ericsson | Proposal 1 The ambiguity issues for PHR time reference and PHR type due to LBT failures are not addressed in Rel-16. |

Given different companies have divergent views on this issue, one suggestion could be to ask companies views on whether we address this issue in R16:

Q7: Do you think we should address the ambiguity issues for PHR time reference and PHR type due to LBT failure in Rel-16?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | No | This issue has been already discussed in LAA and not considered a serious issue, since it will likely not occur persistently. Therefore, this is an issue that the gNB implementation can definitely handle and we do not expect serious link adaptation degradation. Additionally, the solutions proposed in the contributions above are anyhow either suboptimal, e.g. always reporting virtual PHR, or lead to large overhead and spec changes, e.g. reporting time stamp in PHR. Our proposal is to handle this issue as for the BSR case, i.e. it is up to UE implementation how to handle the PHR content. |
| Huawei | Yes | In the previous meeting, we have agreed that the new issue is indeed a problem and hence we sent an LS to RAN1. The new issue is that when SUL is configured, the UE may be configured with PUSCH on one UL carrier and SRS switching in another. In this case, leave alone the exact timing for PHR generation cannot be determined, even the type of the PHR cannot be knows. We think the solution that the UE include the timestamp for the PHR MAC CE solve the problem once and for all, but we are also open to the other solutions. |
| Samsung | No |  |
| Lenovo | Yes | If the ambiguity is not addressed then gNB can only ignore a PHR sent on a configured grant. This will diminish the benefit of the general PHR functionality for NR-U. It should be noted that also RAN1 concluded that a solution is required for the identified problems. |
| Nokia | No |  |
| LG | No | We think this is an unnecessary optimization. |
| OPPO | No | We think it can be postpone to future release. |
| InterDigital | No |  |
| ZTE | No | This issue was discussed in the last meeting, the below agreement was reached: - For BSR/PHR transmitted on configured grant, it is up to the implementation of the UE to handle the content of BSR/PHR. According to the agreement, for PHR transmitted on configured grant, it is up to UE implementation. In addition, considering the ambiguity issues mentioned are minor and the time is limited, we think this ambiguity issues do not need to be addressed in Rel-16. |
| Intel | No | If a solution is needed, it should be a simple one like asking the network to ignore it in the PHR, without regenerating the PHR or sending PHR MAC CE on dynamic grant only. |
| Futurewei | Yes | The ambiguity issue should be addressed. |
| MediaTek | Yes | CG will be used in NR-U extensively. Therefore losing the ability to report PH with CGs should be regarded as a critical issue and should be avoided. |
| Charter Communications | No | RAN2 agreed to leave this issue to UE implementation. |
| vivo | No | To completely solve this issue, a timestamp is needed. However, we don’t think RAN2 has enough time to specify this solution. |
| Qualcomm | No | This was extensively discussed during feLAA and it was decided to leave to the UE implementation. It is also too late do discuss enhancements. |

Q8: If yes to Q7, what’s your preference on the solutions proposed?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei | Yes | Include a timing information or report PH for both UL carriers. |
| Lenovo | Yes | UE reports a virtual PHR for a serving cell for cases when the PHR MAC CE is transmitted on a CG PUSCH on an unlicensed cell. For the case that a serving cell is configured with two UL carriers, e.g. SUL/NUL, UE reports a virtual PHR for a predefined PHR type, e.g. PHR type 1. |
| Futurewei | Yes | Including timing information is straightforward and provide accurate information. |
| MediaTek | Yes | If only virtual PHR is provided to the network, the accuracy of the PHR is compromised. Real PHR should always be reported where available, which is covered by proposal in Tdoc R2-2000842. A timestamp needs to be introduced in the PHR MAC CE. |

Summary: 11 companies out of 15 think we should not address the PHR ambiguity issue and leave it to UE implementation. 4 companies believe it’s a critical issue thus should be addressed, however among the 4 companies, the solutions seems a bit divergent.

- RAN2 postpones the PHR ambiguity issue to future releases. (11/15)

## Other Issues

| R2-2000535, Samsung | Proposal: As a working assumption, the new features introduced for the NR-U are applicable to unlicensed carriers only. As a result, for each RRC configuration for the NR-U feature, its applicability should be clearly specified e.g. in the field description or by the conditional presence. |
| --- | --- |
| R2-2000669, Nokia | Proposal: We do not define abandoning any control plane sending in lower layers and inform this to RAN4 with a LS |

Q9: Regarding the NR-U feature applying to licensed carrier, do you agree NR-U features are applicable to unlicensed carrier only?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | No | First, what are NR-U features? In the NR-U WI we specify a set of features which are beneficial in unlicensed spectrum (or in some cases even critical for unlicensed operation). However, that does not mean that no features added within the NR-U WI are also beneficial in licensed spectrum. Therefore, we don’t agree with a blanket-statement saying that "NR-U features" (whatever that actually means) are only applicable to unlicensed. We think we need to decide on a case-by-case basis whether we shall add wording in the spec which prohibits use of it in licensed spectrum. |
| Huawei | No | If applicable to the other WIs, each feature should be examined one by one |
| Samsung | Yes |  |
| Lenovo | Yes/No | Should be decided on a case-by-case basis |
| Nokia | Yes | Whether some applicable to other WIs should be separately discussed in other WIs. |
| LG | Yes |  |
| Spreadtrum | Yes | At least for R16. |
| OPPO | Yes | NR-U features are only applied for unlicensed band, as a baseline. |
| InterDigital | No | Not sure what the NR-U feature means |
| ZTE | Yes | We may make decision on a case by case basis. If a feature is not useful for NR licensed carrier (e.g. anything related to LBT and LBT failure related enhancements etc), there is no necessity in also supporting this on licensed carrier. Then for other features, which may benefit licensed carrier, we support. |
| Intel | Yes, but | As baseline, it should just be applied to unlicensed unless specified otherwise in the field description for those configurations. If some feature is deemed useful to licensed, we can relax the restriction via proper contribution with clear justification and motivation. |
| Futurewei | Not blindly | Should be checked case by case. |
| MediaTek | Yes, but | In principle, agree with Samsung that which RRC parameters apply only to NR-U should be discussed. However this should be decided on case-by-case basis. For example, CAPC applies only to NR-U, but autonomous retransmissions on configured grants (CGRT) might apply to IIoT as well. |
| Fujistu | No |  |
| Charter Communications | No | No specific NR-U features listed; should be considered case by case. |
| vivo | Yes | It is clear all the features introduced in this WI is originally intended for NR-U. We cannot assume all the features introduced for NRU are naturally inherited in licensed spectrum. Hence, we prefer to make it clear which features are used for NR-U and decide on a case-by-case basis whether an NR-U feature can be applied to licensed spectrum. |
| Qualcomm |  | There will be a wider scope of discussion on this led by RAN1. |

Summary: given the replies, 10 companies think the NR-U features should be discussed case by case to conclude whether it can be applied to licensed operation.

- As a baseline, NR-U features (e.g., CAPC, autonomous retransmission, additional paging occasions, etc) are only applied to unlicensed operation. Case-by-case discussion on whether new features introduced in NR-U can be applied to licensed operation. (10/15)

Regarding the FFS in RAN4 as bellow:

- UL LBT and Measurement Reporting

  - Event triggered reporting:

    - FFS: UE shall abandon the measurement report when the extension UL, i.e., the time period from the time of the first reporting attempt failed due to UL CCA failure until the time of the successful reporting attempt, exceeds UL,max

    - The extension is

      - Option 1: determined by RAN1/RAN2 specifications

      - Option 2: a pre-defined value

    - measurement reporting delay can be further extended to account for DL LBT failures, in the same way as in the measurement period requirements

Q10: Do you agree we should send LS to RAN4 informing them not introducing abandoning procedure?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | Yes | From RAN2 perspective, aborting the measurement report procedure would lead to large spec impact, and it does not seem to be well justified either. |
| Huawei | No | RAN4 is still discussing this and we prefer to wait for the RAN progress |
| Samsung | Yes |  |
| Lenovo | Yes |  |
| Nokia | Yes |  |
| OPPO | No | We can wait for RAN4 progress |
| InterDigital | Yes |  |
| ZTE | Yes |  |
| Intel | Yes | Agree with the LS that it is difficult to retract a measurement report in user plane. |
| Futurewei | No | We should wait for RAN4 to progress; LS can be sent if RAN4 asks for comments/inputs. |
| MediaTek | Yes |  |
| Fujistu | Yes |  |
| Charter Communications | Yes |  |
| vivo | Yes | Agree with Ericsson. |
| Qualcomm | Yes | Note that this is also covered in the CP discussion. |

Summary: 12 companies out of 15 agree to send an LS to RAN4 informing them not introducing abandoning procedure.

- Send an LS to RAN4 not introducing abandoning measurement report procedure. (12/15)

## Proposals may be considered as optimization

| #11 | R2-2000962, Huawei | Proposal 1: UE should switch to a SS group with denser PDCCH occasion when BWP is switched due to initiation of Random Access procedure or consistent UL LBT failure. |
| --- | --- | --- |
| #12 | R2-2000173, Xiaomi | Proposal: RAN2 to agree that we should allow SR transmission on the PUCCH resource colliding with the UL-SCH resource for which LBT fails. |
| #13 | R2-2001450, LG | Proposal. RAN2 enhance the DL opportunity based on the channel busy level dynamically measured by both UE and gNB. |
| #14 | R2-2001451, LG | Proposal. RAN2 only consider the following cases with single active BWP for a carrier in Rel-16 NR-U WI: - UE transmits data only if the CCA is successful at UE for the whole BWP (Alt 1 in RAN1 agreement) - UE transmits data on the only one of sub-bands which the CCA is successful at UE (Method 5 of Alt 2) |

It would be good to gather the companies’ views that whether the above proposals are critical which needs to be addressed in Rel-16 or considered as further optimization thus not be addressed in Rel-16.

Q11: Do you think the above proposals are for further optimization thus will not be addressed in Rel-16? If you answer “No”, please indicate which one in the above table should be further discussed and indicate why.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | Yes |  |
| Huawei | No | #11 The reason why the proposal is necessary is that the SS group is configured per BWP, as shown per 38331 running CR SearchSpace ::= SEQUENCE { ====omitted==== searchSpaceType CHOICE { ===omitted===== ue-Specific SEQUENCE { dci-Formats ENUMERATED {formats0-0-And-1-0, formats0-1-And-1-1}, …, [[ searchSpaceGroupIdList-r16 SEQUENCE (SIZE (1.. 2)) OF INTEGER (0..1) OPTIONAL, -- Need R freqMonitorLocations-r16 BIT STRING (SIZE (5))) OPTIONAL -- Need R ]] } } While RAN1 was designing this feature, it does not consider the RAN2 mechanism for autonomous BWP switching, including with RACH and UL LBT failure. If such BWP switching happens, the PDCCH monitoring between the UE and the network needs to be aligned, while the current MAC spec is missing in this part. Currently, the PDCCH search space group switching mechanism is defined by the following UE behaviour, defined in RAN1: - Based on an indication in DCI format 2-0 - Expiry of a time - Expiry of the COT indicated by DCI 2-0 RAN2 should define the default SS group when BWP is switched autonomously |
| Samsung | Yes |  |
| Lenovo | Yes |  |
| Nokia | Yes |  |
| OPPO | Yes at least for #12,#13,#14 | For SS group, since common search space does not belong to any SS group, when UE performs BWP switching by itself, it should be ok UE only monitors the common search space for RAR. Regarding Huawei’s comments whether need to define a default SS group, we think it’s up to RAN1 discussion. Even if there is no default SS group, I guess network can still configure a search space set belonging to both SS group which are now feasible according to RAN1, so it seems there is no issue. But we are ok to double check with RAN1 |
| InterDigital | Yes |  |
| ZTE | Yes |  |
| Intel | Yes |  |
| Futurewei | Yes for 12, 13, 14. | For #11, an LS may be sent to RAN1 to check their understanding. |
| MediaTek | Yes | We should avoid optimisations in Rel-16 due to limited time. |
| Fujitsu | Yes |  |
| Charter Communications | Yes | Due to limited time, such optimization should be left for future releases. |
| vivo | Yes |  |
| Qualcomm | Yes | They are all within RAN1 domain. |

Summary: all companies agree #12, #13 and #14 are for optimization thus will not be addressed in this released. 3 out 15 think #11 may need further check to RAN1.

- Considering “UE should switch to a SS group with denser PDCCH occasion when BWP is switched due to initiation of Random Access procedure or consistent UL LBT failure” as further optimization thus will not be addressed in this release. (11/15)

- Considering “allow SR transmission on the PUCCH resource colliding with the UL-SCH resource for which LBT fails” as further optimization thus will not be addressed in this release. (15/15)

- Considering “enhance the DL opportunity based on the channel busy level dynamically measured by both UE and gNB” as further optimization thus will not be addressed in this release. (15/15)

- Considering MAC impacts (if any) of multiple CCAs in wideband larger than 20MHz as further optimization thus will not be addressed in this release. (15/15)

# Conclusion

Based on the discussion in section 2 we propose the following:

Proposals with full consensus (aim to agree to those over email) with >80% ratio supportive and thus can be agreeable by Email:

Proposal 2	MAC does not introduce a new timer to consider the DFI as invalid. (15/17)

Proposal 3	Repetitions across multiple CG configurations is not supported in this release. (15/17)

Proposal 4	It’s up to UE implementation on selecting retransmissions, no prioritization is introduced in this release. (16/17)

Proposal 6	When CG type 2 (re-)activation DCI is received, UE implementation selects a HARQ process (as agreed), and stops the CGRT and CGT associated with the selected HARQ process, if running. (14/17)

Proposal 7	No special handling of the transmission delay for confirmation MAC CE due to the previous agreement that UE prioritizes the retransmission over new transmission. (15/17)

Proposal 8	The UE uses RV zero for the initial transmission, and leave it to UE implementation for selection the RV for auto-retransmission as for feLAA. (17/17)

Proposal 10	As already captured in the 38.300 running CR for NR-U, CAPC selection for a MAC PDU containing both data from DTCHs and MAC CEs exclude the LCHs associated with the low priority MAC CEs. (16/16)

Proposal 11	Extend the UE CAPC selection to MsgA PUSCH transmission, Msg3 PUSCH transmission and other UL transmission case where CAT4 is indicated but CAPC is not signaled explicitly. (15/15)

Proposals with almost full consensus and easy to agree and suggest to agree by Email

Proposal 1	When UE performs auto-retransmission on a different CG configuration with the same TBS, it will not consider the LCP restriction. (12/17)

Proposal 12	RAN2 postpones the PHR ambiguity issue to future releases. (11/15)

Proposal 13	As a baseline, NR-U features (e.g., CAPC, autonomous retransmission, additional paging occasions, etc) are only applied to unlicensed operation. Case-by-case discussion on whether new features introduced in NR-U can be applied to licensed operation. (10/15)

Proposal 14	Send an LS to RAN4 not introducing abandoning measurement report procedure. (12/15)

Proposal 15	Considering “UE should switch to a SS group with denser PDCCH occasion when BWP is switched due to initiation of Random Access procedure or consistent UL LBT failure” as further optimization thus will not be addressed in this release. (11/15)

To be agreed over email that these issues are not pursued in rel-16:

Proposal 16	Considering “allow SR transmission on the PUCCH resource colliding with the UL-SCH resource for which LBT fails” as further optimization thus will not be addressed in this release. (15/15)

Proposal 17	Considering “enhance the DL opportunity based on the channel busy level dynamically measured by both UE and gNB” as further optimization thus will not be addressed in this release. (15/15)

Proposal 18	Considering MAC impacts (if any) of multiple CCAs in wideband larger than 20MHz as further optimization thus will not be addressed in this release. (15/15)

Proposals to be discusssed during online:

Proposal 5	With the nrofHARQ-Processes and HPID-offset (introduced in IIoT), UE is allowed to select HPID given by [harq-procID-offset, harq-procID-offset + 1, …, harq-procID-offset + nrofHARQ-Processes – 1]. (9/17)

Proposals postponed to next meeting:

Proposal 9	RAN2 try to agree “For multi-TTI UL grant, UE is allowed to map generated TB(s) internally to different HARQ processes in case of LBT failure(s), i.e. UE may transmit a new TB on any HARQ process in the grants that have the same TBS, the same RV and the NDIs indicate new transmission.”  (7/17), otherwise we revert the agreement (7/17).

# Reference

#### 6.2.2.3	Configured grant operation

Including HARQ aspects, configuration aspects, multiple active configured grants, and conflicts between dynamic and configured grants (NR-U specific).

ONLY NEW CRITICAL OPEN Issues that are not identified in email discussions.  Contributions should NOT discuss open issues in the email discussion

R2-2000417	Remaining issues on NR-U configured grant	OPPO	discussion	Rel-16	NR_unlic-Core	Late   Q4

Proposal 1	The transmission(es) with the associated HARQ process(es) considered as pending should be prioritized over new transmission.

Proposal 2	UE prioritizes retransmission on CG due to CGRT over those transmission(es) (if any) with the associated HARQ process(es) considered as pending.

R2-2000821	HARQ process configuration for configured grants	Lenovo, Motorola Mobility	discussion	Rel-16	NR_unlic-Core

Proposal 1: gNB can configure individual HARQ processes for each CG and thus allowing either shared or separate HARQ processes among CGs.

Proposal 2: introduce a new parameter in the IE ConfiguredGrantConfig, e.g. IE cg-HARQ-Processes-r16 as proposed by the rapporteur, which explicitly indicates which HARQ process IDs are allowed to be selected by the UE for an uplink CG transmission.

R2-2000841	Issues on retransmissions across different configured grant configurations	MediaTek Inc.	discussion	Rel-16	NR_unlic-Core

Proposal 1: When retransmitting a TB on a different CG configuration than the original transmission, the UE checks that the LCH restrictions specified in 38.321 5.4.3.1 are not violated for the TB. If the TB includes data from a LCH that is not allowed to be transmitted on the second (retransmitting) CG, the TB is not retransmitted on that CG.

Proposal 2: RAN2 to adopt the test proposal for 38.321 below.

R2-2000959	Remaining issue on configured grant	Huawei, HiSilicon	discussion	Rel-16	NR_unlic-Core

Proposal 1: a new timer, i.e. cg-HARQ-RTT-Timer, per HARQ process is specified for minimum duration D in MAC layer. The cg-HARQ-RTT-Timer is started for the corresponding HARQ process when the transmission is performed.

Proposal 2: configuredGrantTimer or cg-RetransmissionTimer shall be stopped for the corresponding HARQ process when downlink feedback information is received and cg-HARQ-RTT-Timer is not running.

Proposal 23: cg-RetransmissionTimer shall be started for the corresponding HARQ process after expiry of cg-HARQ-RTT-Timer.

Proposal 4: In case that PDCCH indicates activation or deactivation for a CG, configuredGrantTimer/cg-RetransmissionTimer for a HARQ process is stopped only when that CG previously transmitted that HARQ process.

Proposal 5: repetitions across CGs is not supported in R16.

R2-2001205	Configured Grant remaining issues	Ericsson	discussion	NR_unlic-Core

Proposal 1	It is left to IOT WI to define activation and deactivation of CG configurations. No further work is needed for NR-U in Rel-16.

Proposal 2	Upon receiving a CG activation command, for each HARQ process configured for the CG and not being used by other CG configurations, the UE performs the below actions:

a.	stop the CGT for the HARQ process

b.	stop the CGRT for the HARQ process

c.	flush the HARQ buffer for the HARQ process

d.	consider the NDI to be toggled for the HARQ process.

Proposal 3	The UE can transmit a TB repetition using a different CG configuration/resource from the one used for the initial transmission. The selected resource shall provide same size as the initial TB.

Proposal 4	The CGRT is started and restarted immediately after every TB repetition is transmitted using a configured grant if none of the below condition is fulfilled.

a.	A HARQ acknowledgement (ACK or NACK) for the associated HARQ process is received

b.	A dynamic grant indicating a new transmission associated with the same HARQ process is received

c.	The CGT timer is expired.

R2-2001206	Channel access priority for Configured Grant	Ericsson	discussion	NR_unlic-Core

Proposal 1	For type 1 LBT of a configured grant, in case a MAC PDU doesn’t include low priority MAC CE (i.e. padding BSR or Recommended bit rate query) and DCCH SDU, the UE selects the CAPC for the MAC PDU according to the lowest priority CAPC of LCHs.

Proposal 2	For type 1 LBT of a configured grant, in case a MAC PDU includes not only low priority MAC CE (i.e. padding BSR or Recommended bit rate query) and it does not include DCCH SDU, the UE selects the lowest priority CAPC of LCHs in the MAC PDU excluding the LCHs associated to the low priority MAC CEs.

R2-2001442	Consideration of delayed CG confirmation	LG Electronics Polska	discussion	Rel-16	NR_unlic-Core

Proposal 1. In order to avoid transmission delay of the configured grant confirmation MAC CE, upon activation of the configured grant Type2, the UE flushes the HARQ buffer of the identified HARQ process if the HARQ process is pending.

Proposal 2. The text proposal should be included in the MAC CR in NR-U.

#### 6.2.2.4	Other

Includes wideband operation aspects, HARQ, SR and PHR

ONLY NEW CRITICAL OPEN Issues that are not identified in email discussions.  Contributions should NOT discuss open issues in the email discussion

R2-2000149	Remaining Issues on CAPC Selection for Configured Grant	vivo	discussion

Proposal: For UL CG, if DCCH SDU(s) is included in MAC PDU, UE selects the lowest CAPC index (highest priority) of the DCCH(s) multiplexed in the MAC PDU.

R2-2000154	Consideration on SR transmission colliding with PUSCH transmission	Xiaomi Communications	discussion	Rel-16	R2-1915956	Late

R2-2000172	Consideration on SR transmission colliding with PUSCH transmission	Xiaomi Communications	discussion	Rel-16	R2-1915956	Late

R2-2000173	Consideration on SR transmission colliding with PUSCH transmission	Xiaomi Communications	discussion	Rel-16	R2-1915956

Proposal: RAN2 to agree that we should allow SR transmission on the PUCCH resource colliding with the UL-SCH resource for which LBT fails.

R2-2000176	Remaining issues of CAPC	Huawei, HiSilicon	discussion	Rel-16	NR_unlic-Core

Proposal 1:  UE selects the CAPC of SRB2 if only SRB2 is present.

Proposal 2: The gNB controls the multiplexing of downlink DRB to guarantee the relative fairness.

Proposal 3: Capture the following text into running CR regarding uplink DRB multiplexing.

R2-2000535	Applicability of NR-U features to licensed carrier	Samsung	discussion	Rel-16	NR_unlic-Core	R2-1915222

Proposal: As a working assumption, the new features introduced for the NR-U are applicable to unlicensed carriers only. As a result, for each RRC configuration for the NR-U feature, its applicability should be clearly specified e.g. in the field description or by the conditional presence.

R2-2000669	LBT failure measurement report handling	Nokia, Nokia Shanghai Bell	discussion	Rel-16	NR_unlic-Core

Proposal: We do not define abandoning any control plane sending in lower layers and inform this to RAN4 with a LS

R2-2000838	PHR for NR-U	Lenovo, Motorola Mobility	discussion	Rel-16	NR_unlic-Core

Proposal1: UE reports a virtual PHR for a serving cell for cases when the PHR MAC CE is transmitted on a CG PUSCH on an unlicensed cell. For the case that a serving cell is configured with two UL carriers, e.g. SUL/NUL, UE reports a virtual PHR for a predefined PHR type, e.g. PHR type 1.

R2-2000842	On PHR and autonomous retransmissions	MediaTek Inc.	discussion	Rel-16	NR_unlic-Core	R2-1913262

Proposal 1: The PHR includes a timestamp (SFN and slot) of the time that the reported power headroom corresponds to.

R2-2000960	PHR reporting for NR-U	Huawei, HiSilicon	discussion	Rel-16	NR_unlic-Core

Proposal 1: it is suggested RAN2 to adopt a solution for PHR.

	Option 1: PHR MAC CE is not allowed to be transmitted on any NR-U CG.

	Option 2: a timestamp should be included in each CG transmission.

	Option 3: PHR MAC CE needs to rebuild when LBT fails for CG transmission.

R2-2000961	Reply LS on PHR report	Huawei, HiSilicon	discussion	Rel-16	NR_unlic-Core

R2-2000962	Disucssion PDCCH group switching	Huawei, HiSilicon	discussion	Rel-16	NR_unlic-Core

Proposal 1: UE should switch to a SS group with denser PDCCH occasion when BWP is switched due to initiation of Random Access procedure or consistent UL LBT failure.

R2-2001094	CAPC selection for UL transmissions	Intel Corporation	discussion	Rel-16	NR_unlic-Core

Proposal: Extend the UE CAPC selection to MsgA PUSCH transmission, Msg3 PUSCH transmission and other UL transmission case where CAT4 is indicated but CAPC is not signaled explicitly:

For uplink transmissions in RRC_CONNECTEDon configured grants, the gNB configures the UE for the CAPC to be used for SRB2 and DRBs. For uplink transmissions on configured grants, , Msg3 transmission (UE initiated COT case), MsgA PUSCH transmission and other UL transmission case where CAT4 is indicated but CAPC is not signaled explicitly, the UE shall select the CAPC of DCCH if DCCH SDU is transmitted and otherwise the lowest priority CAPC (i.e. highest number in Table 5.X.2-1) of the logical channel(s) with MAC SDU and of the MAC CE(s) multiplexed in this MAC PDU.

R2-2001108	Remaining CAPC aspects for CG when SRB is multiplexed	NEC Telecom MODUS Ltd.	Discussion

Proposal 1: For UL CG, if SDUs from more than one DCCH are included in MAC PDU, UE selects the lowest CAPC index of these DCCH SDUs

Proposal 2: RAN2 confirm that for UL CG, if DCCH SDU(s) and DTCH SDU(s) are multiplexed in one MAC PDU, UE select CAPC index based on DCCH SDU(s) regardless of SRB type and ignore CAPC of DTCHs.

R2-2001204	Remaining issue on PHR	Ericsson	discussion	NR_unlic-Core

Proposal 1	The ambiguity issues for PHR time reference and PHR type due to LBT failures are not addressed in Rel-16.

R2-2001450	Dynamic DL opportunity enhancement based on channel busy level in NR-U	LG Electronics Polska	discussion	Rel-16	NR_unlic-Core	R2-1915921

Proposal. RAN2 enhance the DL opportunity based on the channel busy level dynamically measured by both UE and gNB.

R2-2001451	MAC impacts of multiple CCAs in wide band operation	LG Electronics Polska	discussion	Rel-16	NR_unlic-Core	R2-1916153

Proposal. RAN2 only consider the following cases with single active BWP for a carrier in Rel-16 NR-U WI:

- UE transmits data only if the CCA is successful at UE for the whole BWP (Alt 1 in RAN1 agreement)

- UE transmits data on the only one of sub-bands which the CCA is successful at UE (Method 5 of Alt 2)