3GPP TSG-RAN2#109e	R2-2001922

- February 24 – March 6 2020

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 0199 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Running Stage-2 CR for NR Shared Spectrum |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rapporteur (Qualcomm Incorporated) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-Core |  |  |  |  |  |  | Date: |  | 2020-02-13 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Following additional changes markup by RAN2#108 , were introduced based on RAN2#108 agreements as well as RAN1 conclusions on top of the RAN2#108 baseline R2-1914520: - Added SRB3 in 5.X.2. - Updated UL LBT failure detection in 5.X.1. - Updated paging stopping in 9.2.5 to include the short message based indication. - Added CG retransmission in 10.3. - Updated 5.1 to include interlaced waveform (from RAN1).. - Updated 5.2.5.4 to include HARQ enhancements for NR-U (from RAN1). - Added in 5.3.1 that hopping is not used for interlaced waveform (from RAN1). - Updated 5.3.3 to include the new PUCCH formats (from RAN1). - Updated Random access 5.3.4 to include RAN1 agreements on preamble lengths (from RAN1). Editor’s Note: The changes in Section 5 need confirmation by RAN1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The agreements on LBT impacts to MAC, RACH, configured grants, CAPC section, UL multiplexing, paging. It is FFS whether the features introduced for NR-U are also applicable to NR licensed. The CR will be updated according to the outcome of that discussion on UE features. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | 3.1, 5.1, 5.2.5.4, 5.3.1, 5.3.3, 5.3.4, 5.3.5.4, 5.X (new), 9.2.1.2, 9.2.5, 9.2.7, 10.3, 11 |  |  |  |  |  |  |  |  |

Start of changes

# 2	References

>>Skipped

[XX]	3GPP TS 37.213: "Physical layer procedures for shared spectrum channel access".

Next change

## 3.1	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], in TS 36.300 [2] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] and TS 36.300 [2].

CAPC				Channel Access Priority Class

>>Skipped

LBT	Listen Before Talk

>>Skipped

NR	NR Radio Access

NR-U	NR Radio Access operating in unlicensed spectrum

>>Skipped

Next change

## 5.1	Waveform, numerology and frame structure

The downlink transmission waveform is conventional OFDM using a cyclic prefix. The uplink transmission waveform is conventional OFDM using a cyclic prefix with a transform precoding function performing DFT spreading that can be disabled or enabled. For operation with shared spectrum access, the uplink transmission waveform subcarrier mapping can map to subcarriers in one or more PRB interlaces.

Figure 5.1-1: Transmitter block diagram for CP-OFDM with optional DFT-spreading

The numerology is based on exponentially scalable sub-carrier spacing f = 2µ × 15 kHz with µ={0,1,3,4} for PSS, SSS and PBCH and µ={0,1,2,3} for other channels. Normal CP is supported for all sub-carrier spacings, Extended CP is supported for µ=2. 12 consecutive sub-carriers form a Physical Resource Block (PRB). Up to 275 PRBs are supported on a carrier.

Table 5.1-1: Supported transmission numerologies.

|  |  | Cyclic prefix | Supported for data | Supported for synch |
| --- | --- | --- | --- | --- |
| 0 | 15 | Normal | Yes | Yes |
| 1 | 30 | Normal | Yes | Yes |
| 2 | 60 | Normal, Extended | Yes | No |
| 3 | 120 | Normal | Yes | Yes |
| 4 | 240 | Normal | No | Yes |

The UE may be configured with one or more bandwidth parts on a given component carrier, of which only one can be active at a time, as described in clauses 7.8 and 6.10 respectively. The active bandwidth part defines the UE's operating bandwidth within the cell's operating bandwidth. For initial access, and until the UE's configuration in a cell is received, initial bandwidth part detected from system information is used.

Downlink and uplink transmissions are organized into frames with 10 ms duration, consisting of ten 1 ms subframes. Each frame is divided into two equally-sized half-frames of five subframes each. The slot duration is 14 symbols with Normal CP and 12 symbols with Extended CP, and scales in time as a function of the used sub-carrier spacing so that there is always an integer number of slots in a subframe.

Timing Advance TA is used to adjust the uplink frame timing relative to the downlink frame timing.

Figure 5.1-2: Uplink-downlink timing relation

Operation on both paired and unpaired spectrum is supported.

Next change

#### 5.2.5.4	HARQ

Asynchronous Incremental Redundancy Hybrid ARQ is supported. The gNB provides the UE with the HARQ-ACK feedback timing either dynamically in the DCI or semi-statically in an RRC configuration. Retransmission for HARQ-ACK feedback is supported for operation with shared spectrum channel access by using enhanced dynamic codebook and/or one-shot triggering of HARQ-ACK transmission for all configured CCs and HARQ processes in the PUCCH group.

The UE may be configured to receive code block group based transmissions where retransmissions may be scheduled to carry a sub-set of all the code blocks of a TB.

Next change

### 5.3.1	Uplink transmission scheme

Two transmission schemes are supported for PUSCH: codebook based transmission and non-codebook based transmission.

For codebook based transmission, the gNB provides the UE with a transmit precoding matrix indication in the DCI. The UE uses the indication to select the PUSCH transmit precoder from the codebook. For non-codebook based transmission, the UE determines its PUSCH precoder based on wideband SRI field from the DCI.

A closed loop DMRS based spatial multiplexing is supported for PUSCH. For a given UE, up to 4 layer transmissions are supported. The number of code words is one. When transform precoding is used, only a single MIMO layer transmission is supported.

Transmission durations from 1 to 14 symbols in a slot is supported.

Aggregation of multiple slots with TB repetition is supported.

Two types of frequency hopping are supported, intra-slot frequency hopping, and in case of slot aggregation, inter-slot frequency hopping. Intra-slot and inter-slot frequency hopping are not supported when PRB interlace uplink transmission waveform is used.

PUSCH may be scheduled with DCI on PDCCH, or a semi-static configured grant may be provided over RRC, where two types of operation are supported:

-	The first PUSCH is triggered with a DCI, with subsequent PUSCH transmissions following the RRC configuration and scheduling received on the DCI, or

-	The PUSCH is triggered by data arrival to the UE's transmit buffer and the PUSCH transmissions follow the RRC configuration.

### 5.3.2	Physical-layer processing for physical uplink shared channel

The uplink physical-layer processing of transport channels consists of the following steps:

-	Transport Block CRC attachment;

-	Code block segmentation and Code Block CRC attachment;

-	Channel coding: LDPC coding;

-	Physical-layer hybrid-ARQ processing;

-	Rate matching;

-	Scrambling;

-	Modulation: π/2 BPSK (with transform precoding only), QPSK, 16QAM, 64QAM and 256QAM;

-	Layer mapping, transform precoding (enabled/disabled by configuration), and pre-coding;

-	Mapping to assigned resources and antenna ports.

The UE transmits at least one symbol with demodulation reference signal on each layer on each frequency hop in which the PUSCH is transmitted, and up to 3 additional DMRS can be configured by higher layers.

Phase Tracking RS may be transmitted on additional symbols to aid receiver phase tracking.

The UL-SCH physical layer model is described in TS 38.202 [20].

For configured grants, described in section 10.3, a CG-UCI (Configured Grant Uplink Control Information) is transmitted in PUSCH scheduled by configured uplink grant.

### 5.3.3	Physical uplink control channel

Physical uplink control channel (PUCCH) carries the Uplink Control Information (UCI) from the UE to the gNB. Five formats of PUCCH exist, depending on the duration of PUCCH and the UCI payload size.

-	Format #0: Short PUCCH of 1 or 2 symbols with small UCI payloads of up to two bits with UE multiplexing capacity of up to 6 UEs with 1-bit payload in the same PRB;

-	Format #1: Long PUCCH of 4-14 symbols with small UCI payloads of up to two bits with UE multiplexing capacity of up to 84 UEs without frequency hopping and 36 UEs with frequency hopping in the same PRB;

-	Format #2: Short PUCCH of 1 or 2 symbols with large UCI payloads of more than two bits with no UE multiplexing capability in the same PRBs;

-	Format #3: Long PUCCH of 4-14 symbols with large UCI payloads with no UE multiplexing capability in the same PRBs;

-	Format #4: Long PUCCH of 4-14 symbols with moderate UCI payloads with multiplexing capacity of up to 4 UEs in the same PRBs.

The short PUCCH format of up to two UCI bits is based on sequence selection, while the short PUCCH format of more than two UCI bits frequency multiplexes UCI and DMRS. The long PUCCH formats time-multiplex the UCI and DMRS. Frequency hopping is supported for long PUCCH formats and for short PUCCH formats of duration of 2 symbols. Long PUCCH formats can be repeated over multiple slots.

For operation with shared spectrum access, PUCCH Format #0, #1, #2, #3 are extended to use resource in one PRB interlace (up to two interlaces for Format #2 and Format #3) in one RB Set PUCCH Format #2 and #3 are enhanced to support multiplexing capacity of up to 4 UEs in the same PRB interlace when one interlace is used.

UCI multiplexing in PUSCH is supported when UCI and PUSCH transmissions coincide in time, either due to transmission of a UL-SCH transport block or due to triggering of A-CSI transmission without UL-SCH transport block:

-	UCI carrying HARQ-ACK feedback with 1 or 2 bits is multiplexed by puncturing PUSCH;

-	In all other cases UCI is multiplexed by rate matching PUSCH.

UCI consists of the following information:

-	CSI;

-	ACK/NAK;

-	Scheduling request.

Multiplexing of CG-UCI and PUCCH carrying HARQ-ACK feedback is configured by the gNB. If not configured, when PUCCH overlaps with PUSCH scheduled by a configured grant within a PUCCH group and PUCCH carries HARQ ACK feedback, PUSCH scheduled by configured grant is skipped.

QPSK and π/2 BPSK modulation can be used for long PUCCH with more than 2 bits of information, QPSK is used for short PUCCH with more than 2 bits of information and BPSK and QPSK modulation can be used for long PUCCH with up to 2 information bits.

Transform precoding is applied to PUCCH Format #3 and Format #4.

Channel coding used for uplink control information is described in table 5.3.3-1.

Table 5.3.3-1: Channel coding for uplink control information

| Uplink Control Information size including CRC, if present | Channel code |
| --- | --- |
| 1 | Repetition code |
| 2 | Simplex code |
| 3-11 | Reed Muller code |
| >11 | Polar code |

### 5.3.4	Random access

Random access preamble sequences, of four different lengths are supported. Long sequence length 839 is applied with subcarrier spacings of 1.25 and 5 kHz and short sequence length 139 is applied with subcarrier spacings of 15, 30, 60 and 120 kHz. Long sequences support unrestricted sets and restricted sets of Type A and Type B, while short sequences support unrestricted sets only. Long sequences are only used for operation with licensed channel access while short sequences can be used for operation with either licensed or shared spectrum channel access. Sequence lengths of 571 and 1151 are applied with subcarrier spacings of 30 kHz and 15 kHz respectively and used only for operation with shared spectrum channel access and support unrestricted sets only.

Multiple PRACH preamble formats are defined with one or more PRACH OFDM symbols, and different cyclic prefix and guard time. The PRACH preamble configuration to use is provided to the UE in the system information.

The UE calculates the PRACH transmit power for the retransmission of the preamble based on the most recent estimate pathloss and power ramping counter.

The system information provides information for the UE to determine the association between the SSB and the RACH resources. The RSRP threshold for SSB selection for RACH resource association is configurable by network.

Next change

#### 5.3.5.4	HARQ

Asynchronous Incremental Redundancy Hybrid ARQ is supported. The gNB schedules each uplink transmission and retransmission using the uplink grant on DCI. For operation in shared spectrum channel access, UE can also retransmit on configured grants.

The UE may be configured to transmit code block group based transmissions where retransmissions may be scheduled to carry a sub-set of all the code blocks of a transport block.

Next change

## 5.X Access to Shared Spectrum

### 5.X.1	Overview

NR Radio Access operating with shared spectrum channel access (referred to as NR-U) can operate in a PCell, an SCell, or a PSCell. In stand-alone mode, all cells are in shared spectrum. When SCells are in shared spectrum, the PCell may be in licensed spectrum. An NR-U SCell may or may not be configured with UL. The DC-type of operation can be configured either with E-UTRAN node (connected to EPC or 5GC) as MN or with NR node as described in TS 37.340 [21].

The gNB operates in either dynamic or semi-static channel access modes as described in TS 37.213 [xx]. In both channel access modes, the gNB and UE may apply Listen-Before-Talk (LBT) before performing a transmission on NR-U cells. When LBT is applied, the transmitter listens to/senses the channel to determine whether the channel is free or busy and performs transmission only if the channel is sensed free.

When the UE detects consistent uplink LBT failures, it takes actions as specified in TS 38.321 [6]. The detection is per BWP and based on all uplink transmissions within this BWP. When consistent uplink LBT failures are detected on SCell(s), the UE reports this to the corresponding gNB (MN for MCG, SN for SCG) via MAC CE on a different serving cell other than the SCell (s) where the failures were detected. If no resources are available to transmit the MAC CE, a Scheduling Request (SR) can be transmitted by the UE. When consistent uplink LBT failures are detected on SpCell, the UE switches to another UL BWP with configured RACH resources on that cell, initiates RACH, and reports the failure via MAC CE. When multiple UL BWPs are available for switching, it is up to the UE implementation which one to select. For PSCell, if consistent uplink LBT failures are detected on all the UL BWPs with configured RACH resources, the UE declares SCG RLF and reports the failure to the MN via SCGFailureInformation. For PCell, if the uplink LBT failures are detected on all the UL BWP(s) with configured RACH resources, the UE declares RLF.

Editor’s Note: It is FFS whether and where to capture NR-U deployment scenarios (e.g. in this section or Annex).

5.X.2	Channel Access Priority Classes

Table 5.X.2-1 shows which CAPC should be used by traffic belonging to the different standardized 5QIs. A non-standardized 5QI (i.e. operator specific 5QI) should use suitable CAPC based on the below table, i.e. CAPC used for a non-standardized 5QI should be the CAPC of the standardized 5QIs which best matches the traffic class of the non-standardized 5QI.

For DRBs, the gNB selects the CAPC by taking into account the 5QIs of all the QoS flows multiplexed in this DRB while considering fairness between different traffic types and transmissions. For SRB0, SRB1, and SRB3, the CAPC is always the highest priority (i.e. the lowest number in Table 5.X.2-1). For uplink transmissions on configured grants, the gNB configures the UE for the CAPC to be used for SRB2 and DRBs. The padding BSR and recommended bit rate MAC CEs use the lowest priority CAPC (i.e. highest number in Table 5.X.2-1) while other MAC CEs use the highest priority CAPC. For uplink transmissions on configured grants, the UE shall select the CAPC as follows:

- highest priority CAPC of MAC CE(s) if only MAC CE(s) are included;

- highest priority CAPC of DCCH(s) if DCCH SDU(s) are included;

- lowest priority CAPC of the logical channel(s) with MAC SDU multiplexed in this MAC PDU otherwise.

- Table 5.X.2-1: Mapping between Channel Access Priority Classes and 5QI

| Channel Access Priority Class (p) | 5QI |
| --- | --- |
| 1 | 1, 3, 5, 65, 66, 67, 69, 70, 79, 80, 82, 83, 84, 85 |
| 2 | 2, 7, 71 |
| 3 | 4, 6, 8, 9, 72, 73, 74, 76 |
| 4 | - |

Next change

#### 9.2.1.2	Cell Reselection

A UE in RRC_IDLE performs cell reselection. The principles of the procedure are the following:

-	Cell reselection is always based on CD-SSBs located on the synchronization raster (see clause 5.2.4).

-	The UE makes measurements of attributes of the serving and neighbour cells to enable the reselection process:

-	For the search and measurement of inter-frequency neighbouring cells, only the carrier frequencies need to be indicated.

-	Cell reselection identifies the cell that the UE should camp on. It is based on cell reselection criteria which involves measurements of the serving and neighbour cells:

-	Intra-frequency reselection is based on ranking of cells;

-	Inter-frequency reselection is based on absolute priorities where a UE tries to camp on the highest priority frequency available;

-	An NCL can be provided by the serving cell to handle specific cases for intra- and inter-frequency neighbouring cells;

-	Black lists can be provided to prevent the UE from reselecting to specific intra- and inter-frequency neighbouring cells;

-	White lists can be provided to request the UE to reselect to only specific intra- and inter-frequency neighbouring cells;

-	Cell reselection can be speed dependent;

-	Service specific prioritisation.

In multi-beam operations, the cell quality is derived amongst the beams corresponding to the same cell (see clause 9.2.4).

Next change

### 9.2.5	Paging

Paging allows the network to reach UEs in RRC_IDLE and in RRC_INACTIVE state through Paging messages, and to notify UEs in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED state of system information change (see subclause 7.3.3) and ETWS/CMAS indications (see subclause 16.4) through Short Messages. Both Paging messages and Short Messages are addressed with P-RNTI on PDCCH, but while the former is sent on PCCH, the latter is sent over PDCCH directly (see subclause 6.5 of TS 38.331 [12]).

While in RRC_IDLE the UE monitors the paging channels for CN-initiated paging; in RRC_INACTIVE the UE also monitors paging channels for RAN-initiated paging. A UE need not monitor paging channels continuously though; Paging DRX is defined where the UE in RRC_IDLE or RRC_INACTIVE is only required to monitor paging channels during one Paging Occasion (PO) per DRX cycle (see TS 38.304 [10]). The Paging DRX cycles are configured by the network:

1)	For CN-initiated paging, a default cycle is broadcast in system information;

2)	For CN-initiated paging, a UE specific cycle can be configured via NAS signalling;

3)	For RAN-initiated paging, a UE-specific cycle is configured via RRC signalling;

-	The UE uses the shortest of the DRX cycles applicable i.e. a UE in RRC_IDLE uses the shortest of the first two cycles above, while a UE in RRC_INACTIVE uses the shortest of the three.

The POs of a UE for CN-initiated and RAN-initiated paging are based on the same UE ID, resulting in overlapping POs for both. The number of different POs in a DRX cycle is configurable via system information and a network may distribute UEs to those POs based on their IDs.

When in RRC_CONNECTED, the UE monitors the paging channels in any PO signalled in system information for SI change indication and PWS notification. In case of BA, a UE in RRC_CONNECTED only monitors paging channels on the active BWP with common search space configured.

For operation with shared spectrum channel access, a UE can be configured for an additional number of PDCCH monitoring occasions in its PO to monitor for paging. However, when the UE detects a PDCCH transmission within the UE’s PO addressed with P-RNTI on PDCCH, the UE is not required to monitor the subsequent PDCCH monitoring occasions within this PO.

Paging optimization for UEs in CM_IDLE: at UE context release, the NG-RAN node may provide the AMF with a list of recommended cells and NG-RAN nodes as assistance info for subsequent paging. The AMF may also provide Paging Attempt Information consisting of a Paging Attempt Count and the Intended Number of Paging Attempts and may include the Next Paging Area Scope. If Paging Attempt Information is included in the Paging message, each paged NG-RAN node receives the same information during a paging attempt. The Paging Attempt Count shall be increased by one at each new paging attempt. The Next Paging Area Scope, when present, indicates whether the AMF plans to modify the paging area currently selected at next paging attempt. If the UE has changed its state to CM CONNECTED the Paging Attempt Count is reset.

Paging optimization for UEs in RRC_INACTIVE: at RAN Paging, the serving NG-RAN node provides RAN Paging area information. The serving NG-RAN node may also provide RAN Paging attempt information. Each paged NG-RAN node receives the same RAN Paging attempt information during a paging attempt with the following content: Paging Attempt Count, the intended number of paging attempts and the Next Paging Area Scope. The Paging Attempt Count shall be increased by one at each new paging attempt. The Next Paging Area Scope, when present, indicates whether the serving NG_RAN node plans to modify the RAN Paging Area currently selected at next paging attempt. If the UE leaves RRC_INACTIVE state the Paging Attempt Count is reset.

### 9.2.6	Random Access Procedure

The random access procedure is triggered by a number of events:

-	Initial access from RRC_IDLE;

-	RRC Connection Re-establishment procedure;

-	DL or UL data arrival during RRC_CONNECTED when UL synchronisation status is "non-synchronised";

-	UL data arrival during RRC_CONNECTED when there are no PUCCH resources for SR available;

-	SR failure;

-	Request by RRC upon synchronous reconfiguration (e.g. handover);

-	Transition from RRC_INACTIVE;

-	To establish time alignment for a secondary TAG;

-	Request for Other SI (see clause 7.3);

-	Beam failure recovery;

-	Consistent UL LBT failure on SpCell.

Furthermore, the random access procedure takes two distinct forms: contention-based random access (CBRA) and contention-free random access (CFRA) as shown on Figure 9.2.6-1 below:

(a)	Contention-Based											(b)	Contention-Free

Figure 9.2.6-1: Random Access Procedures

For random access in a cell configured with SUL, the network can explicitly signal which carrier to use (UL or SUL). Otherwise, the UE selects the SUL carrier if and only if the measured quality of the DL is lower than a broadcast threshold. Once started, all uplink transmissions of the random access procedure remain on the selected carrier.

When CA is configured, the first three steps of CBRA always occur on the PCell while contention resolution (step 4) can be cross-scheduled by the PCell. The three steps of a CFRA started on the PCell remain on the PCell. CFRA on SCell can only be initiated by the gNB to establish timing advance for a secondary TAG: the procedure is initiated by the gNB with a PDCCH order (step 0) that is sent on a scheduling cell of an activated SCell of the secondary TAG, preamble transmission (step 1) takes place on the indicated SCell, and Random Access Response (step 2) takes place on PCell.

### 9.2.7	Radio Link Failure

In RRC_CONNECTED, the UE performs Radio Link Monitoring (RLM) in the active BWP based on reference signals (SSB/CSI-RS) and signal quality thresholds configured by the network. SSB-based RLM is based on the SSB associated to the initial DL BWP and can only be configured for the initial DL BWP and for DL BWPs containing the SSB associated to the initial DL BWP. For other DL BWPs, RLM can only be performed based on CSI-RS.

The UE declares Radio Link Failure (RLF) when one of the following criteria are met:

-	Expiry of a timer started after indication of radio problems from the physical layer (if radio problems are recovered before the timer is expired, the UE stops the timer); or

-	Random access procedure failure; or

-	RLC failure; or

-	Detection of consistent uplink LBT failures for operation with shared spectrum channel accesss described in 5.X.1.

After RLF is declared, the UE:

-	stays in RRC_CONNECTED;

-	selects a suitable cell and then initiates RRC re-establishment;

-	enters RRC_IDLE if a suitable cell was not found within a certain time after RLF was declared.

Next change

## 10.3	Uplink Scheduling

In the uplink, the gNB can dynamically allocate resources to UEs via the C-RNTI on PDCCH(s). A UE always monitors the PDCCH(s) in order to find possible grants for uplink transmission when its downlink reception is enabled (activity governed by DRX when configured). When CA is configured, the same C-RNTI applies to all serving cells.

In addition, with Configured Grants, the gNB can allocate uplink resources for the initial HARQ transmissions and HARQ retransmissions to UEs. Two types of configured uplink grants are defined:

-	With Type 1, RRC directly provides the configured uplink grant (including the periodicity).

-	With Type 2, RRC defines the periodicity of the configured uplink grant while PDCCH addressed to CS-RNTI can either signal and activate the configured uplink grant, or deactivate it; i.e. a PDCCH addressed to CS-RNTI indicates that the uplink grant can be implicitly reused according to the periodicity defined by RRC, until deactivated.

The dynamically allocated uplink transmission overrides the configured uplink grant in the same serving cell, if they overlap in time. Otherwise an uplink transmission according to the configured uplink grant is assumed, if activated.

Retransmissions other than repetitions are explicitly allocated via PDCCH(s) or via configuration of a retransmission timer.

When CA is configured, at most one configured uplink grant can be signalled per serving cell. When BA is configured, at most one configured uplink grant can be signalled per BWP. On each serving cell, there can be only one configured uplink grant active at a time. A configured uplink grant for one serving cell can either be of Type 1 or Type 2. For Type 2, activation and deactivation of configured uplink grants are independent among the serving cells. When SUL is configured, a configured uplink grant can only be signalled for one of the 2 ULs of the cell.

Next change

# 11	UE Power Saving

The PDCCH monitoring activity of the UE in RRC connected mode is governed by DRX and BA.

When DRX is configured, the UE does not have to continuously monitor PDCCH. DRX is characterized by the following:

-	on-duration: duration that the UE waits for, after waking up, to receive PDCCHs. If the UE successfully decodes a PDCCH, the UE stays awake and starts the inactivity timer;

-	inactivity-timer: duration that the UE waits to successfully decode a PDCCH, from the last successful decoding of a PDCCH, failing which it can go back to sleep. The UE shall restart the inactivity timer following a single successful decoding of a PDCCH for a first transmission only (i.e. not for retransmissions);

-	retransmission-timer: duration until a retransmission can be expected;

-	cycle: specifies the periodic repetition of the on-duration followed by a possible period of inactivity (see figure 11-1 below);

-	active-time: total duration that the UE monitors PDCCH. This includes the "on-duration" of the DRX cycle, the time UE is performing continuous reception while the inactivity timer has not expired, and the time when the UE is performing continuous reception while waiting for a retransmission opportunity.

Figure 11-1: DRX Cycle

When BA is configured, the UE only has to monitor PDCCH on the one active BWP i.e. it does not have to monitor PDCCH on the entire DL frequency of the cell. A BWP inactivity timer (independent from the DRX inactivity-timer described above) is used to switch the active BWP to the default one: the timer is restarted upon successful PDCCH decoding and the switch to the default BWP takes place when it expires.

Editor’s Note: For NR-U, a single DRX configuration is used per MAC entity as in NR.

End of changes

Annex (not part of the specification): RAN2 Agreements

This Annex contains the RAN2 agreements on Rel-16 WI for “NR-based Access to Unlicensed Spectrum”. The agreements are provided verbatim for reference.This annex shall be removed once the WI is completed.

## RAN2#105

- Consistent LBT failures can lead to RLF, at least for UL transmissions, for which consistent failures can currently eventually lead to RLF - In NR-U, DRX On-duration starts as in Rel-15 NR (except for potentially have a new switch trigger to go to short DRX - One DRX configuration for one MAC entity (no change) - FFS if DRX active time somehow be extended, or go to short DRX, by a non-data DL transmission (not WUS) - A UE can be configured for an additional number of monitoring occasions at or after or before (FFS) its calculated PO (when the paging message can be transmitted - FFS dynamic extension - FFS dynamic termination

## RAN2#105bis

- Adopt a mechanism in MAC spec to handle the UL LBT failure, where “consistent” UL LBT failures (at least for UL transmissions of SR, RACH, PUSCH) are used for problem detection - R2 assumes that the configured grant timer is not started/restarted when configured grant is not transmitted due to LBT failure. PDU overwrite need to be avoided somehow. - The configured grant timer is not started/restarted when UL LBT fails on PUSCH transmission for grant received by PDCCH addressed to CS-RNTI scheduling retransmission for configured grant - The configured grant timer is not started/restarted when the UL LBT fails on PUSCH transmission for UL grant received by PDCCH addressed to C-RNTI, which indicates the same HARQ process configured for configured uplink grant - Upon UL transmission on configured grant, bwp-InactivityTimer is restarted as today (i.e. at LBT success) - Upon UL transmission on configured grant, sCellDeactivationTimer is restarted as today (i.e. at LBT success) - Content of a MAC PDU (including any PHR value) will not change after it has been built for transmission on dynamic grant even if the LBT fails. - For BSR/PHR transmitted on configured grant, it is up to the implementation of the UE to handle the content of BSR/PHR. - All BSRs triggered prior to MAC PDU assembly shall be cancelled when a MAC PDU is attempted transmission on PUSCH while UL LBT fails and this PDU includes a Long or Short BSR MAC CE which contains buffer status up to (and including) the last event that triggered a BSR prior to the MAC PDU assembly, i.e. no TS change for this case. - dataInactivityTimer should be restarted when any MAC entity attempts transmission of a MAC SDU for DTCH logical channel, or DCCH logical channel regardless UL LBT outcome. - drx-HARQ-RTT-TimerUL should be started/restarted regardless of the LBT outcome for PUSCH transmission with dynamic grant - drx-HARQ-RTT-TimerUL should not be started/restarted when LBT fails for PUSCH transmission with configured grant

- Retransmissions of a TB using configured grant resources, when initial transmission or a retransmission of the TB was previously done using dynamically scheduled resources, is not allowed

- A table for mapping between 5QI and CAPC, similar to Table 5.7.1-1 in 3GPP TS 36.300, shall be specified

- All MAC CEs, except padding BSR MAC CE, uses the highest priority CAPC, that is the lowest number CAPC, FFS for recommended rate for Voice MAC CE

- It is FFS if for CG, when several MAC SDUs are multiplexed, CAPC is selected according to the configuration for the LCH with lowest priority CAPC (for DRB).

- A new timer is introduced for auto retransmission (i.e. timer expiry = HARQ NACK) on configured grant for the case of the TB previous being transmitted on a configured grant “CG retransmission timer”.

- the new timer is started when the TB is actually transmitted on the configured grant and stopped upon reception of HARQ feedback (DFI) or dynamic grant for the HARQ process.

- the legacy configured grant timer and behaviour is kept for preventing the configured grant overriding the TB scheduled by dynamic grant, i.e. it is (re)started upon reception of the PDCCH as well as transmission on the PUSCH of dynamic grant.

- The PREAMBLE_TRANSMISSION_COUNTER is not increased if the preamble is not transmitted due to LBT failure

- SR_COUNTER is increase only when SR is successfully transmitted

- As earlier agreed, The POWER_RAMPING_COUNTER is not increased if the preamble is not transmitted due to LBT failure. For this purpose LBT failure indication or equiv. (used for other LBT outcome dependencies) from PHY is used.

- MAC returns to the resource selection step if LBT fails for Msg1 transmission opportunity(ies)

- From MAC perspective, multiple msg1 transmissions are not supported (does not preclude beam sweeping enhancement if decided for NR)

- Actual transmission for MSG1 (LBT success) is used for starting RAR window

- R2 assumes the maximum RAR window size is extended to [20] ms

- We ask R1 regarding the support of multiple MSG3 transmission opportunities

- R2 assumes the range of ra-ContentionResolutionTimer is not extended for NR-U (note this contradicts earlier assumption)

- Either a) the ra-ContentionResolutionTimer is started regardless of the LBT outcome of msg3 transmission or b) ra-ContentionResolutionTimer is started only at successful LBT outcome of msg3 transmission + immediately the UE to restart from RACH resource selection if all MSG3 transmissions fail. FFS

- As agreed in the SI phase, the sr-ProhibitTimer shall not prohibit SR transmissions due to SR that was not transmitted due to LBT failure.

- As a starting point: If UE receives on PDCCH addressed to P-RNTI in a PDCCH monitoring occasion for paging corresponding to an SSB in a PO, UE is not required to monitor subsequent PDCCH monitoring occasions corresponding to that SSB in that PO

- R2 assumes that missing measurements due to LBT failures do not impact the R2 specification of L3 filtering and the subsequent steps.

- Handling of delayed SSB transmissions due to LBT does not impact L3

- FFS: A new RLF trigger mechanism for missing RLM-RS may be defined at upper layers but RAN2 should wait for RAN1 conclusion on this issue

- RSSI and Channel Occupancy configuration and reporting, in particular measurements over an interval (at least for CO) and periodical reporting, are used as a baseline for NR-U

- Measurement and reporting of WLAN nodes are not supported in NR-U

## RAN2#106

- The UE should also stop monitoring paging for the PO even if it does not decode a P-RNTI if it can detect that the gNB had access to the channel at the pdcch monitoring occasion. FFS if there are additional detection methods to detection of PRNTI and what those are.

- We extend paging monitoring by extending PDCCH occasions for a PO

- For the case when all broadcast SI can be put in one SI message, no optimization is needed.

- RSSI CO measurements are not used in Idle or Inactive in this release.

- If highest ranked or best cell is not suitable in an unlicensed frequency due to the fact that PLMN IDs is not the RPLMN (or EPLMN), only the highest ranked or best cell is considered not candidate for cell reselection for 300s or longer. Other cells in the frequency of the highest ranked or best cell should still be considered for cell reselection. FFS whether we have another limit in addition to Suitability criterion.

- To apply the cell barring and IntraFreqReselection in the MIB, the UE also has to acquire the SIB1 to check the PLMN IDs.

- FFS if the UE should only act on the cell barring and intraFreqReselection in the MIB only if the registered PLMN or selected PLMN matches one of the PLMN IDs in SIB1. Otherwise, the UE should follow Proposal#1 Approach#2

- RSSI and CO measurement quantities can be reported with existing triggers as in LAA

- The reporting for RSSI and Channel Occupancy (CO) for NR-U is an optional UE capability as in LTE LAA.

- For UL CG, select the highest CAPC index (lowest priority) of LCHs multiplexed in a TB, as in LTE LAA (for WiFi coexist)

- For UL CG, FFS if it shall be possible to restrict data of which CAPC can be multiplexed into a TB with high priority data

- SRB0, 1, 3 have highest priority (lowest CAPC index), SRB2 configurable

## RAN2#107

- L2 LBT failure mechanism take into account any LBT failure regardless UL transmission type. - The UL LBT failure mechanism will have the same recovery mechanism for all failures regardless UL transmission type - UL LBT failures are detected per BWP - The UE will report the occurrence of consistent UL LBT failures on PSCell and SCells. The assumption is to reuse SCell failure reporting for BF Baseline Mechanism, further enhancements not precluded: - A “threshold” for the maximum number of LBT failures which triggers the “consistent” LBT failure event will be used. - Both a timer and a counter are introduced, the counter is reset when timer expires and incremented when UL LBT failure happens - The timer is started/restarted when UL LBT failure occur.

- The CG retransmission timer value is configured per configured grant configuration (i.e., ConfiguredGrantConfig) and the CG retransmission timer is maintained per HARQ process. - Autonomous retransmission on CG resource is prohibited for a HARQ process while the CG retransmission timer for the HARQ process is running. - Both CG timer and CG retransmission timer are used at the same time for a HARQ process. - The value of the CG retransmission timer is shorter than the value of the CG timer. - The CG timer is not restarted at autonomous retransmission on CG resource after the CG retransmission timer expiry. - The UE does not stop the CG timer upon NACK feedback reception, and stops the CG timer upon ACK feedback reception. - On LBT failure at TX on CG, the UE transmits the pending TB using same HARQ process, in a CG resource. - CS-RNTI is used for scheduled retransmission, and C-RNTI is used for new transmission, similar to NR CG. To be confirmed by RAN1. - Collisions DG CG is FFS

- Will support extension of RAR window without modifying RA-RNTI. - Include LSBs of SFN in MSG2

- PO consists of ‘S × X’ PDCCH monitoring occasions for paging, where 'S' is the number of actual transmitted SSBs and X is the number of PDCCH monitoring occasions per SSB. - Parameter 'X' is signaled in paging configuration (i.e. pcch-Config). Parameter Name FFS - TP is endorsed as baseline - Text for LS to R1: RAN2 has agree to extend PDCCH monitoring at paging. <ref agreement text> RAN2 think that UE may stop monitoring the additional PDCCH monitoring occasions at paging occasions (PO) if it detects that gNB had access to the channel at the PO and the detection is reliable. RAN2 has agreed to use transmission addressed to P-RNTI for this purpose. RAN2 wonders whether there are other DL signals that can be reliably detected for this purpose.

- We will support whitelist - On NR-U frequencies if the highest ranked cell or best cell is not suitable due to belonging to a PLMN which is not indicated as being equivalent to the registered PLMN, the UE shall not consider this cell for a maximum of 300 seconds for reselection, but shall consider the other cells as candidates for reselection on the same frequency. The UE may consider the current NR-U frequency to be the lowest priority frequency for reselection for 300 seconds after at least <FFS criterion> on that frequency were found not suitable due to belonging to a PLMN which is not indicated as being equivalent to the registered PLMN.

## RAN2#107bis

Agreements:

-	From RAN2 point of view it is beneficial to include LSB of SFN in the DCI.  The same design is desirable to be used for 2-step RACH.   Write LS to RAN1 to ask if there is any feasibility issues.

-	For NR-U, 2 bits are enough for a maximum of 40ms response windows.

-	Multiplexing of responses for more than one SFN is not allowed.

-	ra-ContentionResolutionTimer is started regardless of the LBT outcome of msg3 transmission

Agreements:

- MAC relies on reception of a notification of UL LBT failure from the physical layer to detect a consistent UL LBT failure.

- The UE switches to another BWP and initiates RACH upon declaration of consistent LBT failure on PCell or PSCell if there is another BWP with configured RACH resources.

- The UE shall perform RLF recovery if the consistent UL LBT failure was detected on the PCell and UL LBT failure was detected on “N” possible BWP.   “

- When consistent uplink LBT failures are detected on the PSCell, the UE informs MN via the SCG failure information procedure after detecting a consistent UL LBT failure on “N” BWPs.

- “N” is the number of configured BWPs with configured PRACH resources.   If N is larger than one it is up to the UE implementation which BWP the UE selects.

- When consistent uplink LBT failures are detected on an SCell, a new MAC CE to report this to the node where SCell belongs to is used.  FFS whether the MAC CE can be used to report failure on PCell

Agreements

- The drx-RetransmissionTimerDL is started after the PDSCH scheduled by non-numerical K1

- UE starts the drx-HARQ-RTT-TimerDL after the HARQ A/N transmission opportunity irrespective of the LBT outcome

Agreements

- When configuredGrantTimer expires, the UE should stop the CGretransmission timer (CGRT) if it is still running.

- Upon receiving CG activation command, stop the CG retransmission timer for HARQ processes configured for the CG

- No special handling for HARQ process sharing between configured grant and dynamic grants (i.e. follow licensed specifications)

- HARQ process id selection is based on UE implementation.   Ongoing retransmissions on HARQ processes should be prioritized.

Agreements

- Multiple active CG configurations should be allowed for NR-U.  Details are FFS

=>	Aim to introduce a mechanism for SRBs for CG.  Try to find an easy way to capture this in the specs if possible.

=>  For the determination of CAPC for a DRB, selection of the CAPC should be determined by gNB.  Write in the spec that gNB should try to guarantee fairness in stage 2.

=>	RAN2 acknowledges that with current framework the gNB will not be aware whether the UE transmitted type 1 or type 3.   FFS if and how we address it

Agreements

=>	Wait for RAN1 to understand whether we can use existing channels to determine when the UE should stop monitoring.

=> A UE receives paging messages only in initial BWP or in its active BWP as in NR.

Agreements on cell reselection

- The UE may consider the current NR-U frequency to be the lowest priority frequency for reselection for 300 seconds after at least < the N strongest cells > on that frequency were found not suitable due to belonging to a PLMN which is not indicated as being equivalent to the registered PLMN.  N is UE implementation and the UE should check at least 2 if there is more than one.

- If a cell is barred in NR-U, due to the registered PLMN or selected PLMN does not match one of the PLMN IDs in SIB1, “IntraFreqReselection” shall be always interpreted as “allowed”.    The same applies if SIB1 is not decoded.

Agreements

=>	For normal HO and CHO, no new event triggers will be introduced.  RRSI CO measurements can be included in the measurement reports.

Agreements:

- No additional mechanisms are introduced to help the UE find and identify NR-U target cells. The existing/agreed whitelist/blacklist configuration and CGI reports are considered sufficien.

- No additional mechanisms are introduced to address PCI collisions in Rel-16

## RAN2#108

Agreements

1	sl60 and sl160 are added to configuration of ra-ResponseWindow

Agreements:

- UE can trigger SR if there is no available UL resources for sending the MAC CE for SCell UL LBT problem, using the same framework as BFR.

- MAC CE for UL LBT problem has higher priority than data but lower priority than the BFR MAC CE.

- The MAC CE should be transmitted on a different serving cell other than the SCell which has the UL LBT problem

- The MAC CE can report multiple failed Cells.   The MAC CE format should support multiple entries to indicate all the Cells which have already declared consistent UL LBT failure.   UL LBT MAC CE includes Cell index(s) where UL LBT failure occurs.

- As a baseline, the format of the LBT failure MAC CE is a bitmap to indicate if corresponding serving cell has declared consistent LBT failure.

- Cancel the consistent LTB failure for a serving cell (or BWP(s)) (i.e. do not consider Cell as having LBT failure) upon UE successfully transmit a LBT failure MAC CE indicating the serving cell.  FFS what successfully transmission means (i.e. ideally align with BFR unless there are some issues).

- When consistent UL LBT failure is declared on SpCell, UE triggers MAC CE to indicate where failure happened.  The MAC CE is sent on the BWP that the UE switched to during RA procedure.

- FFS When UE switches to another BWP and initiate RACH upon declaration of consistent LBT failure on SpCell, ONLY RACH is initiated.

- A new failure type for PSCell consistent UL LBT failure is added in the SCGFailureInformation.

- No new re-establishment cause is introduced in the RRC re-establishment message.  “Other” failure will be used

Agreements:

- From MAC perspective, if LBT fails for the preamble, the UE also cancel PUSCH transmission

- The PREAMBLE_TRANSMISSION_COUNTER is not increased if the preamble is not transmitted due to failure of the LBT for the preamble

- If preamble is transmitted but LBT for msgA PUSCH fails, the UE monitors downlink PDCCH for fallback RAR. FFS how and whether to deal with the C-RNTI case for connected mode

- The 2 LSBs for the SFN corresponding to msgA transmission time is included in msgB DCI, as for licenced case (pending RAN1)

Agreements of CG:

- The multiple configured grants of a BWP can be explicitly configured to share a common pool of HARQ processes.    If HARQ processes are shared the same CG timer value has to be configured.

- The processes with TB pending for retransmission shall be prioritized over the processes for new transmissions as already agreed for single CG case.

- Retransmissions can be done on different CG resources as long as they are with the same TBS with the same HARQ process

- cg-RetransmissionTimer is always configured for NR-U

- For a HARQ process, the associated CGT timer is only started when the TB using this HARQ process is initially transmitted, and set to the timer value according to the CG configuration used.

- The cg-RetransmissionTimer for the HARQ process is started and restarted for every transmission attempt of the TB when LBT succeeds, using the timer value according to the CG configuration which is used for the transmission.

=>	For UL CG, if DCCH SDU is included in MAC PDU, UE select the CAPC index of DCCH. Otherwise, UE select the highest CAPC index (lowest priority) of LCHs multiplexed in MAC PDU.

Agreements:

1  	The UE uses CAPC 4 for the MAC CE for Recommended bit rate query.

2	It is up to UE implementation how to prioritize among retransmissions on CGs

Agreements

1	For multi-TTI UL grant, UE is allowed to map generated TB(s) internally to different HARQ processes in case of LBT failure(s), i.e. UE may transmit a TB pending for transmission in a HARQ process due to a failed LBT in a different HARQ process being associated with a PUSCH for which LBT was successful.  FFS how it is captured in the spec

Agreements

1	Increased number of PUCCH Cell Groups and increased number of PUCCH SCells are not supported in Rel-16.

Agreement

- The UE can determine whether it should monitor additional PDCCH monitoring occasions using a mechanism other than just P-RNTI.   We will use existing signalling/mechansims to indicate (e.g. in the paging message itself, short message in the DCI) [CB on Wednesday ]

Agreements

1 	As an additional stopping condition, short message for signalling of paging stopping indication is used.  The existing RRC short message is used.

2	The paging stopping indication is addressed to all the UEs which monitor a given PO, i.e. there is no per UE group indication

3	The indication would be for all the UEs to stop paging monitoring in this PO.  If the short message is sent the bit is always set to ‘1’

Agreements:

1	In case of NR-U carriers, the UE may read the system information of the non-strongest cells and report the found PLMNs to NAS.

2	On NR-U frequencies the UE may search for additional cells to find suitable cells during cell selection

## RAN2#109e

Agreements - Keep the current text in RRC running CR for setting of CAPC priorities for SRBs. Remove the default values in the table in 38.331 Section 9.2.1. - Introduce per-cell signalling in Q in measObjectNR. - For configured uplink grants configured with cg-RetransmissionTimer, a subset of the total HARQ process ID(s) can be configured for CG(s). - For configured uplink grants configured with cg-RetransmissionTimer, same HARQ process ID(s) can be configured for different CG(s). - The guard bands for a cell are signalled by using a starting index and length for each guard band, only when the network wants to configure it. (FFS – move to offline) RAN2 should further discuss the signalling for the cases when there is no guard band, when RAN4 specs should be used, and when/if the UE does not support guard bands. - A single IE for configuring interlaced PUCCH and PUSCH is included in BWP-UplinkCommon. - This IE should also be additionally included in BWP-UplinkDedicated. - For numPagingMonitoringOccasionPerSSB, support at least the values of 2, 3, and 4. - For lbt-FailureInstanceMaxCount, support at least the values of 4, 8, 16, and 32. - For lbt-FailureDetectionTimer, support at least the values of 10ms, 20ms, 40ms, 80ms, 160ms, 320ms. - For UL, if SDUs from multiple DCCHs (i.e. SRB1 and SRB2) are multiplexed in a MAC PDU, the CAPC of the MAC PDU is the highest priority CAPC of the DCCHs. - (FFS move to offline) RAN2 should further discuss the ASN.1 modelling of RSSI reporting. - RAN2 will respond to the RAN1 LS (R2-2000021) that there is no consensus in RAN2 to use our only spare bit in MIB for signalling of Q. RAN2 is also discussing whether a new MIB is needed or not. - The UE can stop paging monitoring if it receives a short message for SI update and PWS. FFS on whether we can set the new bit to zero with SI bit set to 1

Agreements:

1	When cg-RetransmissionTimer is configured and UE receives a CG (re)activation or deactivation, the UE implementation select one corresponding HARQ process.

FFS whether we need to prioritize the MAC CE and transmission/retransmission.

2	Toggle NDI for CG-UCI for new transmissions and not toggle the NDI in the CG-UCI for retransmissions

3	FFS - When RRC BWP switch or DCI BWP switch is received, cancel any triggered consistent LBT failure in this Serving Cell.  Need to deal with overlapping case.

4	FFS - The UE is allowed to transmit other UL transmission after?/while? successful RA procedure triggered by UL LBT failure on SpCell.