3GPP TSG-RAN WG2 Meeting #109 electronic	R2-2001926

E-Meeting, 24th Feb. – 6th Mar. 2020

Title:	[Draft] LS to RAN4 on RRM relaxation in power saving

Response to:	-

Release:	Rel-16

Work Item:	FS_NR_UE_pow_sav

Source:	vivo [to be RAN2]

To:	RAN4

Cc:

- Contact Person:

- Name:	                Chenli

E-mail Address:	 Chenli5g@vivo.com

- 1. Overall Description:

RAN2 discussed NR power saving RRM measurement related issues and the following agreements have been achieved:

- The network broadcasts corresponding parameters of relaxation triggering criteria to enable RRM measurement relaxation feature.

- When network configures the parameters of both low mobility and not-at-cell-edge criteria. UE can perform measurement relaxation according one of the following options, which is indicated by the network:

    - Option a: UE uses both low mobility criterion and not-at-cell-edge criteria, i.e. UE can perform relaxation only when both criteria are fulfilled. And detailed relaxation behaviour is up to RAN4 discussion and decision;

    - Option b: UE uses either low mobility criterion or not-at cell-edge criterion, i.e. UE can perform relaxation when either low mobility or not-at-cell-edge criterion is fulfilled. And detailed relaxation behaviours are same as case that network only configures low mobility or not-at-cell-edge criterion;

- When network configures both RSRP and RSRQ thresholds for not-at-cell-edge criterion, UE can perform relaxation only when both RSRP and RSRQ based criteria are met.

- FFS on RAN4 - if and what parameters we need (e.g. time interval for measurement relaxation since last measurement for cell reselection and the value range for the time interval)

Moreover, RAN2 had the agreement before:

- Whether higher priority frequencies can be relaxed is up to network configuration.  FFS on how the configuration is done.

RAN2 made the decision how to configure and use this highPriorityMeasRelax indication has been postponed to wait for RAN4. RAN2 would like to wait for RAN4 decision on how to perform the relaxation for higher priority frequency.

- 2. Actions:

- To RAN WG4

- RAN2 kindly request RAN4 to:

- take the above information into account during the following work for potential discussion on requirements for RRM in idle/inactive mode;

- provide feedback on how to perform RRM measurement relaxation.

- 3. Date of Next RAN2 Meetings:

TSG RAN WG2 Meeting #109bis		20 – 24 Apr. 2020				e-Meeting

TSG RAN WG2 Meeting #110		25 – 29 May 2020				Athens, GR