3GPP TSG-RAN WG2 Meeting #109 electronic				R2-200xxxx

Elbonia, 24 Feb – 6 Mar 2020

Agenda item:		6.8.2.3

Source:			Intel Corporation

Title:		Report of [AT109e][615][POS] Update RAN1 parameters part of 37.355 CR (Intel)

Document for:	 	Discussion and decision

# Introduction

This is the email discussion report on below email discussion:

- [AT109e][615][POS] Update RAN1 parameters part of 37.355 CR (Intel)

Status: Started

Scope: Update the aspects of the running CR related to RAN1 parameters

Intended outcome: Agreeable draft CR that can be merged with the other running CRs.  Updated draft CR in R2-2001941.

Deadline:  Wednesday 2020-03-04 1300 CET

# Discussion

During online discussion on stage 2 summary and stage 3 summary, some agreements have been made, and some issues will be handled by separate email discussion.

In this email discussion, we focus on the rest issues that have impact on the running LPP CR.

Issue 1: Beam results for NR ECID R2-2000476

We are ok to include beam level results as RRC considering RAN1 has agreed this. Proposal: Beam level measurement results are added in NR ECID method.

During online discussion, companies have different understanding on what RAN1 really agreed.

Companies can check offline with their RAN1 colleagues.  To be considered in updating LPP CR.

Question 1: Did RAN1 agreed to include beam level measurement results for NR ECID method?

| Company | Yes/No | Remark |
| --- | --- | --- |
| Huawei | Yes | Beam level measurement is part of the R15 RRM framework and it is important for the UL/DL PRS configuration in the gNB side. |
| Intel | Yes | RAN1 agreed on RSRP measurements per Resource. Therefore the UE could report beam level measurement results as what we did for RRC if beam level measurement results are available. We can mark it as optional. |
| CATT | Yes |  |

Issue 2: TRP ID handling; R2-2000476/2000966

- Rap: The issue was raised during email discussion on running CR. Assume some changes are needed to make it clear. RAN2 need to decide how to make it clear.

The IE TRP-ID provides the IDs to identify the TRP. (in current running CR)

-- ASN1START

TRP-ID-r16 ::= SEQUENCE {

dl-PRS-ID-r16					INTEGER (0..255)			OPTIONAL,

nr-PhysCellId-r16				NR-PhysCellId-r16			OPTIONAL,

nr-CellGlobalId-r16				NCGI-r15					OPTIONAL,		-- Need ON

nrARFCNRef-r16					ARFCN-ValueNR-r15			OPTIONAL	-- Cond NotSameAsRefServ0

}

-- ASN1STOP

- Option 1: remove the TRP ID IE, and indicate fields inside it separately; R2-2000476

- Option 2: change the name of TRP-ID to TRP-ID-Set, and remove nrARFCN;R2-2000966

Based RAN3 assumption, a TRP is identified by a TRP ID and Cell ID. Based on RAN1 agreement, an ID, which is currently written as dl-PRS-ID, is also introduced to identify the TRP. So in our understanding, {TRP ID, Cell ID} is cell-specific and UE common, which can be used for broadcast assistance data, while dl-PRS-ID may be UE specific, which means that the same TRP can be associated with the different dl-PRS-ID for different UEs. In current running CR, TRP-ID is not same as RAN3 definition, and will lead confusion. Option 1: Proposal: Remove TRP-ID IE, indicates PRS-ID, PCI, CGI and ARFCN individually. R2-2000476 Option 2: Proposal: Adopt the following change to the TRP id configuration 2000966 - Change the name of the IE to TRP-ID-Set - Remove nrARFCNRef

Question 2: How to handle the IE TRP-ID, option 1, 2 or other suggestions ?

| Company | Option1, 2 or other suggestions | Remark |
| --- | --- | --- |
| Huawei | Option2 | The reasoning for this has been provided under R2-200047. We just copy and paste is below: Based RAN3 assumption, a TRP is identified by a TRP ID and Cell ID. Based on RAN1 agreement, an ID, which is currently written as dl-PRS-ID, is also introduced to identify the TRP. So in our understanding, {TRP ID, Cell ID} is cell-specific and UE common, which can be used for broadcast assistance data, while dl-PRS-ID may be UE specific, which means that the same TRP can be associated with the different dl-PRS-ID for different UEs. In light of this, we think the IE name may be a little bit confusing, and TRP ID is not reflected in the IE. Therefore, we suggest to change the name of the IE. Moreover, it is also unclear why nr-ARFCN is included in the ID, we suggest to remove it. |
| Intel | No strong opinion. | The real TRP-ID used in RAN3 is useless in LPP and not contained in the IETRP-ID. Therefore the IE name TRP-ID is incorrect and should be corrected. |
| Qualcomm | Option 2 | The name as proposed in Option 2 is fine. The TRP ID is now defined as INTEGER (1..16384,…) by RAN3, and identifies a TRP within a NG-RAN Node (R3-201290). We understand the nrARFCN can disambiguate the PCI. E.g., different frequency layer may have the same PCI. I.e., a PCI goes together with the ARFCN, similar to LTE. The Need/Cond in the ASN.1 above should be removed. |
| CATT | Option1 | We think ARFCN should be kept here. The ARFCN could be indicated together with PCI to UE by LMF. nrARFCNRef-r16 ARFCN-ValueNR-r15 OPTIONAL -- Cond NotsameasServinggNB |

Issue 3: enhancement on PRS assistance data; R2- 2000241

Rap: This was discussed in last meeting, together with single/multi-methods in LPP. Based on multi-methods approach, it can reduce overhead for hybrid positioning methods, e.g. the LMF asks the UE to perform multi-RTT, DL AoD, DL TDOA simultaneously. There is no benefit or additional overhead if the LMF only request one positioning method.

RAN2 need to discuss whether we need this signalling optimization or not.

Proposal: The ProvideAssistanceData in running CR[2] can be upgraded as below. - The required physical resources are put in: - nr-DL-PRS-ProvideAssistanceData-r16 (nr-DL-PRS-AssistanceDataList-r16, nr-SSB-Config-r16) - The selected physical resources index for some positioning method are put in: - nr-Multi-RTT-ProvideAssistanceData-r16 - nr-DL-AoD-ProvideAssistanceData-r16 - nr-DL-TDOA-ProvideAssistanceData-r16

Question 3: Is the below proposal on the signaling optimization for PRS configuration acceptable? Or any other suggestions?

Proposal: The ProvideAssistanceData in running CR[2] can be upgraded as below.

- The required physical resources are put in:

- nr-DL-PRS-ProvideAssistanceData-r16 (nr-DL-PRS-AssistanceDataList-r16,

nr-SSB-Config-r16)

- The selected physical resources index for some positioning method are put in:

- nr-Multi-RTT-ProvideAssistanceData-r16

- nr-DL-AoD-ProvideAssistanceData-r16

nr-DL-TDOA-ProvideAssistanceData-r16

| Company | Yes/No? | Remark |
| --- | --- | --- |
| Huawei | Yes | We are ok for this but more needs to be discussed on how we should handle the capability (ProvideCapabilities) and the measurement (ProvideLocationInformation). |
| Intel | No | It is only useful when hybrid positioning methods are used and if most of PRS AD are same for different positioning methods. So it is nice to have, but no essential. We should focus on other urgent issues first. |
| Qualcomm | Yes, but with modification | The Proposal is in principle O.K. However, the actual implementation does not fit into LPP design. In this Proposal, the NR-DL-PRS-ProvideAssistanceData are provided at the top level of the message: - [[ - nr-DL-PRS-ProvideAssistanceData-r16 NR-DL-PRS-ProvideAssistanceData-r16 OPTIONAL, -- Need ON - nr-Multi-RTT-ProvideAssistanceData-r16 NR-Multi-RTT-ProvideAssistanceData-r16 OPTIONAL, -- Need ON - nr-DL-AoD-ProvideAssistanceData-r16 NR-DL-AoD-ProvideAssistanceData-r16 OPTIONAL, -- Need ON - nr-DL-TDOA-ProvideAssistanceData-r16 NR-DL-TDOA-ProvideAssistanceData-r16 OPTIONAL -- Need ON - ]] However, at this level there are (a) common IEs, independent on the positioning method, and (b) the individual positioning methods IEs. The proposal generates a new “quasi-common IE”, but without corresponding Request and Capability message. A better implementation would be to keep the LPP as in the current baseline, but add the proposed “selected physical resources index” list to each of the method-ProvideAssistanceData IEs. A field description can be added that in case of multiple methods, the NR-DL-PRS-ProvideAssistanceData-r16 may only be present in one of the method-ProvideAssistanceData IEs. |
| CATT | Yes | The motive of this proposal is that the DL PRS config will be sent several times in multi positioning methods. To simplify the signaling design (to reduce index information), we can consider providing a list of nr-DL-PRS-ProvideAssistanceData-r16. If only one method is used, the list size would be one. If there are two methods are used, the list size could be one (both use the same), two (one common, one specific), or three (one common, two specific). If there are three methods are used, the list size could be 1-7 which covers the cases: common for three, common for two of the three, specific for each method. And introduce a 7-bit map under each method to indicate which index related AD is used for this method. This way can ensure there is no repeated or duplicated information of DL PRS config and can save much signaling overhead. We can discuss the details by email after this meeting. |

Issue 4: Editor’s Note: FFS on whether use critical extension for Rel-16 to branch out message body for LTE and combined LTE and NR; R2-2000476

Rap: This was raised during the email discussion on LPP running CR. RAN2 need to discuss whether we use critical level extension to introduce NR positioning method or use non-critical level extension with prefix “nr” to distinguish NR and LTE.

In current CR, the Rel-16 NR dependent positioning is captured in message body based on non-critical extension, and distinguish LTE and NR based on prefix “nr”. We do not see the problem to follow the way in current CR, and would suggest to keep it as it is and remove the EN. Proposal: Non-critical extension is used in message body to capture Rel-16 NR dependent positioning methods, and prefix “nr” is used to distinguish LTE and NR. The EN is removed;

Question 4: Is the below proposal acceptable? Or any other suggestions?

Proposal: Non-critical extension is used in message body to capture Rel-16 NR dependent positioning methods, and prefix “nr” is used to distinguish LTE and NR. The EN is removed;

| Company | Yes/No? | Remark |
| --- | --- | --- |
| Huawei | Yes | If we use critical extension, the LTE part cannot be included together with the NR part. |
| Intel | Yes | The proposal is same as current CR. |
| Qualcomm | Yes | This is how additional positioning methods have been added in the past. |
| CATT | Yes | Non-critical extension is used in message body to capture Rel-16 NR dependent positioning methods. Should we leave some spare space for LTE in case there is new request on LTE in the future? |

Issue 5: whether NR DL PRS definition is put in the common part, i.e. 6.4.3; R2-2000476

Rap: This was raised during the email discussion on LPP running CR.  “It is questionable to place the NR DL PRS definitions and information elemenets in the common part, given that the specification is for both LTE and NR. As commented by Nokia, there is a need for clear separation between NR and LTE, and to place NR definitions inside the common part is a step towards confusion.”

RAN2 need to discuss whether PRS definition for NR is put under common section or not.

The intention of 6.4 is to contain “Common IEs comprise IEs that are applicable to more than one LPP positioning method.” If methods NR positioning are captured as separate methods, section 6.4 is the correct section to capture the common IEs, e.g. assistance data, capability, etc. But NR-PhysCellId can be moved to 6.4.1 since it is common lower level IE. Proposal: Common NR positioning IEs are captured in section 6 as new sub-clause. NR-PhysCellId is moved to section 6.4.1.

Question 5: Is the below proposal acceptable? Or any other suggestions?

Proposal: Common NR positioning IEs are captured in section 6 as new sub-clause. NR-PhysCellId is moved to section 6.4.1.

| Company | Yes/No? | Remark |
| --- | --- | --- |
| Huawei | Yes | We agree with the above argument that “The intention of 6.4 is to contain “Common IEs comprise IEs that are applicable to more than one LPP positioning method.” There does not seem to be a lot of issues by keeping the current structure. |
| Intel | Yes, | The proposal is same as current CR except NR-PhysCellId. It is ok to move NR-PhysCellId to 6.4.1 since other NR lower IEs are also put there. |
| Qualcomm | Yes | We cannot see any issue with the current section 6.4.3 in the baseline which separates the NR Common IEs. We have a similar section for Common GNSS IEs (section 6.5.2.13). |
| CATT | Yes | Agree the proposal. It’s better to move NR-PhysCellId to section 6.4.1. |

Issue 6: How to group IEs in common section, i.e. 6.4.3; R2-2000476

Rap: This was raised during the email discussion on LPP running CR. Current almost all common IEs are put under the same common sections. Some changes are needed on grouping.

The comments received in the email discussion is “Reporting attribute IE definitions would be better to keep together with the reporting IE, just as for the pos meathods of LTE. With a generic NR positioning section, these are naturally kepot together in the measurement report subsection of such a section.”. To avoid the confusion, we can put sub-clauses under Common NR Positioning Information Elements as, Common NR assistance data Information Elements, Common NR capability Information Elements and Common NR report Information Elements. Proposal: Under Common NR Positioning Information Elements clause, introduce sub-clauses: Common NR assistance data Information Elements, Common NR capability Information Elements and Common NR report Information Elements.

Question 6: Is the below proposal acceptable? Or any other suggestions?

Proposal: Under Common NR Positioning Information Elements clause, introduce sub-clauses: Common NR assistance data Information Elements, Common NR capability Information Elements and Common NR report Information Elements.

| Company | Yes/No? | Remark |
| --- | --- | --- |
| Huawei | Yes | Similar to the issue in Q3, this should be applicable for the multiple positioning methods. |
| Intel | Yes | Not same as current CR. But nice to have in order to make specification clearer. |
| Qualcomm | No | We don’t see a need for this. The IEs should be sorted alphabetically under 6.4.3 Common NR Positioning IEs. This is consistent with LPP and other sections (e.g., Common GNSS IEs 6.5.2.13). |
| CATT | Yes | It’s up to the rapporteur to make the decision. We support Intel to make the specification clearer. |

Issue 7 : How to handle NR-DL-PRS-ReportConfig; R2-2000476

Rap: This was raised during the email discussion on LPP running CR. RAN2 need to discuss whether reportConfig for all positioning methods are put in the same common IE or not.

The comments received in the email discussion is “The fields of this IE need to be included in the individual Request Location Information IEs. E.g., a maxDL-PRS-RxTxTimeDiffMeasPerTRP-r16 would only be needed for Multi-RTT.” We tend to agree, there is no big benefit to group them together and can be indicated in the individual request location information for each positioning methods Proposal: Do not group report configuration, indicate request measurement per positioning method.

Question 7: Is the below proposal acceptable? Or any other suggestions?

Proposal: Do not group report configuration, indicate request measurement per positioning method.

| Company | Yes/No? | Remark |
| --- | --- | --- |
| Huawei | Yes | For example, this allows different measurement request mode of RSRP for DL-TDOA/multi-RTT and DL-AoD. Another example is for multi-RTT operated in CA scenario, it allows explicit pairing of PRS and SRS, while for other techniques, it is not necessary. The comment part of measurement request for different positioning methods is very limited, and we do not see much overhead reduction with it. |
| Intel | Yes | It is the clear way. |
| Qualcomm | Yes | This should follow general LPP principles. Grouping the fields would create unnecessary overhead, since not all fields are applicable to all methods. |
| CATT | Yes | The common part is limited and doesn’t waste too many resources. So no need to group. |

Issue 8  report UE UL carrier information to the LMF R2-2000970

- Rap: The is a new issue.  RAN2 need to discuss whether LMF needs to get UL carrier information, e.g. carrier bandwidth and carrier frequency from UE for SRS related methods.

Therefore, we think that it is beneficial for UE report the UL carrier information to the LMF in the capability so that LMF may request SRS transmission on multiple component carriers that may be received by gNB operated in different frequencies. Without this information, presumably LMF only has knowledge of the serving cell, and the requested SRS transmission characteristics may be only limited to the UL carrier of the serving cell. It may not be possible to configure SRS for positioning on neighbouring cell. - Proposal 1: Support UE to report its UL carrier information to the LMF. - This information at least includes the carrier bandwidth and carrier frequency.

Question 8: Is the below proposal acceptable? Or any other suggestions?

- Proposal 2: Support UE to report its UL carrier information to the LMF.

- This information at least includes the carrier bandwidth and carrier frequency.

| Company | Yes/No? | Remark |
| --- | --- | --- |
| Huawei | Yes | The obvious reason is that the LMF does not know the configured UL carrier frequency and bandwidth for the UE. If this is reported to the LMF, this helps the LMF to select neighbouring gNBs that have the capability of receiving the SRS in SCells of the UE, and for better request of SRS resource as defined below in the NRPPa message of MEASUREMENT REQUEST. Otherwise, LMF would only request SRS in the UL carrier of the PCell. |
| Intel | No | As discussed in 109e-618, the UE should report supported band together with Bandwidth, then the LMF already has sufficient information to configure the UE. DO not need additional information. |
| Qualcomm | Yes, with modification | We think this information is needed at the LMF, since SRS is configured by gNB and LMF does not know BWPs. However, the information could be reported by the serving gNB to the LMF? |
| CATT | No | UE reports the supported band and bandwidth to LMF, so LMF knows UE capability. If the LMF wants to know the configured UL carrier and bandwidth, it can get from serving gNB. So from capability point of view there is no need to report additional information from UE. |

Issue 9 [19], whether UL/DL RSRP measurements are mandatory or not for UL TDOA, DL TDOA and Multi-RTT

Editor’s Note: FFS, whether UL/DL RSRP measurements are mandatory or not.

Editor’s Note: FFS, whether DL RSRP measurements are mandatory or not.

It was discussed in the meeting based on [19], ProposalToDisc8:RAN2 should discuss whether UL PRS-RSRP and DL-PRS RSRP should be optionally reported for UL-TDOA and DL-TDOA and whether both should be optionally reported for multi-RTT.

The conclusion is:

To be discussed as part of the stage 3 topics.

Question 9a: Is UL/DL PRS RSRP measurements mandatory for multi-RTT?

| Company | Yes/No? | Remark |
| --- | --- | --- |
| Huawei | No? | Our understanding is not mandatory. Should we allow LMF to explicitly request UE to report RSRP in RequestLocationInformation, in which case, UE should include RSRP measurement in ProvideLcoationInformation. |
| Intel | No | Rx-Tx is mandatory feature for multi-RTT. UL/DL PRS RSRP should be optional . |
| Qualcomm | No | It is an additional (e.g. quality) measurement which can be reported if supported and requested. |
| CATT | No | RSRP is not mandatory in Multi-RTT. |

Question 9b: Is UL PRS RSRP measurements mandatory for UL TDOA?

| Company | Yes/No? | Remark |
| --- | --- | --- |
| Huawei | No? | It seems NRPPa was written without differentiating UL-AoA/UL-TDOA/Multi-RTT positioning when it comes to MEASUREMENT REQUEST, then we suggest any UL measurement, including - UL RTOA - UL AOA/ZOA - gNB Rx – Tx time difference - UL SRSRSRP Should be individually requested in MEASUREMENT REQUEST message, and optionally present in MEASUREMENT RESPONSE message. In case the specific measurement is requested, gNB should provide the measurement results. |
| Intel | No | RTOA is mandatory feature for UL TDOA. UL PRS RSRP should be optional . |
| Qualcomm | No | It is an additional (e.g. quality) measurement which can be reported if supported and requested. |
| CATT | No | UL RTOA is mandatory in UL TDOA but UL PRS RSRP is optional. |

Question 9c: Is DL PRS RSRP measurements mandatory for DL TDOA?

| Company | Yes/No? | Remark |
| --- | --- | --- |
| Huawei | No? | Our understanding is not mandatory. Should we allow LMF to explicitly request UE to report RSRP in RequestLocationInformation, in which case, UE should include RSRP measurement in ProvideLcoationInformation. |
| Intel | No | RSTD is mandatory feature for DL TDOA. DL PRS RSRP should be optional . |
| Qualcomm | No | It is an additional (e.g. quality) measurement which can be reported if supported and requested. |
| CATT | No | RSTD is mandatory in DL TDOA but DL PRS RSRP is optional. |

# Conclusions

The followings are proposed:

# References

[1]R2-2000241	Design of ProvideAssistantData for RAT-Dependent positioning methods	CATT

[2]R2-2000474	LPP CR Capturing RAN1 parameters for positioning ([108#85][NR Pos])	Intel Corporation

[3]R2-2000475	UE capability on positioning ([108#85][NR Pos])	Intel Corporation

[4]R2-2000476	Open issues in LPP CR	Intel Corporation

[5]R2-2000966	Remaining issues on DL positioning procedure	Huawei, HiSilicon, MediaTek

[6]R2-2000969	Discussion on DL-AoD positioning procedure	Huawei, HiSilicon

[7]R2-2000970	Discussion on SRS capability transfer	Huawei, HiSilicon

[8]R2-2000991	SSB Configuration for UL-PRS and DL-PRS	LG Electronics Inc.

[9]R2-2001168	Introduction of Rel-16 NR positioning	Intel Corporation

[10]R2-2001230	Introduction of PPP-RTK (SSR) 	Qualcomm Incorporated

[11]R2-2001232	posSIBs for NR positioning 	Qualcomm Incorporated

[12]R2-2001278	Single positioning method approach in LPP	Ericsson	CR

[13]R2-2001353	Strongest first path indication with RSTD and UE RxTx measurements 	Ericsson

Moved from other agenda:

[14] R2-2000289	Reduce overhead of RSTD measurement report	vivo

[15] R2-2000290	Remaining issues on support of NR RAT-dependent positioning	vivo

Reference:

[16] R2-2001234	Summary of [108#89][NR/Pos] UE-based downlink positioning assistance data 	Qualcomm Incorportaed

[17] R2-2001279	Summary of [108#86][NR/Pos] Single positioning method approach in LPP	Ericsson

[18] R2-2001659	Summary of [108#87][NR/Rel-16] Additional path reporting	Ericsson

[19] R2-2001931	Summary on the Stage-2 Aspects of R16 Positioning	Huawei, HiSilicon, Intel Corporation