3GPP TSG-RAN WG2 Meeting #109 electronic	 Draft R2-2001944

Elbonia, 24 Feb – 6 Mar 2020

Agenda item:			6.8.2.3

Source:			Intel Corporation

Title:		Report of [AT109e][618][POS] Supported frequency/band list (Intel)

Document for:	 	Discussion and decision

# Introduction

This is the email discussion report on below email discussion:

- [AT109e][618][POS] Supported frequency/band list (Intel)

Status: Started

Scope: Determine if it is needed to provide a supported frequency list or band list to the LMF.

Intended outcome: Summary of agreements on use cases, band vs. frequency list, and what is needed to provide to the LMF.  Summary in R2-2001944.

Deadline:  Wednesday 2020-03-04 1300 CET

# Discussion

RAN2 discussed “Proposal 2.	For RAT dependent positioning method, ask RAN2 to confirm the need of “supported freqs list”;”, but more discussion are needed.

Issue 1: the need of “supported band/frequency list”;

As indicted in Annex, same as OTDOA, the UE may support inter-frequency measurement for DL-TDOA, DL-AoD and Multi-RTT. The supported frequency list was introduced in LTE for OTDOA. The purpose is to let the ESMLC to know which frequencies the UE can support, and then can select proper assistance data to UE, i.e. not configure cells in the frequencies that the UE cannot support.

Copied from TS36.355:

- OTDOA-ProvideCapabilities ::= SEQUENCE {

otdoa-Mode		BIT STRING {	ue-assisted				(0),

ue-assisted-NB-r14		(1),

ue-assisted-NB-TDD-r15	(2) } (SIZE (1..8)),

...,

supportedBandListEUTRA		SEQUENCE (SIZE (1..maxBands)) OF SupportedBandEUTRA		OPTIONAL,

supportedBandListEUTRA-v9a0	SEQUENCE (SIZE (1..maxBands)) OF SupportedBandEUTRA-v9a0

OPTIONAL,

interFreqRSTDmeasurement-r10		ENUMERATED { supported }						OPTIONAL,

additionalNeighbourCellInfoList-r10	ENUMERATED { supported }						OPTIONAL,

prs-id-r14							ENUMERATED { supported }						OPTIONAL,

tp-separation-via-muting-r14		ENUMERATED { supported }						OPTIONAL,

additional-prs-config-r14			ENUMERATED { supported }						OPTIONAL,

prs-based-tbs-r14					ENUMERATED { supported }						OPTIONAL,

additionalPathsReport-r14			ENUMERATED { supported }						OPTIONAL,

densePrsConfig-r14					ENUMERATED { supported }						OPTIONAL,

maxSupportedPrsBandwidth-r14		ENUMERATED { n6, n15, n25, n50, n75, n100, ...}	OPTIONAL,

prsOccGroup-r14						ENUMERATED { supported }						OPTIONAL,

prsFrequencyHopping-r14				ENUMERATED { supported }						OPTIONAL,

maxSupportedPrsConfigs-r14			ENUMERATED { c2, c3 }							OPTIONAL,

periodicalReporting-r14				ENUMERATED { supported }						OPTIONAL,

multiPrbNprs-r14					ENUMERATED { supported }						OPTIONAL,

idleStateForMeasurements-r14		ENUMERATED { required }							OPTIONAL,

numberOfRXantennas-r14				ENUMERATED { rx1, ... }							OPTIONAL,

motionMeasurements-r15				ENUMERATED { supported }						OPTIONAL,

interRAT-RSTDmeasurement-r15		ENUMERATED { supported }						OPTIONAL

}

maxBands INTEGER ::= 64

- SupportedBandEUTRA ::= SEQUENCE {

bandEUTRA							INTEGER (1..maxFBI)

}

SupportedBandEUTRA-v9a0 ::=		SEQUENCE {

bandEUTRA-v9a0						INTEGER (maxFBI-Plus1..maxFBI2)		OPTIONAL

}

maxFBI								INTEGER	::=	64	-- Maximum value of frequency band indicator

maxFBI-Plus1						INTEGER ::= 65	-- lowest value extended FBI range

maxFBI2								INTEGER ::= 256	-- highest value extended FBI range

-- ASN1STOP

SupportedBandEUTRA

This field specifies the frequency bands for which the target device supports RSTD measurements. One entry corresponding to each supported E-UTRA band as defined in TS 36.101 [21]. In case the target device includes bandEUTRA-v9a0, the target device shall set the corresponding entry of bandEUTRA (i.e. without suffix) to maxFBI.

Same as LTE OTDOA, NR RAT dependent positioning methods also support inter-freq measurement. The LMF also needs to know the frequency bands that supported by the target device.

Question 1: Do companies agree that the LMF needs to know the frequency bands that supported by the target device?

| Company | Yes/No | Remark |
| --- | --- | --- |
| Intel | Yes | Otherwise, the LMF has no idea which frequency the UE can support, and cannot configure TRPs in different frequencies than serving cell. |
| Qualcomm | Yes |  |
| Nokia | Yes | Inter-frequency RSTD measurement is indeed possible for DL-TDOA. So the band support capability of UE is needed to be known at LMF for proper TRP assistance data. We should also make it clear that this supportedBandListNR-r16 is for DL-TDOA and provided in the DL-TDOA-ProvideCapabilities IE. |
| CATT | Yes | LMF should config the TRPs to UE based on the frequency capabilities of UE. |
| Huawei/HiSilicon | Yes | In addition, we think different positioning method (for multiple positioning methods representation) should have its own supported frequency bands, since UE may support DL-TDOA in some bands, but not Multi-RTT. (This needs to be examine for different positioning methods case-by-case) |
| OPPO | Yes | LMF needs to know the supported frequency bands of UE since inter-freq measurement is required. |

Issue 2: If answer to question 1 is yes, what structure should be for “supported band/frequency list”;

Option 1: In current running CR on RAN1 parameters [2], the supportedBandListNR is captured as

supportedBandListNR-r16			SEQUENCE (SIZE (1..nrMaxBands)) OF SupportedBandNR		OPTIONAL, --- not in RAN1 list

- SupportedBandNR-r16 ::= SEQUENCE {

FreqBandIndicatorNR-r16 ::=             INTEGER (1..1024)

}

nrMaxBands-r16                                INTEGER ::= 1024    -- Maximum number of supported bands in UE capability.

Option 2: In TS38.331, the supportedBandListNR is captured as

supportedBandListNR                 SEQUENCE (SIZE (1..maxBands)) OF BandNR,

BandNR ::=                          SEQUENCE {

bandNR                              FreqBandIndicatorNR,

modifiedMPR-Behaviour               BIT STRING (SIZE (8))                           OPTIONAL,

mimo-ParametersPerBand              MIMO-ParametersPerBand                          OPTIONAL,

extendedCP                          ENUMERATED {supported}                          OPTIONAL,

multipleTCI                         ENUMERATED {supported}                          OPTIONAL,

bwp-WithoutRestriction              ENUMERATED {supported}                          OPTIONAL,

bwp-SameNumerology                  ENUMERATED {upto2, upto4}                       OPTIONAL,

bwp-DiffNumerology                  ENUMERATED {upto4}                              OPTIONAL,

crossCarrierScheduling-SameSCS      ENUMERATED {supported}                          OPTIONAL,

pdsch-256QAM-FR2                    ENUMERATED {supported}                          OPTIONAL,

pusch-256QAM                        ENUMERATED {supported}                          OPTIONAL,

ue-PowerClass                       ENUMERATED {pc1, pc2, pc3, pc4}                 OPTIONAL,

rateMatchingLTE-CRS                 ENUMERATED {supported}                          OPTIONAL,

channelBWs-DL-v1530                 CHOICE {

fr1                                 SEQUENCE {

scs-15kHz                           BIT STRING (SIZE (10))                      OPTIONAL,

scs-30kHz                           BIT STRING (SIZE (10))                      OPTIONAL,

scs-60kHz                           BIT STRING (SIZE (10))                      OPTIONAL

},

fr2                                 SEQUENCE {

scs-60kHz                           BIT STRING (SIZE (3))                       OPTIONAL,

scs-120kHz                          BIT STRING (SIZE (3))                       OPTIONAL

}

}                                                                                   OPTIONAL,

channelBWs-UL-v1530                 CHOICE {

fr1                                 SEQUENCE {

scs-15kHz                           BIT STRING (SIZE (10))                      OPTIONAL,

scs-30kHz                           BIT STRING (SIZE (10))                      OPTIONAL,

scs-60kHz                           BIT STRING (SIZE (10))                      OPTIONAL

},

fr2                                 SEQUENCE {

scs-60kHz                           BIT STRING (SIZE (3))                       OPTIONAL,

scs-120kHz                          BIT STRING (SIZE (3))                       OPTIONAL

}

}                                                                                   OPTIONAL,

...,

[[

maxUplinkDutyCycle-PC2-FR1                  ENUMERATED {n60, n70, n80, n90, n100}   OPTIONAL

]],

[[

pucch-SpatialRelInfoMAC-CE          ENUMERATED {supported}                          OPTIONAL,

powerBoosting-pi2BPSK               ENUMERATED {supported}                          OPTIONAL

]],

[[

maxUplinkDutyCycle-FR2          ENUMERATED {n15, n20, n25, n30, n40, n50, n60, n70, n80, n90, n100}     OPTIONAL

]]

}

To our understanding, for option 1, channel BW is missing, and would be needed for the LMF to make proper decision. For option 2, some parameters are not needed, for instance pucch, pusch, cross-scheduling, etc.

Maybe an option could be:

Option 3:

supportedBandListNR-r16			SEQUENCE (SIZE (1..nrMaxBands)) OF SupportedBandNR		OPTIONAL, --- not in RAN1 list

- SupportedBandNR-r16 ::= SEQUENCE {

FreqBandIndicatorNR-r16 ::=             INTEGER (1..1024)

bwp-WithoutRestriction              ENUMERATED {supported}                          OPTIONAL,

bwp-SameNumerology                  ENUMERATED {upto2, upto4}                       OPTIONAL,

bwp-DiffNumerology                  ENUMERATED {upto4}                              OPTIONAL,

channelBWs-DL-v1530                 CHOICE {

fr1                                 SEQUENCE {

scs-15kHz                           BIT STRING (SIZE (10))                      OPTIONAL,

scs-30kHz                           BIT STRING (SIZE (10))                      OPTIONAL,

scs-60kHz                           BIT STRING (SIZE (10))                      OPTIONAL

},

fr2                                 SEQUENCE {

scs-60kHz                           BIT STRING (SIZE (3))                       OPTIONAL,

scs-120kHz                          BIT STRING (SIZE (3))                       OPTIONAL

}

}                                                                                   OPTIONAL,

channelBWs-UL-v1530                 CHOICE {

fr1                                 SEQUENCE {

scs-15kHz                           BIT STRING (SIZE (10))                      OPTIONAL,

scs-30kHz                           BIT STRING (SIZE (10))                      OPTIONAL,

scs-60kHz                           BIT STRING (SIZE (10))                      OPTIONAL

},

fr2                                 SEQUENCE {

scs-60kHz                           BIT STRING (SIZE (3))                       OPTIONAL,

scs-120kHz                          BIT STRING (SIZE (3))                       OPTIONAL

}

}                                                                                   OPTIONAL,

...,

}

nrMaxBands-r16                                INTEGER ::= 1024    -- Maximum number of supported bands in UE capability.

- Option 4: others.

Question 2: Which option is preferred? And whether anything is missing or not needed?.

| Company | Yes/No | Remark |
| --- | --- | --- |
| Intel | Option 3 |  |
| Qualcomm |  | This should ideally come from RAN1. From the various RAN1 feature list proposals, we understand that some capabilities may be per UE, per band, and per band combination. It may be better to handle this issue together with the RAN1 feature/capability list and decide the detailed Option later. |
| Nokia |  | At a high level it looks like just the band support information is sufficient for LMF to decide on the TRP frequency or positioning frequency layer to choose for providing TRP assistance. I am not sure if BWP, channel BW etc are needed. This needs to be come from RAN1 and RAN4. From ASN.1 perspective we can leave a place holder to fill in the details later but for now just include a skeleton supportedBandListNR-r16. I expect 38.306 impacts also. So, as Qualcomm indicated, we can discuss it as part of feature/capability list discussions. |
| CATT |  | Agree with QC. It is better to check and decide Option later. |
| Huawei/HiSilicon |  | Tend to have similar view with QC. The band list should be associated with other features reported per band, e.g., PRS processing capability. We do not think what is in RRC should be considered as the starting point for what is to be transferred in LPP. |
| OPPO |  | We share similar view with QC. And regarding to option 3, we have two questions: - For bwp-SameNumerology and bwp-DiffNumerology, they are used for BWP switching, which relying on DCI/timer, why they are related to the measurement operation requirement here? - It seems the issue here is more about DL measurement, why UL BWP is needed here? |

# Conclusion

The followings are proposed:

# References

[1] R2-2000475	UE capability on positioning ([108#85][NR Pos])	Intel Corporation

[2] R2-2000474	LPP CR Capturing RAN1 parameters for positioning ([108#85][NR Pos])	Intel Corporation

# Annex

Capabilities listed in RAN1 capability table:

- NR positioning

| - Features | - Index | - Feature group | - Components | - Prerequisite feature groups | - Need for the gNB to know if the feature is supported | - Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | - Need of FDD/TDD differentiation | - Need of FR1/FR2 differentiation | - Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | - Note | - Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 13. NR DL positioning | 13-1 | DL PRS RSRP Measurement and Reporting | - Max number of positioning frequency layers supported by UE for DL PRS RSRP measurement report Values = {1,4}, other values are FFS - Max number of DL PRS Resource Sets per TRP per frequency layer supported by UE for DL PRS RSRP measurement report. Values = {1,2} - Max number of DL PRS Resources per DL PRS Resource Set a UE can be configured for DL PRS RSRP measurement report Values are FFS Maximum number of DL PRS RSRP measurements on different PRS from the same TRP supported by the UE Values {1,2,3,4,5,6,7,8} - Max number of DL PRS Resources supported by UE for DL PRS RSRP report across all frequency layers, TRPs and DL PRS Resource Sets. - Values are FFS - Max number of TRPs across all positioning frequency layers per UE for DL PRS RSRP measurement report. Values are FFS - Max number of DL PRS Resources per positioning frequency layer for DL PRS RSRP measurement report. Values are FFS Support of inter-frequency DL PRS RSRP measurement report in RRC_CONNECTED state - Support of SSB from neighbor cell as QCL Type C source of a DL PRS resource - Support of SSB from neighbor cell as QCL Type D source of a DL PRS resource - Support of a DL PRS resource as QCL Type D source of another DL PRS resource |  |  | N/A |  | [ #1 – per UE (FFS FR1/FR2 differentiation) #2 – per UE (FFS FR1/FR2 differentiation) #3 – per UE (FFS FR1/FR2 differentiation) #4 – per UE (FFS FR1/FR2 differentiation) #5 – per UE (FFS FR1/FR2 differentiation) #6 – per UE (FFS FR1/FR2 differentiation) #7 – per UE #8 – per UE ] | No | TBD |  |  | Optional with capability signaling |
|  | 13-2 | DL PRS RSTD Measurement and Reporting | - Max number of positioning frequency layers supported by UE for DL PRS RSTD measurement report Values = {1,4}, other values are FFS - Max number of DL PRS Resource Sets per TRP per frequency layer supported by UE for DL PRS RSTD measurement report. Values = {1,2} - Max number of DL PRS Resources per DL PRS Resource Set a UE can be configured for DL PRS RSTD measurement report. Values are FFS - Max number of DL PRS Resources supported by UE for DL PRS RSTD report across all frequency layers, TRPs and DL PRS Resource Sets. Values are FFS - Max number of TRPs across all positioning frequency layers per UE for DL PRS RSTD measurement report. Values are FFS - Max number of DL PRS Resources per positioning frequency layer for DL PRS RSTD measurement report. Values are FFS - Max number of DL PRS RSTD measurements per pair of TRPs. Values {1,2,3,4}. Note: This is a max number of DL PRS RSTD measurements per pair of TRPs with each measurement between a different pair of DL PRS resources or DL PRS resource sets. All the RSTD measurements in a single report should have a single reference timing. - Support of inter-frequency DL PRS RSTD measurement report in RRC_CONNECTED state - Support of DL PRS RSTD measurement quality metric - Support of SSB from neighbor cell as QCL Type C source of a DL PRS resource - Support of SSB from neighbor cell as QCL Type D source of a DL PRS resource - Support of a DL PRS resource as QCL Type D source of another DL PRS resource | 13-1 |  | N/A |  | [ #1 – per UE (FFS FR1/FR2 differentiation) #2 – per UE (FFS FR1/FR2 differentiation) #3 – per UE (FFS FR1/FR2 differentiation) #4 – per UE (FFS FR1/FR2 differentiation) #5 – per UE (FFS FR1/FR2 differentiation) #6 – per UE (FFS FR1/FR2 differentiation) #7 – per UE #8 – per UE ] | No | NA |  |  | Optional with capability signaling |
|  | 13-3 | DL SSB RRM measurements for NR Positioning | Prerequisite – support of SSB measurements for RRM (SS-RSRP, SS-RSRQ) based on Rel.15 for NR Positioning in Rel.16 Configuration and reporting of SSB RRM measurements (SS-RSRP, SS-RSRQ) based on Rel.15 for NR Positioning in Rel.16 |  |  | N/A |  |  | No | NA |  |  | Optional with capability signaling |
|  | 13-4 | CSI-RS RRM measurements for NR Positioning | Prerequisite – support CSI-RS for RRM (CSI-RSRP, CSI-RSRQ) based on Rel.15 for NR Positioning in Rel.16 Configuration and reporting of CSI-RS for RRM (CSI-RSRP, CSI-RSRQ) for NR Positioning in Rel.16 |  |  | N/A |  |  | No | NA |  |  |  |
|  | 13-5 | UE Based Positioning | TBD specific content in RAN2 WG | 13-2 |  | N/A |  | [per UE] | No | NA |  |  | Optional with capability signaling |
| 13a. NR UL positioning | 13a-1 | Transmission of SRS for NR positioning | - Max number of SRS Resource Sets for positioning supported by UE per BWP. Values for the UE capability include {1, 16}. Other values FFS - Max number of aperiodic SRS Resources for positioning supported by UE across all SRS Resource Sets per BWP. Value {64} is agreed. - Max number of semi-persistent SRS Resources for positioning supported by UE across all SRS Resource Sets per BWP. Value {64} is agreed. - Max number of periodic SRS Resources for positioning supported by UE across all SRS Resource Sets per BWP. Value {64} is agreed. - Support of SSB for serving and neighbour cell DL pathloss estimation and SRS for positioning OLPC. - Support of DL PRS for serving and neighbour cell DL pathloss estimation and SRS for positioning OLPC. - Max number of pathloss estimates across all SRS resource sets for positioning. It is indicated from the following set of values {0, 4, 8, 16} - Spatial Relation of SRS Resources for positioning with DL PRS from serving and neighbour cells - Spatial Relation of SRS Resources for positioning with SSB from serving and neighbour cells - Spatial Relation of SRS Resources for positioning with other SRS resources for positioning - Max number of active spatial relations including both DL PRS and SSB. Values FFS |  |  | N/A |  | [ #1 – per UE #2 – per UE #3 – per UE #4 – per UE #5 – per UE #6 – per UE #7 – per UE ] | No | NA |  |  | Optional with capability signaling |
| 13b. NR DL and UL positioning | 13b-1 | UE Rx-Tx Time Difference Measurement and Reporting | - Support of UE Rx-Tx time difference measurement with serving cell - Support of UE Rx-Tx time difference measurement with neighbouring cells - Support of inter-frequency UE Rx-Tx time difference measurements - Max number of UE Rx – Tx time difference measurements per TRP DL PRS Resource Set/Resource - A UE can be configured to report multiple Rx–Tx time difference measurements corresponding to a single SRS resource/resource set for positioning with each measurement corresponding to a single DL PRS resource/resource set. The DL PRS resource/resource sets can be in different positioning frequency layers - Support of UE Rx-Tx time difference measurement quality metric | 13-2 13a-1 |  | N/A |  | [ #1 - per UE #2 - FFS #3 - per UE ] | No | NA |  |  | Optional with capability signaling |
| 13c. NR RAT Independent Positioning | 13c-1 | TBD in RAN2 WG | [TBD in RAN2 WG] |  |  | N/A |  |  | No | NA |  |  | Optional with capability signaling |